3GPP TSG-SA5 Meeting #125 	S5-193139

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0082 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove attribute availabilityStatus in NRCellDU IOC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-03-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Attribute “availabilityStatus” included in NRCellDU IOC in clause 4.3.5.2, however, such “availabilityStatus” is not descibed in Annex A.1 “Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell" “ and Annex A.2 “Combined state diagram for gNB cell”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove attribute “availabilityStatus” included in NRCellDU IOC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be published with inconsistent content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1, 4.3.5.2,C.4.3, D.4.3, E.4.2, E.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

4.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| 3GPP TS 28.622 [30], IOC, EP_RP | EP_RP |
| 3GPP TS 28.662 [11], IOC, SectorEquipmentFunction | SectorEquipmentFunction |
| 3GPP TS 28.658 [19], IOC, ExternalENBFunction | ExternalENBFunction |
| 3GPP TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| 3GPP TS 28.658 [19], IOC, EUtranCellFDD | EUTranCellFDD |
| 3GPP TS 28.658 [19], IOC, EUtranCellTDD | EUTranCellTDD |

Second change

4.3.5	NRCellDU

4.3.5.1	Definition

This IOC represents the information of a cell known by DU. The procedure for cell setup is initiated from the DU, thus requiring a notion of a cell. The DU owns the resources realizing the cell, thus requiring a notion of a cell owning the specific resources instances.

An NR cell transmits SS/PBCH block and always requires downlink transmission at a certain carrier frequency with a certain channel bandwidth. Transmission may be performed from multiple sector-carriers using different transmission points, and these may be configured with different carrier frequencies and channel bandwidths, as long as they are aligned to the cell's downlink resource grids as defined in subclause 4.4 in TS 38.211 [32]. The values of arfcnDL and bSChannelBwDL attributes define the resource grids which each sector-carrier needs to be aligned to. See subclauses 5.3 and 5.4.2 of TS 38.104 for definitions of BS channel bandwidth and NR-ARFCN, respectively.

An NR cell requires an uplink in order to provide initial access. In case of TDD, the values of arfcnUL and bSChannelBwUL have to always be set to the same values as for the corresponding DL attributes. For both FDD and TDD, the arfcnUL and bSChannelBwUL define uplink resource grids to which each sector-carrier needs to align to.

An NR cell can in addition be configured with a supplementary uplink, which has its own arfcnSUL and bSChannelBwSUL, which define resource grids for supplementary uplink sector-carriers.

Each of downlink, uplink and supplementary uplink (if configured) need an initial bandwidth part (BWP), which defines resources to be used by UEs during and immediately after initial access. Additional BWPs can be either configured or calculated by gNB internally and be applied to UEs dynamically by gNB based on e.g. UE capability and bandwidth need of each UE.

4.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| operationalState | M | T | F | F | T (see Note 2) |
| administrativeState | M | T | T | F | T (see Note 2) |
| cellState | M | T | F | F | T (see Note 2) |
| pLMNIdList | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| nRPCI | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| arfcnDL | M | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| arfcnSUL | CM | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| ssbFrequency | M | T | T | F | T |
| ssbPeriodicity | M | T | T | F | T |
| ssbSubCarrierSpacing | M | T | T | F | T |
| ssbOffset | M | T | T | F | T |
| ssbDuration | M | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| bSChannelBwSUL | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRSectorCarrier | M | T | T | F | T |
| bWP | M | T | T | F | T |
| Note 1: No state propagation is implied. Note 2: The attribute value change is conveyed by the notifyStateChange notification. |  |  |  |  |  |

4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |
| arfcnUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| arfcnSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| bSChannelBwUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| bSChannelBwSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| nrTAC Support Qualifier | Condition: 5G Standalone solution is supported. |

4.3.5.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

Third change

C.4.3	XML schema "nRNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 NR Network Resource Model

XML schema definition

nrNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:epc="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:sp="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"/>

<simpleType name="GnbId">

<restriction base="unsignedLong">

<maxInclusive value="4294967295"/>

</restriction>

</simpleType>

<simpleType name="GnbIdLength">

<restriction base="integer">

<minLength value="22"/>

<maxLength value="32"/>

</restriction>

</simpleType>

<simpleType name="Nci">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

</restriction>

</simpleType>

<simpleType name="Pci">

<restriction base="unsignedShort">

<maxInclusive value="503"/>

<!-- Minimum value is 0, maximum value is 3x167+2=503 -->

</restriction>

</simpleType>

<simpleType name="NrTac">

<restriction base="unsignedLong">

<maxInclusive value="16777215"/>

<!—5G TAC is 3-octets length -->

</restriction>

</simpleType>

<simpleType name="GnbDuId">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

<!-- Minimum value is 0, maximum value is 2^36-1=68719476735 -->

</restriction>

</simpleType>

<simpleType name="GnbName">

<restriction base="string">

<minLength value="1"/>

<maxLength value="150"/>

</restriction>

</simpleType>

<simpleType name="CyclicPrefix">

<restriction base="integer">

<enumeration value="15"/>

<enumeration value="30"/>

<enumeration value="60"/>

<enumeration value="120"/>

</restriction>

</simpleType>

<simpleType name="QuotaType">

<restriction base="string">

<enumeration value="STRICT"/>

<enumeration value="FLOAT"/>

</restriction>

</simpleType>

<complexType name="IpEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

<element name="vlanId" type="integer"/>

</sequence>

</complexType>

<simpleType name="CellState">

<restriction base="string">

<enumeration value="IDLE"/>

<enumeration value="INACTIVE"/>

<enumeration value="ACTIVE"/>

</restriction>

</simpleType>

<simpleType name="BwpContext">

<restriction base="string">

<enumeration value="DL"/>

<enumeration value="UL"/>

<enumeration value="SUL"/>

</restriction>

</simpleType>

<simpleType name="IsInitialBwp">

<restriction base="string">

<enumeration value="INITIAL"/>

<enumeration value="OTHER"/>

</restriction>

</simpleType>

<complexType name="RRMPolicyRation2">

<sequence>

<element name="groupId" type="integer"/>

<element name="sNSSAIList" type="ngc:SnssaiList"/>

<element name="quotaType" type="QuotaType"/>

<element name="rRMPolicyMaxRation" type="integer"/>

<element name="rRMPolicyMarginMaxRation" type="integer"/>

<element name="rRMPolicyMinRation" type="integer"/>

<element name="rRMPolicyMarginMinRation" type="integer"/>

</sequence>

</complexType>

<element name="GNBDUFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="gnbId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbDUId" type="nn:GnbDuId"/>

<element name="gnbDuName" type="nn:GnbName" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellDU"/>

<element ref="nn:BWP"/>

<element ref="nn:NRSectorCarrier"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_F1U"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUCPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="gnbId" type="nn:GnbId" />

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbCuName" type=" nn:GnbName" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellCU"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_E1"/>

<element ref="nn:EP_XnC"/>

<element ref="nn:EP_X2C"/>

<element ref="nn:EP_NgC"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUUPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="gNBId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:EP_E1"/>

<element ref="nn:EP_F1U"/>

<element ref="nn:EP_XnU"/>

<element ref="nn:EP_NgU"/>

<element ref="nn:EP_X2U"/>

<element ref="nn:EP_S1U"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellCU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="nCGI" type="nn:Ncgi"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="rRMPolicyType" type="integer" minOccurs="0"/>

<element name="rRMPolicyNSSIId" type="xn:dn" minOccurs="0"/>

<element name="rRMPolicyRatio" type="integer" minOccurs="0"/>

<element name="rRMPolicy" type="string" minOccurs="0"/>

<element name="rRMPolicyRatio2" type="RRMPolicyRation2" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellDU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="nCGI" type="nn:Ncgi" minOccurs="0"/>

<element name="operationalState" type="sm:operationalStateType" minOccurs="0"/>

<element name="administrativeState" type="sm:administrativeStateType"

minOccurs="0"/

<element name="cellState" type="nn:CellState"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="nRpci" type="nn:Pci" />

<element name="nRTac" type="nn:NrTac" />

<element name="arfcnDL" type="integer"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="arfcnSUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwSUL" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element name="nRSectorCarrier" type="xn:dn" minOccurs="0"/>

<element name="bWP" type="xn:dn" minOccurs="0"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRSectorCarrier">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="txDirection" type="nn:TxDirection"/>

<element name="configuredMaxTxPower" type="integer"/>

<element name="arfcnDL" type="integer" minOccurs="0"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer" minOccurs="0"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element name="sectorEquipmentFunction" type="xn:dn" minOccurs="0"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="BWP">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="bwpContext" type="nn:BwpContext"/>

<element name="isInitialBwp" type="nn:IsInitialBwp"/>

<element name="subCarrierSpacing" type="integer"/>

<element name="cyclicPrefix" type="nn:CyclicPrefix"/>

<element name="startRB" type="integer"/>

<element name="numberOfRBs" type="integer"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_E1">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Fourth change

D.4.3	JSON schema "nrNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#,"

"id": "http://3gpp.org/28541/nrNrm.json",

"description": "JSON based solution set definitions for NR NRM",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"gnbDuFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbDuId": {

"$ref": "#/definitions/GnbDuId"

},

"gnbDuName": {

"$ref": "#/definitions/GnbName"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

}

},

"required": [

"userLabel",

"gnbDuId",

"gnbId",

" gnbIdLength"

]

},

"gnbCuCpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

},

"gnbCuName": {

"$ref": "#/definitions/GnbName"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

" gnbIdLength",

"plmnIdList"

]

},

"gnbCuUpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

"gnbIdLength",

"plmnIdList"

]

},

"nrCellCu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"nCi": {

"$ref": "#/definitions/NCi"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

},

"snssaiList": {

"$ref": "#/definitions/SnssaiList"

},

"rrmPolicyType": {

"type": "integer"

},

"rrmPolicyNSSIId": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"rrmPolicyRatio": {

"type": "integer"

},

"rrmPolicy": {

"$ref": "#/definitions/RrmPolicy"

},

"rrmPolicyRatio2": {

"$ref": "#/definitions/RrmPolicyRatio2"

}

},

"required": [

"userLabel",

"nCI",

"plmnIdList"

]

},

"nrCellDu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/AdministrativeState"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/OperationalState"

},

},

"cellState": {

"$ref": "#/definitions/CellState"

},

"nCi": {

"$ref": "#/definitions/NCi"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

},

"snssaiList": {

"$ref": "#/definitions/SnssaiList"

},

"nrPci": {

"$ref": "#/definitions/NrPci"

},

"nrTac": {

"$ref": "#/definitions/NrTac"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"arfcnSUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

"bSChannelBwSUL": {

"type": "integer"

}

},

"required": [

"userLabel",

"ngi",

"plmnIdList",

"nrPci",

"nrTac",

"arfcnDL",

"bSChannelBwDL"

]

},

"nrSectorCarrier": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"txDirection": {

"$ref": "#/definitions/TxDirection"

},

"configuredMaxTxPower": {

"type": "integer"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

},

"required": [

"userLabel",

"configuredMaxTxPower",

"txDirection"

]

},

"bwp": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"bwpContext": {

"$ref": "#/definitions/BwpContext"

},

"isInitialBwp": {

"$ref": "#/definitions/IsInitialBwp"

},

"subCarrierSpacing": {

"type": "integer"

},

"cyclicPrefix": {

"$ref": "#/definitions/CyclicPrefix"

},

"startRB": {

"type": "integer"

},

"numberOfRBs": {

"type": "integer"

},

},

"required": [

"userLabel",

"bwpContext",

"isInitialBwp",

"subCarrierSpacing",

"cyclicPrefix",

"startRB",

"numberOfRBs"

]

},

"eP_E1": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_XnC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_XnU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_NgC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_NgU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_F1C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_F1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_S1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_X2C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_X2U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"GnbId": {

"type": "string",

"description": "gNBId identifies a gNB within a PLMN."

},

"GnbIdLength": {

"type": "integer",

"description":  "This indicates the number of bits for encoding the gNB ID.",

"minimum": 22,

"maximum": 32

},

"GnbName": {

"type": "string",

"maxLength": 150

},

"GnbDuId": {

"type": "number",

"description": "GnbDuId represents the NB-DU ID defined in TS 38.473.",

"minimum": 0,

"maximum": 68719476735

},

"NCi": {

"type": "object",

"description": "NCgi uniquely identifies a NR cell  within a PLMN.",

"properties": {

"plmnId": {

"$ref": "#/definitions/PlmnId"

},

"nCI": {

"$ref": "#/definitions/NrCellId"

}

}

},

"SnssaiList": {

"type": "array",

"item": {

"$ref": "#/definitions/Snssai"

}

},

"RrmPolicy": {

"type": "string",

"description": "RrmPolicy represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports. The RRM policy is implementation dependent.",

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"type": "string"

},

"ipv6Address": {

"type": "string"

},

"ipv6Prefix": {

"type": "string"

},

"transport": {

"type": "string"

},

"vlanId": {

"type": "integer"

}

}

},

"NrPci": {

"type": "integer",

"maximum": 503

},

"NrTac": {

"type": "integer",

"maximum": 16777215

},

"NrCellId": {

"type": "integer",

"maximum": 68719476735

},

"Sst": {

"type": "integer",

"maximum": 255

},

"Snssai": {

"type": "object",

"properties": {

"sst": {

"$ref": "#/definitions/Sst"

},

"sd": {

"type": "string"

}

}

},

"CellState": {

"enum": ["IDLE", "INACTIVE", "ACTIVE"]

},

"CyclicPrefix": {

"enum": ["15", "30", "60", "120"]

},

"TxDirection": {

"enum": ["DL", "UL", "DL and UL"]

},

"BwpContext": {

"enum": ["DL", "UL", "SUL"]

},

"IsInitialBwp": {

"enum": ["INITIAL", "OTHER", "SUL"]

},

"QuotaType": {

"enum": ["STRICT", "FLOAT"]

},

"RrmPolicyRatio2": {

"type": "object",

"properties": {

"groupId": {

"type": "integer"

},

"sNSSAIList": {

"$ref": "#/definitions/SnssaiList"

},

"quotaType": {

"$ref ": "#/definitions/QuotaType"

},

"rRMPolicyMaxRation": {

"type": "integer"

},

"rRMPolicyMarginMaxRation": {

"type": "integer"

},

"rRMPolicyMinRation": {

"type": "integer"

},

"rRMPolicyMarginMinRation": {

"type": "integer"

}

}

},

"Mcc": {

"type": "string",

"pattern": "[02-79][0-9][0-9]"

},

"Mnc": {

"type": "string",

"pattern": "[0-9]{3}|[0-9]{2}"

},

"PlmnId": {

"type": "object",

"properties": {

"mcc": {

"$ref": "#/definitions/Mcc"

},

"mnc": {

"$ref": "#/definitions/Mnc"

}

}

},

"PlmnIdList": {

"type": "array",

"item": {

"$ref": "#/definitions/PlmnId"

}

}

}

}

Fifth change

E.4.2	Graphical representation

module: ngran

+--rw ManagedElement

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro vendorName                string

+--rw userDefinedState          string

+--ro swVersion                 string

+--ro dnPrefix                  string

+--rw userLabel                 string

+--ro locationName              string

+--ro managedBy*                nrm-type:t_dn

+--ro managedElementTypeList*   string

+--rw GNBDUFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw gNBDUId              uint32

|  +--rw gNBDUName           string

|  +--ro gNBId                uint32

|  +--ro gNBIdLength          uint32

|  +--rw EP_F1C

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_F1U

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw NRCellDU* [k_nCI]

|  |  +--rw k_nCI                          string

|  |  +--ro objectClass                     string

|  |  +--ro objectInstance                  nrm-type:t_dn

|  |  +--ro id                              nrm-type:t_dn

|  |  +--rw userLabel                      string

|  |  +--rw vnfParametersList

|  |  |  +--rw vnfInstanceId    string

|  |  |  +--rw vnfdId          string

|  |  |  +--rw flavourId       string

|  |  |  +--rw autoScalable    boolean

|  |  +--rw peeParametersList

|  |  |  +--rw siteIdentification    string

|  |  |  +--rw siteLatitude         decimal64

|  |  |  +--rw siteLongitude        decimal64

|  |  |  +--rw siteDescription       string

|  |  |  +--rw equipmentType         enumeration

|  |  |  +--rw environmentType       enumeration

|  |  |  +--rw powerInterface        enumeration

|  |  +--rw nCI		t_NCI

|  |  +--rw pLMNId* [MCC]

|  |  |  +--rw MCC    t_mcc

|  |  |  +--rw MNC   t_mnc

|  |  +--rw s-NSSAI*                        nrm-type:t_s-NSSAI

|  |  +--ro operationalState               nrm-type:t_operationalState

|  |  +--rw administrativeState            nrm-type:t_administrativeState

|  |  +--ro cellState                       nrm-type:t_cellState

|  |  +--rw nRPCI                           nrm-type:t_nRPCI

|  |  +--rw nRTAC                           nrm-type:t_tAC

|  |  +--rw bSChannelBw                     uint16

|  |  +--rw fDDCarrierDL                    uint16

|  |  +--rw fDDCarrierUL                    uint16

|  |  +--rw tDDCarrier                      uint16

|  |  +--rw subcarrierSpacingDL             nrm-type:t_subcarrierSpacing

|  |  +--rw subcarrierSpacingUL             nrm-type:t_subcarrierSpacing

|  |  +--rw cyclicPrefixDL                  nrm-type:t_cyclicPrefix

|  |  +--rw cyclicPrefixUL                  nrm-type:t_cyclicPrefix

|  |  +--rw fDDBwpCommonCarrierFreqDL      uint16

|  |  +--rw fDDBwpCommonCarrierFreqUL      uint16

|  |  +--rw fDDBwpCommonChannelBwDL        uint16

|  |  +--rw fDDBwpCommonChannelBwUL        uint16

|  |  +--rw tDDBwpCommonCarrierFreq        uint16

|  |  +--rw tDDBwpCommonChannelBw          uint16

|  |  +--rw fDDBwpDedicatedCarrierFreqDL   uint16

|  |  +--rw fDDBwpDedicatedCarrierFreqUL   uint16

|  |  +--rw fDDBwpDedicatedChannelBwDL     uint16

|  |  +--rw fDDBwpDedicatedChannelBwUL     uint16

|  |  +--rw tDDBwpDedicatedCarrierFreq     uint16

|  |  +--rw tDDBwpDedicatedChannelBw       uint16

|  |  +--rw NRSectorCarrier*                nrm-type:t_dn

|  +--rw NRSectorCarrier* [sectorEquipmentFunction]

|     +--ro configuredMaxTxPower       uint16

|     +--ro pointAArfcnDLFDD          uint32

|     +--ro pointAArfcnULFDD          uint32

|     +--ro pointAArfcnTDD            uint32

|     +--ro fDDBwpTxStartDL           uint16

|     +--ro fDDBwpTxStartUL           uint16

|     +--ro fDDBwpTxBwDL              uint16

|     +--ro fDDBwpTxBwUL              uint16

|     +--ro tDDBwpTxStart             uint16

|     +--ro tDDBwpTxBw                uint16

|     +--rw sectorEquipmentFunction    nrm-type:t_dn

+--rw GNBCUCPFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw gNBCUName           string

|  +--ro gNBId                uint32

|  +--ro gNBIdLength          uint32

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw (gNB-type)

|  |  +--:(gNB)

|  |  |  +--rw EP-NgC

|  |  |  |  +--ro objectClass       string

|  |  |  |  +--ro objectInstance    nrm-type:t_dn

|  |  |  |  +--rw id                nrm-type:t_dn

|  |  |  |  +--rw userLabel        string

|  |  |  |  +--rw farEndEntity     nrm-type:t_dn

|  |  |  |  +--rw localAddress

|  |  |  |  |  +--rw ip_address   inet:ip-address

|  |  |  |  |  +--rw vlan_id      uint16

|  |  |  |  +--rw remoteAddress    inet:ip-address

|  |  |  +--rw EP-XnC

|  |  |     +--ro objectClass       string

|  |  |     +--ro objectInstance    nrm-type:t_dn

|  |  |     +--rw id                nrm-type:t_dn

|  |  |     +--rw userLabel        string

|  |  |     +--rw farEndEntity     nrm-type:t_dn

|  |  |     +--rw localAddress

|  |  |     |  +--rw ip_address   inet:ip-address

|  |  |     |  +--rw vlan_id      uint16

|  |  |     +--rw remoteAddress    inet:ip-address

|  |  +--:(en-gNB)

|  |     +--rw EP-X2C

|  |        +--ro objectClass       string

|  |        +--ro objectInstance    nrm-type:t_dn

|  |        +--rw id                nrm-type:t_dn

|  |        +--rw userLabel        string

|  |        +--rw farEndEntity     nrm-type:t_dn

|  |        +--rw localAddress

|  |        |  +--rw ip_address   inet:ip-address

|  |        |  +--rw vlan_id      uint16

|  |        +--rw remoteAddress    inet:ip-address

|  +--rw EP_F1C

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_E1

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw NRCellCU* [k_nCI]

|     +--rw k_nCI               string

|     +--ro objectClass          string

|     +--ro objectInstance       nrm-type:t_dn

|     +--ro id                   nrm-type:t_dn

|     +--rw userLabel           string

|     +--rw vnfParametersList

|     |  +--rw vnfInstanceId    string

|     |  +--rw vnfdId          string

|     |  +--rw flavourId       string

|     |  +--rw autoScalable    boolean

|     +--rw peeParametersList

|     |  +--rw siteIdentification    string

|     |  +--rw siteLatitude         decimal64

|     |  +--rw siteLongitude        decimal64

|     |  +--rw siteDescription       string

|     |  +--rw equipmentType         enumeration

|     |  +--rw environmentType       enumeration

|     |  +--rw powerInterface        enumeration

|     +--rw nCGI		t_NCI

|     +--rw pLMNId* [MCC]

|     |  +--rw MCC    t_mcc

|     |  +--rw MNC   t_mnc

|     +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|     +--rw rRMPolicy           string

+--rw GNBCUUPFunction

+--ro objectClass          string

+--ro objectInstance       nrm-type:t_dn

+--ro id                   nrm-type:t_dn

+--rw userLabel           string

+--rw vnfParametersList

|  +--rw vnfInstanceId    string

|  +--rw vnfdId          string

|  +--rw flavourId       string

|  +--rw autoScalable    boolean

+--rw peeParametersList

|  +--rw siteIdentification    string

|  +--rw siteLatitude         decimal64

|  +--rw siteLongitude        decimal64

|  +--rw siteDescription       string

|  +--rw equipmentType         enumeration

|  +--rw environmentType       enumeration

|  +--rw powerInterface        enumeration

+--ro gNBId                uint32

|  +--ro gNBIdLength                uint32

+--rw pLMNId* [MCC MNC]

|  +--rw MCC    t_mcc

|  +--rw MNC    t_mnc

+--rw EP_F1U

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw EP_E1

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw (gNB-type)

+--:(gNB)

|  +--rw EP_NgU

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_XnU

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

+--:(en-gNB)

+--rw EP_X2U

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw EP_S1U

+--ro objectClass       string

+--ro objectInstance    nrm-type:t_dn

+--rw id                nrm-type:t_dn

+--rw userLabel        string

+--rw farEndEntity     nrm-type:t_dn

+--rw localAddress

|  +--rw ip_address   inet:ip-address

|  +--rw vlan_id      uint16

+--rw remoteAddress    inet:ip-address

Sixth change

E.4.3.8	NGRAN submodule, "ngran- nRCellDU.yang"

submodule ngran-nRCellDU {

belongs-to ngran { prefix ngran; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "NRCellDU derived from basic ManagedFunction";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRCellDU {

uses mf:ManagedFunction;

leaf nCI {

config "true";

description "It uniquely identifies a NR cell within a PLMN.";

reference "See NR Cell identifier (NCI) subclause 8.2 of 3GPP TS 38.300 [3].";

type nrm-type:t_NCI;

}

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key MCC;

uses nrm-type:pLMNId;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

type nrm-type:t_s-NSSAI;

}

leaf operationalState  {

config "false";

description "CM, It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled).";

type nrm-type:t_operationalState;

}

leaf administrativeState  {

description "CM, It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services.";

config "true";

type nrm-type:t_administrativeState;

}

leaf cellState  {

description "It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active)";

mandatory "true";

config "false";

type nrm-type:t_cellState;

}

leaf nRPCI {

description "";

mandatory "true";

config "true";

type nrm-type:t_nRPCI;

}

leaf nRTAC {

mandatory "true";

config "true";

type nrm-type:t_tAC;

}

leaf arfcnDL {

mandatory "true";

config "true";

type uint32;

}

leaf arfcnUL {

config "true";

type uint32;

}

leaf arfcnSUL {

config "true";

type uint32;

}

leaf bSChannelBwDL {

mandatory "true";

config "true";

type uint32;

}

leaf bSChannelBwUL {

config "true";

type uint32;

}

leaf bSChannelBwSUL {

config "true";

type uint32;

}

//refer to nRSectorCarrier

leaf-list NRSectorCarrier {

min-elements "0";

description "";

config "true";

type nrm-type:t_dn;

}

//refer to bwp

leaf-list bWP {

min-elements "0";

description "";

config "true";

type nrm-type:t_dn;

}

}

}

End change