3GPP TSG-SA5 Meeting #125 	S5-193417

# Newport Beach, US, 8-12 April 2019												Revision of S5-193144

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.425 | CR | 0189 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on kbits abbreviation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13 |  |  |  |  |  | Date: |  |  | 2019-04-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some undefined kbits and kb used in several performance measurment definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Unify the use of kbit in the corresponding performance measurements and add a clear abbreviation of kbit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mixed use of the undefined kbits and kb may lead to interoperation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.4.1.1, 4.4.1.2, 4.4.1.3, 4.4.1.4, 4.4.1.5, 4.4.1.6, 4.4.6.1, 4.4.6.2, 4.4.6.3, 4.4.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3GPP	3G Partnership Project

BLER	Block Error Rate

CRC	Cyclic Redundancy Check

EPS	Evolved Packet System

EQPT	Equipment

E-UTRAN	Evolved UTRAN

E-RAB	E-UTRAN Radio Access Bearer

HO	Handover

kbit	kilobit (1000 bits)

MCE	Multi-cell/multicast Coordination Entity

MOP	Master Operator

PCell	Primary Cel

PEE	Power, Energy and Environmental l

QoS	Quality of Service

RN	Relay Node

SCell	Secondary Cell

TB		Transport Block

UTRAN	Universal Terrestrial Radio Access Network

NOTE:	Below there's a list of abbreviations used within the measurement types for field E of the measurement template (see 3GPP TS 32.404 [6]).

Alloc	Allocation

AOA	Angle of Arrival

Att	Attempt(s,ed)

Conn	Connection

Ded	Dedicated

DL	Downlink

EE	Energy Efficiency

ENB	eNodeB

Err		Error

Estab	Establish (ed,ment)

Fail	Fail(ed, ure)

Freq	Frequency

Inc	Incoming

Out	Outgoing

Pkt	Packet(s)

Prep	Prepare(/Preparation)

Late	Latency

Mod	Modify(/Modification)

Nbr	Number

Rel	Release(s,d)

Res	Resource

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

Succ	Success(es,ful)

Tot	Total

UL	Uplink

Next change

4.4.1.1	Average DL cell PDCP SDU bit-rate

a)	This measurement provides the average cell bit-rate of PDCP SDUs on the downlink.  This represents the ingress rate of user plane traffic to the eNodeB/RN (via X2 or S1).  The measurement is split into subcounters per E-RAB QoS level (QCI).

b)	CC

c)	This measurement is obtained by accumulating the number of bits entering the eNodeB/RN, and then dividing the sum by the measurement period . The measurement is performed at the PDCP SDU level. PDCP SDUs that are forwarded over the X2/S1 to another eNodeB during handover shall be deducted from the bit count – if this results in a negative bit count the bit count shall be set to zero.  Separate counters are maintained for each QCI. The sum of all supported per QCI measurements shall equal the total DL cell PDCP SDU bit-rate.  In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

d)	Each measurement is an integer value representing the bit-rate measured in kbit/s. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form DRB.PdcpSduBitrateDl.QCI
where QCI identifies the E-RAB level quality of service class.

f)	EUtranCellFDD
EUtranCellTDD

g)	Valid for packet switched traffic

h)	EPS

i)	One usage of this measurement is to support  the KPI "E-UTRAN data Energy Efficiency" defined in [13].

4.4.1.2	Average UL cell PDCP SDU bit-rate

a)	This measurement provides the average cell bit-rate of PDCP SDUs on the uplink.  This represents successful transmissions of user plane traffic; control signalling and retransmissions are excluded from this measure.  The measurement is split into subcounters per E-RAB QoS level (QCI).

b)	CC

c)	This measurement is obtained by accumulating the number of bits leaving the eNodeB/RN on the X2 or S1 interface, and then dividing the sum by the measurement period.  The measurement is performed at the PDCP SDU level.  PDCP SDUs that were not received over the air interface in the cell (but were forwarded from another eNodeB during handover) are excluded from the count.  Separate counters are maintained for each QCI. The sum of all supported per QCI measurements shall equal the total UL cell PDCP SDU bit-rate.  In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

d)	Each measurement is an integer value representing the bit-rate measured in kbit/s. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form DRB. PdcpSduBitrateUl.QCI
where QCI identifies the E-RAB level quality of service class.

f)	EUtranCellFDD
EUtranCellTDD

g)	Valid for packet switched traffic

h)	EPS

i)	One usage of this measurement is to support  the KPI "E-UTRAN data Energy Efficiency" defined in [13].

4.4.1.3	Maximum DL cell PDCP SDU bit-rate

- This measurement provides the maximum cell bit-rate of PDCP SDUs on the downlink.  This represents the maximum ingress rate of user plane traffic to the eNodeB/RN (via X2 or S1).  This is a sum counter measured across all QCIs.

- SI

- This measurement is obtained by sampling at pre-defined intervals the DL cell PDCP SDU bit-rate summed across all QCIs (see clause 4.4.1.1), and then taking the arithmetic maximum of these samples.

- A single integer value representing the maximum bit-rate measured in kbit/s.

- DRB.PdcpSduBitrateDlMax

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

4.4.1.4	Maximum UL cell PDCP SDU bit-rate

- This measurement provides the maximum cell bit-rate of PDCP SDUs measured on the uplink.  This represents successful transmissions of user plane traffic; control signalling and retransmissions are excluded from this measure.  This is a sum counter measured across all QCIs.

- SI

- The measurement is obtained  by sampling at pre-defined intervals the UL cell PDCP SDU bit-rate summed across all QCIs (see clause 4.4.1.2), and then taking the arithmetic maximum of these samples.

- A single integer value representing the maximum bit-rate measured in kbit/s.

- DRB.PdcpSduBitrateUlMax

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

4.4.1.5	Average DL cell control plane PDCP SDU bit-rate

- This measurement provides the average cell bit-rate of control plane PDCP SDUs on the downlink.

- CC.

- This measurement is obtained by accumulating the number of received control plane PDCP SDU bits by the eNodeB/RN, including the control plane PDCP SDU bits received from S1 and RRC SAP, and then dividing the sum by the measurement period.

- A single integer value in kbit/s.

- SRB.PdcpSduBitrateDl

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

4.4.1.6	Average UL cell control plane PDCP SDU bit-rate

- This measurement provides the average cell bit-rate of control plane PDCP SDUs on the uplink.  
This represents successful transmissions of control plane traffic;

- CC.

- This measurement is obtained by accumulating the number of transmitted uplink control plane PDCP SDU bits by the eNodeB/RN, and then dividing the sum by the measurement period.

- A single integer value in kbit/s.

- SRB.PdcpSduBitrateUl

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

Next change

4.4.6.1	IP Throughput in DL

- This measurement provides IP throughput in downlink. For an eNodeB serving one or more RNs, packets transmitted between the E-UTRAN and RNs are excluded, i.e., only packets transmitted between the eNodeB (or RN) and UEs are counted. The measurement is also applicable to RN.

- DER(N=1)

- This measurement is obtained according to the following formula based on the “ThpVolDl” and “ThpTimeDl” defined for the “Scheduled IP Throughput in DL” in 3GPP TS 36.314 [11] for each QCI.

- Each measurement is a real value representing the throughput in kbit/s. The number of measurements is equal to the number of QCIs.

- The measurement name has the form 
DRB.IPThpDl.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- This measurement is to support the Integrity KPI “E-UTRAN IP Throughput” defined in [13].

4.4.6.2	IP Throughput in UL

- This measurement provides IP throughput in uplink. For an eNodeB serving one or more RNs, packets transmitted between the E-UTRAN and RNs are excluded, i.e., only packets transmitted between the eNodeB (or RN) and UEs are counted. The measurement is also applicable to RN

- DER(N=1)

- This measurement is obtained according to the following formula based on the “ThpVolUl” and “ThpTimeUl” defined for the “Scheduled IP Throughput inUL” in 3GPP TS 36.314 [11] for each QCI.

- Each measurement is a real value representing the volume in kbit. The number of measurements is equal to the number of QCIs.

- The measurement name has the form 
DRB.IPThpUl.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- This measurement is to support the Integrity KPI “E-UTRAN IP Throughput” defined in [13].

4.4.6.3	Scheduled IP Throughput in DL

a)	This measurement provides the volume of a data burst during IP throughput measurement, excluding the data transmitted in the TTI when the buffer is emptied in downlink. For an eNodeB serving one or more RNs, packets transmitted between the E-UTRAN and RNs are excluded, i.e., only packets transmitted between the eNodeB (or RN) and UEs are counted. The measurement is also applicable to RN.

b)	DER(n=1)

c)	This measurement is obtained according to the definition in 3GPP TS 36.314 [11] clause 4.1.6 as sum of ThpVolDl. Separate counters are maintained for each QCI.

d)	Each measurement is a real value representing volume of a data burst in kbit. The number of measurements is equal to the number of QCIs.

e)	The measurement name has the form 
DRB.IPVolDl.QCI
where QCI identifies the E-RAB level quality of service class.

f)	EUtranCellFDD
EUtranCellTDD

g)	Valid for packet switched traffic

h)	EPS

i)	This measurement is to support the Integrity KPI "E-UTRAN IP Throughput" defined in [13].

4.4.6.4	Scheduled IP Throughput in UL

a)	This measurement provides the volume of a data burst during IP throughput measurement, excluding the data transmitted in the TTI when the buffer is emptied in uplink. For an eNodeB serving one or more RNs, packets transmitted between the E-UTRAN and RNs are excluded, i.e., only packets transmitted between the eNodeB (or RN) and UEs are counted. The measurement is also applicable to RN.

b)	DER(n=1).

c)	This measurement is obtained according to the definition in 3GPP TS 36.314 [11] clause 4.1.6 as sum of ThpVolUl. Separate counters are maintained for each QCI.

d)	Each measurement is a real value representing volume of a data burst in kbit. The number of measurements is equal to the number of QCIs.

e)	The measurement name has the form 
DRB.IPVolUl.QCI
where QCI identifies the E-RAB level quality of service class.

f)	EUtranCellFDD
EUtranCellTDD

g)	Valid for packet switched traffic.

h)	EPS.

i)	This measurement is to support the Integrity KPI "E-UTRAN IP Throughput" defined in [13].

End of changes