3GPP TSG-SA WG2 Meeting #132	S2-1903171

# 8 - 12 April 2019, Xi’an, China	(Revision of S2-19xxxxx)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1152 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slicing and delegated discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-03-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A clarification on SCP in the context of Network Slicing is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the DP presented at SA2#131 in S2-1901588, and the CR 1056 to 23.501 S2-1903170 the EN in 4.17.9 regarding SCP and slicing is removed. |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 4.17.9	Delegated service discovery when NF service consumer and NF service producer are in same PLMN

Figure 4.17.9-1: Delegated NF service discovery when NF service consumer and NF service producer are in same PLMN

1.	The NF service consumer intends to communicate with an NF service producer. The NF service consumer sends the service request to an SCP. The request may include discovery and selection parameters necessary to discover and select a NF service producer instance. The discovery and selection parameters are included in the request by the NF service consumer in a way that the SCP does not need to parse the request body.

2.	The SCP may perform discovery upon the request either by interacting with an NRF using Nnrf_NFDiscovery service NRF or may use information collected during the previous interactions with an NRF (by the Nnrf_NFDiscovery service or Nnrf_NFManagement_NFStatusNotify service operation). The SCP together with the NRF authorizes the request. The SCP selects the target NF service producer.

Editor's note:	Authorization for Indirect Communication and Delegated Discovery need to be defined together with SA WG3 and are therefore FFS.

3.	If the NF service consumer is authorized to communicate with the NF service producer, the SCP forwards the request to the selected NF service producer according to the configuration of the Network Slice, e.g. the expected NF instances are only reachable by NFs in the same network slice.

4.	The NF service producer sends a response to the SCP. If the request in step 3 creates a resource in the NF service producer, such as depicted in Figure 4.17.9-1, the NF service producer responds with a resource address identifying the created resource.

5.	The SCP routes the response to the NF service consumer.

If the NF service consumer receives a resource address, it uses it for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

6.	On a subsequent operation on the created resource, the NF service consumer addresses the resource via the resource address returned by the NF service producer at step 4.

7.	The SCP resolves the NF service producer address and selects a target NF service producer instance. The SCP then routes the request to the selected NF service producer instance.

If the resource address indicates a specific target set of instances (e.g. the request URI points to a set of instances) for a service request, the SCP selects an instance from the target set and route the service request to the selected target instance but within the NF service consumer selected set.

If the resource address indicates a specific target instance for a service request (e.g. the request URI points to a specific target instance, the SCP resolves the URI to a specific target instance), the SCP shall route the service request to the requested target unless the target is not available (e.g. in the case of instance failure, operator configuration).

8.	The SCP delivers the request to the NF service producer.

9.	The NF service producer sends a response to the SCP. The NF service producer may respond with an updated resource address different to the one received in the previous response.

10.	The SCP sends a response to the NF service consumer. If the resource address was updated, the NF service consumer uses the received resource address for subsequent operations (requests) on the resource.

*** END of CHANGES ***