3GPP TSG-SA5 Meeting #126	S5-195708

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0124 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify PDU Sessions setup related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PDU Sessions Setup are inlcuded both in the PDU SESSION RESOURCE SETUP procedures and INITIAL COTEXT SETUP procedures.However, the existing PDU Sessions Setup related measurements did not consider the Initial Context Setup procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the INITIAL COTEXT SETUP procedures to measure the PDU sessions setup related measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurements related to PDU Session Management are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.5.1, 5.1.1.5.2, 5.1.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First of changes

5.1.1.5	PDU Session Management

5.1.1.5.1	Number of PDU Sessions requested to setup

a)	This measurement provides the number of PDU Sessions by the gNB. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	On receipt of PDU SESSION RESOURCE SETUP REQUEST message, INITIAL CONTEXT SETUP REQUEST message (see 3GPP TS 38.413 [11]) by the gNB from the AMF. Each PDU Session requested to setup increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupReq.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

5.1.1.5.2	Number of PDU Sessions successfully setup

a)	This measurement provides the number of PDU Sessions successfully setup by the gNB from AMF. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	On transmission of PDU SESSION RESOURCE SETUP RESPONSE message, INITIAL CONTEXT SETUP RESPONSE message containing the "PDU Session Resource Setup Response List" IE (see 3GPP TS 38.413 [11]) by the gNB to the AMF. Each PDU Session listed in the "PDU Session Resource Setup Response List" IE increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupSucc.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

5.1.1.5.3	Number of PDU Sessions failed to setup

a)	This measurement provides the number of PDU Sessions failed to setup by the gNB. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On transmission of PDU SESSION RESOURCE SETUP RESPONSE message, INIITAL CONTEXT SETUP FAILURE message containing the "PDU Session Resource Failed to Setup List" IE (see 3GPP TS 38.413 [11]) by the gNB to the AMF. Each PDU Session listed in the "PDU Session Resource Failed to Setup List" IE increments the relevant subcounter per failure cause (see clause 9.3.1.2 of 3GPP TS 38.413 [11]) by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupFail.Cause.

Where Cause identifies the cause of the PDU Sessions Resource Setup failure, per the "PDU Session Resource Setup Unsuccessful Transfer" IE. Encoding of the Cause is defined in clause 9.3.1.2 of 3GPP TS 38.413 [11].

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

End of changes