3GPP TSG-SA5 Meeting #126	S5-195575

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0010 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of detailed message format for converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_NEFCH |  |  |  |  |  | Date: |  |  | 2019-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Detailed message format for converged charging is not specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of detailed message format for converged charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The converged charging for NEF would be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.x (new after 6.3.3) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a revision of S5-195251 |  |  |  |  |  |  |  |  |

First change

### 6.3.x	Detailed message format for converged charging

The Operation types are listed in the following order: I [Initial] / U (Update)/T [Termination]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (e.g. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (e.g. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.x.1 illustrates the basic structure of the supported fields in the Charging Data Request for exposure function API online charging.

Table 6.3.x.1: Supported fields in Charging Data Request message

| Information Element | Service Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | -UT- |
| Originator Host |  | IUTE |
| Originator Domain |  | IUTE |
| Destination Domain |  | IUTE |
| Operation Identifier |  | IUTE |
| Operation Token |  | IUTE |
| Operation Type |  | IUTE |
| Operation Number |  | IUTE |
| Destination Host |  | IUTE |
| User Name |  | IUTE |
| Origination State |  | IUTE |
| Origination Timestamp |  | IUTE |
| Subscriber Identifier |  | IUTE |
| Termination Cause |  | --TE |
| Requested Action |  | IUTE |
| Multiple Unit Information |  | IU-- |
| Proxy Information |  | IUTE |
| Route Information |  | IUTE |
| Service Information |  | IUTE |
| Supported Features |  | IUTE |
| Node Functionality |  | IUTE |
| SCEF Node |  | IUTE |
| SCEF Address |  | ---E |
| SCS/AS Address |  | IUTE |
| API Identifier |  | IUTE |
| TLTRI |  | IUTE |
| Event Timestamp |  | IUTE |
| API Invocation Timestamp |  | IUTE |
| API Direction |  | IUTE |
| API Network Service Node |  | IUTE |
| API Content |  | IUTE |
| API Reference |  | IU-E |
| API Result Code |  | --TE |
| External Identifier |  | IUTE |

Table 6.3.x.2 illustrates the basic structure of the supported fields in the Charging Data Response for exposure function API converged charging.

Table 6.3.x.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | IUTE |
| Operation Result |  | IUTE |
| Originator Host |  | IUTE |
| Originator Domain |  | IUTE |
| Operation Identifier |  | IUTE |
| Operation Type |  | IUTE |
| Operation Number |  | IUTE |
| Operation Failover |  | IU-E |
| Multiple Unit Information |  | IU-- |

Editor's note:	The full list of information elements is FFS.

End of changes