3GPP TSG-SA1 Meeting #88	S1-193584

# Reno, Nevada, United States, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.866 | CR | 0003 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Establishment of an indirect network connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NOVAMINT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_REFEC |  |  |  |  |  | Date: |  |  | 2019-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution proposes to re add the requirement related to establishing an indirect network connection coming from the metering use case in the list of consolidated new requirements. This requirement was removed at the SA1#87 meeting as the requirement exists as part of 22.278 “The Evolved ProSe Remote UE supporting E-UTRA and other RAT(s) can use Indirect 3GPP Communication either in E-UTRAN coverage or out of coverage of all supported 3GPP RAT(s) of the UE” However, it is only applying for Evolved ProSe. It is therefore proposed to include the requirement as potential new requirement with the objective to add as generic in TS 22.261 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirement in consolidated functional requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G system may not provide a solution that can meet several use cases expectations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S1-193294, S1-193274, S1-193110 |  |  |  |  |  |  |  |  |

## ********************* First Change *************************

## 8.1	Functional new requirements

### 8.1.1	General

[PR 8.1.1-1] The 5G system shall support the relaying of traffic between a remote UE and a gNB using one or more UE-to-Network Relay UEs.

[PR 8.1.1-2] The 5G system shall support different traffic flows of a remote UE to be relayed via different Indirect 3GPP Communication paths.

[PR 8.1.1-3] The 5G system shall enable the network operator to define the maximum number of hops supported when using UE-to-Network Relay UEs.

[PR 8.1.1-4] The 5G system shall be able to provide indication to a remote UE (alternatively, an authorized user) on the quality of currently available Indirect 3GPP Communication paths.

### [PR 8.1.1-5] The 5G system shall be able to support an indirect network connection even when the UE is in E-UTRAN or NG-RAN coverage.

### 8.1.2	Relay Selection

[PR 8.1.2-1] The 3GPP system shall support selection and reselection of a UE-to-Network Relay UE based on a combination of different criteria e.g.,

-	the characteristics of the traffic that is intended to be relayed (e.g. expected message frequency and required QoS),

-	the subscriptions of UE-to-Network Relay UEs and remote UE,

-	the capabilities/capacity/coverage when using the UE-to-Network Relay UE,

-	the QoS that is achievable by selecting the UE-to-Network Relay UE,

-	the power consumption required by UE-to-Network Relay UE and remote UE,

-	the pre-paired UE-to-Network Relay UE,

-	the 3GPP or non-3GPP access the UE-to-Network Relay UE uses to connect to the network,

-	the 3GPP network the UE-to-Network Relay UE connects to (either directly or indirectly),

-	the overall optimization of the power consumption/performance of the 3GPP system, or

-	battery capabilities and battery lifetime of the UE-to-Network Relay UE and the remote UE.

NOTE 1:	Reselection may be triggered by any dynamic change in the selection criteria, e.g. by the battery of a UE-to-Network Relay UE getting depleted, a new relay capable UE getting in range, a remote UEs requesting additional resources or higher QoS, etc.

### 8.1.3	Permission and Authorization

[PR 8.1.3-1] The 5G system shall enable the network operator to authorize a UE to use Indirect 3GPP Communication. The authorization shall be able to be restricted to using only UE-to-Network Relay UEs belonging to the same network operator. The authorization shall be able to be restricted to using only UE-to-Network Relay UEs belonging to the same application layer group.

[PR 8.1.3-2] The 5G system shall enable the network operator to authorize a UE to relay traffic as UE-to-Network Relay. The authorization shall be able to be restricted to relaying only for remote UEs belonging to the same network operator. The authorization shall be able to be restricted to relaying only for remote UEs belonging to the same application layer group.

[PR 8.1.3-3] The 5G system shall support a mechanism for an end user to provide/revoke permission to an authorized UE to act as a UE-to-Network Relay UE.

[PR 8.1.3-4] The 5G system shall provide a suitable API by which an authorized 3rd party shall be able to authorize (multiple) UEs under control of the 3rd party to act as a UE-to-Network Relay UE or remote UE.

[PR 8.1.3-5] The 5G system shall provide a suitable API by which an authorized 3rd party shall be able to enable/disable (multiple) UEs under control of the 3rd party to act as a UE-to-Network Relay UE or remote UE.

### 8.1.4	Services and Service Continuity

[PR 8.1.4-1] A 5G system shall support all types of data e.g., voice, data, multimedia, MCX for Indirect 3GPP communication.

[PR 8.1.4-2] The 5G system shall maintain service continuity of Indirect 3GPP Communication for a remote UE when the communication path to the network changes (i.e. change of one or more of the UE-to-Network Relay UEs, change of the gNB).

## 8.2	Functional existing requirements

[PR 8.2-1] The 5G system shall support a UE acting as both a remote UE and as a UE-to-Network Relay UE at the same time.

[PR 8.2-2] The 5G system shall maintain service continuity when the remote UE moves from a direct network connection to an Indirect 3GPP communication using UE-to-Network Relay UEs and vice-versa.

## ********************* End of First Change *************************