3GPP TSG-WG SA2 Meeting #136 	S2-1912176

# Reno, USA, November 18 – 22, 2019	(revision of S2-1911188)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1889 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add NWDAF as consumer of NF Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As defined in the approved S2-1909257, TS 23.288, NWADF could consume the services e.g. Namf_EventExposure, Nsmf_EventExposure, Npcf_PolicyAuthorization_Subscribe etc. for data collection. However, the descriptions for these NF services in TS 23.502 are not aligned. Furthermore, during the SA2#135, it is approved in the S2-1910560 for which NF Services such as Nudm_ECM or Nbsf_Management should be consumed by NWDAF to determine which NF instances are serving a UE. However, the descriptions for these NF services in TS 23.502 are not aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NWDAF as consumer of NF Services to to collect data from the NF. Add NWDAF as consumer of the NF Services to determine which NF instances are serving a UE. Revision 1: removed the added example in 5.2.3.2.4 Nudm_UECM_Get service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Technically not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.2.2.1, 5.2.3.1, 5.2.5.1, 5.2.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.2.2	AMF Services

#### 5.2.2.1	General

The following table shows the AMF Services and AMF Service Operations.

Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) |
| --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Request/ Response | Peer AMF |
|  | CreateUEContext | Request/ Response | Peer AMF |
|  | ReleaseUEContext | Request/ Response | Peer AMF |
|  | RegistrationCompleteNotify | Subscribe / Notify | Peer AMF |
|  | N1MessageNotify | Subscribe / Notify | SMF, SMSF, PCF, LMF, Peer AMF |
|  | N1MessageSubscribe |  | SMF, SMSF, PCF |
|  | N1MessageUnSubscribe |  | SMF, SMSF, PCF |
|  | N1N2MessageTransfer | Request/ Response | SMF, SMSF, PCF, LMF |
|  | N1N2TransferFailureNotification | Subscribe / Notify | SMF, SMSF, PCF, LMF |
|  | N2InfoSubscribe | Subscribe / Notify | NOTE 1 |
|  | N2InfoUnSubscribe |  | NOTE 1 |
|  | N2InfoNotify |  | AMF, LMF |
|  | EBIAssignment | Request/Response | SMF |
|  | AMFStatusChangeSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeUnSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeNotify | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
| Namf_EventExposure | Subscribe | Subscribe / Notify | NEF, SMF, UDM, NWDAF |
|  | Unsubscribe | Subscribe / Notify | NEF, SMF, UDM, NWDAF |
|  | Notify | Subscribe / Notify | NEF, SMF, UDM, NWDAF |
| Namf_MT | EnableUEReachability | Request/Response | SMSF |
|  | ProvideDomainSelectionInfo | Request/Response | UDM |
| Namf_Location | ProvidePositioningInfo | Request/Response | GMLC |
|  | EventNotify | Subscribe / Notify | GMLC |
|  | ProvideLocationInfo | Request/Response | UDM |
|  | CancelLocation | Request/Response | GMLC |
| NOTE 1: In this Release of the specification no known consumer is identified to use this service operation. |  |  |  |

# * * * * Next change * * * *

### 5.2.3	UDM Services

#### 5.2.3.1	General

The following table illustrates the UDM Services and Service Operations.

Table 5.2.3.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Subscriber Data | Get | Request/Response | AMF, SMF, SMSF, NEF |
| Management | Subscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF |
| (SDM) | Unsubscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF |
|  | Notification | Subscribe/Notify | AMF, SMF, SMSF, NEF |
|  | Info | Request/Response | AMF, NEF |
|  | Update | Request/Response | NEF |
| UE Context | Registration | Request/Response | AMF, SMF, SMSF |
| Management | DeregistrationNotification | Subscribe/Notify | AMF |
| (UECM) | Deregistration | Request/Response | AMF, SMF, SMSF |
|  | Get | Request/Response | NEF, SMSF, GMLC, NWDAF |
|  | Update | Request/Response | AMF, SMF |
|  | PCscfRestoration | Subscribe/Notify | AMF, SMF |
| UE | Get | Request/Response | AUSF |
| Authentication | ResultConfirmation | Request/Response | AUSF |
| EventExposure | Subscribe | Subscribe/Notify | NEF (NOTE), NWDAF |
|  | Unsubscribe |  | NEF (NOTE), NWDAF |
|  | Notify |  | NEF (NOTE), NWDAF |
| Parameter | Update | Request/Response | NEF |
| Provision | Create | Request/Response | NEF |
|  | Delete | Request/Response | NEF |
|  | Get | Request/Response | NEF |
| NIDDAuthorisation | Get | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| NOTE: Other NFs are allowed to consume the service based on roaming agreement or operator policy. |  |  |  |

# * * * * Next change * * * *

### 5.2.5	PCF Services

#### 5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF |
|  | Subscribe |  | AF, NEF, NWDAF |
|  | Unsubscribe |  | AF, NEF, NWDAF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |

# * * * * End of changes * * * *

### 5.2.13	BSF Services

#### 5.2.13.1	General

The following table shows the BSF Services and Service Operations:

Table 5.2.13.1-1: NF services provided by the BSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nbsf_management | Register | Request/Response | PCF |
|  | Deregister | Request/Response | PCF |
|  | Discovery | Request/Response | NEF, AF, NWDAF |
|  | Update | Request/Response | PCF |

# * * * * End of changes * * * *