3GPP TSG-SA5 Meeting # 128	S5-197648

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0040 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add notification subscription control NRM fragment – Stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 27-11-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR implements the stage 3 changes for the stage 2 CR in S5-197829 “Add notification subscription control fragment” |  |  |  |  |  |  |  |  |
| Summary of change: |  | The XML, JSON and YANG descriptions of the NtfSubscriptionControl IOC are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 in TS 28.623 does not match stage 2 in TS 28.622. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.3.3, C.4.3, D2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR 28.622 CR 0063rev2 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

## B.3.3	XML schema "genericNrm.xsd"

<?xml version="1.1" encoding="UTF-8"?>

<!--

3GPP TS 28.623 Generic Network Resources IRP

Bulk CM Configuration data file NRM-specific XML schema

genericNrm.xsd

-->

<schema

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:sp="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"

>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"/>

<!-- Base XML type for all NRM class associated XML elements -->

<complexType name="NrmClass">

<attribute name="id" type="string" use="required"/>

<attribute name="modifier" use="optional">

<simpleType>

<restriction base="string">

<enumeration value="create"/>

<enumeration value="delete"/>

<enumeration value="update"/>

</restriction>

</simpleType>

</attribute>

</complexType>

<!-- Generic Network Resources IRP NRM attribute related XML types -->

<simpleType name="dn">

<restriction base="string">

<maxLength value="400"/>

</restriction>

</simpleType>

<complexType name="dnList">

<sequence minOccurs="0" maxOccurs="unbounded">

<element name="dn" type="xn:dn"/>

</sequence>

</complexType>

<simpleType name="linkType">

<list>

<simpleType>

<restriction base="string">

<enumeration value="Signalling"/>

<enumeration value="Bearer"/>

<enumeration value="OAM_AND_P"/>

<enumeration value="Other"/>

</restriction>

</simpleType>

</list>

</simpleType>

<complexType name="linkListType">

<sequence minOccurs="0" maxOccurs="unbounded">

<element name="dn" type="xn:dn"/>

</sequence>

</complexType>

<complexType name="managedElementTypeListType">

<sequence minOccurs="0" maxOccurs="unbounded">

<element name="managedElementType" type="string"/>

</sequence>

</complexType>

<complexType name="vnfParametersListType">

<sequence minOccurs="1" maxOccurs="unbounded">

<element name="vnfInstanceId" type="string"/>

<element name="vnfdId" type="string" minOccurs="0"/>

<element name="flavourId" type="string" minOccurs="0"/>

<element name="autoScalable" type="boolean"/>

</sequence>

</complexType>

<simpleType name="latitude">

<restriction base="decimal">

<fractionDigits value="4"/>

<minInclusive value="-90.0000"/>

<maxInclusive value="90.0000"/>

</restriction>

</simpleType>

<simpleType name="longitude">

<restriction base="decimal">

<fractionDigits value="4"/>

<minInclusive value="-180.0000"/>

<maxInclusive value="180.0000"/>

</restriction>

</simpleType>

<complexType name="peeParametersListType">

<sequence minOccurs="1" maxOccurs="unbounded">

<element name="siteIdentification" type="string"/>

<element name="siteLatitude" type="xn:latitude" minOccurs="0"/>

<element name="siteLongitude" type="xn:longitude" minOccurs="0"/>

<element name="siteDescription" type="string"/>

<element name="equipmentType" type="string"/>

<element name="environmentType" type="string"/>

<element name="powerInterface" type="string"/>

</sequence>

</complexType>

<simpleType name="pMAdministrativeStateType">

<restriction base="string">

<enumeration value="LOCKED"/>

<enumeration value="SHUTTINGDOWN"/>

<enumeration value="UNLOCKED"/>

</restriction>

</simpleType>

<simpleType name="pMOperationalStateType">

<restriction base="string">

<enumeration value="ENABLED"/>

<enumeration value="DISABLED"/>

</restriction>

</simpleType>

<complexType name="MeasurementTypeList">

<sequence minOccurs="1" maxOccurs="unbounded">

<element name="measurementType" type="string"/>

</sequence>

</complexType>

<complexType name="GPList">

<sequence minOccurs="1" maxOccurs="unbounded">

<element name="gP" type="integer"/>

</sequence>

</complexType>

<complexType name="MeasurementTypesAndGPs">

<sequence>

<element name="measurementTypes" type="xn:MeasurementTypeList"/>

<element name="GPs" type="xn:GPList"/>

</sequence>

</complexType>

<complexType name="MeasurementTypesAndGPsList">

<sequence>

<element name="measurementTypesAndGPs" type="xn:MeasurementTypesAndGPs" minOccurs="1" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="GPListType">

<sequence minOccurs="1" maxOccurs="unbounded">

<element name="GP" type="integer"/>

</sequence>

</complexType>

<simpleType name="directionType">

<list>

<simpleType>

<restriction base="string">

<enumeration value="increasing"/>

<enumeration value="decreasing"/>

</restriction>

</simpleType>

</list>

</simpleType>

<complexType name="thrsholdPackType">

<sequence minOccurs="1" maxOccurs="unbounded">

<element name="thresholdPackElement" type="xn:thresholdPackElementType"/>

</sequence>

</complexType>

<complexType name="thresholdPackElementType">

<all>

<element name="thresholdValue" type="string"/>

<element name="thresholdLevel" type="integer"/>

<element name="hysteresis" type="decimal" minOccurs="0"/>

</all>

</complexType>

<complexType name="thresholdInfoType">

<all>

<element name="measurementType" type="string"/>

<element name="direction" type=" xn:directionType"/>

<element name="thresholdPack" type=" xn:thrsholdPackType"/>

</all>

</complexType>

<complexType name="thresholdInfoListType">

<sequence minOccurs="1" maxOccurs="unbounded">

<element name="ThresholdInfoElement" type="xn:thresholdInfoType"/>

</sequence>

</complexType>

<simpleType name="ScopeType">

<restriction base="string">

<enumeration value="BASE_ONLY"/>

<enumeration value="BASE_ALL"/>

<enumeration value="BASE_NTH_LEVEL"/>

<enumeration value="BASE_SUBTREE"/>

</restriction>

</simpleType>

<complexType name="Scope">

<sequence>

<element name="scopeType" type="xn:ScopeType"/>

<element name="scopeLevel" type="integer" minOccurs="0"/>

</sequence>

</complexType>

<!-- Generic Network Resources IRP NRM class associated XML elements -->

<element name="SubNetwork">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="dnPrefix" minOccurs="0"/>

<element name="userLabel"/>

<element name="userDefinedNetworkType"/>

<element name="setOfMcc" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:SubNetwork"/>

<element ref="xn:ManagedElement"/>

<element ref="xn:MeContext"/>

<element ref="xn:ManagementNode"/>

<element ref="xn:IRPAgent"/>

<element ref="xn:SubNetworkOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:ThresholdMonitoringCapability"/>

<element ref="xn:ThresholdMonitor"/>

<element ref="xn:MeasurementControl"/>

<element ref="xn:NtfSubscriptionControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ManagedElement">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="dnPrefix"/>

<element name="managedElementTypeList" type="xn: managedElementTypeListType" minOccurs="0"/>

<element name="userLabel"/>

<element name="vendorName"/>

<element name="userDefinedState"/>

<element name="locationName"/>

<element name="swVersion"/>

<element name="managedBy" type="xn:dnList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:IRPAgent"/>

<element ref="xn:ManagedElementOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:ThresholdMonitoringCapability"/>

<element ref="xn:ThresholdMonitor"/>

<element ref="xn:MeasurementControl"/>

<element ref="xn:NtfSubscriptionControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ManagedFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType"/>

<element name="peeParametersList" type="xn:peeParametersListType"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="xn:EP_RP"/>

<element ref="xn:ThresholdMonitoringCapability"/>

<element ref="xn:ThresholdMonitor"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="MeContext">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="dnPrefix" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:ManagedElement"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ManagementNode">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel"/>

<element name="vendorName"/>

<element name="locationName"/>

<element name="managedElements" type="xn:dnList" minOccurs="0"/>

<element name="swVersion"/>

<element name="userDefinedState"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:IRPAgent"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="MeasurementControl">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="pMAdministrativeState" type="xn:pMAdministrativeStateType"/>

<element name="pMOperationalState" type="xn:pMOperationalStateType"/>

<element name="defaultFileBasedGP" type="integer"/>

<element name="defaultFileReportingPeriod" type="integer"/>

<element name="defaultFileLocation" type="string"/>

<element name="defaultStreamBasedGP" type="integer"/>

<element name="defaultStreamTarget" type="string"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementReader"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="MeasurementReader">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="measurementTypes"/>

<element name="fileBasedGP" type="integer" minOccurs="0"/>

<element name="fileReportingPeriod" type="integer" minOccurs="0"/>

<element name="fileLocation" type="string" minOccurs="0"/>

<element name="streamBasedGP" type="integer" minOccurs="0"/>

<element name="streamTarget" type="string" minOccurs="0"/>

<element name="managedObjectDNsBasic" type="xn:dnList" minOccurs="0"/>

<element name="managedObjectDNs" type="xn:dnList" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="IRPAgent">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element ref="xn:systemDN" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_RP">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="VsDataContainer">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="vsDataType"/>

<element name="vsDataFormatVersion"/>

<element ref="xn:vsData"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ThresholdMonitoringCapability">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="supportedMonitoringGPs" type="xn:GPListType"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:ThresholdMonitoringCapabilityOptionallyContainedNrmClass"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ThresholdMonitor">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="thresholdInfoList" type="xn:thresholdInfoListType"/>

<element name="monitoringGP" type="integer"/>

<element name="monitoringNotifTarget" type="string"/>

<element name="monitoredIOCName" type="string"/>

<element name="monitoredObjectDNs" type="xn:dnList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:ThresholdMonitorOptionallyContainedNrmClass"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NtfSubscriptionControl">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="notificationRecipientAddress" type="string"/>

<element name="notificationTypes" type="string" minOccurs="0" />

<element name="scope" type="Scope"/>

<element name="notificationFilter" type="string" minOccurs="0" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="1">

<element ref="xn:HeartbeatControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<!--

IRPAgent IOC attributes

-->

<element name="systemDN" type="xn:dn"/>

<!--

VsDataContainer NRM class vsData attribute associated empty XML element

-->

<complexType name="vsData"/>

<element name="vsData" type="xn:vsData"/>

<!--

Abstract head XML element for all XML elements associated to further

NRM classes optionally contained under SubNetwork NRM class

-->

<element

name="SubNetworkOptionallyContainedNrmClass"

type="xn:NrmClass"

abstract="true"

/>

<!--

Abstract head XML element for all XML elements associated to further

NRM classes optionally contained under ManagedElement NRM class

-->

<element

name="ManagedElementOptionallyContainedNrmClass"

type="xn:NrmClass"

abstract="true"

/>

<!--

Abstract head XML element for all XML elements associated to further

NRM classes optionally contained under ThresholdMonitoringCapability NRM class

-->

<element

name="ThresholdMonitoringCapabilityOptionallyContainedNrmClass"

type="xn:NrmClass"

abstract="true"

/>

<!--

Abstract head XML element for all XML elements associated to further

NRM classes optionally contained under ThresholdMonitor NRM class

-->

<element

name="ThresholdMonitorOptionallyContainedNrmClass"

type="xn:NrmClass"

abstract="true"

/>

</schema>

Next modification

## C.4.3	JSON schema "genericNrm.json"

{

"openapi": "3.0.1",

"info": {

"title": "3GPP generic NRM",

"version": "16.2.0",

"description": "OAS 3.0.1 specification compatible schema for 3GPP generic NRM"

},

"paths": {},

"components": {

"schemas": {

"Dn": {

"type": "string",

"maxLength": 400

},

"DnList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/Dn"

}

},

"Mcc": {

"type": "string",

"pattern": "^[0-9]{3}$"

},

"AdministrativeState": {

"type": "string",

"enum": [

"LOCKED",

"SHUTTING_DOWN",

"UNLOCKED"

]

},

"OperationalState": {

"type": "string",

"enum": [

"ENABLED",

"DISABLED"

]

},

"UsageState": {

"type": "string",

"enum": [

"IDEL",

"ACTIVE",

"BUSY"

]

},

"RegistrationState": {

"type": "string",

"enum": [

"REGISTERED",

"DEREGISTERED"

]

},

"SetOfMcc": {

"type": "array",

"items": {

"$ref": "#/components/schemas/Mcc"

}

},

"ManagedElementType": {

"type": "string"

},

"ManagedElementTypeList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ManagedElementType"

}

},

"VnfParameter": {

"type": "object",

"properties": {

"vnfInstanceId": {

"type": "string"

},

"vnfdId": {

"type": "string"

},

"flavourId": {

"type": "string"

},

"autoScalable": {

"type": "boolean"

}

}

},

"VnfParametersList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/VnfParameter"

}

},

"SiteLatitude": {

"type": "number",

"format": "float",

"minimum": -90,

"maximum": 90

},

"SiteLongitude": {

"type": "number",

"format": "float",

"minimum": -180,

"maximum": 180

},

"PeeParameter": {

"type": "object",

"properties": {

"siteIdentification": {

"type": "string"

},

"siteDescription": {

"type": "string"

},

"siteLatitude": {

"$ref": "#/components/schemas/SiteLatitude"

},

"siteLongitude": {

"$ref": "#/components/schemas/SiteLongitude"

},

"equipmentType": {

"type": "string"

},

"environmentType": {

"type": "string"

},

"powerInterface": {

"type": "string"

}

}

},

"PeeParametersList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/PeeParameter"

}

},

"MonitoringGPList": {

"type": "array",

"items": {

"type": "integer"

}

},

"ThresholdInfoList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ThresholdInfo"

}

},

"ThresholdInfo": {

"type": "object",

"properties": {

"measurementType": {

"type": "string"

},

"direction": {

"$ref": "#/components/schemas/Direction"

},

"thresholdPack": {

"$ref": "#/components/schemas/ThresholdPack"

}

}

},

"Direction": {

"enum": [

"Increasing",

"Decreasing"

]

},

"ThresholdPack": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ThresholdPackElement"

}

},

"ThresholdPackElement": {

"type": "object",

"properties": {

"thresholdLevel": {

"type": "integer"

},

"thresholdValue": {

"type": "number"

},

"hysteresis": {

"type": "number"

}

}

},

"Operation": {

"type": "object",

"properties": {

"name": {

"type": "string"

},

"allowedNFTypes": {

"$ref": "#/components/schemas/NFType"

},

"operationSemantics": {

"$ref": "#/components/schemas/OperationSemantics"

}

}

},

"OperationList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/Operation"

}

},

"NFType": {

"type": "string",

"description": " NF name defined in TS 23.501",

"enum": [

"NRF",

"UDM",

"AMF",

"SMF",

"AUSF",

"NEF",

"PCF",

"SMSF",

"NSSF",

"UDR",

"LMF",

"GMLC",

"5G_EIR",

"SEPP",

"UPF",

"N3IWF",

"AF",

"UDSF",

"DN"

]

},

"Fqdn": {

"type": "string"

},

"OperationSemantics": {

"type": "string",

"enum": [

"REQUEST_RESPONSE",

"SUBSCRIBE_NOTIFY"

]

},

"SAP": {

"type": "object",

"properties": {

"host": {

"$ref": "#/components/schemas/HostAddr"

},

"port": {

"type": "integer"

}

}

},

"NFServiceType": {

"type": "string",

"enum": [

"Namf_Communication",

"Namf_EventExposure",

"Namf_MT",

"Namf_Location",

"Nsmf_PDUSession",

"Nsmf_EventExposure",

"Others"

]

},

"HostAddr": {

"oneOf": [

{

"$ref": "#/components/schemas/Ipv4Addr"

},

{

"$ref": "#/components/schemas/Ipv6Addr"

},

{

"$ref": "#/components/schemas/Fqdn"

}

]

},

"Ipv4Addr": {

"type": "string",

"pattern": "^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$",

"example": "198.51.100.1"

},

"Ipv4AddrRm": {

"type": "string",

"pattern": "^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$",

"example": "198.51.100.1",

"nullable": true

},

"Ipv6Addr": {

"type": "string",

"allOf": [

{

"pattern": "^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$"

},

{

"pattern": "^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$"

}

],

"example": "2001:db8:85a3::8a2e:370:7334"

},

"Ipv6AddrRm": {

"type": "string",

"allOf": [

{

"pattern": "^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$"

},

{

"pattern": "^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$"

}

],

"example": "2001:db8:85a3::8a2e:370:7334",

"nullable": true

},

"Ipv6Prefix": {

"type": "string",

"allOf": [

{

"pattern": "^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$"

},

{

"pattern": "^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\\/.+)$"

}

],

"example": "2001:db8:abcd:12::0/64"

},

"Uri": {

"type": "string"

},

"TransportProtocol": {

"anyOf": [

{

"type": "string",

"enum": [

"TCP"

]

},

{

"type": "string"

}

]

},

"MeasurementTypeAndGPs": {

"type": "object",

"properties": {

"measurementType": {

"type": "string"

},

"gPs": {

"type": "array",

"items": {

"type": "integer"

}

}

}

},

"Scope": {

"type": "object",

"properties": {

"scopeType": {

"type": "string",

"enum": [

"BASE_ONLY",

"BASE_ALL",

"BASE_NTH_LEVEL",

"BASE_SUBTREE"

]

},

"scopeLevel": {

"type": "integer"

}

}

},

"Top-Attributes": {

"type": "object",

"properties": {

"id": {

"type": "string"

}

}

},

"SubNetwork-Attributes": {

"type": "object",

"properties": {

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"$ref": "#/components/schemas/SetOfMcc"

},

"priorityLabel": {

"type": "integer"

},

"measurements": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MeasurementTypeAndGPs"

}

}

}

},

"ManagedElement-Attributes": {

"type": "object",

"properties": {

"dnPrefix": {

"type": "string"

},

"managedElementTypeList": {

"$ref": "#/components/schemas/ManagedElementTypeList"

},

"userLabel": {

"type": "string"

},

"locationName": {

"type": "string"

},

"managedBy": {

"$ref": "#/components/schemas/DnList"

},

"vendorName": {

"type": "string"

},

"userDefinedState": {

"type": "string"

},

"swVersion": {

"type": "string"

},

"priorityLabel": {

"type": "integer"

},

"measurements": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MeasurementTypeAndGPs"

}

}

}

},

"ManagedFunction-Attributes": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "#/components/schemas/VnfParametersList"

},

"peeParametersList": {

"$ref": "#/components/schemas/PeeParametersList"

},

"priorityLabel": {

"type": "integer"

},

"measurements": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MeasurementTypeAndGPs"

}

}

}

},

"EP_RP-Attributes": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"measurements": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MeasurementTypeAndGPs"

}

}

}

},

"SubNetwork-ContainingObjects": {

"type": "object",

"properties": {

"ManagementNode": {

"$ref": "#/components/schemas/ManagementNode-Multiple"

},

"MeContext": {

"$ref": "#/components/schemas/MeContext-Multiple"

},

"MeasurementControl": {

"$ref": "#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "#/components/schemas/VsDataContainer-Multiple"

},

"ThresholdMonitor": {

"$ref": "#/components/schemas/ThresholdMonitor-Multiple"

},

"ThresholdMonitoringCapability": {

"$ref": "#/components/schemas/ThresholdMonitoringCapability"

},

"NtfSubscriptionControl": {

"$ref": "#/components/schemas/NtfSubscriptionControl-Multiple"

}

}

},

"ManagedElement-ContainingObjects": {

"type": "object",

"properties": {

"MeasurementControl": {

"$ref": "#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "#/components/schemas/VsDataContainer-Multiple"

},

"ThresholdMonitor": {

"$ref": "#/components/schemas/ThresholdMonitor-Multiple"

},

"ThresholdMonitoringCapability": {

"$ref": "#/components/schemas/ThresholdMonitoringCapability"

},

"NtfSubscriptionControl": {

"$ref": "#/components/schemas/NtfSubscriptionControl-Multiple"

}

}

},

"ManagedFunction-ContainingObjects": {

"type": "object",

"properties": {

"MeasurementControl": {

"$ref": "#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "#/components/schemas/VsDataContainer-Multiple"

},

"ThresholdMonitor": {

"$ref": "#/components/schemas/ThresholdMonitor-Multiple"

},

"ThresholdMonitoringCapability": {

"$ref": "#/components/schemas/ThresholdMonitoringCapability"

},

"ManagedNFService": {

"$ref": "#/components/schemas/ManagedNFService-Multiple"

}

}

},

"ManagedNFService-Attributes": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"nFServiceType": {

"$ref": "#/components/schemas/NFServiceType"

},

"sAP": {

"$ref": "#/components/schemas/SAP"

},

"operations": {

"$ref": "#/components/schemas/OperationList"

},

"administrativeState": {

"$ref": "#/components/schemas/AdministrativeState"

},

"operationalState": {

"$ref": "#/components/schemas/OperationalState"

},

"usageState": {

"$ref": "#/components/schemas/UsageState"

},

"registrationState": {

"$ref": "#/components/schemas/RegistrationState"

}

}

},

"ManagedNFService-ContainingObjects": {

"type": "object",

"properties": {

"VsDataContainer": {

"$ref": "#/components/schemas/VsDataContainer-Multiple"

}

}

},

"ManagedNFService-Single": {

"allOf": [

{

"$ref": "#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "#/components/schemas/ManagedNFService-Attributes"

},

{

"type": "object",

"properties": {}

}

]

}

}

}

]

},

"ManagedNFService-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ManagedNFService-Single"

}

},

"ManagementNode-Single": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"managedElements": {

"$ref": "#/components/schemas/DnList"

},

"vendorName": {

"type": "string"

},

"userDefinedState": {

"type": "string"

},

"locationName": {

"type": "string"

},

"swVersion": {

"type": "string"

}

}

}

}

},

"ManagementNode-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ManagementNode-Single"

}

},

"MeContext-Single": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"dnPrefix": {

"type": "string"

}

}

}

}

},

"MeContext-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MeContext-Single"

}

},

"VsDataContainer-Single": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"vsDataType": {

"type": "string"

},

"vsDataFormatVersion": {

"type": "string"

},

"vsData": {

"type": "object",

"properties": {}

}

}

}

}

},

"VsDataContainer-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/VsDataContainer-Single"

}

},

"MeasurementControl-Single": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"pMAdministrativeState": {

"$ref": "#/components/schemas/AdministrativeState"

},

"pMOperationalState": {

"$ref": "#/components/schemas/OperationalState"

},

"defaultFileBasedGp": {

"type": "integer"

},

"defaultFileReportPeriod": {

"type": "integer"

},

"defaultStreamBasedGp": {

"type": "integer"

},

"defaultFileLocation": {

"type": "string"

},

"defaultStreamTarget": {

"type": "string"

}

}

},

"MeasurementReader": {

"$ref": "#/components/schemas/MeasurementReader-Multiple"

}

}

},

"MeasurementControl-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MeasurementControl-Single"

}

},

"MeasurementReader-Single": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"measurementTypes": {

"type": "string"

},

"fileBasedGp": {

"type": "integer"

},

"fileReportingPeriod": {

"type": "integer"

},

"streamBasedGp": {

"type": "integer"

},

"fileLocation": {

"type": "string"

},

"streamTarget": {

"type": "string"

},

"managementObjectDNsBasic": {

"$ref": "#/components/schemas/DnList"

},

"managementObjectDNs": {

"$ref": "#/components/schemas/DnList"

}

}

}

}

},

"MeasurementReader-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MeasurementReader-Single"

}

},

"ThresholdMonitoringCapability": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"supportedMonitoringGPs": {

"$ref": "#/components/schemas/MonitoringGPList"

}

}

}

}

},

"ThresholdMonitor-Single": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"thresholdInfoList": {

"$ref": "#/components/schemas/ThresholdInfoList"

},

"monitoringGP": {

"type": "integer"

},

"monitoringNotifTarget": {

"type": "string"

},

"monitoredIOCName": {

"type": "string"

},

"monitoredObjectDNs": {

"$ref": "#/components/schemas/DnList"

}

}

}

}

},

"ThresholdMonitor-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ThresholdMonitor-Single"

}

},

"NtfSubscriptionControl-Single": {

"allOf": [

{

"$ref": "#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"type": "object",

"properties": {

"notificationRecipientAddress": {

"$ref": "#/components/schemas/Uri"

},

"notificationTypes": {

"type": "string"

},

"scope": {

"$ref": "#/components/schemas/Scope"

},

"notificationFilter": {

"type": "string"

}

}

},

"HeartbeatControl": {

"$ref": "#/components/schemas/HeartbeatControl-Single"

}

}

}

]

},

"NtfSubscriptionControl-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NtfSubscriptionControl-Single"

}

}

}

}

}

Next modification

# D.2	Modules

This is the list of YANG modules for the Generic NRM.

_3gpp-common-ep-rp.yang

_3gpp-common-managed-element.yang

_3gpp-common-managed-function.yang

_3gpp-common-measurements.yang

_3gpp-common-subnetwork.yang

_3gpp-common-subscription-control

_3gpp-common-top.yang

_3gpp-common-yang-extensions.yang

_3gpp-common-yang-types.yang

This is the YANG modules for the Generic NRM.

module _3gpp-common-ep-rp {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-ep-rp";

prefix "eprp3gpp";

import _3gpp-common-yang-types { prefix types3gpp ; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-measurements { prefix meas3gpp; }

organization "3GPP SA5";

description "Common/basic class/grouping to be inherited/reused.

This IOC represents an end point of a link used across a reference

point between two network entities.";

reference

"3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

3GPP TS 28.620

Umbrella Information Model (UIM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.620 V15.X.XX

3GPP TS 28.622 V15.X.XX";

}

grouping EP_RPGrp {

description "Abstract class, represents an end point of a link used

across a reference point between two network entities.

For naming the subclasses of EP_RP, the following rules shall apply:

-	The name of the subclassed IOC shall have the form “EP_<rp>”,

where <rp> is a string that represents the name of the reference point.

Thus, two valid examples of EP_RP subclassed IOC names would be:

EP_S1 and EP_X2.";

leaf userLabel {

type string;

description "A user-friendly (and user assignable) name of this object.";

}

leaf farEndEntity {

config false;

type types3gpp:DistinguishedName;

}

}

grouping EP_Common {

uses EP_RPGrp;

uses meas3gpp:Measurements;

list localAddress {

description "Local IP address and VLAN ID.";

key "ipAddress vlanId";

min-elements 1;

max-elements 1;

uses types3gpp:AddressWithVlan;

}

leaf remoteAddress {

description "Remote IP address.";

mandatory true;

type inet:ip-address;

}

}

}

module _3gpp-common-managed-element {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-common-managed-element;

prefix "me3gpp";

import _3gpp-common-yang-types { prefix types3gpp ; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-measurements { prefix meas3gpp; }

organization "3GPP SA5";

description "Defines ManagedElement which will be augmented

by other IOCs";

reference "3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

3GPP TS 28.620

Umbrella Information Model (UIM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.620 V15.X.XX

3GPP TS 28.622 V15.X.XX";

}

feature MeasurementsUnderManagedElement {

description "The MeasurementSubtree shall be contained under ManageElement";

}

grouping ManagedElement_Grp {

description "Abstract class representing telecommunications resources.

An ME communicates with a manager (directly or indirectly) for the

purpose of being monitored and/or controlled. MEs may perform element

management functionality.

An ME (and its contained Function_(s)) may or may not be geographically

distributed. An ME (and its contained Function_(s)) is often referred

to as a Network Element";

leaf dnPrefix {

description "Provides naming context that allows the Managed

Elements to be partitioned into logical domains.

A Distingushed Name(DN) is defined by 3GPP TS 32.300,

which splits the DN into a DN Prefix and Local DN";

type types3gpp:DistinguishedName;

}

leaf userLabel {

description "A user-friendly (and user assignable) name of this object.";

type string;

}

leaf locationName {

description "The physical location (e.g. an address) of an entity

represented by a (derivative of) ManagedElement_. It may contain no

information to support the case where the derivative of

ManagedElement_ needs to represent a distributed multi-location NE.";

config false;

type string;

}

leaf-list managedBy {

description "Relates to the role played by ManagementSystem_ in the

between ManagedSystem_ and ManagedElement_. This attribute contains

a list of the DN(s) of the related subclasses of

ManagementSystem_ instance(s).";

config false;

type types3gpp:DistinguishedName;

}

leaf-list managedElementTypeList {

description "The type of functionality provided by the ManagedElement.

It may represent one ME functionality or a combination of

more than one functionality.

1) The allowed values of this attribute are the names of the IOC(s)

that are (a) derived/subclassed from ManagedFunction and (b) directly

name-contained by ManagedElement IOC (on the first level below

ManagedElement), but with the string “Function” excluded.

2) If a ManagedElement contains multiple instances of a ManagedFunction

this attribute will not contain repeated values.

3) The capitalisation (usage of upper/lower case) of characters in this

attribute is insignificant.  Thus, the NodeB should be case insensitive

when reading these values.

4) Two examples of allowed values are:

•  NodeB;

•  HLR, VLR.";

config false;

min-elements 1;

type string;

}

}

grouping ManagedElementGrp {

description "Represents telecommunications equipment or

TMN entities within the telecommunications network providing support

and/or service to the subscriber.";

uses ManagedElement_Grp;

uses meas3gpp:Measurements {

if-feature MeasurementsUnderManagedElement ;

}

leaf vendorName {

config false;

type string;

}

leaf userDefinedState {

type string;

description "An operator defined state for operator specific usage";

}

leaf swVersion {

config false;

type string;

}

leaf priorityLabel {

type uint32;

mandatory true;

}

}

list ManagedElement {

description "Represents telecommunications equipment or

TMN entities within the telecommunications network providing support

and/or service to the subscriber.

An ME communicates with a manager (directly or indirectly) over one or

more management interfaces for the purpose of being monitored and/or

controlled. MEs may or may not additionally perform element management

functionality.

An ME contains equipment that may or may not be geographically

distributed. An ME is often referred to as a Network Element.

A telecommunication equipment has software and hardware components.

The IOC described above represents the case when the software component

is designed to run on dedicated hardware component. In the case when the

software is designed to run on ETSI NFV defined NFVI [15], the IOC

description would exclude the NFVI component supporting the above

mentioned subject software. A ManagedElement may be contained in either

a SubNetwork or in a MeContext instance. A single ManagedElement may also

exist stand-alone with no parent at all.

The ManagedElement IOC may be used to represent combined ME functionalit

y (as indicated by the managedElementType attribute and the contained

instances of different functional IOCs).";

key id;   // TODO: should this have a min-element/max-elements?

uses top3gpp:Top_Grp;

container attributes {

uses ManagedElementGrp;

}

uses meas3gpp:MeasurementSubtree {

if-feature MeasurementsUnderManagedElement ;

}

}

}

module _3gpp-common-managed-function {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-common-managed-function;

prefix mf3gpp;

organization "3GPP SA5";

description "The module defines a base class/grouping for major 3GPP functions.";

reference

"3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

3GPP TS 28.620

Umbrella Information Model (UIM)";

revision 2019-06-18 {

description "Initial revision";

reference "Based on

3GPP TS 28.620 V15.X.XX

3GPP TS 28.622 V15.X.XX ";

}

grouping Function_Grp {

description "A base grouping for 3GPP functions.";

leaf userLabel {

type string;

description "A user-friendly (and user assignable) name of this object.";

}

}

grouping ManagedFunctionGrp {

description "Abstract root class to be inherited/reused by classes

representing 3GPP functions.";

uses Function_Grp;

container vnfParametersList {

description "Contains the parameter set of the VNF

instance(s) corresponding to an NE.";

presence "The presence of this container indicates that the ManagedFunction

represented is realized by one or more VNF instance(s). Otherwise it

shall be absent.";

leaf vnfInstanceId {

type string ;

mandatory true;

description "VNF instance identifier";

reference "ETSI GS NFV-IFA 008 v2.1.1:

Network Functions Virtualisation (NFV); Management and Orchestration;

Ve-Vnfm reference point - Interface and Information Model Specification

section 9.4.2

ETSI GS NFV-IFA 015 v2.1.2: Network Functions Virtualisation (NFV);

Management and Orchestration; Report on NFV Information Model

section B2.4.2.1.2.3";

}

leaf vnfdId {

type string ;

description "Identifier of the VNFD on which the VNF instance is based.

The absence of the leaf or a string length of zero for vnfInstanceId

means the VNF instance(s) does not exist (e.g. has not been

instantiated yet, has already been terminated).";

reference "ETSI GS NFV-IFA 008 v2.1.1:

Network Functions Virtualisation (NFV); Management and Orchestration;

Ve-Vnfm reference point - Interface and Information Model Specification

section 9.4.2";

}

leaf flavourId {

type string ;

description "Identifier of the VNF Deployment Flavour applied to this

VNF instance.";

reference "ETSI GS NFV-IFA 008 v2.1.1:

Network Functions Virtualisation (NFV); Management and Orchestration;

Ve-Vnfm reference point - Interface and Information Model Specification

section 9.4.3";

}

leaf autoScalable                        {

type boolean ;

mandatory true;

description "Indicator of whether the auto-scaling of this

VNF instance is enabled or disabled.";

}

}

container peeParametersList {

description "Contains the parameter set for the control

and monitoring of power, energy and environmental parameters of

ManagedFunction instance(s).";

presence "Present supported if the control and monitoring of PEE

parameters is supported by the ManagedFunction or sub-class instance.";

leaf siteIdentification {

type string;

mandatory true;

description "The identification of the site where the

ManagedFunction resides.";

}

leaf siteLatitude {

type decimal64 {

fraction-digits 4;

range "-90.0000..+90.0000";

}

description "The latitude of the site where the ManagedFunction

instance resides, based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to

the northern hemisphere. This attribute is optional in case of

BTSFunction and RNCFunction instance(s).";

}

leaf siteLongitude {

type decimal64 {

fraction-digits 4;

range "-180.0000..+180.0000";

}

description "The longitude of the site where the ManagedFunction

instance resides, based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to

degrees east of 0 degrees longitude. This attribute is optional in

case of BTSFunction and RNCFunction instance(s).";

}

leaf siteDescription {

type string;

mandatory true;

description "An operator defined description of the site where

the ManagedFunction instance resides.";

}

leaf equipmentType {

type string;

mandatory true;

description "The type of equipment where the managedFunction

instance resides.";

reference "clause 4.4.1 of ETSI ES 202 336-12";

}

leaf environmentType {

type string;

mandatory true;

description "The type of environment where the managedFunction

instance resides.";

reference "clause 4.4.1 of ETSI ES 202 336-12";

}

leaf powerInterface {

type string;

mandatory true;

description "The type of power.";

reference "clause 4.4.1 of ETSI ES 202 336-12";

}

}

leaf priorityLabel {

mandatory true;

type uint32;

}

}

}

module _3gpp-common-measurements {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-measurements";

prefix "meas3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

description "Defines Measurement related groupings

Any list/class intending to use this should include 2 uses statements

controlled by a feature:

A)

+++     feature MeasurementsUnderMyClass {

+++       description 'Indicates whether measurements are supported for this class.';

+++     }

B) include the attribute supportedMeasurementsGPs indicating the

supported measurmentTypes and GPs. Note that for classes inheriting from

ManagedFunction, EP_RP or SubNetwork this attribute is already inherited,

so there is no need to include it once more.E.g.

grouping MyClassGrp {

+++        uses meas3gpp:Measurements;

}

C) include the classes MeasurementControl & MeasuremntReader to

control the measurements. E.g.

list MyClass {

container attributes {

uses MyClassGrp;

}

+++       uses meas3gpp:MeasurementSubtree {

+++         if-feature MeasurementsUnderMyClass ;

+++       }

}

Measurements can be contained under ManagedElement, SubNetwork, or

any list representing a class inheriting from EP_RP, Subnetwork or

ManagedFunction.";

reference "3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2019-06-17 {

reference "Based on

3GPP TS 28.622 V15.X.XX";

}

feature StreamingSupported {

description "The delivery of measurement data using streaming is supported.";

}

grouping Measurements {

description "Identifies the supported Measurement types and their

supported GPs.";

list supportedMeasurementsGPs {

config false;

key measurementType;

description "List of supported Measurement types and their

supported GPs for the parent function/class";

leaf measurementType {

type string;

}

leaf-list supportedGPs {

type uint32 ;

min-elements 1;

units second;

description "GP (granularity period) is the time between the initiation

of two successive gatherings of measurement data.";

}

}

}

grouping MeasurementControlGrp {

description "represents the capabilities to produce and deliver Measurements

identified by a MeasurementReader.

There are two delivery methods (i.e. file-based and stream-based) via

which the consumer(s) can receive the Measurements.";

leaf pMAdministrativeState {

default LOCKED;

type types3gpp:AdministrativeState ;

description "It describes the permission to use or prohibition against

using the capability of MeasurementControl, imposed through the consumer

of OAM services produced by MeasurementControl,

The measurement report production would begin when pMadministrativeState

is UNLOCKED and pMoperationalState is ENABLED.";

}

leaf pMOperationalState {

config false;

mandatory true;

type types3gpp:OperationalState ;

description "Indicates whether the MeasurementControl is working.";

}

leaf defaultFileLocation {

type string ;

description "It is the path to the location where produced

measurement reports (containing PM data) are stored. File based

measurement delivery will not start if this leaf does not have a

valid value.

It is the path to a location on either the producer’s file system or a

URI to a network file location that is not part of the producer’s file

system. In case it points to a location on the producer’s file system,

it is a relative path based on a vendor-specified root directory for

measurement files.

The size of this fileLocation is decided by consumer and producer.

The producer is expected to remove old files to make room for

new files, when necessary.

This value is ignored in case the fileLocation leaf in the

MeasurementReader is defined.";

}

leaf defaultFileBasedGP {

when '../defaultFileLocation' ;

mandatory true;

type uint32 ;

units second;

description "GP (granularity period) defines the frequency of producing

measurement data. Measurement data would be produced immediately at the

end of each fileBasedGP.

This value is ignored in case the fileBasedGP leaf in the

MeasurementReader is defined.";

}

leaf defaultFileReportingPeriod {

when '../defaultFileLocation' ;

must '. *60 >= ../defaultFileBasedGP' {

error-message "defaultFileBasedGP must not be greater than defaultFileReportingPeriod";

}

mandatory true;

type uint32 ;

units minute;

description "The frequency of producing the measurement report files.

A measurement report file contains multiple measurement data.

This value is ignored in case the fileReportingPeriod leaf in the

MeasurementReader is defined.";

}

leaf defaultStreamTarget {

if-feature StreamingSupported ;

type string ;

description "Identifies the target of the notifications carrying the

content of the measurement report.

This value is ignored in case the streamTarget leaf in the

MeasurementReader is defined.";

}

leaf defaultStreamBasedGP {

if-feature StreamingSupported ;

when '../defaultStreamTarget' ;

mandatory true;

type uint32 ;

units second;

description "It defines the frequency of producing and sending the

Measurement to the streamTargets.

This value is ignored in case the streamBasedGP leaf in the

MeasurementReader is defined.";

}

}

grouping MeasurementReaderGrp {

description "Identifies the entity whose Measurements are required by

consumer, the types of Measurements required and the delivery method for

the measurements.";

leaf-list measurementTypes {

type string ;

description "The Measurement type to be reported. It must be one of the

measurmentTypes supported by the containing function/class.

The Measurement type can be those specified in TS 28.552,

TS 32.404 and can be those specified by other SDOs or can be

vendor-specific.";

}

leaf-list managedObjectDNsBasic {

type types3gpp:DistinguishedName ;

description "Identifies the managed functions whose Measurements are

required to be produced.

It identifies specific managed entities (say X, Y, Z). It would mean

Measurements type specified in MeasurementReader.measurementTypes,

are required to be produced if X, Y, Z are capable of supporting

the Measurement types.

If managedObjectDNs of the same MeasurementReader instance has valid

information, the information of this leaf-list is ignored.";

}

leaf-list managedObjectDNs {

type types3gpp:DistinguishedName ;

description "Identifies the managed entities whose Measurements are

required to be produced.

In case the base is SubNetwork, it identifies all, including the base,

managed entities that are subordinates, in the sense of name-containment,

of the base.

In case the base is NetworkSliceSubnet, it identifies all, including

the base, managed entities that has aggregation association relation

with the base.

We called the identified entities a collection. It would mean

Measurement types specified in attribute

MeasurementReader.measurementTypes, are required to be produced if the

member (of the collection) is capable of supporting the Measurement

types.";

}

leaf fileLocation {

type string ;

description "It is the path to the location where produced

measurement reports (containing PM data) are stored. File based

measurement delivery will not start if this leaf does not have a

valid value.

It is the path to a location on either the producer’s file system or a

URI to a network file location that is not part of the producer’s file

system. In case it points to a location on the producer’s file system,

it is a relative path based on a vendor-specified root directory for

measurement files.

The size of this fileLocation is decided by consumer and producer.

The producer is expected to remove old files to make room for

new files, when necessary.

If defined this value overrides the value of defaultFileLocation in the

parent MeasuremnetControl.";

}

leaf fileBasedGP {

when '../fileLocation' ;

mandatory true;

type uint32 ;

units second;

description "GP (granularity period) defines the frequency of producing

measurement data.";

}

leaf fileReportingPeriod {

when '../fileLocation' ;

must '.  *60 >= ../fileBasedGP' {

error-message "fileBasedGP must not be greater than fileReportingPeriod";

}

mandatory true;

type uint32 ;

units minute;

description "The frequency of producing the measurement report files.";

}

leaf streamTarget {

when 'not(../fileLocation)' ;

type string ;

description "Identifies the target of the notification carrying the

content of the measurement report.

There are two delivery methods (i.e. file-based and stream-based) via

which the consumer(s) can receive the Measurements. This attribute

is used for the stream-based delivery method.";

}

leaf streamBasedGP {

when '../streamTarget' ;

mandatory true;

type uint32 ;

units second;

description "Defines the frequency of producing and sending the

Measurement to the consumer.";

}

}

grouping MeasurementSubtree {

description "Contains classes that define measurements.

Should be used in al  classes (or classes inheriting from)

- SubNnetwork

- ManagedElement

- ManagedFunction

- NetworkSliceSubnet

- EP_RP

If some YAM wants to augment these classes/list/groupings they must

augment all user classes!

If a class uses this grouping in its list it shall also use the

grouping Measurements to add supportedMeasurementsGPs as

an attribute to its grouping";

list MeasurementControl {

description "Represents the properties of the file-based and stream-based

measurement delivery methods.

The file-based delivery method has properties for the file location,

the file reporting period and the file-based GP.

The stream-based delivery method has properties for the stream target

and the stream-based GP.

These properties are labelled as default (e.g. defaultFileBasedGP) in

that they will be ignored in case the same properties captured in

MeasurementReader are used.

Instance of this list shall not be created nor deleted by client. It

shall be created and deleted by the system.

Depending on particular deployment context and agreement between

operator and vendor, all attributes could be config=false.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses MeasurementControlGrp ;

}

list MeasurementReader {

description "Identifies the entity (derivatives of ManagedFunction)

whose Measurements are required by consumer to be produced and

captured. The types of Measurements required are identified by the

measurementNames.

It captures the properties of the two delivery methods for delivering

the Measurements.

The file-based delivery method has properties: fileLocation,

reportingPeriod and fileBasedGP.

The stream-based delivery method has properties: streamTarget and

streamBasedGP.

The parent MeasurementControl list entry also can capture the

properties of the file-based and stream-based delivery methods.

If the MeasurementReader instance’s file-based delivery method has

valid properties, the file-based delivery method is used and the

MeasurementControl instance’s file-based delivery method is ignored.

If the MeasurementReader instance’s stream-based delivery method has

valid properties, the stream-based delivery method is used and the

MeasurementControl instance’s stream-based delivery is ignored.

The file-based and stream-based delivery methods can be active at the

same time for a MeasurementReader instance.

The activity of a MeasurementReader instance is independent of that

of other MeasurementReader instances.

This IOC uses managedObjectDNs or managedObjectDNsBasic to identify

specific managed entities whose Measurements are required by consumer.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses MeasurementReaderGrp;

}

}

}

}

}

module _3gpp-common-subnetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-subnetwork";

prefix "subnet3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-measurements { prefix meas3gpp; }

import ietf-yang-schema-mount { prefix yangmnt; }

organization "3GPP SA5";

description "Defines basic SubNetwork which will be augmented by other IOCs";

reference "3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

3GPP TS 28.620

Umbrella Information Model (UIM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.620 V15.X.XX

3GPP TS 28.622 V15.X.XX";

}

feature ExternalsUnderSubNetwork {

description "Classes representing external entities like EUtranFrequency,

ExternalGNBCUCPFunction, ExternalENBFunction

are contained under a Subnetwork list/class.";

}

feature MeasurementsUnderSubNetwork {

description "The MeasurementSubtree shall be contained under SubNetwork";

}

grouping Domain_Grp {

description "A domain is a partition of instances of managed entities

such that :

-  the group represents a topological structure which describes the

potential for connectivity

-  Subject to common administration

-  With common characteristics";

leaf dnPrefix {

type types3gpp:DistinguishedName;

reference "Annex C of 32.300 ";

}

leaf userLabel {

type string;

description "A user-friendly (and user assignable) name of this object.";

}

leaf userDefinedNetworkType {

type string;

description "Textual information indicating network type, e.g. 'UTRAN'.";

}

}

grouping SubNetworkGrp {

uses Domain_Grp;

uses meas3gpp:Measurements;

leaf-list setOfMcc {

description "Set of Mobile Country Code (MCC).

The MCC uniquely identifies the country of domicile

of the mobile subscriber. MCC is part of the IMSI (3GPP TS 23.003)

This list contains all the MCC values in subordinate object

instances to this SubNetwork instance.

See clause 2.3 of 3GPP TS 23.003 for MCC allocation principles.

It shall be supported if there is more than one value in setOfMcc

of the SubNetwork. Otherwise the support is optional.";

type types3gpp:Mcc;

}

leaf priorityLabel {

mandatory true;

type uint32;

}

}

list SubNetwork {

key id;

description "Represents a set of managed entities";

uses top3gpp:Top_Grp;

container attributes {

uses SubNetworkGrp;

leaf-list parents {

description "Reference to all containg SubNetwork instances

in strict order from the root subnetwork down to the immediate

parent subnetwork.

If subnetworks form a containment hierarchy this is

modeled using references between the child SubNetwork and the parent

SubNetworks.

This reference MUST NOT be present for the top level SubNetwork and

MUST be present for other SubNetworks.";

type leafref {

path "../../../SubNetwork/id";

}

}

leaf-list containedChildren{

description "Reference to all directly contained SubNetwork instances.

If subnetworks form a containment hierarchy this is

modeled using references between the child SubNetwork and the parent

SubNetwork.";

type leafref {

path "../../../SubNetwork/id";

}

}

}

uses meas3gpp:MeasurementSubtree {

if-feature MeasurementsUnderSubNetwork ;

}

yangmnt:mount-point children-of-SubNetwork {

description "Mountpoint for ManagedElement";

reference "RFC8528 YANG Schema Mount";

}

// augment external parts here

}

} module _3gpp-common-top {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-common-top;

prefix top3gpp;

organization "3gpp SA5";

description "The model defines a YANG mapping of the top level

information classes used for management of 5G networks and

network slicing.";

reference

"3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

3GPP TS 28.620

Umbrella Information Model (UIM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.620 V15.X.XX

3GPP TS 28.622 V15.X.XX";

}

grouping Top_Grp {

description "Abstract class supplying a naming attribute";

reference "3GPP TS 28.620";

leaf id {

type string;

description "Key leaf (namingAttribute) for a class/list.

Should be used as a key leaf for lists representing

stage 2 classes.";

reference "3GPP TS 32.300 Name convention for managed objects";

}

}

}

module _3gpp-common-subscription-control {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-subscription-control";

prefix "subscr3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

description "Defines IOCs for subscription and heartbeat control.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.623";

revision 2019-11-29 {

description "Initial revision";

reference "S5-197648 S5-197647 S5-197829 S5-197828";

}

grouping NtfSubscriptionControlGrp {

description "Attributes of a specific notification subscription";

leaf notificationRecipientAddress {

type string;

mandatory true;

}

leaf-list notificationTypes {

type string;

description "Defines the types of notifications that are candidates

for being forwarded to the notification recipient.

If the notificationFilter attribute is not supported or not present

all candidate notifications types are forwarded to the notification;

discriminated by notificationFilter attribute.";

}

list scope {

key "scopeType";

min-elements 0;

max-elements 1;

description "Describes which object instances are selected with

respect to a base object instance.";

leaf scopeType {

type enumeration {

enum BASE_ONLY;

enum BASE_ALL;

enum BASE_NTH_LEVEL;

enum BASE_SUBTREE;

}

description "If the optional scopeLevel parameter is not supported

or absent, allowed values of scopeType are BASE_ONLY and BASE_ALL.

The value BASE_ONLY indicates only the base object is selected.

The value BASE_ALL indicates the base object and all of its

subordinate objects (incl. the leaf objects) are selected.

If the scopeLevel parameter is supported and present, allowed

values of scopeType are BASE_ALL, BASE_ONLY, BASE_NTH_LEVEL

and BASE_SUBTREE.

The value BASE_NTH_LEVEL indicates all objects on the level,

which is specified by the scopeLevel parameter, below the base

object are selected. The base object is at scopeLevel zero.

The value BASE_SUBTREE indicates the base object and all of its

subordinate objects down to and including the objects on the level,

which is specified by the scopeLevel parameter, are selected.

The base object is at scopeLevel zero.";

}

leaf scopeLevel {

when '../scopeType = "BASE_NTH_LEVEL" or ../scopeType = "BASE_SUBTREE"';

type uint16;

mandatory true;

description "See description of scopeType.";

}

}

leaf notificationFilter {

type string;

description "Defines a filter to be applied to candidate notifications

identified by the notificationTypes attribute.

If notificationFilter is present, only notifications that pass the

filter criteria are forwarded to the notification recipient; all other

notifications are discarded.

The filter can be applied to any field of a notification.";

}

}

grouping NtfSubscriptionControlWrapper {

list NtfSubscriptionControl {

description "A NtfSubscriptionControl instance represents the

notification subscription of a particular notification recipient.

The scope attribute is used to select managed object instances.

The base object instance of the scope is the object instance

name-containing the NtfSubscriptionControl instance.

The notifications related to the selected managed object instances

are candidates to be sent to the address specified by the

notificationRecipientAddress attribute.

The notificationType attribute and notificationFilter attribute

allow MnS consumers to exercise control over which candidate

notifications are sent to the notificationRecipientAddress.

If the notificationType attribute is supported and present, its

value identifies the

types of notifications that are candidate to be sent to the

notificationRecipientAddress. If the notificationType attribute is

not supported or not present, all types of notifications are

candidate to be sent to notificationRecipientAddress.

If supported, the notificationFilter attribute defines a filter that

is applied to the set of candidate notifications. Only candidate

notifications that pass the filter criteria are sent to the

notificationRecipientAddress. If the notificationFilter attribute is

not supported all candidate notificatios are sent to the

notificationRecipientAddress.

To receive notifications, a MnS consumer has to create

NtfSubscriptionControl object instancess on the MnS producer.

A MnS consumer can create a subscription for another MnS consumer

since it is not required the notificationRecipientAddress be his own

address.

When a MnS consumer does not wish to receive notifications any more

the MnS consumer shall delete the corresponding NtfSubscriptionControl

instance.

Creation and deletion of NtfSubscriptionControl instances by MnS

consumers is optional; when not supported, the NtfSubscriptionControl

instances may be created and deleted by the system or be pre-installed.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NtfSubscriptionControlGrp;

}

}

}

augment /subnet3gpp:SubNetwork {

uses NtfSubscriptionControlWrapper;

}

augment /me3gpp:ManagedElement {

uses NtfSubscriptionControlWrapper;

}

}

module _3gpp-common-yang-extensions {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-common-yang-extensions ;

prefix yext3gpp ;

organization "3GPP SA5";

description "The module defines YANG extensions needed

3GPP YANG modeling.

Copyright (c) 2019 3GPP. All rights reserved.”;

Extensions MUST be defined with the following structure in the

description statement:

- What is this statement.

- Newline,

- This statement can be a substatement of the xxx statements with

cardinality x..y.

- This statement can have the following substatements with

cardinality x..y.

- Newline

- Is changing this statement an editorial, BC(backwards compatible)

or NBC(non-BC) change?

- Newline.

- The argument its meaning and type. Preferably use YANG types and

constraints to define the argument's type.

Any extension statement can be added with a

deviation/deviate add statement. In this case the restriction about

the parent statement of the extension SHALL be evaluated based on the

target of the deviation statement.

Support for this module does not mean that a YANG server implements

support for each of these extensions.

Implementers of each specific module using an extensions MUST check

if the server implements support for the used extension.

Note: modules use many extensions which individual

implementations MAY or MAY NOT support.

If support for an extension is missing the extension statement needs

individual handling or it SHOULD be removed from the module using

the extension e.g. with a deviation.

";

revision "2019-06-23" {

description "Initial version";

}

extension inVariant {

description

"Indicates that the value for the data node can only be set when its

parent data node is being created. To change the value after that, the

parent data node must be deleted and recreated with the data node

having the new value.

It is unnecessary to use and MUST NOT be used for key leafs.

The statement MUST only be a substatement of a leaf, leaf-list, list

statements that is config=true.

Zero or one inVariant statement is allowed per parent statement.

NO substatements are allowed.

Adding this statement is an NBC change, removing it is BC.";

}

extension initial-value {

description "Specifies a value that the system will set for a leaf

leaf-list if a value is not specified for it when its parent list

or container is created. The value has no effect in any other

modification e.g. changing or removing the value.

The description statement of the parent statement SHOULD contain

the label 'Initial-value: ' followed by the text from the argument.

The statement MUST only be a substatement of a leaf or leaf-list.

The statement MUST NOT be present if the leaf or the leaf-list

has a default statement or the type used for the data node

has a default value.

The statement MUST NOT be used for config=false data or in an

action, rpc or notification.

Zero or one initial-value statements are allowed for a leaf parent

statement. Zero or more initial-value statements are allowed for a

leaf-list parent statement. If the leaf-list is ordered-by user, the

initial values are stored in the order they appear in the YANG definition.

NO substatements are allowed.

Always consider using a YANG-default statement instead.

Modification of the initial-value is a non-backwards-compatible change.

The argument specifies a single initial value for a leaf or leaf-list.

The value MUST be part of the valuespace of the leaf/leaf-list.

It follows the same rules as the argument of the default statement.";

argument "initial-value";

}

}

module _3gpp-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-yang-types";

prefix "types3gpp";

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

description "The model defines a YANG mapping of the top level

information classes used for management of 5G networks and

network slicing.";

reference "3GPP TS 28.541";

revision 2019-06-23 {

description "Initial version.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

typedef Mcc {

description "The mobile country code consists of three decimal digits,

The first digit of the mobile country code identifies the geographic

region (the digits 1 and 8 are not used):";

type string {

pattern '[02-79][0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

typedef Mnc {

description "The mobile network code consists of two or three

decimal digits (for example: MNC of 001 is not the same as MNC of 01)";

type string {

pattern '[0-9][0-9][0-9]|[0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

grouping PLMNId {

leaf mcc {

mandatory true;

type Mcc;

}

leaf mnc {

mandatory true;

type Mnc;

}

reference "TS 23.658";

}

typedef Nci {

description "NR Cell Identity. The NCI shall be of fixed length of 36 bits

and shall be coded using full hexadecimal representation.

The exact coding of the NCI is the responsibility of each PLMN operator";

reference "TS 23.003";

type union {

type string {

length 36;

pattern '[01]+';

}

type string {

length 9;

pattern '[a-fA-F0-9]*';

}

}

}

typedef OperationalState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum DISABLED {

value 0;

description "The resource is totally inoperable.";

}

enum ENABLED {

value 1;

description "The resource is partially or fully operable.";

}

}

}

typedef AdministrativeState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum LOCKED {

value 0;

description "The resource is administratively prohibited from performing

services for its users.";

}

enum UNLOCKED {

value 1;

description "The resource is administratively permitted to perform

services for its users. This is independent of its inherent

operability.";

}

enum SHUTTINGDOWN {

value 2;

description "Use of the resource is administratively permitted to

existing instances of use only. While the system remains in

the shutting down state the manager or the managed element

may at any time cause the resource to transition to the

locked state.";

}

}

}

typedef AvailabilityStatus {

type enumeration {

enum IN_TEST;

enum FAILED;

enum POWER_OFF;

enum OFF_LINE;

enum OFF_DUTY;

enum DEPENDENCY;

enum DEGRADED;

enum NOT_INSTALLED;

enum LOG_FULL;

}

}

typedef CellState {

type enumeration {

enum IDLE;

enum INACTIVE;

enum ACTIVE;

}

}

typedef SNssai {

type union {

type uint8;

type uint32;

}

description "Single Network Slice Selection Assistance Information.";

reference "TS 23.501 clause 5.15.2";

}

typedef Sst {

type uint8;

}

typedef Nrpci {

type uint32;

description "Physical Cell Identity (PCI) of the NR cell.";

reference "TS 36.211 subclause 6.11";

}

typedef Tac {

type int32 {

range 0..16777215 ;

}

description "Tracking Area Code";

reference "TS 23.003 clause 19.4.2.3";

}

typedef AmfRegionId {

type union {

type uint8 ;

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfSetId {

type union {

type uint16 {

range '0..1023';

}

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfPointer {

type union {

type uint8 {

range '0..63';

}

type string {

length 6;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

grouping AmfIdentifier {

leaf amfRegionId {

type AmfRegionId;

}

leaf amfSetId {

type AmfSetId;

}

leaf amfPointer {

type AmfPointer;

}

description "The AMFI is constructed from an AMF Region ID,

an AMF Set ID and an AMF Pointer.

The AMF Region ID identifies the region,

the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and

the AMF Pointer uniquely identifies the AMF within the AMF Set. ";

}

// type definitions especially for core NFs

typedef NfType {

type enumeration {

enum NRF;

enum UDM;

enum AMF;

enum SMF;

enum AUSF;

enum NEF;

enum PCF;

enum SMSF;

enum NSSF;

enum UDR;

enum LMF;

enum GMLC;

enum 5G_EIR;

enum SEPP;

enum UPF;

enum N3IWF;

enum AF;

enum UDSF;

enum BSF;

enum CHF;

}

}

typedef NotificationType {

type enumeration {

enum N1_MESSAGES;

enum N2_INFORMATION;

enum LOCATION_NOTIFICATION;

}

}

typedef Load {

description "Latest known load information of the NF, percentage ";

type uint8 {

range 0..100;

}

}

typedef N1MessageClass {

type enumeration {

enum 5GMM;

enum SM;

enum LPP;

enum SMS;

}

}

typedef N2InformationClass {

type enumeration {

enum SM;

enum NRPPA;

enum PWS;

enum PWS_BCAL;

enum PWS_RF;

}

}

grouping DefaultNotificationSubscription {

leaf notificationType {

type NotificationType;

}

leaf callbackUri {

type inet:uri;

}

leaf n1MessageClass {

type N1MessageClass;

}

leaf n2InformationClass {

type N2InformationClass;

}

}

grouping Ipv4AddressRange {

leaf start {

type inet:ipv4-address;

}

leaf end {

type inet:ipv4-address;

}

}

grouping Ipv6PrefixRange {

leaf start {

type inet:ipv6-prefix;

}

leaf end {

type inet:ipv6-prefix;

}

}

typedef NsiId {

type string;

}

typedef UeMobilityLevel {

type enumeration {

enum STATIONARY;

enum NOMADIC;

enum RESTRICTED_MOBILITY;

enum FULLY_MOBILITY;

}

}

typedef ResourceSharingLevel {

type enumeration {

enum SHARED;

enum NOT_SHARED;

}

}

typedef TxDirection {

type enumeration {

enum DL;

enum UL;

enum DL_AND_UL;

}

}

grouping AddressWithVlan {

leaf ipAddress {

type inet:ip-address;

}

leaf vlanId {

type uint16;

}

}

typedef DistinguishedName {    // TODO is this equivalent to TS 32.300 ?

type string {

pattern '([a-zA-Z][a-zA-Z0-9-]*=(\\( |#|\\|>|<|;|"|\+|,|[a-fA-F0-9]{2})|[^\\><;"+,# ])'

+ '((\\( |#|\\|>|<|;|"|\+|,|[a-fA-F0-9]{2})|[^\\><;"+,])*'

+ '(\\( |#|\\|>|<|;|"|\+|,|[a-fA-F0-9]{2})|[^\\><;"+, ]))?'

+ '[,\+])*[a-zA-Z][a-zA-Z0-9-]*=(\\( |#|\\|>|<|;|"|\+|,|[a-fA-F0-9]{2})|[^\\><;"+,# ])'

+ '((\\( |#|\\|>|<|;|"|\+|,|[a-fA-F0-9]{2})'

+ '|[^\\><;"+,])*(\\( |#|\\|>|<|;|"|\+|,|[a-fA-F0-9]{2})|[^\\><;"+, ]))?';

}

description "Represents the international standard for the representation

of Distinguished Name (RFC 4512).

The format of the DistinguishedName REGEX is:

{AttributeType = AttributeValue}

AttributeType consists of alphanumeric and hyphen (OIDs not allowed).

All other characters are restricted.

The Attribute value cannot contain control characters or the

following characters : \\ > < ; \" + , (Comma) and White space

The Attribute value can contain the following characters if they

are excaped : \\ > < ; \" + , (Comma) and White space

The Attribute value can contain control characters if its an escaped

double digit hex number.

Examples could be

UID=nobody@example.com,DC=example,DC=com

CN=John Smith,OU=Sales,O=ACME Limited,L=Moab,ST=Utah,C=US";

reference  "RFC 4512 Lightweight Directory Access Protocol (LDAP):

Directory Information Models";

} // recheck regexp it doesn't handle posix [:cntrl:]

typedef QOffsetRange  {

type int8 {

range "-24 | -22 | -20 | -18 | -16 | -14 | -12 | -10 | -8 | -6 | " +

" -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 10 | " +

" 12 | 14 | 16 | 18 | 20 | 22 | 24";

}

units dB;

}

}

End of modificatios