TSG SA Meeting #SP-87E	SP-200029

17 - 20 March 2020, Electronic meeting

3GPP TSG-SA WG1 Meeting #89e 	S1-201087

Electronic Meeting, 10 - 14 February 2020	(revision of S1-201078)

Title:	LS on Questions on onboarding requirements

Response to:	LS S2-2001729 / S1-201015 on Questions on onboarding requirements from SA2

Release:	Release 17

Work Item:	FS_eNPN

Source:	SA1

To:	SA2

Cc:	SA, CT1, SA3, CT6

Contact Person:

#### Name:	Kurt Bischinger

Tel. Number:

E-mail Address:	kurt.bischinger@magenta.at

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA1 thanks SA2 for their LS on onboarding requirements.

SA2 raised several questions concerning the following onboarding requirement:

The 5G system shall support a secure mechanism for a network operator of an NPN to remotely provision the non-3GPP identities and credentials of a uniquely identifiable and verifiably secure IoT device.

SA1 would like to provide the following answers:

Q1) SA2 would like to verify with SA1 whether the above-quoted requirement includes the provisioning of the following for Stand-alone non-public networks (SNPNs):

- IMSI accompanied by AKA credentials, both used for SNPN authentication

- IMSI accompanied by AKA credentials, the IMSI being used to derive a Network Specific Identifier that will be used for SNPN authentication with the AKA credentials

A1) The quoted requirement applies to non-3GPP identities and credentials only, while SA2’s question refers to 3GPP identities and credentials. As such, the answer is no, the above-quoted requirement does not include provisioning of the mentioned identities and credentials to SNPNs. However, SA1 would like to point out that a requirement for remote provisioning has been included in TS 22.261, clause 6.14.2, since Release 15:

The 5G system shall support a secure mechanism for a home operator to remotely provision the 3GPP credentials of a uniquely identifiable and verifiably secure IoT device.

This requirement was acknowledged as being part of "Existing features partly or fully covering the use case functionality" during FS_AVPROD study (see TR 22.827).

Q2) SA2 would like to verify with SA1 whether the above-quoted requirement applies to PNI-NPN, which is the NPN “hosted by a PLMN” as described in TS 22.261 clause 6.25.1, or not, and what would be the corresponding use cases.

A2) SA1 requests clarification on the question from SA2, specifically, is SA2 asking if the above quoted question is related to primary or secondary authentication for the PNI-NPN.

Q3) If SA1 confirm the above-quoted requirement applies to PNI-NPN in Q2, SA2 have further Q3 as below.

For PNI-NPN, a UE may perform secondary PDU session authentication using 3rd party credentials, if the NPN is integrated in PLMN by means of dedicated DNNs, and/or a UE may perform Network specific slice authentication and authorisation (NSSAA) using 3rd party credentials if the NPN is integrated in PLMN by means of network slice. Given the authentication procedures already specified in TS 23.501, TS 24.501 and TS 33.501, SA2 would also like to ask whether provisioning for identities and credentials used for Network specific slice authentication and authorisation (NSSAA) and secondary PDU session authentication should be considered to be covered as part of NPN service requirement for onboarding and remote provisioning solution.

A3) SA1 requests clarification on the question from SA2, specifically, is SA2 asking whether 3rd party credentials may be used for secondary network slice authentication and authorization or

Is SA2 asking whether these 3rd party credentials for secondary authentication can be provisioned via the 3GPP system, or is SA2 asking something else.

2. Actions:

To SA2 group.

ACTION: 	SA1 asks SA2 group to take the answer to A1 into account and provide the requested clarifications to Q2 and Q3

3. Date of Next TSG SA WG1 Meetings:

SA1#90	18-22 May 2020		TBD, North-America

SA1#91	24-28 August 2020 (TBC)	Wroclaw, Poland