SA WG2 Meeting #136AH	S2-2001611

# Jan 13th-17th, 2020 ; Incheon, Korea		                               	 (revision of S2-2001125)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2028 | rev | 3 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the 5G VN usage of IP Multicast mechanisms from TS 23.316 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5.8.2.13.3.1 refers to 23.316 for the handling of IP Multicast. The conditions need to be clarified e.g. that this applies for any UE on any access and is not meant for IPTV services. Clarify when and where PIM signalling is exchanged in case of IP Multicast |  |  |  |  |  |  |  |  |
| Summary of change: |  | See Reason for change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on the cases where § 5.8.2.13.3.1 can apply and possibly mis understanding at stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.13.3.1 ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 - Wording: They are not limited to Wireline, is not meant for IPTV services - 5.8.2.13.3.0 General removed and modified text put in 5.8.2.13.3.1 Rev2 - Text on PIM slimmed down |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.8.2.13.3	Support for user plane traffic replication in a 5G VN

5.8.2.13.3.1	User plane traffic replication based on UPF internal functionality

For Ethernet PDU Sessions, the SMF may instruct the UPF to route traffic to be replicated as described in clause 5.8.2.5.

For IP PDU Session types, the SMF may instruct the UPF to manage IP multicast traffic as described in TS 23.316 [84] clauses 4.6.6 and 7.7.1. The UPF replicates the IP multicast traffic received from PDU Sessions or N6 interface and sends the packets over other PDU Sessions and other N6 interface subscribed to the IP Multicast groups.

Mechanisms described in TS 23.316 [84] clauses 4.6.6 and 7.7.1 apply to support 5G VN group communication with following clarifications:

- These mechanisms are not limited to Wireline access and can apply on any access,

- IP Multicast traffic allowed for a PDU Session is not meant for IPTV services reachable over N6,

- IGMP /MLD signalling does not relate with STB or 5G-RG: TS 23.316 [84] clauses 4.6.6 and 7.7.1, apply to UE members of a 5G VN group instead of 5G-RG, and

- TS 23.316 [84] clauses 7.7.1.1.2 and 7.7.1.1.4 are not applicable to 5G VN groups: members of the 5G VN groups may receive any multicast traffic associated with the (DNN, S-NSSAI) of the 5G VN group.

- UPF exchange of signalling such as PIM (Protocol-Independent Multicast) may apply as defined in TS 23.316, with following clarification:

-	PIM signalling is generally exchanged over N6 but may be sent towards the PDU Session supporting the source address of multicast traffic identified by IGMP / MLD signalling for Source Specific Multicast. In case of IGMP / MLD signalling not related with Source Specific Multicast no PIM signalling is sent towards any PDU Session

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES