3GPP TSG-SA3 Meeting #99-e	S3-201422

# Online, 11th May 2020 - 15th May 2020	        Revision of S3-201045

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0069 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Elliptic Curve Group Size |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CryptPr |  |  |  |  |  |  | Date: |  | 2020-04-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The formulation "Elliptic curve groups of less than 255 bits shall not be supported" seems to be unclear. The intention is to allow usage of curve-25519/ed25519/W-25519 while preventing usage of other curves having less than 256 bits. 3GPP should support a higher security level and so recommend ecdsa_secp384r1_sha384. Typo "extention" instead of "extension". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified text about elliptic curve group size Text added to recommend ecdsa_secp384r1_sha384 Corrected typo |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Critical 3GPP systems might use non-recommended elliptic curve groups leading to weaknesses. Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***	BEGIN CHANGES	***

## 6.2.3	Profiling for TLS 1.2

TLS 1.2 (RFC 5246 [50]) shall support the following restrictions and extensions:

TLS cipher suites

-	The rules on allowed  cipher suites given in TLS 1.2 (RFC 5246 [50]) shall be followed.

-	In addition, the following cipher suites are mandatory to support and recommended to use:

-	TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 as defined in RFC 5289 [55]

-	TLS_DHE_RSA_WITH_AES_128_GCM_SHA256 as defined in RFC 5288 [54]

-	Support of the following cipher suites is recommended:

-	TLS_ECDHE_ECDSA_WITH_AES_256_GCM_SHA384 as defined in RFC 5289 [55]

-	TLS_ECDHE_RSA_WITH_AES_256_GCM_SHA384 as defined in RFC 5289 [55]

-	Only cipher suites with AEAD (e.g. GCM) and PFS (e.g. ECDHE, DHE) shall be supported.

Diffie-Hellman groups

-	For ECDHE, the curve secp256r1 (P-256) as defined in RFC 8422 [71] shall be supported, secp384r1 (P-384) as defined in RFC 8422 [71] should be supported.  Except curve25519, ed25519, and W-25519, elliptic curve groups of less than 256 bits shall not be supported.

-	For DHE, Diffie-Hellman groups of at least 4096 bits should be supported. Diffie-Hellman groups smaller than 2048 bits shall not be supported.

TLS hash algorithms and signature algorithms

-	Hash algorithms: SHA-256 shall be supported. SHA-384 should be supported. MD5 and SHA-1 shall not be supported.

-	Signature algorithms: ecdsa, rsa_pss_rsae, and rsa_pkcs1 shall be supported. Usage of rsa_pkcs1 is not recommended.

-    ecdsa_secp384r1_sha384 should be supported.

TLS compression

-	The “null” compression method as specified in TLS 1.2 RFC 5246 [50] is mandatory to support. All other compression methods shall not be supported.

TLS extensions

-	If TLS Extensions are used in conjunction with TLS, then for RFC 6066 [57] shall apply.

-	The Server Name Indication (SNI) extension defined in RFC 6066 [57] shall be supported.

-	The Truncated HMAC extension, defined in RFC 6066 [57] shall not be supported.

-	TLS Session Resumption based on RFC 5246 [50] or RFC 5077 [59] should be supported.

-	TLS servers and TLS clients shall support RFC 5746 [60]. The server shall accept client-initiated renegotiation only if secured according to RFC 5746 [60].

- The Extended Master Secret extension, defined in RFC 7627 [61] shall be supported.

- Signature Algorithms, defined in RFC 5246 [50] shall be supported.

-	The Supported Groups extension, defined in RFC 8422 [71] and RFC 7919 [62] shall be supported.

-	The OCSP Status (a.k.a. certificate status request) extension, defined in RFC 6066 [57] should be supported.

PSK cipher suites

-	If pre-shared key (psk) cipher suites are implemented in TLS, then RFC 4279 [63] and RFC 5489 [64] shall apply and the following cipher suites are mandatory to support and recommended to use:

-	TLS_DHE_PSK_WITH_AES_128_GCM_SHA256 as defined in RFC 5487 [65].

-	TLS_ECDHE_PSK_WITH_AES_128_GCM_SHA256 as defined in RFC 8442 [51].

-	Support of the following cipher suite is recommended:

-	TLS_ECDHE_PSK_WITH_AES_256_GCM_SHA384 as defined in RFC 8442 [51].

***	END OF CHANGES	***