3GPP TSG-SA1 Meeting #90	S1-202181

Electronic Meeting, 18 - 22 May 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.263 | CR | 006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of CMED KPIs tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | b<>com |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CMED |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CR agreed at SA1#88 but not implemented in TS22.263 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Service Performance tables 6.2.2-1, 6.3.2-1 are modified according to the following: - At SA1#87, SA4 provided a feedback on bitrate assumptions made earlier as part of FS_CMED study item work. This feedback triggered a modification of required bitrates for 4k 60 fps 12 bits per pixel colour coded lossy compression video streams in the TR that now needs to be reflected in TS 22.263. - As to other bitrates figures involving uncompressed or lossless compressed video streams the assumptions made were correct and the brackets around a number of bitrate figures have to be removed, - alignment with CT/MRI scan bitrate in the table 7.2.1-3 of TR 22.826, - alignment with #UE density figures of TR 22.826, - alignment with reliability figures of TR 22.826, - alignment with other network dependability related parameters in TR22.826 as follow: relaxation of survival time for non life critical compressed video streams and relaxation of few network availability figures for non life critical data streams, - addition of a new profile for ultrasound images/video transmitted over a PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect services requirements would remain in TS22.263 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2; 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------            START FIRST CHANGE         -----------------------------

### 6.2.2	Medical applications

Table 6.2.2-1: Performance requirements for low latency ultra-reliable imaging/video traffic for medical applications

| Profile | Characteristic parameter |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate | Direction | Message Size [byte] | Survival time | UE speed (km/h) | # of active UEs connection | Service Area |
| UHD medical video over NPNs | >99.99999 | >1 year | <1 ms | < 50 Gbit/s | UL; DL | ~1500 - ~9000 (note 1) | ~8ms | stationary | 1 | 100 m2 |
| Ultrasound images over NPNs | >99.9999 | >1 year | <10ms | 500 Mbit/s - 4 Gbit/s (note 2) | UL; DL | ~1500 | 20-100 ms (note 2) | stationary | 1 | 100 m2 |
| UHD video for telesurgery over PLMNs | >99.9999 | >1 year | < 20 ms | < 6 Gbit/s | UL; DL | ~1500 - ~9000 (note 1) | ~16 ms | stationary | <2 per 1000 km2 | <400 km (note 3) |
| UHD video for medical examination over PLMNs | >99.99 | >1 month | <20 ms | <4 Gbit/s | UL; DL | ~1500 -9000 | ~16 ms | stationary | <20 per 100 km2 | <50 km (note 3) |
| Ultrasound images over PLMNs | >99.999 | >>1 month (<1 year) | <20 ms | <200 Mbit/s | UL; DL | ~1500 | ~16 ms | stationary | <20 per 100 km2 | <50 km (note 3) |
| CT/MR real time scan over PLMNs | >99.999 | >>1 month (<1 year) | < 100ms | <670 Mbit/s | UL, DL | ~1500 | <100 ms | <150 | <20 per 100 km2 | <50 km (note 3) |
| NOTE 1: MTU size of 1500 bytes is not generally suitable to gigabits connections as it induces many interruptions and loads on CPUs. On the other hand, Ethernet jumbo frames of up to 9000 bytes require all equipment on the forwarding path to support that size in order to avoid fragmentation. NOTE 2: lower values considered for 2D ultrasounds images and higher values for 3D ultrasound images NOTE 3: Higher values are needed for telesurgery systems NOTE 3: Maximum straight-line distance between UEs. |  |  |  |  |  |  |  |  |  |  |

-------------------------            END FIRST CHANGE         -----------------------------

-------------------------            START SECOND CHANGE         -----------------------------

### 6.3.2	Medical applications

Table 6.3.2-1: Performance requirements for low latency ultra-reliable imaging/video multicast traffic for medical applications

| Requirement | Characteristic parameter |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate | Direction | Message Size [byte] | Survival time | UE speed (km/h) | # of active UEs connection | Service Area |
| UHD medical video over NPNs | >99.99999 | >1 year | <1 ms | <50 Gbits/s | DL | ~1500 - ~9000 (note 1) | ~8ms | stationary | <10 | 100 m2 |
| UHD medical video over PLMNs | >99.999 | >>1 month (<1 year) | < 250 ms | <2 Gbit/s | DL | ~1500 - ~9000 (note 1) | ~16 ms | stationary | <5 per 100m2 (note 2) | 400 km (note 3) |
| NOTE 1: MTU size of 1500 bytes is not generally suitable to gigabits connections as it induces many interruptions and loads on CPUs. On the other hand, Ethernet jumbo frames of up to 9000 bytes require all equipment on the forwarding path to support that size in order to avoid fragmentation. NOTE 2: This comprises a maximum of 5 displays gathered in the same 100m2 room with an expected room density of 2 per 1000km2 NOTE 3: Maximum straight-line distance between two UEs |  |  |  |  |  |  |  |  |  |  |

-------------------------            END SECOND CHANGE         -----------------------------