3GPP TSG-WG SA2 Meeting #88E e-meeting  	SP-200529

# Elbonia, June 30 -July 3, 2020	(revision of S2-2003288)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2278 | rev | 2 | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Deregistration procedures for SMS over NAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR proposes the following changes: - Consider the Access Type over which the SMS is deactivated. According to the clause 4.13.3.2, the SMS deactivation is triggered by Deregistration or Registration with UE’s capability change for SMS over NAS. The general Registration or Deregistration is associated with Access Type (i.e., 3GPP Access or non-3GPP Access). It is clarified the UE is deregistered for SMS for both access type at one AMF if only one Registration misses the "SMS supported" indication. - Some editorial changes to correct the Nudr_DM_Unsubscribe service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to consider the Access Type over which the SMS is deactivated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Deregistration on one of the access types will cause SMS service become completely unusable to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2 updates the SA2 agreed CR in S2-2003288 to avoid the possiblity the Support of SMS over NAS at a single AMF is slective per access type when the UE is registered for both. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.3.2	Deregistration procedures for SMS over NAS

If:

a)	the UE indicates to the AMF that it no longer wants to send and receive SMS over NAS (e.g., not including "SMS supported" indication in a Registration Request message); or

b)	 when the UE is registered at the AMF for one Access Type only, the AMF determines locally that UE is deregistered for this  Access Type or the AMF receives for this Access Type a Deregistration Notification from UDM indicating UE Initial Registration, Subscription Withdrawn or 5GS to EPS Mobility as specified in clause 5.2.3.2.2,

then:

-	The AMF may unsubscribe from SMS Subscription data changes notification with the UDM by means of the Nudm_SDM_Unsubscribe service operation. The UDM may remove the corresponding subscription of data change notification in UDR by Nudr_DM_Unsubscribe service operation.

-	The AMF invokes the Nsmsf_SMService_Deactivate service operation to trigger the release of the UE Context for SMS on SMSF based on local configurations.

-	The AMF shall delete or deactivate the stored SMSF address in its UE Context.

The SMSF unsubscribes from SMS Management Subscription data changes notification with the UDM by means of the Nudm_SDM_Unsubscribe service operation. The UDM may remove the corresponding subscription of data change notification in UDR by Nudr_DM_Unsubscribe service operation. The SMSF shall invoke, for the applicable access type(s) Nudm_UECM_Deregistration (SUPI, NF ID, Access Type) service operation from UDM to trigger UDM to delete SMSF address of the UE. The UDM may update UE context in SMSF in UDR by Nudr_DM_Update (SUPI, Subscription Data, SMS Subscription data, SMSF address). The SMSF also removes the UE Context for SMS, including AMF address.

# * * * no more changes * * * *