3GPP TSG-SA5 Meeting #138e	S5-214148

# 23 August to 31 August 2021, E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0312 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to applying policy for NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | AF may need to negotiate the policies for future background data transfer with 5GS via NEF, and apply the negotiated policies for a future PDU Session at some point. The measurements for background data transfer policy negotiation have been defined, however the measurements on background data transfer policy application are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the measurements related to background data transfer policy application for NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of background data transfer policy application for NEF cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.9.x (new), A.87 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

#### 5.9.9.x	Background data transfer policy application

##### 5.9.9.x.1	Number of background data transfer policy application requests

a)	This measurement provides the number of background data transfer policy application requests received by the NEF from AF.

b)	CC.

c)	Receipt by the NEF of an Nnef_ApplyPolicy_Create request message from AF (see 3GPP TS 23.502 [7]).

d)	A single integer value.

e)	BDTP.NbrApplyCreatReq

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.2	Number of successful background data transfer policy applications

a)	This measurement provides the number of successful background data transfer policy applications by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ApplyPolicy_Create response message to SMF indicating a successful background data transfer policy application (see 3GPP TS 29.522 [44]).

d)	A single integer value.

e)	BDTP.NbrApplyCreatSucc

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.3	Number of failed background data transfer policy applications

a)	This measurement provides the number of failed background data transfer policy applications by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ApplyPolicy_Create response message to AF indicating a failed background data transfer policy application (see 3GPP TS 29.522 [44]), each message increments the relevant subcounter per failure cause by 1.

d)	Each measurement is an integer value.

e)	BDTP.NbrApplyCreatFail.cause
Where cause indicates the failure cause of the background data transfer policy application.

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.4	Number of background data transfer policy update requests

a)	This measurement provides the number of background data transfer policy update requests received by the NEF from AF.

b)	CC.

c)	Receipt by the NEF of an Nnef_ApplyPolicy_Update request message from AF (see 3GPP TS 23.502 [7]).

d)	A single integer value.

e)	BDTP.NbrApplyUpdateReq

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.5	Number of successful background data transfer policy updates

a)	This measurement provides the number of successful background data transfer policy updates by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ApplyPolicy_Update response message to AF indicating a successful background data transfer policy update (see 3GPP TS 29.522 [44]).

d)	A single integer value.

e)	BDTP.NbrApplyUpdateSucc

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.6	Number of failed background data transfer policy updates

a)	This measurement provides the number of failed background data transfer policy updates by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ApplyPolicy_Update response message to AF indicating a failed background data transfer policy update (see 3GPP TS 29.522 [44]), each message increments the relevant subcounter per failure cause by 1.

d)	Each measurement is an integer value.

e)	BDTP.NbrApplyUpdateFail.cause
Where cause indicates the failure cause of the failed background data transfer policy update.

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.7	Number of background data transfer policy deletion requests

a)	This measurement provides the number of background data transfer policy deletion requests received by the NEF from AF.

b)	CC.

c)	Receipt by the NEF of an Nnef_ApplyPolicy_Delete request message from AF (see 3GPP TS 23.502 [7]).

d)	A single integer value.

e)	BDTP.NbrApplyDelReq

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.8	Number of successful background data transfer policy deletions

a)	This measurement provides the number of successful background data transfer policy deletions by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ApplyPolicy_Delete response message to AF indicating a successful background data transfer policy deletion (see 3GPP TS 29.522 [44]).

d)	A single integer value.

e)	BDTP.NbrApplyDelSucc

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.9.x.9	Number of failed background data transfer policy deletions

a)	This measurement provides the number of failed background data transfer policy deletions by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ApplyPolicy_Delete response message to AF indicating a failed background data transfer policy deletion (see 3GPP TS 29.522 [44]), each message increments the relevant subcounter per failure cause by 1.

d)	Each measurement is an integer value.

e)	BDTP.NbrApplyDelFail.cause
Where cause indicates the failure cause of the failed background data transfer policy deletion.

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

Next modified section

# A.87	Monitoring of background data transfer policy negotiation and application

AF may need to negotiate the policies for future background data transfer with 5GS via NEF, before the UE's PDU Session establishment. Per the request from AF, the NEF negotiates with H-PCF about the transfer policies for the future background data transfer. The transfer policies may contain a desired time window for the background data transfer, a reference to a charging rate for the time window, network area information, and optionally a maximum aggregated bitrate, as described in clause 6.1.2.4 of TS 23.503 [46].

The AF may apply the negotiated policies for a future PDU Session at some point.

The policies for the background data transfer have strong relevance to users’ experience, therefore the measurements are needed to monitor the performance of policy negotiation and application for background data transfer.

End of modified sections