3GPP TSG SA Meeting #93E e-meeting  	SP-211081

# Electronic, 14-20 September 2021	(revision of S2-2106712)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3035 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update UE subscription to contain information indicating authentication towards AAA server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-09-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When AUSF in SNPN supports primary authentication and authorization of UEs that use credentials from an AAA Server in a Credentials Holder (CH), the UDM decides that the primary authentication is performed by AAA Server in CH based on UE subscription data and UE’s SUPI and instructs AUSF that primary authentication by a AAA server in a CH is required. The information that indicates primary authentication and authorization is performed by AAA server needs to be captured as part of UE subscription. |  |  |  |  |  |  |  |  |
| Summary of change: |  | clarify that Nudm_UEAuthentication is used to instruct the AUSF to execute authentication towards AAA-S |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incomplete introduction of Nudm_UEAuthentication service |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.2.3.4	Nudm_UEAuthentication Service

##### 5.2.3.4.1	General

This service is used by the requester NF to get authentication data and provide UDM with the result of the authentication procedure success. In case of SNPN, this service may be used to instruct the AUSF to execute authentication towards AAA-S.

##### 5.2.3.4.2	Nudm_UEAuthentication_Get service operation

See TS 33.501 [15].

##### 5.2.3.4.3	Nudm_UEAuthentication_ResultConfirmation service operation

See TS 33.501 [15].

# * * * * End of changes * * * *