3GPP TSG-SA #93E e-meeting	SP-211002

Sep 14 – 20, 2021, Electronic meeting

# Source:	China Mobile, AT&T, Vodafone, CATT, Tencent, Deutsche Telekom, SK Telecom, Sandvine, Matrixx

# Title:	New SID: Study on UPF enhancement for control and SBA

# Document for:	Approval

# Agenda Item:	6.4

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title: 	Study on UPF enhancement for control and SBA

Acronym:	FS_UPCAS

Unique identifier:

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don’t know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| TR 23.742 | Study on Enhancements to the Service-Based Architecture (Release 16) | Study of extending SBA to user plane but without standardization |

3	Justification

Improve ease for exposure for reporting of information by the UPF:

1) Saving duplicate data transfer and reducing transmission path

  - Example 1: PCF services can directly subscribe/unsubscribe on UPF services for QoS monitoring latency report

2) Retrieving original status or real-time service flow information from UPF:

  - Example 2: NWDAF services can subscribe/unsubscribe to UPF for retrieving real-time service flow information e., to help big-data collection and analysis considering efficient sampling intervals for the different services

3) UPF event exposure:

  - Example 3: 5G IoT solutions would require interfacing of UPF to NEF/Local NEF for network information exposure to 3rd party.

4	Objective

The study item will consider following aspects to support better integration of UPF into the 5GC SBA:

-	Study UPF services registration/deregistration, and discovery with NRF

-	Study UPF event exposure service (s) that would support, e.g.

-	Communication from UPF services to SMF services, PCF services, NWDAF services, CHF services, NEF services, Trusted AF services.

-	Relevant Event IDs.

-	Study and evaluate usage of event exposure service(s) and potential architectural impacts

NOTE 1: SMF is responsible for controlling UPF packet processing

NOTE 2: The performance of UP traffic handling shall not be degraded due to mechanisms defined in this study.

NOTE 3: The study shall address generic UPF data exposure mechanisms via SBA based mechanisms, and the coordination with other SIDs for this aspect may be needed in study phase.

This study shall maintain the Rel-17 backward compatibility on the N3, N6, N9 interfaces.

3TUs for study phase and 2 TUs for normative phase

| Objective | Estimated TUs for SI (3) | Estimated TUs for WI (2) |
| --- | --- | --- |
| 1 | 0.5 | 0.5 |
| 2 | 2 | 1 |
| 3 | 0.5 | 0.5 |

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Rapporteur(s) (see note 2) | For info at TSG# | For approval at TSG# | Remarks |
| New TR 23.7xy | Internal TR |  |  |  | Including the study outputs on key issues, solutions and architecture impacts. |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs. 
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as “Remarks”.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA2

8	Aspects that involve other WGs

Charging aspects will be addressed by SA5

9	Supporting Individual Members

Supporting IM name

China Mobile

AT&T

CATT

Deutsche Telekom

Sandvine

SK Telecom

Tencent

Vodafone

Matrixx

Charter Communications

Verizon UK Ltd

Intel

KDDI

Samsung

Orange