3GPP TSG-SA WG6 Meeting #46-e	S6-212575

e-meeting, 15th – 23rd November 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0016 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Request for updating MBS resources for group communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the work done in TS 23.247 related to MBS sessions usage, the MC service server is capable of updating an MBS session once the need has emerged. The update can either be on media or service requirements, or service area (if applicable). For the case of multicast MBS sessions, the MC service server is also capable of updating the corresponding session state, i.e., active or inactive. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A general description of MBS session update and the corresponding procedure are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No information or content regarding the ability of the MC service server of updating the MBS sessions once it is needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x.y (new), 7.x.y.1 (new), 7.x.y.2 (new), 7.x.y.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.x.y Request for updating MBS resources for group communications

#### 7.x.y.1 General

The MC service server can create one or several MBS sessions based on certain service requirements (e.g., QoS profile), a certain service area, or the activity status of multicast MBS sessions. However, during the life cycle of the MBS sessions, the MC service server may need to update the sessions to meet emerging needs, such as service requirements or service area.

In case of dynamic PCC rule, the MC service server needs to determine what aspects are needed to be updated (either QoS profile, MBS service area, or both) in order to interact with the required entity in the 5GC (either directly in case the MC service server is in trusted domain) or indirectly via NEF/MBSF.

#### 7.x.y.2 Procedure for updating MBS resources without dynamic PCC rule

The procedure shown in figure 7.x.y.2-1 presents an MBS session update procedure triggered by the MC service server (either directly to the MB-SMF, or indirectly via NEF/MBSF). Within the update request, either the service requirements, MBS service area, activity status of multicast MBS session, or all three are done.

Pre-conditions:

-	The MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	The MC service server has obtained the required information related to the MB-SMF, either locally configured or during initial session configuration.

-	The MBS session is created with certain service requirements and optionally with a certain broadcast/multicast service area. The MBS session is announced to be associated with the MC service group for group communication purposes.

Figure 7.x.y.2-1: MBS session update without dynamic PCC.

1.	An MBS session is established (either a multicast or a broadcast session) and associated with a certain active MC group for group communication purposes. In the case of a multicast MBS session, the MC service clients have already joined the session.

2.	The MC service server invokes an MBS session update request towards the 5GC (either directly to the MB-SMF or indirectly via NEF/MBSF) once the need has emerged to modify some aspects for the given MBS session under consideration. In case the MBS session update is done without dynamic PCC rule: the MC service server sends an MBS session update request (either directly to the MB-SMF or indirectly via NEF/MBSF) indicating the MBS session ID to be updated. Along with the update request, the updated aspects are sent, which are either service requirements (required QoS), service area, or both. In case of multicast MBS sessions, the MC service server may as well update the status (active or inactive) of the multicast MBS session once needed within the update request.

NOTE 1:	the updated service area information is required for local MBS and for broadcast MBS services.

3.	Based on the needed requirements, the corresponding MBS session is accordingly modified. The update may lead to QoS Flow(s) addition, modification, or removal.

4.	The MC service server receives an MBS session update response once the requested modifications are performed, and the indicated MBS session is updated accordingly.

5.	The MC service server may initiate a service announcement towards the MC service clients associated with the ongoing session in order to announce the updated information if required, e.g., the updated service area or SDP information.

NOTE 2: the updated service area information is required for local MBS and for broadcast MBS services.

6.	The MC service server sends an MapGroupToSessionStream over the configured MBS session providing the required information to receive the media related to the established MC service group communication.

7.	The MC service clients process the received information over the MapGroupToSessionStream in order to receive the associated MC media over the specific MBS session stream.

8.	MC service client 1 sends media to the MC service server over unicast to be distributed for the established group communication.

9.	The MC service server distributes the MC media to the MC services clients 2 to n over the indicated streams.

#### 7.x.y.3 Procedure for updating MBS resources with dynamic PCC rule

The procedure shown in figure 7.x.y.3-1 presents an MBS session update procedure triggered by the MC service server to the 5GC, either directly or via NEF/MBSF. Based on the required updates to be done, the MC service server needs to interact with the MB-SMF to update the MBS service area and multicast activity status, with the PCF to update the required QoS requirements, or sequentially both to update all the above.

Pre-conditions:

-	The MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	The MC service server has obtained the required information related to the MB-SMF, either locally configured or during initial session configuration.

-	The MBS session is created with certain service requirements and optionally with a certain broadcast/multicast service area. The MBS session is announced to be associated with the MC service group for group communication purposes.

Figure 7.x.y.3-1: MBS session update without dynamic PCC.

1.	An MBS session is established (either a multicast or a broadcast session) and associated with a certain active MC group for group communication purposes. In the case of a multicast MBS session, the MC service clients have already joined the session.

2.	In case that updating only the MBS service area and/or the multicast MBS session activity status are needed to be updated:  the MC service server sends an MBS session update request, (either directly to the MB-SMF or indirectly via NEF/MBSF) along with the MBS session ID, the updated MBS service area and/or session activity status.

NOTE 1:	the updated service area information is required for local MBS and for broadcast MBS services.

3.	Based on the needed requirements, the corresponding MBS session is accordingly modified. The update may lead to QoS Flow(s) addition, modification, or removal.

4.	The MC service server may receive an MBS session update response once the requested modifications are performed, and the indicated MBS session is updated accordingly.

5.	In case of only updating the service requirements: the MC service server sends an MBS policy authorization update request, (either directly to the PCF or indirectly via NEF/MBSF) with the MBS session ID and the updated QoS requirements.

6. Based on the needed requirements, the corresponding MBS session is accordingly modified. The update may lead to QoS Flow(s) addition, modification, or removal.

7. The MC service server may receive an MBS policy authorization response once the requested modifications are performed, and the indicated MBS session is updated accordingly.

NOTE 2: in case of updating the MBS service area and/or the multicast MBS session activity status as well as the service requirements: first, the MC service server sends an MBS session update request towards MB-SMF (or indirectly via NEF/MBSF) with the required updates, afterwards it sends an MBS policy authorization update request towards the PCF (either directly or via NEF/MBSF) indicating the required QoS requirements. In other words, steps 2 to 7 are all performed.

8. The MC service server may initiate a service announcement towards the MC service clients associated with the ongoing session in order to announce the updated information if required, e.g., the updated service area or SDP information.

NOTE 2: the updated service area information is required for local MBS and for broadcast MBS services.

9.	The MC service server sends an MapGroupToSessionStream over the MBS session providing the required information to receive the media related to the established MC service group communication.

10.	The MC service clients process the received information over the MapGroupToSessionStream in order to receive the associated MC media over the specific MBS session stream.

11.	MC service client 1 sends media to the MC service server over unicast to be distributed for the established group communication.

12.	The MC service server distributes the MC media to the MC services clients 2 to n over the indicated streams.