3GPP TSG-SA WG6 Meeting #47-e	S6-220438

e-meeting, 14th – 22nd February 2022	(revision of S6-220319, S6-220175)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0027 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and correction on clause 8.8 Other MSGin5G messaging related procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH |  |  |  |  |  | Date: |  |  | 2022-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Two different procedures for Messaging Topic unsubscription are specified in both 8.8.1 and 8.8.3, some statement about the procedure choice is needed. Some text in clause 8.8.2 is duplicate and needed to be removed. Some wording is needed to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification and correction on clause 8.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Two different procedures for Messaging Topic unsubscription are specified and how to choice a procedure is not mentioned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.1, 8.8.2 and 8.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: the Expiration=0 descriptions in clause 8.8.1 are removed and NOTE in 8.8.3 proposed in S6-220175 is removed. Rev2: the coversheet problems are fixed. |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.8	Other MSGin5G messaging related procedures

8.8.1	Messaging Topic Subscription

An MSGin5G Client or an Application Server can subscribe one or more Messaging Topic(s) on the MSGin5G Server.  The Messaging Topic IE will be populated by the Application Client or the Application Server and the content of this IE is out of scope.

When an MSGin5G Client or an Application Server is subscribed to a Messaging Topic, then the MSGin5G Server will deliver messages that contain the same Messaging Topic to the subscriber.

Figure 8.8.1-1 shows the MSGin5G Client/Application Server subscribing to Messaging Topic(s) on the MSGin5G Server.

Pre-conditions:

1.	The MSGin5G Client or Application Server has registered to the MSGin5G Server.

Figure 8.8.1-1: MSGin5G Service endpoint subscribes to Messaging topic(s)

1.	The MSGin5G Client or Application Server sends a Messaging Topic subscription request to the MSGin5G Server. The request includes the information elements listed in Table 8.8.1-1.

Table 8.8.1-1: Messaging Topic subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client or the sending Application Server. |
| Messaging Topic | M | A list of Messaging Topic(s) that is to be subscribed. The number of Messaging Topic(s) included in this IE can be one or more. |
| Expiration | O | The date and time when the subscription expires. This date and time apply to all Messaging Topic(s) subscribed in this request. If this IE is included, the value of it should be larger than 0.- If this IE is not included, the expiration time is subject to operator policy. |
| NOTE: The content of the Messaging Topic is out of scope of the present document. |  |  |

2.	The MSGin5G Server validates the Messaging Topic subscription request and checks the local stored Messaging Topic(s).

a)	If the subscribed Messaging Topic has already been created, the MSGin5G Server checks whether the UE Service ID/AS Service ID of the subscriber is already included in the subscribers list of this Messaging Topic.

1.	If not, the MSGin5G Server adds the UE Service ID/AS Service ID of the subscriber to the subscribers list of this Messaging Topic. The MSGin5G Server sets the validity time of this subscription to the value of the Expire IE or to a default value according to the service policy.

2.	Else, 	the MSGin5G Server updates the validity time of this subscription.

b)	If the subscribed Messaging Topic has not been already created, the MSGin5G Server creates this Messaging Topic, and adds the UE Service ID/AS Service ID of the subscriber to the subscribers list of this Messaging Topic. The MSGin5G Server sets the validity time of this subscription to the value of the Expire IE or to a default value according to the service policy.

3.	The MSGin5G Server sends a Messaging Topic Subscription response to the originator of the request. The response includes the information listed in Table 8.8.1-2.

Table 8.8.1-2: Messaging Topic Subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | Indicates whether the subscription was successfully added or deleted on the MSGin5G Server. |
| Expiration | O | The validity date and time of this subscription set by the MSGin5G Server. |

### 8.8.2	Message delivery based on Messaging Topic

If an MSGin5G Client or an Application Server is in the subscribers list of a Messaging Topic, the MSGin5G Server delivers messages that contain this Messaging Topic to it.

Figure 8.8.2-1 shows the Message delivery to a subscribing service endpoint based on Messaging Topic.

Pre-conditions:

1.	The MSGin5G Client or Application Server subscribed to a Messaging Topic with the MSGin5G Server. A Messaging Topic with the UE Service ID/AS Service ID of the subscriber has been created.

Figure 8.8.2-1: Message delivery to subscribing service endpoint based on Messaging Topic

1.	The MSGin5G Server receives an MSGin5G message request or an API message request corresponding to step 2 in figures 8.3.2-1 or 8.3.2-2 which includes the IEs as listed in table 8.3.2-1. The MSGin5G message request or API message request contains a Messaging Topic IE corresponding to the Messaging Topic for which the subscription exists.

2.	The MSGin5G Server uses the procedure described in clause 8.3.3 to deliver the message to all subscriber(s) of this Messaging Topic. In each outbound message, the UE Service ID/AS Service ID of subscriber should be added as the Recipient UE Service ID/AS Service ID IE specified in table 8.3.3-1.

8.8.3	Messaging Topic Unsubscription

Corresponding to message topic unsubscription, an MSGin5G Client or an Application Server can unsubscribe one or more Messaging Topic(s) on the MSGin5G Server.

Figure 8.8.3-1 shows the MSGin5G Client/Application Server unsubscribing to Messaging Topic(s) on the MSGin5G Server.

Pre-conditions:

1.	The MSGin5G Client or Application Server has subscribed one or more message topic(s) on the MSGin5G Server.

Figure 8.8.3-1: MSGin5G Service endpoint unsubscribes to Messaging topic(s)

1.	The MSGin5G Client or Application Server sends a Messaging Topic unsubscription request to the MSGin5G Server. The request includes the information listed in Table 8.8.3-1.

Table 8.8.3-1: Messaging Topic unsubscription request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client or the sending Application Server. |
| Messaging Topic(s) | M | A list of Messaging Topic(s) that is to be subscribed. The number of Messaging Topic(s) included in this IE can be one or more. |

2.	The MSGin5G Server validates the Messaging Topic unsubscription request and checks the local stored Messaging Topic(s). If the subscribed Messaging Topic has already been created and if the UE Service ID/AS Service ID of the subscriber is already included in the subscribers list of this Messaging Topic, the MSGin5G Server removes the UE Service ID/AS Service ID from the subscribers list of this Messaging Topic.

3.	The MSGin5G Server sends a Messaging Topic Unsubscription response to the originator of the request. The response includes the information listed in Table 8.8.3-2.

Table 8.8.3-2: Messaging Topic Unsubscription response

| Information element | Status | Description |
| --- | --- | --- |
| subscription status | M | Indicates whether the subscription was successfully deleted on the MSGin5G Server |