SA WG2 Meeting #151e	S2-2203674

# May 16th – 20th, 2022; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0526 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing UDM as consumer of expected UE behavioural parameters analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2022-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specification for expected UE behavioural parameters related network data analytics in clause 6.7.4 indicates that UDM is a possible consumer for these analytics while Nnwdaf services description in clause 7.1 does not list UDM as example consumer. As specified in TS 23.502 clause 4.15.6.2, the AF subscribes to expected UE behaviour analytics from NWDAF and may then use these analytics to provision expected UE behaviour in UDM. It was not intended that UDM consumes expected UE behavioural parameters related network data analytics directly from NWDAF. CT3 confirmed there is no backward compatibility issue with removing UDM as a consumer of these analytics in their reply LS in C3-222597 / S2-2203656. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing UDM as a possible consumer for expected UE behavioural parameters related network data analytics in clause 6.7.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification, leading to useless stage 3 effort. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.1, 6.7.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.7.4	Expected UE behavioural parameters related network data analytics

#### 6.7.4.1	General

The clause 6.7.4 defines how a service consumer learns from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE.

The service consumer may be an NF (e.g. AMF, AF), or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "UE Mobility" or "UE Communication".

-	The Target of Analytics Reporting can be a UE or an Internal Group Identifier.

NOTE:	In the case of untrusted AF the Target of Analytics Reporting can be a GPSI or an External Group Identifier that is mapped in the 5GC to a SUPI or an Internal Group Identifier

-	An Analytics target period, which indicates the time period over which the statistics or predictions are requested.

-	Optional maximum number of objects;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.7.4.3.

The clause 6.7.4 defines how a service consumer learns from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3 of TS 23.502 [3] for a group of UEs or a specific UE.

NEXT CHANGE

#### 6.7.4.4	Procedures

##### 6.7.4.4.1	NWDAF-assisted expected UE behavioural analytics

Figure 6.7.4.4.1-1: NWDAF assisted expected UE behavioural analytics procedure

1.	5GC NF (e.g., AMF, SMF and AF) to NWDAF: Nnwdaf_AnalyticsInfo_Request (Analytics ID, Target of Analytics Reporting=SUPI) or Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Target of Analytics Reporting =SUPI).

The Analytics ID is set to "UE Mobility" or to "UE Communication"," and the consumer request analytics.

2.	If Analytics ID is set to "UE Mobility", the NWDAF collects data from OAM, AMF and/or AF as specified in clause 6.7.2.4 step 2, unless the information is already available.

If Analytics ID is set to "UE Communication", the NWDAF collects data from AMF, SMF and/or AF as specified in clause 6.7.3.4 step 2, unless the information is already available.

3.	The NWDAF derives requested analytics.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

The NWDAF provides requested Expected UE behaviour to the NF, using either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-6.	If the NF subscribed to at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

END OF CHANGES