3GPP TSG-SA5 Meeting #143-e 	S5-223317

# e-meeting, 9 - 17 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0070 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | OpenAPI file name and dependence change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-04-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the endorsed DP SP-220341 in SA#95, one of the action is to update OpenAPI YAML file names to be prefixed with the TS number |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the file names in spec and update the depended reference across yaml files for PerMeasJobCtlMnS.yaml |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS number is missing in the file name. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | ForgeLink: https://forge.3gpp.org/rep/sa5/MnS/-/tree/Rel17_OPENAPI_Filename_Change_142e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

Annex E (normative): 
OpenAPI specification

E.1	Introduction

This clause describes the capabilities of the Management Services in the structure of the OpenAPI Specification Version 3.0.1. The OpenAPI document is represented in the YAML format option.

E.2	OpenAPI document " TS28550_PerMeasJobCtlMnS.yaml"

openapi: 3.0.1

info:

title: TS 28.550 Performance Measurement Job Control Service

version: 16.8.0

description: >-

OAS 3.0.1 specification of the Performance Measurement Job Control Service

@ 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.550 V16.8.0; Performance assurance

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.550/

servers:

- url: '{MnSRoot}/PerfMeasJobCtrlMnS/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/measJobs:

post:

summary: Create a measurement job

description: To create a measurement job the representation of the measurement job is POSTed on the /measJobs collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-RequestType'

responses:

'201':

description: Success case ("201 Created"). The representation of the newly created measurement job resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-ResponseType'

'202':

description: Partial success case ("202 Partically created"). The representation of the newly created measurement job resource with unsupported list shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

get:

summary: Read resources of measurement jobs

description: 'With HTTP GET, resources of measurement jobs are read. The resources to be read are identified with the path component (base resource) and the query component (jobIdList) of the URI. The fields query component allows to select the resource properties to be returned.'

parameters:

- name: jobIdList

in: query

description: This parameter identifies the list of jobId to select the resources from the collection resources identified with the path component of the URI.

required: true

schema:

type: array

items:

type: string

responses:

'200':

description: 'Success case ("200 OK"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected resources are returned.'

content:

application/json:

schema:

$ref: '#/components/schemas/measJobsRetrieval-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

'/measJobs/{jobId}':

get:

summary: Read resource of a single measurement job

description: 'With HTTP GET, resource of a measurement job is read. The resource to be read is identified with the path component of the URI.'

parameters:

- name: jobId

in: path

description: Identifies the measurement job to be read.

required: true

schema:

$ref: '#/components/schemas/uri-Type'

responses:

'200':

description: 'Success case ("200 OK"). The resource identified in the path for retrieval is returned in the response message body. '

content:

application/json:

schema:

$ref: '#/components/schemas/measJobsRetrieval-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

delete:

summary: Delete a single measurement job

description: The measurement job is deleted by deleting the corresponding measurement job resource. The resource to be deleted is identified with the path component of the URI.

parameters:

- name: jobId

in: path

description: Identifies the measurement job to be deleted.

required: true

schema:

$ref: '#/components/schemas/uri-Type'

responses:

'204':

description: Success case ("204 No Content"). The measurement job resource has been deleted. The response message body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

components:

schemas:

dateTime-Type:

type: string

format: date-Time

uri-Type:

type: string

measJobCreation-RequestType:

type: object

properties:

iOCName:

type: string

iOCInstanceList:

type: array

items:

$ref: '#/components/schemas/uri-Type'

measurementCategoryList:

type: array

items:

type: string

reportingMethod:

$ref: '#/components/schemas/reportingMethod-Type'

granularityPeriod:

type: integer

reportingPeriod:

type: integer

startTime:

$ref: '#/components/schemas/dateTime-Type'

stopTime:

$ref: '#/components/schemas/dateTime-Type'

schedule:

$ref: '#/components/schemas/schedule-Type'

streamTarget:

type: string

priority:

$ref: '#/components/schemas/priority-Type'

reliability:

type: string

measJobCreation-ResponseType:

type: object

properties:

unsupportedList:

type: array

items:

$ref: '#/components/schemas/unsupportedMeas-Type'

measJobsRetrieval-ResponseType:

type: object

properties:

jobInfoList:

type: array

items:

$ref: '#/components/schemas/measJobInfo-ResourceType'

error-ResponseType:

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

measJobInfo-ResourceType:

type: object

properties:

href:

$ref: '#/components/schemas/uri-Type'

iOCName:

type: string

iOCInstanceList:

type: array

items:

$ref: '#/components/schemas/uri-Type'

measurementCategoryList:

type: array

items:

type: string

reportingMethod:

$ref: '#/components/schemas/reportingMethod-Type'

granularityPeriod:

type: integer

reportingPeriod:

type: integer

startTime:

$ref: '#/components/schemas/dateTime-Type'

stopTime:

$ref: '#/components/schemas/dateTime-Type'

schedule:

$ref: '#/components/schemas/schedule-Type'

streamTarget:

type: string

priority:

$ref: '#/components/schemas/priority-Type'

reliability:

type: string

schedule-Type:

type: object

properties:

scheduleOption:

$ref: '#/components/schemas/scheduleOption-Type'

dailySchedule:

type: array

items:

$ref: '#/components/schemas/timeInterval-Type'

weeklySchedule:

type: array

items:

$ref: '#/components/schemas/scheduleOfDay-Type'

timeInterval-Type:

type: object

properties:

intervalStart:

type: string

format: Time

intervalEnd:

type: string

format: Time

scheduleOfDay-Type:

type: object

properties:

dayOfWeek:

$ref: '#/components/schemas/dayOfWeek-Type'

intervalsOfDay:

type: array

items:

$ref: '#/components/schemas/timeInterval-Type'

unsupportedMeas-Type:

type: object

properties:

iOCInstance:

$ref: '#/components/schemas/uri-Type'

measurementTypeName:

type: string

reason:

type: string

reportingMethod-Type:

type: string

enum:

- file

- streaming

priority-Type:

type: string

enum:

- low

- medium

- high

scheduleOption-Type:

type: string

enum:

- daily

- weekly

dayOfWeek-Type:

type: string

enum:

- Monday

- Tuesday

- Wednesday

- Thursday

- Friday

- Saturday

- Sunday

End of Change