3GPP TSG-SA5 Meeting #143-e S5-223679 # e-meeting, 9 - 17 May 2022 CR-Form-v12.1. CHANGE REQUEST. 32.291; CR; 0404; rev; 1; Current version:; 17.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Introduce Data Type for 5G ProSe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  |  |  |  |  | Date: |  |  | 2022-4-29 |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |
| Reason for change: |  | The corresponding Data type of 5G ProSe needs to be introduced |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce specific Data Type for 5G ProSe. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No converged charging of 5G ProSe. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6, 6.1.6.2.x (new), 6.1.6.3.4, 6.1.6.3.a-e (new) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1st modified section

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: “Telecommunication management; Charging management; Charging architecture and principles”.

[2] – [13]	Void.

[14]	3GPP TS 32.254: “Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging “.

[15] – [28]	Void.

[29]	3GPP TS 32.274: “Telecommunication management; Charging management;Short Message Service (SMS) charging”.

[30]	3GPP TS 32.255: “Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2”.

[31]	3GPP TS 32.256: “Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2”.

[32]	3GPP TS 32.260: “Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging”.

[33]	3GPP TS 32.275: “Telecommunication management; Charging management; MultiMedia Telephony (MMTel) charging”.

[34]	3GPP TS 32.281: “ Telecommunication management; Charging management; Announcement

[35]	3GPP TS 32.277: “Telecommunication management; Charging management; Proximity-based Services (ProSe) charging”.

[36] – [49]	Void.

[50] – [57]	Void.

[58]	3GPP TS 32.290: “Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI).

[59] – [69]	Void.[70]	3GPP TS 28.201: “Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2”.

[71]	3GPP TS 28.202: “Charging management; Network slice management charging in the 5G System (5GS); Stage 2”.

[72] – [99]	Void.

[100]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[101]	3GPP TR 21.900: “Technical Specification Group working methods”.

[102]	3GPP TS 24.605: “Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification”.

[103] – [199]	Void

[200] – [252]	Void

[253]	3GPP TS 28.532: “Management and orchestration; Management services”.

[254]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3”.

[255]	3GPP TS 32.300: “Telecommunication management; Configuration Management (CM); Name convention for Managed Objects”.

[256]	3GPP TS 28.554: “Management and orchestration;5G end to end Key Performance Indicators (KPI)”.

[257]	3GPP TS 28.623: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions”.

[258]	3GPP TS 24.229: “IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3”.

[259]	3GPP TS 29.078: “Customised Applications for Mobile network Enhanced Logic (CAMEL); CAMEL Application Part (CAP) specification”.”.

[260]	3GPP TS 29.228: “IP Multimedia (IM) Subsystem Cx and Dx interface; signalling flows and message contents”.

[261]	3GPP TS 29.002 : « Mobile Application Part (MAP) specification ».

[262] – [298]	Void

[299] 	3GPP TS 29.500: “5G System; Technical Realization of Service Based Architecture; Stage 3”.

[300]	3GPP TS 29.501: “5G System; Principles and Guidelines for Services Definition; Stage 3”.

[301]	3GPP TS 29.594: “5G System; Spending Limit Control Service; Stage 3”.

[302]	3GPP TS 29.512: “5G System; Session Management Policy Control Service; Stage 3”.

[303]	3GPP TS 24.501: “Non-Access-Stratum (NAS) Protocol for 5G System (5GS); Stage 3”.

[304]	3GPP TS 38.413: “NG-RAN; NG Application Protocol (NGAP)”.

[305]	3GPP TS 29.510: “Network Function Repository Services; Stage 3”.

[306]	3GPP TS 29.520: “5G System; Network Data Analytics Services;Stage 3”.

[307]	3GPP TS 38.331: “NR; Radio Resource Control (RRC); Protocol specification”.

[308]	3GPP TS 24.334: “ Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3”.

[309] – [370]	Void

[371]	3GPP TS 29.571: “5G System; Common Data Types for Service Based Interfaces; Stage 3”.

[372] – [389]	Void

[390] 	3GPP TS 33.501: “Security architecture and procedures for 5G System”.

[391] – [399]	Void

[400]	Void.

[401]	IETF RFC 7540:  “Hypertext Transfer Protocol Version 2 (HTTP/2) “.

[402]	IETF RFC 8259:  “The JavaScript Object Notation (JSON) Data Interchange Format “.

[403]	IETF RFC 6749: “The Oauth 2.0 Authorization Framework”.

[404] 	IETF RFC 3986: “Uniform Resource Identifiers (URI): Generic Syntax”.

[405]	IETF RFC 7315: “Private Extensions to the Session Initiation Protocol (SIP) for the 3rd Generation Partnership Projects (3GPP)”.

[406]	IETF RFC 3261: “SIP: Session Initiation Protocol”.

[407]	IETF RFC 8866: “SDP: Session Description Protocol".

[408]	IETF RFC 5646: "Tags for Identifying Languages".

[409] - [499]	Void.

[500]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[501] - [599]	Void.

Next change

6.1.6.1	General

This subclause specifies the application data model supported by the API.

The Nchf_ConvergedCharging Service API allows the NF consumer to consume the converged charging service from the CHF as defined in 3GPP TS 32.290 [58].

Table 6.1.6.1-1 specifies the data types defined for the ConvergedCharging service based interface protocol.

Table 6.1.6.1-1: Nchf_ ConvergedCharging specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ChargingDataRequest |  | 6.1.6.2.1.1 6.1.6.2.2.1 |  | Describes the attributes of Charging Data Request to CHF for initial, update and termination of the charging session. |  |  |  |
| ChargingDataResponse |  | 6.1.6.2.1.2 6.1.6.2.2.2 |  | Describes the attributes of Charging Data Response from CHF on charging session initial, update and termination. |  |  |  |
| ChargingNotifyRequest |  | 6.1.6.2.1.3 |  | Describes Notifications about events that occurred in request message. |  |  |  |
| ChargingNotifyResponse |  | 6.1.6.2.1.16 |  | Describes the response of notification. |  |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Nchf_ConvergedCharging service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nchf_ConvergedCharging service based interface.

Table 6.1.6.1-2: Nchf_ConvergedCharging re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supi |  | 3GPP TS 29.571 [371] |  | The identification of the user (i.e. IMSI, NAI, GLI, GCI). (NOTE 1) |  |  |  |
| Uint32 |  | 3GPP TS 29.571 [371] |  | Unsigned 32-bit integers |  |  |  |
| Uint64 |  | 3GPP TS 29.571 [371] |  | Unsigned 64-bit integers |  |  |  |
| PduSessionId |  | 3GPP TS 29.571 [371] |  | The identification of the PDU session. |  |  |  |
| PduSessionType |  | 3GPP TS 29.571 [371] |  | the type of a PDU session |  |  |  |
| Uri |  | 3GPP TS 29.571 [371] |  | String providing an URI |  |  |  |
| AccessType |  | 3GPP TS 29.571 [371] |  | The identification of the type of access network. |  |  |  |
| DateTime |  | 3GPP TS 29.571 [371] |  | The time. |  |  |  |
| ChargingId |  | 3GPP TS 29.571 [371] |  | Charging identifier allowing correlation of charging information |  |  |  |
| RatType |  | 3GPP TS 29.571 [371] |  | The identification of the RAT type. |  |  |  |
| RatingGroup |  | 3GPP TS 29.571 [371] |  | The identification of the rating group |  |  |  |
| IpAddr |  | 3GPP TS 29.571 [371] |  | Ipv4 address, Ipv6 address, or Ipv6Prefix |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [371] |  | Ipv4 address. |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [371] |  | The Ipv6 prefix allocated for the user. |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [371] |  | Ipv6 Address. |  |  |  |
| Pei |  | 3GPP TS 29.571 [371] |  | The Identification of a Permanent Equipment. |  |  |  |
| TimeZone |  | 3GPP TS 29.571 [371] |  | Time zone information |  |  |  |
| NfInstanceId |  | 3GPP TS 29.571 [371] |  | String uniquely identifying a NF instance. |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [371] |  | String identifying a Gpsi |  |  |  |
| DefaultQosInformation |  | 3GPP TS 29.571 [371] |  | Identifies the information of the default QoS. |  |  |  |
| SubscribedDefaultQos |  | 3GPP TS 29.571 [371] |  | subscribed default QoS. |  |  |  |
| AuthorizedDefaultQos |  | 3GPP TS 29.512 [302] |  | Authorized default QoS. |  |  |  |
| Ambr |  | 3GPP TS 29.571 [371] |  | Aggregate Maximum Bit rate |  |  |  |
| QosData |  | 3GPP TS 29.512 [302] |  | Contains QoS parameters |  |  |  |
| UserLocation |  | 3GPP TS 29.571 [371] |  | User location information |  |  |  |
| PlmnId |  | 3GPP TS 29.571 [371] |  | PLMN id |  |  |  |
| Guami |  | 3GPP TS 29.571 [371] |  | Globally Unique AMF Identifier |  |  |  |
| DurationSec |  | 3GPP TS 29.571 [371] |  | Identifies a period of time in units of seconds. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [371] |  | SNSSAI |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [371] |  | additional details of the error |  |  |  |
| ServiceId |  | 3GPP TS 29.571 [371] |  | Identifier of service |  |  |  |
| SscMode |  | 3GPP TS 29.571 [371] |  | SSC Mode type |  |  |  |
| PresenceInfo |  | 3GPP TS 29.571 [371] |  | PRA information including PRAId, PRA element list and PRA status |  |  |  |
| Qfi |  | 3GPP TS 29.571 [371] |  | QoS flow identifier designated as "Qfi". |  |  |  |
| AmfId |  | 3GPP TS 29.571 [371] |  | AMF identifier |  |  |  |
| Dnn |  | 3GPP TS 29.571 [371] |  | Data Network Name |  |  |  |
| GroupId |  | 3GPP TS 29.571 [371] |  | Network internal Identifier for a group of IMSIs |  |  |  |
| ExternalGroupId |  | 3GPP TS 29.571 [371] |  | External Group Identifier for one or more subscriptions associated to a group of IMSIs |  |  |  |
| Bytes |  | 3GPP TS 29.571 [371] |  | String with format "byte" |  |  |  |
| Tai |  | 3GPP TS 29.571 [371] |  | Tracking Area Identifier |  |  |  |
| Area |  | 3GPP TS 29.571 [371] |  | List of TACs or Operator specific codes |  |  |  |
| CoreNetworkType |  | 3GPP TS 29.571 [371] |  | 5GC or EPC |  |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [371] |  | Service Area restriction |  |  |  |
| GlobalRanNodeId |  | 3GPP TS 29.571 [371] |  | Global RAN Node Id |  |  |  |
| QosCharacteristics |  | 3GPP TS 29.512 [302] |  | Map of QoS characteristics for non standard 5QIs and non-preconfigured 5QIs. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [371] |  | See 3GPP TS 29.500 [4] clause 6.6 |  |  |  |
| NsiLoadLevelInfo |  | 3GPP TS 29.520 [306] |  | Represents the load level information for an S-NSSAI and the associated network slice instance |  |  |  |
| ServiceExperienceInfo |  | 3GPP TS 29.520 [306] |  | ServiceExperience |  |  |  |
| ApplicationChargingId |  | 3GPP TS 29.571 [371] |  | Application provided charging identifier allowing correlation of charging information. |  | AF_Charging_Identifier |  |
| SharingLevel |  | 3GPP TS 28.541 [254] |  | Ressources sharing level |  |  |  |
| MobilityLevel |  | 3GPP TS 28.541 [254] |  | UE mobility Level |  |  |  |
| SsT |  | 3GPP TS 28.541 [254] |  | Slice Service type (SST) |  |  |  |
| Support |  | 3GPP TS 28.541 [254] |  | Supported, Not Supported indicator |  |  |  |
| Float |  | 3GPP TS 29.571 [371] |  | Number with format "float" |  |  |  |
| MaPduIndication |  | 3GPP TS 29.512 [302] |  | MA PDU session indication |  | ATSSS |  |
| AtsssCapability |  | 3GPP TS 29.571 [371] |  | ATSSS capabilities |  | ATSSS |  |
| SteeringFunctionality |  | 3GPP TS 29.571 [371] |  | Steering functionalities for MA PDU session |  | ATSSS |  |
| SteeringMode |  | 3GPP TS 29.512 [302] |  | Steering mode for MA PDU session |  | ATSSS |  |
| OperationalState |  | 3GPP TS 28.623 [257] |  | Operational state |  |  |  |
| AdministrativeState |  | 3GPP TS 28.623 [257] |  | Administrative state |  |  |  |
| RanNasRelCause |  | 3GPP TS 29.512 [302] |  | Indicates the RAN or NAS release cause code information. |  | EnhancedDiagnostics |  |
| Ecgi |  | 3GPP TS 29.571 [371] |  | E-UTRA Cell Id |  |  |  |
| Ncgi |  | 3GPP TS 29.571 [371] |  | NR Cell Id |  |  |  |
| NOTE 1: A SUPI containing GLI or GCI is used to support 5G-RG and FN-RG in scenarios of wireline network. |  |  |  |  |  |  |  |

Next change

6.1.6.2.x	5G ProSe Specified Data Type

6.1.6.2.x.1	Type ChargingDataRequest

This clause is additional attributes of the type ChargingDataRequest defined in clause 6.x.2.2 for 5G ProSe charging described in 3GPP TS 32.277[35].

Table 6.1.6.2.x.1-1: 5G ProSe Specified attribute of type ChargingDataRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseCharging Information | ProSeChargingInformation | OM | 0..1 | This field holds the 5G ProSe specific information. |  |

6.1.6.2.x.2	Type ChargingDataResponse

This clause is additional attributes of the type ChargingDataResponse defined in clause 6.x.2.2 for 5G ProSe charging described in 3GPP TS 32.277[35].

Table 6.1.6.2.x.2-1: 5G ProSe Specified attribute of type ChargingDataResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

6.1.6.2.x.3	Type UsedUnitContainer

Table 6.1.6.2.x.3-1: 5G ProSe Specified portion of type UsedUnitContainer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pC5Container Information | PC5Container Information | OC | 0..1 | This field holds the PC5 container information |  |

6.1.6.2.x.4	Type PC5ContainerInformation

This clause is additional portion of the type PC5ContainerInformation defined in clause 6.x.2.2 for 5G ProSe charging described in 3GPP TS 32.277[35].

Table 6.1.6.2.x.4-1: 5G ProSe Specified portion of type PC5ContainerInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| coverageInfoList | array (CoverageInfo) | OC | 0..N | This IE provides information on the coverage information. |  |
| radioParameter SetInfoList | array (RadioParameter SetInfo) | OC | 0..N | This IE provides information on a radio parameter set configured in the UE for direct communication use |  |
| transmitterInfoList | array (TransmitterInfo) | OC | 0..N | This IE provides information on a transmitter detected for direct communication |  |
| timeOfFirstTransmission | DateTime | OC | 0..1 | This IE holds the time in UTC format for the first packet transmitted |  |
| timeOfFirstReception | DateTime | OC | 0..1 | This IE holds the time in UTC format for the first packet received. |  |

6.1.6.2.x.5	Type CoverageInfo

Table 6.1.6.2.x.5-1: 5G ProSe Specified portion of type CoverageInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| coverageStatus | boolean | OC | 0..1 | Whether the UE is served by NG-RAN or not |  |
| changeTime | DateTime | OC | 0..1 | The time when the coverage status changed to its current state. |  |
| locationInfo | array(LocationInfo) | OC | 0..N | It provides UE location Information. When in NG-RAN coverage, additionally includes a list of location changes |  |

6.1.6.2.x.6	Type RadioParameterSetInfo

This clause is additional portion of the type RadioParameterSetInfo defined in clause 6.x.2.2 for 5G ProSe charging described in 3GPP TS 32.277[35].

Table 6.1.6.2.x.6-1: 5G ProSe Specified portion of type RadioParameterSetInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| radioParameterSetValues | array(OctetString) | OC | 0..N | It provides the radio parameter set configured in the UE for direct communication. The format of the value is according to the SL-Preconfiguration data type. |  |
| changeTimestamp | DateTime | OC | 0..1 | The time when associated time stamp of when Radio Parameters became active. |  |

6.1.6.2.x.7	Type TransmitterInfo

This clause is additional portion of the type TransmitterInfo defined in clause 6.x.2.2 for 5G ProSe charging described in 3GPP TS 32.277[35].

Table 6.1.6.2.x.7-1: 5G ProSe Specified portion of type TransmitterInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseSourceIPAddress | IpAddr | OC | 0..1 | Source IP address of ProSe UE |  |
| proseSourceL2Id | string | OC | 0..1 | Source L2 Id of ProSe |  |

6.1.6.2.x.8	Type ProseChargingInformation

Table 6.1.6.2.x.8-1: Definition of type ProseChargingInformation

| Attribute name |  | Data type |  |  | P | Cardinality |  | Description | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| announcingPlmnID |  | PlmnId |  |  | OC |  | 0..1 | PLMN identity of the serving PLMN which signalled the carrier frequency. |  |  |
| announcingUeHplmnIdentifier |  | PlmnId |  |  | OM |  | 0..1 | PLMN identity of HPLMN for announcing UE. |  |  |
| announcingUeVplmnIdentifier |  | PlmnId |  |  | OC |  | 0..1 | PLMN identity of VPLMN for announcing UE |  |  |
| monitoringUeHplmnIdentifier |  | PlmnId |  |  | OC |  | 0..1 | PLMN identity of HPLMN for monitoring UE. |  |  |
| monitoringUeVplmnIdentifier |  | PlmnId |  |  | OC |  | 0..1 | PLMN identity of VPLMN for monitoring UE. |  |  |
| discovererUeHplmnIdentifier |  | PlmnId |  |  | OM |  | 0..1 | PLMN identity of Discoverer UE HPLMN. |  |  |
| discovererUeVplmnIdentifier |  | PlmnId |  |  | OC |  | 0..1 | PLMN identity of Discoverer UE VPLMN. |  |  |
| discovereeUeHplmnIdentifier |  | PlmnId |  |  | OC |  | 0..1 | PLMN identity of Discoveree UE HPLMN. |  |  |
| discovereeUeVplmnIdentifier |  | PlmnId |  |  | OC |  | 0..1 | PLMN identity of Discoveree UE VPLMN. |  |  |
| monitoredPlmnIdentifier |  | PlmnId |  |  | OC |  | 0..1 | Monitored PLMN ID in Match_Report request |  |  |
| proseApplicationID |  | string |  |  | OC |  | 0..1 | The identities used for ProSe Direct Discovery, identifying application related information for the ProSe-enabled UE |  |  |
| applicationID |  | string |  |  | OC |  | 0..1 | The identifier a specific 3rd party application. |  |  |
| applicationSpecificDataList |  | array(OctetString) |  |  | OC |  | 0..N | This IE contains a data block provided by the application in the UE as specified in clause 11.3.3 of 3GPP TS 24.334 [308] |  |  |
| proseFunctionality |  | ProseFunctionality |  |  | OC |  | 0..1 | This IE holds the ProSe functionality UE is requesting |  |  |
| proseEventType |  | ProseEventType |  |  | OC |  | 0..1 | This IE holds the event which triggers the charging message delivery |  |  |
| directDiscoveryModel |  | DirectDiscoveryModel |  |  | OC |  | 0..1 | This IE holds the model of the Direct Discovery used by the UE. |  |  |
| validityPeriod |  | integer |  |  | OC |  | 0..1 | Time interval during which user is authorized for using ProSe Direct Discovery |  |  |
| roleOfUE |  | RoleOfUE |  |  | OC |  | 0..1 | Role of the UE using ProSe |  |  |
| proseRequestTimestamp |  | DateTime |  |  | OC |  | 0..1 | The time when ProSe Request is received from UE. |  |  |
| pC3ProtocolCause |  | integer | OC |  |  |  | 0..1 | This IE holds the particular reason why a DISCOVERY_REQUEST or Match_Report messages from the UE have been rejected by the 5G DDNMF in PC3 interface. |  |  |
| monitoringUEIdentifier |  | Supi | OM |  |  |  | 0..1 | Identifier of the party who initiate monitor/match report |  |  |
| requestedPLMNIdentifier |  | PlmnId | OC |  |  |  | 0..1 | The PLMN identifier of the user who is targeted in proximity request. |  |  |
| timeWindow |  | integer | OC |  |  |  | 0..1 | The time interval in minutes during which a proximity request is valid. |  |  |
| rangeClass |  | RangeClass | OC |  |  |  | 0..1 | A range class for the first proximity request. |  |  |
| proximityAlertIndication | Boolean | OC | 0..1 | Indication of whether proximity alert has been sent before proximity request cancellation. |  |
| proximityAlertTimestamp | DateTime | OC | 0..1 | The time stamp when proximity alert is sent, to indicate two UEs are in proximity. |  |
| proximityCancellationTimestamp | DateTime | OC | 0..1 | The time stamp when proximity request cancellation is requested. |  |
| relayIPAddress | IpAddr | OC | 0..1 | The IP address UE used as ProSe UE-to-Network Relay UE address |  |
| proseUEToNetworkRelayUEID | string | OC | 0..1 | A link layer identifier that uniquely represents the ProSe UE-to-Network Relay UE |  |
| proseDestinationLayer2ID | string | OC | 0..1 | The identifier of a link-layer that identifies a device or a group of devices that are recipients of ProSe communication frames. |  |
| pFIContainerInformation | array(pFIContainerInformation) | OC | 0..N | This field holds the PFI data container information |  |
| transmissionDataContainer | array(PC5DataContainer) | OC | 0..N | The container associated to a trigger conditions |  |
| receptionDataContainer | array(PC5DataContainer) | OC | 0..N | This field holds the container associated to a trigger conditions |  |

6.1.6.2.x.9	Type PFIContainerInformation

This clause is additional portion of the type PFIContainerInformation defined in clause 6.x.2.2 for 5G ProSe charging described in 3GPP TS 32.277[35].

Table 6.1.6.2.x.9-1: 5G ProSe Specified portion of type PFIContainerInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pFI | Qfi | OM | 0..1 | PC5 QoS flow Identifier (PFI) |  |
| reportTime | DateTime | OM |  | 1 | the UTC time indicating time stamp when the QFI data container was closed |  |
| timeofFirstUsage | DateTime | OC |  | 0..1 | the UTC time indicating time stamp for the first IP packet to be transmitted and mapped to the PFI container |  |
| timeofLastUsage | DateTime | OC |  | 0..1 | the UTC time indicating time stamp for the last IP packet to be transmitted and mapped to the PFI container. |  |
| qoSInformation | QoSData | OC |  | 0..1 | the PC5 QoS applied to PFI container. In case gbrUl or gbrDl are present for GBR QoS flow, the GBR targets are "GUARANTEED", otherwise, are " NOT_GUARANTEED". |  |
| qoSCharacteristics | QosCharacteristics | OC |  | 0..1 | Map of PC5 QoS characteristics for non standard PQIs and non-preconfigured PQIs. |  |
| userLocationInformation | UserLocation | OC | 0..1 | provides information on the location |  |
| uetimeZone | TimeZone | OC | 0..1 | UE Time Zone the UE is currently located |  |
| presenceReportingAreaInformation | map(PresenceInfo) | OC | 0..N | the Presence Reporting Area status of UE during the PFI container interval. |  |

6.1.6.2.x.10	Type PC5DataContainer

Table 6.1.6.2.x.10-1: 5G ProSe Specified portion of type PC5DataContainer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| localSequenceNumber | string | OC | 0..1 | The sequence number of the Direct Communication data container |  |
| changeTime | DateTime | OC | 0..1 | The time when the container is closed and reported due to ProSe charging condition change. |  |
| coverageStatus | boolean |  | OC | 0..1 | Whether UE is served by NG-RAN or not |  |
| userLocationInformation | UserLocation |  | OC | 0..1 | The location of the UE |  |
| dataVolume | Uint64 |  | OC | 0..1 | This field holds the amount of volume transmitted or received |  |
| changeCondition | string |  | OC | 0..1 | ProSe specific reason for closing the container |  |
| usageInfoReportSN | string |  | OC | 0..1 | The sequence number of usage information report, which is used to generate the container. |  |
| radioResourcesId | RadioResourcesIndicator |  | OC | 0..1 | This IE identifies whether the operator-provided radio resources or the configured radio resources were used for ProSe direct communication. |  |
| radioFrequency | string |  | OC | 0..1 | This IE identifies the radio frequency used for ProSe direct communication as specified in clause 9.3 of 3GPP TS 38.331 [307] |  |
| pC5RadioTechnology | string |  | OM | 0..1 | The PC5 radio technology used by UE |  |

Next change

6.1.6.3.4	Enumeration: NodeFunctionality

Table 6.1.6.3.4-1: Enumeration NodeFunctionality

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SMF | This field identifies that NF is a SMF. |  |
| AMF | This field identifies that NF is a AMF. |  |
| SMSF | This field identifies that NF service consumer is a SMSF. |  |
| PGW_C_SMF | This field identifies that NF is a SMF+PGW-C. |  |
| NEF | This field identifies that NF is a NEF. |  |
| SGW | This field identifies that node is an SGW, only applicable for interworking with EPC. |  |
| I_SMF | This field identifies that node is an I-SMF, only applicable for PDU session served by SMF + I-SMF. | ETSUN |
| ePDG | This field identifies that node is an ePDG, only applicable for interworking with EPC/ePDG. | 5GIEPC_CH |
| CEF | This field identifies that NF is a CEF. |  |
| MnS_Producer | This field identifies that NF is a MnS Producer |  |
| SGSN | This field identifies that node is an SGSN, only applicable when SMF+PGW-C serves GERAN/UTRAN access. | TEI17_NIESGU |
| 5G DDNMF | This field identifies that NF is a 5G DDNMF | 5G ProSe |

Next change

6.1.6.3.a	Enumeration: ProseFunctionality

Table 6.1.6.3.a -1: Enumeration ProseFunctionality

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DIRECT_DISCOVERY | Indicates the UE is requesting for ProSe direct discovery. |  |
| DIRECT_COMMUNICATION | Indicates the UE is requesting for ProSe direct communication. |  |

6.1.6.3.b	Enumeration: ProseEventType

Table 6.1.6.3.b -1: Enumeration ProseEventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ANNOUNCING | Indicates the ProSe ProSe charging announcing event. |  |
| MONITORING | Indicates the ProSe ProSe charging monitoring event. |  |
| MATCH_REPORT | Indicates the ProSe ProSe charging match report event. |  |

6.1.6.3.c	Enumeration: DirectDiscoveryModel

Table 6.1.6.3.c -1: Enumeration DirectDiscoveryModel

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MODEL_A | Indicates model A of the Direct Discovery used by the UE |  |
| MODEL_B | Indicates model B of the Direct Discovery used by the UE. |  |

6.1.6.3.d	Enumeration: RoleOfUE

Table 6.1.6.3.d -1: Enumeration RoleOfUE

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ANNOUNCING_UE | Indicates role of the UE using ProSe for announcing. |  |
| MONITORING_UE | Indicates role of the UE using ProSe for monitoring. |  |
| REQUESTOR_UE | Indicates role of the UE using ProSe for sending requst. |  |
| REQUESTED_UE | Indicates role of the UE using ProSe for receive requst. |  |

6.1.6.3.e	Enumeration: RangeClass

Table 6.1.6.3.e -1: Enumeration RangeClass

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| RESERVED | This value is reserved |  |
| 50_METER | Indicates a range class for a specific proximity request in 50m |  |
| 100_METER | Indicates a range class for a specific proximity request in 100m |  |
| 200_METER | Indicates a range class for a specific proximity request in 200m |  |
| 500_METER | Indicates a range class for a specific proximity request in 500m |  |
| 1000_METER | Indicates a range class for a specific proximity request in 1000m |  |
| UNUSED | Indicates a range class not used. |  |

6.1.6.3.f	Enumeration: RadioResourcesIndicator

Table 6.1.6.3.f -1: Enumeration RadioResourcesIndicator

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| OPERATOR_PROVIDED | Indicates the operator-provided radio resources for direct communication. |  |
| CONFIGURED | Indicates the configured radio resources for direct communication. |  |

End of changes