3GPP TSG-SA/CT/RAN Meeting #96	SP-220585

Budapest, Hungary, 6 – 10 June 2022

Title:	Summary for 5G Charging for Local breakout roaming of data connectivity

Type:	Work Item Summary

Agenda Item:	21.5 (SA)

Source:	Ericsson AB, Robert Törnkvist

Contact:	robert.tornkvist@ericsson.com

This document is a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary.

Summary based on the input provided by Ericsson in SP-220585.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 950037 | 5G Charging for Local breakout roaming of data connectivity | CHROAM | S5 | SP-220156 | Ericsson |

Introduction

This Work Item enables charging for roaming local breakout scenario, it will also support some scenarios involving non-roaming MVNO (with its own CHF).

Description

To support charging for roaming local breakout the following has been introduced from the SMF in the visited MNO:

-	Collection of charging information for the purpose of wholesale charging towards the home MNO

-	Collection of charging information and conveying it to the home MNO network for the purpose of retail charging

Figure 1: 5G data connectivity converged charging architecture in Local breakout scenario reference point representation

The N40 reference point is defined for the interactions between V-SMF and V-CHF, the N47 reference point is defined for the interactions between V-SMF and H-CHF, specified in TS 32.255 [2]. Wherever the V-SMF will use only N40, or both N40 and N47, depends on agreement and SMF configuration. Both reference points use the NchfConvergedCharging service, specified in TS 32.290 [3] and TS 32.291 [4], however online charging is only possible on the N47 when both N40 and N47 is used for the same chargeable data connectivity session.

References

List of related CRs: select "TSG Status = Approved" in: 
https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=950037

[1]	TS 32.240: “Telecommunication management; Charging management; Charging architecture and principles”

[2]	TS 32.255: “Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2”

[3]	TS 32.290: “Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)”

[4]	TS 32.291: “Telecommunication management; Charging management; 5G system, charging service; Stage 3”