3GPP TSG SA Meeting #96	SP-220642

June 7th – 10th, 2022 ; Budapest, Hungary

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on 5G ProSe Control Plane Security procedures

Document for: 	Agreement

Agenda Item: 	3.5 (Discussion papers on work in Rel-17)

Work Item / Release:	5G_ProSe / Rel-17

Abstract of the contribution: This contribution proposes way forward on the 5G ProSe CP security procedures

# 1 Progress in SA2 #150E and SA2 #151E

SA3 had informed SA2 in LS S2-2200067 of their intention to work on 5G ProSe security solution via providing both Control Plane and User Plane security solutions. An early version of CP security solution was included in TS 33.503 at an early stage.

Proposals were made to complete the SA2 work in SA2 #150E and SA2 #151E, but as stated in LS SP-220371 / S2-2204904, the SA2 CRs failed to get agreed in the meeting. The concerns expressed in SA2 #150E and SA2 #151E included a risk that the CP security solution might not be completed in time for Rel-17 or it might not include secondary authentication procedure.

# 2 How things have changed since SA2 #151E

The originators would like to ask TSG SA to re-consider the CR provided to this meeting in SP-220643, since some important points have changed already since the SA2 meeting week:

- SA3 have sent TS 33.503 to TSG SA #96 for approval in SP-220541

- The new TS 33.503 contains both CP and UP solutions. The new TS 33.503 has addressed the main pending issues as identified in Work Item Exception for 5G_ProSe Security Aspects in SP-220349 (ProSe keys storage, secondary authentication procedure details).

- The slice-based selection of AMF that supports the CP security procedure is updated as follows:

  - The updated proposal is aligned with the latest version of TS 33.503 that is presented for approval

  - The proposed procedure is very light weight and it does not require any additional RSC to S-NSSAI provisioning other than what has been specified already (i.e. the RSC binding of earlier revisions is removed)

  - The procedure for AUSF selection refers to the one that is documented in the lastest version of TS 33.503 (i.e. the proposal to add AUSF CP security capability in NF profile is removed)

Revised version of SA2 CR merging the contents of two CRs (TS 23.304 CR 0102 and 0104) according to the above principles is provided as a company contribution to TSG SA #96 in SP-220643.

# 3. Proposal

It is proposed to keep both CP and UP security solutions as part of 5G ProSe in Rel-17 and to approve the related CR in SP-220643 (TS 23.304 CR 0102r4) that is needed to align the SA2 work with the latest version of TS 33.503 provided by SA3 for approval to this meeting.