3GPP TSG-S4   Meeting #120-e	S4-221114

# Online,  , 17th–26th August 2022	revision of S4-S4-220930

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0026 | rev | 1 | Current version: | 16.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS3] Rel-16 API corrections |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  | 2022-08-19 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-16 |
| Reason for change: |  | Compliance with 3GPP OpenAPI conventions. |  |  |  |  |  |  |  |
| Summary of change: |  | OpenAPI corrections: - Fix camelcasing in Content Hosting Configuration, Policy Template and Service Access Information. - Bump OpenAPI version numbers. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correction to V16.1.0 is not signalled by a diffeent version number. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.3.1, 7.9.3.1, 11.2.3.1, 11.6.2, C.3, C.4. |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-220930 -> S4-221114. |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.6.3.1	ContentHostingConfiguration resource

The data model for the ContentHostingConfiguration resource is specified in table 7.6.3.1-1 below:

Table 7.6.3.1-1: Definition of ContentHostingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| Name | String | 1..1 | A name for this Content Hosting Configuration. |
| ingestConfiguration | Object | 1..1 | Describes the 5GMSd Application Provider's origin server from which media resources will be ingested via interface M2d. |
| path | String | 1..1 | The relative path which will be used to address the media resources at interface M2d. This path is provided by the 5GMSd AF in the case of Push-based ingest. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSd AS shall use Pull or Push for ingesting the content. |
| protocol | Uri | 1..1 | A fully-qualified term identifier allocated in the name space urn:3gpp:5gms:content-protocol that identifies the content ingest protocol. The set of supported protocols is defined in clause 8. |
| entryPoint | Url | 1..1 | An entry point to ingest the content. The semantics of the entry point are dependent on the selected ingest protocol. In the case of Push ingest (pull flag is set to False), this parameter is returned by the 5GMSd AF to the 5GMSd Application Provider and indicates the entry point for pushing the content. In case of Pull (pull flag is set to True), the entryPoint shall be provided to the 5GMSd AF to indicate the location from which content is to be pulled. In this case, the entryPoint shall be used as the base URL. A request received by the 5GMSd AS is mapped to a URL using the provided base URL to fetch the content from the origin server. |
| distributionConfigurations | Array(Object) | 1..1 | Specifies the distribution method and configuration for the ingested content. More than one distribution may be configured for the ingested content, e.g. to offer different distribution configurations such as DASH and HLS. |
| contentPreparationTemplateId | ResourceId | 0..1 | Indicates that content preparation prior to distribution is requested by the 5GMSd Application Provider. It identifies the Content Preparation Template that shall be used as defined in clause 7.4 |
| canonicalDomainName | String | 1..1 | All resources of the current distribution shall be accessible through this default FQDN assigned by the 5GMSd AF. |
| domainNameAlias | String | 1..1 | The 5GMSd Application Provider may assign another FQDN through which media resources are additionally accessible at M4d. This domain name is used by the 5GMSd AS to select an appropriate Server Certificate to present at M4d, and to set appropriate CORS HTTP response headers at M4d. If this property is present, the 5GMSd Application Provider is responsible for providing in the DNS a CNAME record that resolves domainNameAlias to canonicalDomainName. |
| pathRewriteRules | Array(Object) | 0..1 | An ordered list of rules for rewriting the request URL paths of media resource requests handled by the 5GMSd AS. If multiple rules match a particular resource's path, only the first matching rule, in order of appearance in this array, shall be applied. |
| requestPathPattern | String | 1..1 | A regular expression [5] against which the path part of each 5GMSd AS request URL, including the leading "/", and up to and including the final "/", shall be compared. (Any leaf path element following the final "/" shall be excluded from this comparison.) In the case of Pull-based ingest, the M4d download request path is used in the comparison. In the case of Push-based ingest, the M2d upload request path is used in the comparison. In either case, if the request path matches this pattern, the path mapping specified in the corresponding mappedPath shall be applied. |
| mappedPath | String | 1..1 | A replacement for the portion of the 5GMSd AS request path that matches requestPathPattern. In the case of Pull-based ingest, ingestConfiguration.entryPoint is concatenated with the mapped path and any leaf path element from the original M4d download request to form the M2d origin request URL. In the case of Push-based ingest, canonicalDomainName (and, optionally, domainNameAlias) are concatenated with the mapped path and any leaf path element from the original M2d upload request to form the distribution URL(s) exposed over M4d. |
| cachingConfigurations | Array(Object) | 0..1 | Defines a configuration of the 5GMSd AS cache for a matching subset of media resources ingested in relation to this Content Hosting Configuration. |
| urlPatternFilter | String | 1..1 | A pattern that will be used to match media resource URLs to determine whether a given media resource is eligible for caching by the 5GMSd AS. The format of the pattern shall be a regular expression as specified in [5]. |
| cachingDirectives | Object | 1..1 | If a urlPatternFilter applies to a resource, then the provided cachingDirectives shall be applied by the 5GMSd AS at M4d, potentially overwriting any origin caching directives ingested at M2d. |
| statusCodeFilters | Array(Integer) | 0..1 | The set of HTTP origin response status codes to which these cachingDirectives apply. The filter shall be provided as a regular expression as specified in [5]. If the list is empty, the cachingDirectives shall apply to all HTTP origin response status codes at M2d. |
| noCache | Boolean | 1..1 | If set to True, this indicates that the media resources matching the filters shall not be cached by the 5GMSd AS and shall be marked as not to be cached when served by the 5GMSd AS at M4d. |
| maxAge | Integer | 0..1 | The caching time-to-live period that shall be set on ingested media resources matching the filters. This determines the minimum period for which the 5GMSd AS shall cache matching media resources as well as the time-to-live period signalled by the 5GMSd AS at interface M4d when it serves such media resources. The time-to-live for a given media resource shall be calculated relative to the time it was ingested. |
| geoFencing | Object | 0..N | Limit access to the content to the indicated geographic areas. |
| locatorType | Uri | 1..1 | The type of the locators shall be indicated using a fully-qualified term identifier URI from the controlled vocabulary urn:3gpp:5gms:‌locator-type, as specified in clause 7.6.4.6, or else from a vendor-specific vocabulary. |
| locators | Array(String) | 1..1 | Array of locators from which access to the resources is to be allowed. The format of the locator strings shall be determined by the value of locatorType, as specified in clause 7.6.4.6. |
| urlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content resource at M4d. |
| urlPattern | String | 1..1 | A pattern that shall be used to match M4d media resource URLs. The 5GMSd AS shall not serve a matching media resource at M4d unless it includes a valid authentication token. The format of the pattern shall be a regular expression as specified in [5]. |
| tokenName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the authentication token when required to do so. |
| passphraseName | String | 1..1 | The name of the query parameter that is used to refer to the passphrase when constructing the authentication token. Note that the token is not included in the cleartext part of the M4d URL query component. |
| passphrase | String | 1..1 | The shared secret between the 5GMSd Application Provider and the 5GMSd AS for this distributionConfiguration. The passphrase is used in the computation and verification of the M4d authentication token but is never sent in-the-clear over that interface. |
| tokenExpiryName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the token expiry field. |
| useIPAddress | Boolean | 1..1 | If set to True, the IP address of the UE is included in the computation of the authentication token for resources that match urlPattern and access to matching media resources shall be allowed by the 5GMSd AF only when the M4d request is made from a UE with this IP address. |
| ipAddressName | String | 0..1 | The name of the M4d request query parameter that is encoded as part of the authentication token if the useIPAddress flag is set to True. Note that the IP address is not passed in the cleartext part of the M4d URL query component. |
| certificateId | ResourceId | 0..1 | When content is distributed using TLS [16], the X.509 [8] certificate for the origin domain is shared with the 5GMSd AF so that it can be presented by the 5GMSd AS in the TLS handshake at M4d. This attribute indicates the identifier of the certificate to use. |

NEXT CHANGE

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Usage | Visibility | Description |
| --- | --- | --- | --- | --- | --- |
| policyTemplateId | ResourceId | 1..1 | C: RO R: RO U: RO |  | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 | C: RO R: RO U: RO |  | A Policy Template may be in the PENDING, INVALID, READY, or SUSPENDED state. Only a Policy Template in the READY state may be instantiated as a Dynamic Policy Instance and applied to media streaming sessions. |
| apiEndPoint | String | 1..1 | C: RW R: RO U: RW | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | C: RW R: RO U: RW | MNO Admin | N5: Npcf_PolicyAuthorization Service. N33: AsSessionWithQoS or CHargableParty. |
| externalReference | String | 1..1 | C: RW R: RO U: RW |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the media streaming session. |
| qoSSpecification | M1QoSSpecification | 0..1 | C: RW R: RO U: RW |  | Specifies the network quality of service to be applied to media streaming sessions at this Policy Template. |
| applicationSession‌Context | Object | 1..1 |  |  | Specifies information about the application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 | C: RW R: RW U: RW |  | As defined in clause 5.6.2.3 of TS 29.514 [34] and clause 5.3.2 of TS 29.571 [12]. |
| sliceInfo | Snssai | 0..1 | C: RW R: RW U: RW |  |  |
| dnn | Dnn | 0..1 | C: RW R: RW U: RW |  |  |
| aspId | AspId | 1..1 | C: RW R: RW U: RW |  |  |
| chargingSpecification | ChargingSpecification | 0..1 | C: RW R: RW U: RW |  | Provides information about the charging policy to be used for this Policy Template. |

NEXT CHANGE

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformation resource is specified in table 11.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session from which the Service Access Information is derived (as indicated in the provisioningSessionType property) and this is specified in the Applicability column.

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | RO | Unique identification of the M1 Provisioning Session. | All types |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | RO | The type of Provisioning Session. | All types. |
| streamingAccess | Object | 0..1 | RO |  | downlink |
| mediaPlayerEntry | Url | 0..1 | RO | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |  |
| clientConsumptionReporting‌Configuration | Object | 0..1 | RO |  | downlink |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the media streaming session has ended. |  |
| serverAddresses | Array(Url) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| locationReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |  |
| samplePercentage | Percentage | 1..1 | RO | The percentage of media streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |  |
| dynamicPolicyInvocation‌Configuration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddresses | Array(Url) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| scheme | Uri | 1..1 | RO | The metrics reporting scheme that metrics reports shall use (see clause 4.7.5). |  |
| validPolicyTemplateIds | Array(ResourceId) | 1..1 | RO | A list of Policy Template identifiers which the 5GMS Client is authorized to use. |  |
| sdfMethods | Array(SdfMethod) | 1..1 | RO | A list of recommended service data flow description methods (descriptors), e.g. 5-Tuple, ToS, 2-Tuple, etc., which should be used by the Media Session Handler to describe the service data flows for the traffic to be policed. |  |
| externalReferences | Array(String) | 0..1 | RO | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the media streaming session. Example: "HD_Premium". |  |
| clientMetricsReporting‌Configurations | Array(Object) | 0..1 | RO |  | downlink, uplink |
| serverAddresses | Array(Url) | 1..1 | RO | A list of 5GMS AF addresses to which metrics reports shall be sent. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| scheme | Uri | 1..1 | RO | The metrics reporting scheme that metrics reports shall use (see clause 4.7.5). |  |
| dataNetworkName | Dnn | 0..1 | RO | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |  |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the media streaming session has ended. |  |
| samplePercentage | Percentage | 1..1 | RO | The percentage of media streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |  |
| urlFilters | Array(String) | 0..1 | RO | A non-empty list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |  |
| metrics | Array(String) | 1..1 | RO | A list of metrics which shall be reported. |  |
| networkAssistanceConfiguration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddress | Url | 1..1 | RO | Address of the 5GMS AF that offers the APIs for 5GMS AF-based Network Assistance, for access by the 5GMSd Media Session Handler. See NOTE. This address shall be an opaque URL, following the 5GMS URL format. |  |
| NOTE: In deployments where multiple instances of the 5GMSd AF expose the Media Session Handling APIs at M5, the 5G System may use a suitable mechanism (e.g. HTTP load balancing or DNS resolution) to direct requests to a suitable AF instance. |  |  |  |  |  |

NEXT CHANGE

### 11.6.2	Resource structure

The Network Assistance API is accessible via the following URL base path:

{apiRoot}/3gpp-m5/v1/network-assistance/

Table 11.6.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the URL base path.

Table 11.6.2-1: Operations supported by the Network Assistance API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Network Assistance Session resource |  | POST | Provision a new Network Assistance Session. If the operation succeeds, the URL of the created Network Assistance Session resource shall be returned in the Location header of the response. |
| Fetch a Network Assistance Session resource | {naSessionId} | GET | Fetch the properties of an existing Network Assistance Session. |
| Update a Network Assistance Session resource | {naSessionId} | PUT, PATCH | Update the properties of an existing Network Assistance Session. |
| Request a bit rate recommendation | {naSessionId}/recommendation | GET | Obtain a bit rate recommendation for the next recommendation window. |
| Request a delivery boost | {naSessionId}/boost-request | POST | Request a delivery boost for the next recommendation window. |
| Terminate Network Assistance Session | {naSessionId} | DELETE | Terminate a Network Assistance session. |

NEXT CHANGE

# C.3	OpenAPI representation of the M1 APIs

## C.3.1	M1_ProvisioningSessions API

openapi: 3.0.0 info: title: M1_ProvisioningSessions version: 1.1.0 description: ¦ 5GMS AF M1 Provisioning Sessions API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ProvisioningSessions description: '5G Media Streaming: Provisioning (M1) APIs: Provisioning Sessions' externalDocs: description: 'TS 26.512 V16.7.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions: post: operationId: createProvisioningSession summary: 'Create a new Provisioning Session' responses: '201': description: 'Provisioning Session Created' headers: Location: description: 'URL including the resource identifier of the newly created Provisioning Session.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' /provisioning-sessions/{provisioningSessionId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' get: operationId: getProvisioningSessionById summary: 'Retrieve an existing Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' delete: operationId: destroyProvisioningSession summary: 'Destroy an existing Provisioning Session' responses: '204': description: 'Provisioning Session Destroyed' # No Content components: schemas: ProvisioningSession: type: object description: "A representation of a Provisioning Session." required: - provisioningSessionId - provisioningSessionType properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' serverCertificateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 contentPreparationTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 metricsReportingConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 policyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1

## C.3.2	M1_ServerCertificatesProvisioning API

(NO CHANGES IN THIS CLAUSE)

## C.3.3	M1_ContentPreparationTemplatesProvisioning API

(NO CHANGES IN THIS CLAUSE)

## C.3.4	M1_ContentProtocolsDiscovery API

openapi: 3.0.0 info: title: M1_ContentProtocolsDiscovery version: 1.0.1 description: ¦ 5GMS AF M1 Content Protocols Discovery API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentDiscovery description: '5G Media Streaming: Provisioning (M1) APIs: Content Protocols Discovery' externalDocs: description: 'TS 26.512 V16.7.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/protocols: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' get: operationId: retrieveContentProtocols summary: 'Retrieve the set of Content Protocols supported by the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentProtocols' components: schemas: ContentProtocolDescriptor: type: object description: "A descriptor describing a content protocol." required: - termIdentifier properties: termIdentifier: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' descriptionLocator: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' ContentProtocols: type: object description: "A representation of the Content Protocols resource." properties: downlinkIngestProtocols: type: array items: $ref: '#/components/schemas/ContentProtocolDescriptor' minItems: 1 uplinkEgestProtocols: type: array items: $ref: '#/components/schemas/ContentProtocolDescriptor' minItems: 1 geoFencingLocatorTypes: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1

## C.3.5	M1_ContentHostingProvisioning API

openapi: 3.0.0 info: title: M1_ContentHostingProvisioning version: 1.1.0 description: ¦ 5GMS AF M1 Content Hosting Provisioning API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentHostingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Hosting Provisioning' externalDocs: description: 'TS 26.512 V16.7.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentHostingConfiguration summary: 'Create (and optionally upload) the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '201': description: 'Content Hosting Configuration Created' headers: Location: description: 'URL of the newly created Content Hosting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' get: operationId: retrieveContentHostingConfiguration summary: 'Retrieve the Content Hosting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' put: operationId: updateContentHostingConfiguration summary: 'Update the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '204': description: 'Updated Content Hosting Configuration' '404': description: 'Not Found' patch: operationId: patchContentHostingConfiguration summary: 'Patch the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' delete: operationId: destroyContentHostingConfiguration summary: 'Destroy the current Content Hosting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Hosting Configuration' '404': description: 'Not Found' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/purge: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: A unique identifier of the Provisioning post: operationId: purgeContentHostingCache summary: 'Purge the content of the cache for the Content Hosting Configuration of the specified Provisioning Session' requestBody: description: 'The regular expression pattern for resources to purge from the cache' required: true content: application/x-www-form-urlencoded: schema: properties: pattern: description: 'Keyword' type: string value: description: 'The regular expression' type: string responses: '200': description: 'Content Purged' components: schemas: IngestConfiguration: type: object description: 'A configuration for content ingest.' properties: path: type: string pull: type: boolean protocol: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' entryPoint: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' PathRewriteRule: type: object description: 'A rule to manipulate URL paths.' required: - requestPattern - mappedPath properties: requestPattern: type: string mappedPath: type: string CachingConfiguration: type: object description: 'A content caching configuration.' properties: urlPatternFilter: type: string cachingDirectives: type: object required: - urlPatternFilter - noCache properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 DistributionConfiguration: type: object description: 'A content distribution configuration.' required: - canonicalDomainName - domainNameAlias properties: contentPreparationTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' canonicalDomainName: type: string domainNameAlias: type: string pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cachingConfigurations: type: array items: $ref: '#/components/schemas/CachingConfiguration' geoFencing: type: object required: - locatorType - locators properties: locatorType: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' locators: type: array items: type: string description: 'Format of individual locators depends on the locatorType.' minItems: 1 urlSignature: type: object required: - urlPattern - tokenName - passphraseName - passphrase - tokenExpiryName - useIPAddress properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' # Schema for the resource itself ContentHostingConfiguration: type: object description: "A representation of a Content Hosting Configuration resource." required: - name - ingestConfiguration - distributionConfigurations properties: name: type: string ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfigurations: type: array items: $ref: '#/components/schemas/DistributionConfiguration'

## C.3.6	M1_ConsumptionReportingProvisioning API

openapi: 3.0.0 info: title: M1_ConsumptionReportingProvisioning version: 1.0.1 description: ¦ 5GMS AF M1 Consumption Reporting Provisioning API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ConsumptionReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Consumption Reporting Provisioning' externalDocs: description: 'TS 26.512 V16.2.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/consumption-reporting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateConsumptionReporting summary: 'Activate the consumption reporting procedure for the specified Provisioning Session by providing the Consumption Reporting Configuration' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '201': description: 'Consumption Reporting Configuration Created' headers: Location: description: 'URL of the newly created Consumption Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' get: operationId: retrieveConsumptionReportingConfiguration summary: 'Retrieve the Consumption Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' put: operationId: updateConsumptionReportingConfiguration summary: 'Update the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '204': description: 'Updated Consumption Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchConsumptionReportingConfiguration summary: 'Patch the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '200': description: 'Patched Consumption Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyConsumptionReportingConfiguration summary: 'Destroy the current Consumption Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Consumption Reporting Configuration' '404': description: 'Not Found' components: schemas: ConsumptionReportingConfiguration: type: object description: "A representation of a Consumption Reporting Configuration resource." properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' locationReporting: type: boolean

## C.3.7	M1_MetricsReportingProvisioning API

openapi: 3.0.0 info: title: M1_MetricsReportingProvisioning version: 1.0.1 description: ¦ 5GMS AF M1 Metrics Reporting Provisioning API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_MetricsReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Metrics Reporting Provisioning' externalDocs: description: 'TS 26.512 V16.7.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateMetricsReporting summary: 'Activate the Metrics reporting procedure for the specified Provisioning Session by providing the Metrics Reporting Configuration' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '201': description: 'Metrics Reporting Configuration Created' headers: Location: description: 'URL of the newly created Metrics Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations/{metricsReportingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: metricsReportingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of a Metrics Reporting Configuration.' get: operationId: retrieveMetricsReportingConfiguration summary: 'Retrieve the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' put: operationId: updateMetricsReportingConfiguration summary: 'Update the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '204': description: 'Updated Metrics Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchMetricsReportingConfiguration summary: 'Patch the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '200': description: 'Patched Metrics Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyMetricsReportingConfiguration summary: 'Destroy the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Metrics Reporting Configuration' '404': description: 'Not Found' components: schemas: MetricsReportingConfiguration: type: object description: "A representation of a Metrics Reporting Configuration resource." required: - metricsReportingConfigurationId - scheme properties: metricsReportingConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 1 metrics: type: array items: type: string minItems: 1

## C.3.8	M1_PolicyTemplatesProvisioning API

openapi: 3.0.0 info: title: M1_PolicyTemplatesProvisioning version: 1.1.0 description: ¦ 5GMS AF M1 Policy Templates Provisioning API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_PolicyTemplatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Policy Templates Provisioning' externalDocs: description: 'TS 26.512 V16.7.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/policy-templates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createPolicyTemplate summary: 'Create (and optionally upload) a new Policy Template' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '201': description: 'Policy Template Created' headers: Location: description: 'URL of the newly created Policy Template resource.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/policy-templates/{policyTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A unique identifier of the Provisioning Session.' - name: policyTemplateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A resource identifier of a Policy Template.' get: operationId: retrievePolicyTemplate summary: 'Retrieve a representation of an existing Policy Template in the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' put: operationId: updatePolicyTemplate summary: 'Update a Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '204': description: 'Updated Policy Template' '404': description: 'Not Found' patch: operationId: patchPolicyTemplate summary: 'Patch the Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' application/json-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' delete: operationId: destroyPolicyTemplate responses: '204': description: 'Destroyed Policy Template' '404': description: 'Not Found' components: schemas: PolicyTemplate: type: object description: "A representation of a Policy Template resource." required: - policyTemplateId - state - apiEndPoint - apiType - externalReference - applicationSessionContext properties: policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' state: anyOf: - type: string enum: [PENDING, INVALID, READY, SUSPENDED] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. apiEndPoint: type: string apiType: anyOf: - type: string enum: [N5, N33] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. externalReference: type: string qoSSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M1QoSSpecification' applicationSessionContext: type: object properties: afAppId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId' sliceInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai' dnn: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' chargingSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/ChargingSpecification'

NEXT CHANGE

# C.4	OpenAPI representation of the M5 APIs

## C.4.1	M5_ServiceAccessInformation API

openapi: 3.0.0 info: title: M5_ServiceAccessInformation version: 1.1.0 description: ¦ 5GMS AF M5 Service Access Information API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ServiceAccessInformation description: '5G Media Streaming: Media Session Handling (M5) APIs: Service Access Information' externalDocs: description: 'TS 26.512 V16.7.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /service-access-information/{provisioningSessionId}: parameters: - name: provisioningSessionId description: 'The resource identifier of an existing Provisioning Session.' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveServiceAccessInformation summary: 'Retrieve the Service Access Information resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ServiceAccessInformationResource' '404': description: 'Not Found' components: schemas: ServerAddresses: description: "A set of application endpoint addresses." type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' minItems: 1 ServiceAccessInformationResource: description: "A representation of a Service Access Information resource." type: object required: - provisioningSessionId - provisioningSessionType properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' streamingAccess: type: object properties: mediaPlayerEntry: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' clientConsumptionReportingConfiguration: type: object required: - serverAddresses - locationReporting - samplePercentage properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' serverAddresses: $ref: '#/components/schemas/ServerAddresses' locationReporting: type: boolean samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' dynamicPolicyInvocationConfiguration: type: object required: - serverAddresses - validPolicyTemplateIds - sdfMethods properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' validPolicyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 0 sdfMethods: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/SdfMethod' minItems: 0 externalReferences: type: array items: type: string minItems: 1 clientMetricsReportingConfiguration: type: array items: type: object required: - serverAddresses - scheme - samplePercentage - urlFilters - metrics properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 0 metrics: type: array items: type: string networkAssistanceConfiguration: type: object required: - serverAddress properties: serverAddress: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url'

## C.4.2	M5_ConsumptionReporting API

openapi: 3.0.0 info: title: M5_ConsumptionReporting version: 1.0.0 description: ¦ 5GMS AF M5 Consumption Reporting API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ConsumptionReporting description: '5G Media Streaming: Media Session Handling (M5) APIs: Consumption Reporting' externalDocs: description: 'TS 26.512 V16.2.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /consumption-reporting/{aspId}: parameters: - name: aspId in: path required: true schema: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' description: 'See 3GPP TS 26.512 clause 11.3.2.' post: operationId: submitConsumptionReport summary: 'Submit a Consumption Report' requestBody: description: 'A Consumption Report' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReport' responses: '204': description: 'Consumption Report Accepted' '400': description: 'Bad Request' '415': description: 'Unsupported Media Type' components: schemas: ConsumptionReport: description: "A representation of a Consumption Report." type: object required: - mediaPlayerEntry - reportingClientId - consumptionReportingUnits properties: mediaPlayerEntry: type: string reportingClientId: type: string consumptionReportingUnits: type: array items: $ref: '#/components/schemas/ConsumptionReportingUnit' ConsumptionReportingUnit: description: "A Consumption Reporting Unit." type: object required: - mediaConsumed - startTime - duration properties: mediaConsumed: type: string startTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' duration: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' locations: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/TypedLocation' minItems: 1

## C.4.3	M5_MetricsReporting API

(NO CHANGES IN THIS CLAUSE.)

## C.4.4	M5_DynamicPolicies API

openapi: 3.0.0 info: title: M5_DynamicPolicies version: 1.1.0 description: ¦ 5GMS AF M5 Dynamic Policy API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_DynamicPolicies description: '5G Media Streaming: Media Session Handling (M5) APIs: Dynamic Policies' externalDocs: description: 'TS 26.512 V16.3.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /dynamic-policies: post: operationId: createDynamicPolicy summary: 'Create (and optionally upload) a new Dynamic Policy resource' requestBody: description: 'An optional JSON representation of a Dynamic Policy resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '201': description: 'Created Dynamic Policy Resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' headers: Location: description: 'The URL of the newly created Dynamic Policy resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' '400': description: 'Bad Request' '401': description: 'Unauthorized' /dynamic-policies/{dynamicPolicyId}: parameters: - name: dynamicPolicyId description: 'The resource identifier of a Dynamic Policy resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveDynamicPolicy summary: 'Retrieve an existing Dynamic Policy resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateDynamicPolicy summary: 'Update an existing Dynamic Policy resource' requestBody: description: 'A replacement JSON representation of a Dynamic Policy resource' required: true content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchDynamicPolicy summary: 'Patch an existing Dynamic Policy resource' requestBody: description: 'A JSON patch to a Dynamic Policy resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' application/json-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '200': description: 'Patched Dynamic Policy' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '204': description: 'Patched Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' delete: operationId: destroyDynamicPolicy summary: 'Destroy an existing Dynamic Policy resource' responses: '204': description: 'Destroyed Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: DynamicPolicy: description: "A representation of a Dynamic Policy resource." type: object required: - dynamicPolicyId - policyTemplateId - serviceDataFlowDescriptions - provisioningSessionId properties: dynamicPolicyId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescriptions: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' qosSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' enforcementMethod: type: string enforcementBitRate: type: integer

## C.4.5	M5_NetworkAssistance API

openapi: 3.0.0 info: title: M5_NetworkAssistance version: 1.0.1 description: ¦ 5GMS AF M5 Network Assistance API © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_NetworkAssistance description: '5G Media Streaming: Media Session Handling (M5) APIs: Network Assistance' externalDocs: description: 3GPP TS 26.512 V16.7.0; 5G Media Streaming (5GMS); Protocols. url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v1' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. paths: /network-assistance/: post: operationId: createNetworkAssistanceSession summary: 'Create a new Network Assistance Session.' responses: '201': description: 'Created Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' headers: Location: description: 'The URL of the nely created Network Assistance Session resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' '400': description: 'Bad Request' '401': description: 'Unauthorized' /network-assistance/{naSessionId}: parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveNetworkAssistanceSession summary: 'Retrieve an existing Network Assistance Session resource' responses: '200': description: 'A JSON representation of a Network Assistance Session resource' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateNetworkAssistanceSession summary: 'Update an existing Network Assistance Session resource' requestBody: description: 'A replacement JSON representation of a Network Assistance Session resource' required: true content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchNetworkAssistanceSession summary: 'Patch an existing Network Assistance Session resource' requestBody: description: 'A JSON patch to a Network Assistance Session resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' application/json-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '200': description: 'Patched Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '204': description: 'Patched Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' delete: operationId: destroyNetworkAssistanceSession summary: 'Destroy an existing Network Assistance Session resource' responses: '204': description: 'Destroyed Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/recommendation: get: operationId: requestBitRateRecommendation summary: 'Obtain a bit rate recommendation for the next recommendation window' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Success' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/boost-request: post: operationId: requestDeliveryBoost summary: 'Request a delivery boost' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Delivery Boost Request Processed' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/OperationSuccessResponse' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: NetworkAssistanceSession: description: "A representation of a Network Assistance Session resource." type: object required: - naSessionId properties: naSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescription: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' minItems: 1 policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' requestedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' recommendedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' notficationURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url'

END OF CHANGES