| 3GPP TR 23.700-99 V2.0.0 (2022-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Network Slice Capability Exposure for Application Layer Enablement (NSCALE) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Application architecture for network slice capability enablement	10

4.1	Architectural requirements	10

4.1.1	General requirements	10

4.1.2	Security requirements	10

4.1.3	Registration requirement	10

4.1.4	Discovery requirement	10

4.1.5	Lifecycle management requirement	10

4.2	Application architecture	11

4.2.1	General	11

4.2.2	Architecture	11

4.2.3	Functional elements	13

4.2.3.1	Network slice Capability Enablement client	13

4.2.3.2	Network slice Capability Enablement server	13

4.2.4	Service-based interfaces	13

4.2.5	Reference points description	13

4.2.5.1	VAL-UU	13

4.2.5.2	NSCE-UU	14

4.2.5.3	NSCE-C	14

4.2.5.4	NSCE-S	14

4.2.5.5	NSCE-E	14

4.3	Cardinality rules	14

4.3.1	General	14

4.3.2	Functional Entity Cardinality	14

4.3.2.1	VAL Client	14

4.3.2.2	NSCE Client	14

4.3.2.3	NSCE Server	14

4.3.2.6	VAL Server	14

4.3.3	Service Cardinality	15

5	Key issues	15

5.1	Key issue 1: Network slice capability management enhancements	15

5.2	Key issue 2: Application layer exposed network slice lifecycle management	15

5.3	Key issue 3: Discovery & registration aspects for management service exposure	16

5.4	Key issue 4: Network slice fault management capability	16

5.5	Key issue 5: Communication service management exposure	17

5.6	Key issue 6: Application layer QoS verification capability enablement	18

5.7	Key issue 7: Network slice related performance and analytics exposure	18

5.8	Key issue 8: Support for requirements translation	19

5.9	Key issue 9: Support for trust enablement	19

5.10	Key Issue 10: Support for managing trusted third-party owned application(s)	20

5.11	Key issue 11: Slice requirement alignment	20

5.12	Key issue 12: Network slice capability exposure in the edge data network	21

5.13	Key issue 13: Delivery of the existing Network Slice information to the trusted third-party	21

5.14	Key issue 14: Network Slice creation to the third-party and UE	22

6	Solutions	23

6.0	Mapping of Solutions to Key Issues	23

6.1	Solution 1: Automatic application layer network slice management	24

6.1.1	Solution description	24

6.1.1.1	General	24

6.1.1.2	Automatic application layer network slice lifecycle management	25

6.1.2	Solution evaluation	26

6.2	Solution 2: Network slice fault management capability	26

6.2.1	Solution description	26

6.2.1.1	General	26

6.2.1.2	Network slice fault management capability exposure	27

6.2.2	Solution evaluation	28

6.3	Solution 3: Slice API configuration and translation	28

6.3.1	Solution description	28

6.3.1.1	General	28

6.3.1.2	slice API configuration	29

6.3.1.3	Slice API translation	30

6.3.2	Solution evaluation	31

6.4	Solution 4: QoS verification capability	32

6.4.1	Solution description	32

6.4.1.1	General	32

6.4.1.2	QoS verification capability	32

6.4.2	Solution evaluation	33

6.5	Solution 5: Network slice related performance and analytics exposure	33

6.5.1	Solution description	33

6.5.1.1	General	33

6.5.1.2	Network slice related performance and analytics exposure	33

6.5.1.2.1	KQI and performance data report from NSCE client	35

6.5.2 	Solution evaluation	36

6.6	Solution 6: VAL server authorization and authentication via slice enabler layer	37

6.6.1	Solution description	37

6.6.1.1	General	37

6.6.1.2	VAL server authorization and authentication via slice enabler layer	37

6.6.2	Solution evaluation	38

6.7	Solution 7:  network slice capability registration	38

6.7.1	Solution description	38

6.7.1.1	 General	38

6.7.1.2 	Capability exposure registration	38

6.7.2	Solution evaluation	39

6.8	Solution 8: Discovery of management service exposure	39

6.8.1	Solution description	39

6.8.1.1	General	39

6.8.1.2	Procedure	39

6.8.2	Solution evaluation	41

6.9	Solution 9: Support for managing trusted third-party owned application(s)	41

6.9.1	Solution description	41

6.9.1.1	General	41

6.9.1.2	Network slice quota management capability exposure	41

6.9.2	Solution evaluation	42

6.10	Solution 10: Network slice application policy management capability	42

6.10.1	Solution description	42

6.10.1.1	General	42

6.10.1.2	Network slice application policy management capability	42

6.10.2	Solution evaluation	44

6.11	Solution 11: Communication service management exposure	44

6.11.1	Solution description	44

6.11.1.1	General	44

6.11.1.2	Use Case of Communication service lifecycle management	44

6.11.1.3	Communication service creation	47

6.11.1.4	Communication service modification	47

6.11.1.5	Communication service disengagement	48

6.11.2	Solution evaluation	48

6.12	Solution 12: SEAL enhancement	48

6.12.1	Solution description	48

6.12.2	Solution evaluation	49

6.13	Solution 13: Network Slice Allocation by VAL server	50

6.13.1	Solution description	50

6.13.1.1	General	50

6.13.1.2	Network Slice Allocation by VAL Server	50

6.13.2	Solution evaluation	52

6.14	Solution 14: Interaction between the NSCE servers	52

6.14.1	Solution description	52

6.14.2	Information collection from multiple NSCE servers	52

6.14.3	Solution evaluation	52

6.15	Solution 15: UE triggered network slice adaptation	53

6.15.1	Solution description	53

6.15.1.1	General	53

6.15.2	Solution evaluation	57

6.16	Solution 16: Multi-Network slice management capability	57

6.16.1	Solution description	57

6.16.1.1	General	57

6.16.1.2	Multi-Network slice management capability	57

6.16.2	Solution evaluation	59

6.17	Solution 17: Multi-Network slice resource optimization	59

6.17.1	Solution description	59

6.17.1.1	General	59

6.17.1.2	Multi-Network slice management capability	59

6.17.2	Solution evaluation	61

6.18	Solution 18: Network Slice Information Delivery	61

6.18.1	Solution description	61

6.18.1.1	General	61

6.18.1.2	Network Slice Information delivery	62

6.18.1.3	Network Slice Information subscription	63

6.18.1.4	Network Slice Information Notify	64

6.18.1.5	Network Slice Information delivery while registration	65

6.18.2	Solution evaluation	66

6.19	Solution 19: Slice requirements alignment capability	67

6.19.1	Solution description	67

6.19.1.1	General	67

6.19.1.2	Slice requirements alignment capability	67

6.19.2	Solution evaluation	68

6.20	Solution 20: Network slice optimization based on AF policy	68

6.20.1	Solution description	68

6.20.1.1	General	68

6.20.1.2	 OAM event triggered network slice optimization	69

6.20.1.3	 NWDAF event triggered network slice optimization	70

6.20.1.4	 NSCE server triggered network slice optimization	71

6.20.1.5	 AF policy provisioning	72

6.20.2	Solution evaluation	73

6.21	Solution 21: Solution on predictive slice modification in edge based NSCE deployments	73

6.21.1	Solution description	73

6.21.1.1	General	73

6.21.1.1	Procedure	74

6.21.2	Solution evaluation	75

6.22	Solution #22: Slice policy and configuration alignment	75

6.22.1	Solution description	75

6.22.1.1	General	75

6.22.1.2	 Procedure	75

6.22.2	Solution evaluation	76

7	Identities and commonly used values	77

7.1	General	77

7.2	VAL server ID	77

7.3	NSCE server ID	77

7.4	NSCE client ID	77

7.5	VAL client ID	77

7.6	UE ID	77

7.7	slice coverage area	77

7.8	NSCE service area	77

8	Overall evaluation	78

8.1	General	78

8.2	Solution evaluations	78

8.2.1	General	78

8.2.2	Overall Evaluation for KI#1	79

8.2.3	Overall Evaluation for KI#10	79

8.2.4	Overall Evaluation for KI#12	79

8.3	Architecture evaluations	80

9	Conclusions	80

Annex A	Deployment models	83

A.1	Deployment scenarios	83

A.1.1	General	83

A.1.2	Centralized NSCE deployment	83

A.1.3	Distributed NSCE deployment	83

A.1.3.1	NPN NSCE deployment	84

A.1.3.2	Edge NSCE deployment	84

A.2	Deployment of NSCE server(s) in relation to VAL server and 3GPP system	85

Annex B (informative): Business models and relationships for NSCALE	86

Annex B.1: Relevance to SA5 models	86

Annex B.2: Business relationships	88

Annex C:Change history	90

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The study identifies the SA1 requirements and potential vertical requirements of network slice capability exposure that have yet to be realized, and proposes application architecture aspects solutions and enhancements to SEAL. Based on any gaps found between requirements and existing capabilities, the present document includes: identification of key issues, solutions, corresponding evaluations and conclusions to ensure the efficient network slice capability exposure.

The study takes into consideration the SEAL architecture specified in 3GPP TS 23.434 [2],  network slicing related architecture and services(such as services of NEF) specified in 3GPP TS 23.501 [3] and 3GPP TS 23.502 [4], network slicing management capability provisioning and architecture specified in 3GPP TS 28.531 [5] and 3GPP TS 28.533 [6], requirements specified in TS 22.261 [7].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[6]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9]	3GPP TR 22.835: "Study on enhanced access to and support of network slices".

[10]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS) ".

[11]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[14]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[15]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[16]	3GPP TS 32.111-1: "Management and orchestration; Fault management, Part 1: 3G fault management requirements".

[17]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[18]	3GPP TS 22.263: "Services and System Aspects; Service requirements for video, imaging and audio for professional applications (VIAPA)".

[19]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[20]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[21]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[22]		3GPP TS 29.536: "5G System; Network Slice Admission Control Service; Stage 3".

[23]	3GPP TS 32.404: "Performance Management (PM); Performance measurements; Definitions and template".

[24]	3GPP TS 28.554: "Management and orchestration ; 5G end to end Key Performance Indicators (KPI)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

ASP	Application Service Provider

Auto-NS-LCM	Automatic Application Layer Network Slice Lifecycle Management

EDN	Edge Data Network

EGMF	Exposure Governance Management Function

GST	Generic Network Slice Template

KPI	Key Performance Indicator

KQI	Key Quality Indicator

MnS	Management Service

NEF	Network Exposure Function

NEST	Network Slice Template

NOP	Network Operator

NSCE	Network Slice Capability Enablement

NSaaS	Network Slice as a Service

NSI	Network Slice Instance

NSSI	Network Slice Subnet Instance

NWDAF	Network Data Analytics Function

NSACF	Network Slice Admission Control Function

NSC	Network Slice consumer

NSP	Network Slice Provider

NSMF	Network Slice Management Function

OAM	Operation, administration and maintenance

QoE	Quality of Experience

S-NSSAI	Single Network Slice Selection Assistance Information

VAL	Vertical Application Layer

# 4	Application architecture for network slice capability enablement

## 4.1	Architectural requirements

### 4.1.1	General requirements

[AR.4.1.1-a]	The application enablement layer shall support interaction with 3GPP network management system to consume network slice management service.

NOTE: The consuming of the network slice management service related procedures are specified in TS 28.531[5].

NOTE: The Network Slice Capability Enablement (NSCE) layer acts a service consumer utilizing the management services exposed by EGMF as defined in SA5 TS 28.533[6] if authorized.

[AR-4.1.1-b] The NSCE architecture shall support one or more applications from the same vertical.

[AR-4.1.1-c] The NSCE client shall be able to communicate to multiple NSCE servers.

[AR-4.1.1-d] The API interactions between the vertical application server(s) and NSCE server(s) shall conform to CAPIF as specified in 3GPP TS 23.222 [20].

[AR-4.1.1-e] The NSCE server(s) shall provide a service API compliant with CAPIF as specified in 3GPP TS 23.222 [20].

### 4.1.2	Security requirements

[AR-4.1.2-a] The application architecture shall provide mechanisms to authorize the usage of network slicing related services by the VAL servers and NSCE clients.

[AR-4.1.2-b] The application architecture shall support mutual authentication and authorization check between clients and servers, servers and servers that interact.

NOTE: The authentication and authorization aspects related to VAL servers and NSCE enablers are out scope of this study and to be addressed by SA3, in TS 33.434[21].

### 4.1.3	Registration requirement

[AR.4.1.3-a] The application layer architecture shall provide mechanisms for an VAL server to register onto the NSCE servers.

### 4.1.4	Discovery requirement

[AR.4.1.4-a] The application layer architecture shall provide mechanisms for the discovery of NSCE services exposure to the VAL server.

### 4.1.5	Lifecycle management requirement

[AR.4.1.5-1] The application layer architecture shall provide a mechanism for lifecycle management of network slice to VAL server.

[AR.4.1.5-2] The application layer architecture shall provide a mechanism for lifecycle management of network slice communication service to VAL server.

4.1.6		Performance data retrieval requirement

[AR.4.1.6-1] The application layer architecture shall provide a mechanism to the VAL server to compare between the requested and actual slice QoS.

[AR.4.1.6-2] The application layer architecture shall provide a mechanism to support aggregation of slice performance information retrieved from all applicable sources.

[AR.4.1.6-3] The application layer architecture shall provide a mechanism to the VAL server to retrieve that aggregated slice performance data.

## 4.2	Application architecture

### 4.2.1	General

This clause provides the overall architecture description:

-	Clause 4.2.2 describes the application architecture in the service based representation and reference point representation;

-	Clause 4.2.3 describes the functional entities;

### 4.2.2	Architecture

# Figure 4.2.2-1 Architecture for network slice capability enablement – Service based representation

Figure 4.2.2-1 exhibits the service-based interfaces for providing and consuming network slice capability enablement services.

The mechanisms for service discovery in the service-based representation depicted in figure 4.2.2-1 are as follows:

-	The network slice capability enablement server could provide service to VAL server and NSCE client through interface Snsce.

Figure 4.2.2-2 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501[3] and TS 28.533[6].

Figure 4.2.2-2: Architecture for network slice capability enablement utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 4.2.2-3 depicts the network slice capability enablement architecture in the non-roaming case, using the reference point representation showing how various entities interact with each other.

Figure 4.2.2-3: Architecture for network slice capability enablement – reference points representation

The network slice capability enablement client communicates with the network slice capability enablement server over the NSCE-UU reference point. The network slice capability enablement client provides the support for network slice capability enablement functions to the VAL client(s) over NSCE-C reference point. The VAL server(s) communicates with the network slice capability enablement server over the NSCE-S reference point. The network slice capability enablement server, acting as AF, may communicate with the 5G Core Network functions via NEF (N33) reference point (for interactions with PCF, NSACF, etc.), or interacting with PCF directly via N5, if permitted. The network slice capability enablement server may interact with OAM system over NSCE-OAM reference point, as consumer in both NSaaS and NoP model (for Network Slice Provisioning capabilities, Performance Assurance,   Fault Supervision etc.).

NOTE:	The NSCE-OAM reference point can be realized by the services exposed by EGMF as defined in TS 28.533[6].

Figure 4.2.2-4 illustrates the architecture for interconnection between NSCE servers.

Figure 4.2.2-4: Interconnection between NSCE servers

The NSCE server could interact with another NSCE server over NSCE-E reference point.

### 4.2.3	Functional elements

#### 4.2.3.1	Network slice Capability Enablement client

The network slice capability enablement client functional entity acts as the application client for the slice enablement. The network slice capability enablement client interacts with the network slice capability enablement server to trigger a network slice related operations. This trigger may be due to an application QoS requirement change, a service operation change, a network slice status change, etc. The NSCE client may receive a network slice related notification from the NSCE server. The NSCE client may optionally notify the VAL client on the network slice / DNN change.

#### 4.2.3.2	Network slice Capability Enablement server

The network slice capability enablement server functional entity provides application layer enablement to support the network slice management and control with/without invoking control and management plane capabilities from SA2 and SA5 pertaining to network slicing. Such enablement supports the network slice related operations such as the mapping or migration of one or more vertical applications to one or more network slices, triggering the dynamic network slice lifecycle management, NSI/NSSI monitoring, etc.

### 4.2.4	Service-based interfaces

The architecture for enabling network slice capability enablement service contains the following service-based interfaces:

Sval:	Service-based interface exhibited by VAL server.

Snsce:	Service-based interface exhibited by NSCE server.

Unsce-c:	Service-based interface exhibited by NSCE client.

NOTE:	Detailed specification of Unsce-c is based on implementation.

### 4.2.5	Reference points description

#### 4.2.5.1	VAL-UU

The interactions related to vertical application layer support functions between VAL client and VAL server are supported by VAL-UU reference point. This reference point is an instance of Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

NOTE:	The details of VAL-UU reference point is out of scope of the present document.

#### 4.2.5.2	NSCE-UU

The interactions between a NSCE client and the corresponding NSCE server are generically referred to as NSCE-UU reference point.  This reference point supports fault data, QoE data and KQI data provisioning etc.

#### 4.2.5.3	NSCE-C

The interactions between the VAL client(s) and the NSCE client(s) within a VAL UE are generically referred to as NSCE-C reference point. This reference point supports obtaining information about network slice that VAL client(s) require, application client information (such as its KQI) provisioning etc.

#### 4.2.5.4	NSCE-S

The interactions between the VAL server and the SEAL server are generically referred to as SEAL-S reference point.  This reference point supports network slice capability exposure such as: application layer slice lifecycle management, fault diagnosis, slice API configuration and mapping, QoS verification, slice performance analytics exposure etc.

#### 4.2.5.5	NSCE-E

The interactions between the NSCE servers are generically referred to as NSCE-E reference point. his reference point supports information collection from other NSCE servers.

Editor's Note:	The supported capabilities for NSCE-E are FFS.

## 4.3	Cardinality rules

### 4.3.1	General

The cardinality rules are applied to the architecture specified in clause 4.2. The cardinality rules are based on functional elements. The functional elements cardinality specifies the multiplicity of the functional elements that can exist as per the architecture.

### 4.3.2	Functional Entity Cardinality

#### 4.3.2.1	VAL Client

The following cardinality rules apply for VAL Clients:

a)	One or more VAL Clients may be located in a UE.

#### 4.3.2.2	NSCE Client

The following cardinality rules apply for NSCE Clients:

a)	One or more NSCE Client(s) may be located in a UE.

#### 4.3.2.3	NSCE Server

The following cardinality rules apply for NSCE Server:

a)	One or more NSCE Server(s) may be located in an PLMN; and

b)	One or more NSCE Server(s) may be located in an PLMN per Network slice Provider.

#### 4.3.2.6	VAL Server

The following cardinality rules apply for VAL Servers:

a)	One or more VAL Server(s) may be served by one NSCE server.

### 4.3.3	Service Cardinality

a)	One NSCE server may enable one or more Network slice(s) identified by S-NSSAI(s);

b)	One Network slice can only be enabled and served by one NSCE server.

c)	The NSCE servers may provide service in the same or in different Tracking Areas.

# 5	Key issues

## 5.1	Key issue 1: Network slice capability management enhancements

SEAL is the service enabler architecture layer common to all vertical applications over 3GPP systems. It provides the functions like location management, group management, configuration management, identity management, key management, network resource management and network slice capability management as defined in 3GPP TS 23.434 [2].

Network slicing is a general network capability which can be applied for many vertical industries. The network slice capability management service in 3GPP TS 23.434 [2] only provides capability of network slice adaptation between NSCE server and the NSCE client. However, for such capability both control plane and management plane interactions need to be considered since there is close coupling between the per UE session and the per slice related actions. For example, the SEAL/NSCE layer may need to be aware of the slice provisioning parameters (NSI/NSSI configuration) which can be provided by the slice management system. Furthermore, NSI/NSSI performance monitoring from management system (e.g. NSI/ NSSI status) may be useful to be known at the enabler layer, since this may affect the application to slice re-mapping triggering (e.g. to re-map to the least congested slice); and may also impact other SEAL provided functionalities (e.g. QoS/resource control, group management etc).

Open issues:

-	Whether and which enhancement to SEAL network slice capability management service is required having in mind SA2 and SA5 slice related exposure?

-	Whether a potential enhancement to SEAL network slice capability management functional model is required?

-	Whether new or enhanced APIs are needed to support the potential SEAL enhancements?

-	How the network slice capability (such as management service (MnS)) consumption may trigger and impact the value-add services provided by the SEAL layer?

## 5.2	Key issue 2: Application layer exposed network slice lifecycle management

It is specified in clause 6.10 of 3GPP TS 22.261 [7] that 5G network is required to provide suitable APIs to allow a trusted third-party to create, modify, scale and delete network slices used for the third-party. As specified in 28.531 [5], SA5 has defined the network slice lifecycle management service. SA6 can provide a more concise application layer exposed network slice lifecycle management with additional functionality for verticals. It can help vertical do the lifecycle management without knowledge of SA5 O&M defined interfaces and APIs. In addition, with the network slice related information (such as network slice status reported from NSACF), the SA6 could trigger some slice lifecycle management operations automatically to provide a value-added lifecycle management service. It is not clear how to expose the network slice lifecycle management service from application perspective with providing additional value on top of existing SA5 solution.

NOTE: The interface which may be used in this KI is specified in TS 28.532 [8], and the enabler layer could use the interface exposed by EGMF defined in TS 28.533[6].

Hence, it is required to study the following issues:

-	How to integrate and expose the SA5 and SA2 network slice related capability to provide some value-added lifecycle management service from application layer?

-	Whether and how additional service APIs are required to be supported for application layer enablement of network slice lifecycle management?

-	Whether and how CAPIF can be leveraged for additional service APIs.

## 5.3	Key issue 3: Discovery & registration aspects for management service exposure

There are use cases (being discussed also in SA1, TR 22.835 [9]), where the applications (e.g. gaming or online video applications) may access the 5GS over multiple slices for different services (e.g. based on the user membership); or have different priorities on different slices based on Application Service Provider (ASP) request. As an example, a mobile network operator has provisioned a set of network slices (Slice#1, Slice#2, Slice#3) which may be used by different ASPs (e.g. Slice#1 for online video services, Slice#2 for gaming.  Slice#3 for eMBB or IOT service). Different ASPs may use these slices (or a subset of them) for different services that they offer. Furthermore, when an application changes the network slices to be accessed, it should be agnostic to the UEs accessing the service and should be performed automatically.

The vertical enablement layer (SEAL, vertical-specific enablers) supports the exposure of telco provided services to the vertical / application service provider (ASP). Such telco-provided services traditionally covered the 3GPP control plane services (provided by SA2); however, these can be extended to 3GPP management domain services (provided by SA5) which may be useful for allowing the ASP to monitor and manage the slices used by the UEs. The enabler layer can be seen as a trusted application entity that interacts with the management system on behalf of ASP to allow the exposure of management services related to the offered slices. It can be used to trigger dynamic slice-related actions and also reduce complexity at the ASP side.

In this extended notion of vertical enablement (to cover the 3GPP management service exposure), the vertical/ slice customer needs to be able to 1) discover the relevant Network Slice Instances and the respective capabilities such as coverage offered, RAT/ frequencies, to 2) discover the management services (based on TS 28.533 [6]) which can be exposed as part of the offered slices, and 3) register to the 3GPP management domain via the vertical enabler layer for consuming the management services.

This study needs to investigate:

-	Whether and how SEAL needs to be enhanced to support the discovery of the offered slices and the offered management services related to these slices, to the vertical/ASP;

-	Whether and how SEAL needs to be enhanced to support the registration of the vertical applications to the 3GPP management domain.

## 5.4	Key issue 4: Network slice fault management capability

As the requirement captured in TS 22.261 [7], based on operator policy, the 5GS shall provide suitable APIs to allow a trusted third-party to monitor the network slice used for the third-party according to operator policy.

In addition, the descriptions of network diagnostics are captured in clause F.2, TS 22.261 [7] as following:

Network diagnostics helps with scanning, diagnosing and identifying problems within a network. Diagnostics includes gathering data and continuously providing sufficient performance parameters that characterize the quality of the network connection. This includes data of the physical connection as well as of logical links and sub-networks. Exposure of relevant (and possibly aggregated) performance parameters ensures a quick reaction in case of failure as well as identifying network connectivity, performance and other related problems.

Network diagnostic information needs to be generated automatically and, in case of a hosted or virtual network deployment, be made available to the tenant of the network via a suitable API.

The alarm data can be used to help the third-party to diagnose the fault problem of the services, locate the fault causes, and to be aware of the potential fault. In TS 28.545 [10], the fault supervision management services are standardized by which the alarm of the network slice instance from network resource aspects can be subscribed and reported. This alarm information together with the application function's fault report and communication service related knowledge can be utilized by the SEAL/NSCE to diagnose and locate the cause of the service performance deterioration and the fault of the communication services, and then exposed the fault report to the third-party. For example, if the status of the required communication is not correct, the SEAL/NSCE derives this alarm information from application functions. In this case, it is the SEAL/NSCE's responsibility to detect whether this fault is caused by the 5GS network or not and exposed the fault report to the third-party. If it is, then the SEAL/NSCE may inform the management functions the location of the fault and ask for the maintenance of the managed functions to clear the fault.

In addition, the two business relationships of network slice as a service and network slice as NOP internals may considered separately for the capabilities exposed in each scenarios. The coordination with SA5 is needed to fill the gap between the communication services (from the verticals' point of view) and the 5G network.

Open issues:

-	Whether and how additional APIs dedicated to network slice fault management capability are required?

-	How to define the APIs to expose the fault report to verticals client/server

-	How to coordinate with fault supervision management services provided by O&M systems defined in SA5 to fill the gap in fault management.

## 5.5	Key issue 5: Communication service management exposure

In clause 6.10 of TS 22.261 [7], following requirements are defined:

Based on operator policy, the 5G network shall expose a suitable API to allow an authorized third-party to define and reconfigure the properties of the communication services offered to the third-party.

The 5G system shall support the means for disengagement (tear down) of communication services by an authorized third-party

In addition, in clause 6.23.1 of TS 22.261 [7], it defines that QoS monitoring can be used for assessing and assuring the dependability of the communication services.

In SA5 TS 28.535 [11], management services to assure the communication service as per agreement (for example a SLS) with a communication service (only network slice as a service scenario) consumer (e.g. enterprise) have been defined. For other scenarios, the communication services from the verticals aspects required by SA1 (e.g., vertical automation communication services, URLLC services) are not discussed in SA5

From the vertical industry perspective, they may be more concerned about how to operate the communication services provided by 5GS to meet the business requirements on application level. Take V2X service as an example, there is a specific service of cooperative driving for vehicle platooning information exchange, and the vertical has some requirements on this service, including end-to-end latency between two UEs. Therefore, the service enable layer needs to provide such capabilities for vertical industries, including lifecycle management and quality assurance of communication services from the verticals' perspective. For example, with the QoE/QoS data of communication services collected from vertical application layer, SA6 may need to provide some value-added communication services management and/or assurance services.

It is not clear how to expose the communication service management service from application perspective with providing additional value on top of existing SA5 solution.

Hence, it is required to study the following issues:

-	What kinds of service APIs are required to be supported for application layer exposure of communication services life cycle management?

-	What kinds of additional service APIs are required to be supported for application layer exposure of communication services SLA assurance?

-	How to support above capabilities in the enable layer?

-	How to coordinate with SA5 functionalities to fill the gap between the communication services (from the verticals' point of view) and the 5G network.

## 5.6	Key issue 6: Application layer QoS verification capability enablement

In clause 6.23 of TS 22.261 [7], it is defined the requirements about QoS monitoring that 5G system shall be able to assessing and assuring the dependability of the communication services.

In clause F.1 of TS 22.261 [7], it is discussed how QoS monitoring information can be used for assurance purposes. In step "Customer rating of QoS", it mentions that the customer can compare the QoS achieved by the provider with the QoS requirements and its own experience of the QoS.

In clause SA5 TS 28.552 [12], it defined QoS measurements reports with different Filters from perspective of OAM, e.g. 5QI, QIC, S-NSSAI, PLMN.

In some cases, the verticals and the service provider reach an agreement on the SLA, however with this SLA requirement, the VAL client may also suffer unsatisfied experience. Hence, it could be possible to provide Vertical application layer the capability of comparing the QoS achievement status together with the OAM QoS data versus real customer QoS data (e.g., MOS) collected from VAL client to check whether the existing QoS data is able to satisfy the VAL client's.

It is not clear how to expose the QoS verification capabilities on top of existing SA5 solution.

Hence, it is required to study the following issues:

-	What kinds of additional service APIs are required to be supported for application layer enablement of QoS verification?

-	What kinds of additional service APIs are required to obtain the real vertical QoS to support the QoS verification?

-	How to support above capabilities in the enable layer?

## 5.7	Key issue 7: Network slice related performance and analytics exposure

As specified in clause 6.10 of 3GPP TS 22.261 [7], 5G network is requested to support a 3rd party to get the network status information of a private slice dedicated for the 3rd party. The 5G network collects various kinds of data, such as performance measurements in TS 28.552 [12] and analytics data as specified in 3GPP TS 28.104 [15] and the analytics data from NWDAF and NSACF exposed by NEF. However, to get the information efficiently, the verticals are supposed to know, which entity to interact with to get the desired information, how to extract useful information from data which is collected using different statistical methods from different entities, which is challenging for some verticals. The network slice capability exposure enabler layer can aggregate and process the data from different source, making the network information exposure more orderly and easier to read. For example, for the slice related performance and analytics come from multiple sources, the enabler layer could help to organize and aggregate the information. For some applications utilizing multiple S-NSSAI, the enabler layer could help to organize and aggregate the information so that it is exposed and displayed based on the application level rather than slices level.

Hence, it is required to study the following:

-	Whether and how information about available (SA2 and SA5) slice performance and analytics should be exposed to a third party with added value?

-	Whether and how available slice performance and analytics related information could be aggregated or processed to support an efficient information exposure?

-	Whether and how additional service APIs are required to be supported at SEAL for the network slice measurements and analytics exposure?

-	Whether and how CAPIF can be leveraged for additional service APIs?

## 5.8	Key issue 8: Support for requirements translation

Requirements for network slice from different verticals may vary from one to another, in terms of performance and capability requirements. For example, for the performance related requirements, the live video streaming cares more on bandwidth while V2X cares more on latency and jitters. For the capabilities related requirements, V2X may requests the positioning while future factory may request the self-control and management. To satisfy the requirements, vertical has to interact with several 5GS entities and understand the specific network parameters, such as the attributes in the service profile as defined in TS 28.541 [14]. Slice enabler layer could act as a mediator between the vertical customer and the 5GS to decompose and translate the requirements.

Besides, the verticals are more focused on the KQI or QoE of the applications and services. For example, in a scenario of future factory, the robots are used to enable the intelligent delivering, the third-parties have a requirement on the application latency between the robots and the robots control system, the slice enabler may decompose this application latency requirement to 5GS network latency requirement (e.g., the network latency requirements specified in TS 28.541[14] as packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network or the UL/DL packet delay measurement between UE and PSA UPF for a QoS Flow defined in TS 23.501[3]) and forward it to the 5GS systems. Another example is that to support the service of the imaging for professional applications as defined in TS 22.261[7], the imaging system latency of the application as defined in TS 22.263[x] is required which may not only depends on the 5GS network transmission latency but also will be influenced by the network throughput and network bandwidth. The NSCE are responsible for the translation from the vertical's KQI/QoE requirements to 5GS network requirements based on the knowledge of the industry profiles and 5G network (the definition of the KQI/OoE is service and application specific and is out the scope of this study).

However, to cope with the various requirements, it is needed to specify the operations and procedure on how to translate them more efficiently, such as in the unified manner with unified format/model/template. In such way, the slice enabler layer is capable of translating requirements from the vertical, to service consumption or service API invocation and configurations to the respective 5GS domains. The service could be provided from control plane, management plane and SA6 slice enabler layer itself, or the combinations of services above.

There are two main aspects in requirements translation: 1) one is how the vertical requirements could be collected, whether and how the template is needed? 2) The other is how to transfer the requirements into actions, whether and how API translation is needed?

This key issue aims to discuss how to configure and translate the requirements to service consumption and configuration in a way that the slice capability exposure is 1) agnostic to the underlying telecom infrastructure, 2) hides the complexity of telecom infrastructure, 3) doesn't impact/restrict the level of exposure to the vertical and 4) that is resilient to dynamic changes that may happen due to application portability or telco-provided API status changes.

Therefore, the open issues include:

-	Whether and how the vertical requirements could be collected at the slice enabler to allow the translation to network service consumption and configuration,

-	Whether and how API translation is needed to support the requirements translation.

## 5.9	Key issue 9: Support for trust enablement

A vertical application may use the slice enabler services (which can be seen as a trusted 3rd party to the MNO) to request management services as well as control plane services for a new slice on demand, based on an agreement between the vertical and the network slice provider.

However, the creation of a new slice will require a form of trust between the vertical/end application and the 5GS (management and control plane) for authorizing/authenticating the application request and enabling the vertical app to consume management / control services related to the requested slice.

The vertical application may not be trusted by the OAM or the 5GC, and is therefore not able to access management and control services. Also, there can be two way of accessing these services after authorization, via direct exposure or indirectly via the enabler server.

So, the key issue will study:

-	How to enable the authorization/authentication of the vertical application to consume telco-provided services (management and control plane), based on the vertical applications request.

- 	How to enable the authorization/authentication of the vertical application to consume telco-provided services (management and control plane) indirectly via the slice enabler layer, based on the vertical applications request.

## 5.10	Key Issue 10: Support for managing trusted third-party owned application(s)

As per 3GPP TS 22.261 [7], it is possible for trusted third-party to use a dedicated network slice for diverse use cases. Further, it provides following requirement to manage applications:

"Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third-party to manage this trusted third-party owned application(s) in the operator's Service Hosting Environment."

It is also possible for the third-party to offer its consumers different contract qualities level (e.g. gold, silver and bronze). In clause 5.7.1 of 3GPP TR 22.835 [9], following use case has been specified:

"For gaming or online video applications, the end users, who have subscription with MNOs who may provide multiple network slices to different users or services, may still have different priority or membership e.g. VIP maintained by 3rd party Service Provider (SP). And depending on the priority or membership information from 3rd party SP perspective, based on the agreement between SP and MNO, the UE have different priority for the available network slices."

In clause 4.2.11.2 of 3GPP TS 23.502 [4] specifies following:

"When for all the Requested S-NSSAI(s) provided in step 2 the NSACF returned the maximum number of UEs per network slice has been reached and if one or more subscribed S-NSSAIs are marked as default in the subscription data and not subject to Network Slice Admission Control, the AMF can decide to include these Default Subscribed S-NSSAIs in the Allowed NSSAI. Otherwise, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, and for each rejected S-NSSAI the AMF includes a reject cause to indicate that the maximum number of UEs per network slice has been reached and optionally a back-off timer."

Upon reaching maximum UEs slice quota, the 5GC may reject the registration request on the S-NSSAI from the gold quality level customer which may not be desirable by the trusted third party. This happens since 5GC is not aware of the relevant application information e.g. contract qualities level of the UE making the registration request.

The third party application needs to provide high priority to the higher level of contract qualities and so it needs to manage such connections.

It is also specified in 3GPP TS 22.261 [7] that the 5G system shall support a mechanism to optimize resources of network slices (e.g., due to operator deploying different frequency to offer different network slices) based on network slice usage patterns and policy (e.g., application preference) of a UE or group of UEs.

In particular, this KI will address:

-	Whether and how the AF can provide application policy information to the operator's Service Hosting Environment to enable automatic management of application resources?

-	What application policy information needs to be specified by the trusted third-party AF that can be automatically evaluated by the operator's Service Hosting Environment?

-	Whether and how the AF can manage use of the application resources in the operator's Service Hosting Environment on a per user characteristic?

## 5.11	Key issue 11: Slice requirement alignment

As described in clause 6.1.2.2, TS 22.261[7], the 5G system shall allow the operator to create, modify, and delete a network slice, verticals have strong desires for the slice/service self-management. Initially, they may translate the communication service parameters to slice parameters (serviceProfile provided to OAM as defined SA5) and order a slice with certain slice requirements parameters and their values.

The verticals will put their best effort into slice requirements translation. However, they are not able to guarantee that all the potential factors will be considered to generate the optimal slice requirements parameters on the first try. After the service is executed on the required slice, the slice may not fully match the service real-time running conditions, for example, maybe only 60% of the slice resource is used to support the service, and rest of the slice resource is always idle, or the slice resource is insufficient due to under-provisioning. Or in some cases, there may be some unforeseen exceptions (e.g., unexpected traffic changes) and the current configured slice requirements parameters are not able to fulfil the requirements, for example, more resources are required to address the exceptions.

In order to achieve the maximum return of investment and ensure the slice/service self-management, the verticals expect a more optimal service profile which contains the exact value of those slice requirements parameters to support to the executing services' demands, i.e., by monitoring the network performance statistics to align the slice requirements between verticals and network providers. Furthermore, the action of alignment is not triggered by a single event but considers the statistics of the network performance during a certain time period. From the management perspective, third-party is able to modify the slice requirements (serviceProfile defined in TS 28.541[14]) by perform the operation of modifyMOIAttributes listed in table 6.1-1 in TS 28.531[5] to support the slice requirements alignment.

This key issue is to study how to enable better alignment between the vertical needs and the slice requirements parameters.

NOTE:	All the potential changes to the slice are based on existing SA2 and SA5 service/capability.

Open issues:

-	Whether and How the SEAL network slice capability management service supports better alignment between vertical needs and slice requirements parameters for initial service requirements and ongoing service conditions?

-	How does the SEAL network slice capability management service determine and indicate that VAL server policy (e.g., requirements provided when adding or changing slices) requests or actions result in suggested changes to the NSCE service provider policy (e.g., allowed NS profile operations or parameter ranges) or to VAL policies?

## 5.12	Key issue 12: Network slice capability exposure in the edge data network

The network slice deployed in the core network has a certain distance from the customer, so the delay will be affected to a certain extent and cannot meet the operation requirements of low latency equipment. Moreover, a variety of service data need to be processed in the core network, the scale of data traffic is large, the backhaul network needs to bear a large load and consume more bandwidth. For example, the differential protection service has strict requirements for latency in power industry.

In order to meet the personalized services requirements of vertical industries, network slices are deployed by using the computing, storage and communication capabilities of the edge date network, so as to realize the localized processing of the service, reduce the service transmission latency and enhance the service performance.

How to expose the network slice capability deployed in the edge data network to the vertical industry or the third parties is worthy of our study.

Open issues:

-	How could the NSCE server deployed inside the EDN interact with the NSCE server outside the EDN? Whether and how could the NSCE server interact with other NSCE serve?

-	Whether and how could the NSCE server inside the EDN manage the network slice that has resource outside the EDN?

-	Whether and how does SEAL need to be enhanced to support NSCE client to interact with NSCE server in the EDN and NSCE server outside the EDN?

## 5.13	Key issue 13: Delivery of the existing Network Slice information to the trusted third-party

There are many requirements on Network Slice Exposure specified in clause 6.10 of 3GPP TS 22.261 [7].

Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third-party to create, modify, and delete network slices used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to monitor the network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to define and update the set of services and capabilities supported in a network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to configure the information which associates a UE to a network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to configure the information which associates a service to a network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to assign a UE to a network slice used for the third-party, to move a UE from one network slice used for the third-party to another network slice used for the third-party, and to remove a UE from a network slice used for the third-party based on subscription, UE capabilities, and services provided by the network slice.

In order to satisfy the above requirements, the third party should know the Network Slice existence in prior to the proper operation related to the Network Slice.

The 3rd party service providers may know the existence of Network Slice through a way of delivery from the NSCE as an entity of NSP, which manages information of Network Slice for the 3rd party service providers. The concept of Network slice delivery is described in TS 28.530 clause 4.1.8.

Which information should be contained and delivered is another important point for delivery of the existing Network Slice. When 3rd party service providers activate the existing Network slice, 3rd party service providers should analyze the specific information of Network Slice such as set of service, capability, QoE, bandwidth and so on in order to figure out to meet their service requirements. What kind of information is necessary to 3rd party service providers may vary according to the services of the 3rd party service providers. With the regards, NSCE servers collect and manage information on Network Slice which 5GS may provide. It may be enough to indicate the standardized NEST (e.g, MBB, URLLC, massive IOT, etc.). Security aspects have to be considered. If there is a need for resource reservation per customer, then a dedicated slice is needed.

Open issues:

-	When does the Network Slice delivery occur?

-	Which information of Network Slice does need to be contained when delivered? Or is it enough the delivery indicates the standardized NEST such MBB, URLLC, massive IOT?

-	Whether and how management domain capabilities (e.g. MnS) corresponding to the network slice need to be delivered via the NSCE server?

-	How the deployment of NSCE server (e.g. at NSP or NSC side) affects the level of slice information to be delivered?

## 5.14	Key issue 14: Network Slice creation to the third-party and UE

The requirements specified in clause 6.10 of 3GPP TS 22.261 [7] says,

Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third-party to create, modify, and delete network slices used for the third-party.

This requirement interprets that the third-party can allocate the Network Slice for its service with suitable APIs provided by 5G Network. It can be implemented based on whether an appropriate Network Slice for the third-party service does exist or not. In case the appropriate Network Slice exists, the third-party request to use the Network Slice for the service, and the corresponding entities modifies the info of the Network Slice properly. In case the appropriate Network Slice does not exist, then the third-party request to create the new Network slice. This procedure should be handled with interaction between VAL server, NSCE and 5G network. Security aspects have to be considered. If there is a need for resource reservation per customer, then dedicated slice is needed.

After the Network Slice allocation is requested, UE should be notified that the Network Slice for the third-party service is used. It is obvious that the UE utilizes the info of Network Slice in URSP or local configuration to transport the data of the third-party service. With the regard, without notification to the UE, the Network Slice will not be used for the service. When notified, SA6 needs to take it consideration whether the notification is transported over NSCE layer. If transported over NSCE layer, then it should be considered whether UE applies the new allocated Network Slice into URSP.

NOTE 1:	The application enablement layer will not circumvent nor try to replace solutions required at the network layer defined by SA2 nor the management layer defined by SA5.

Open Issues

-	What kind of information should be required for creating a new Network Slice for the third-party service (between NSP and NSC)? Any minimum set of information should be considered such as Service/Slice Type, Device Type or QoS Index?

-	Whether and how is the notification transported over NSCE layer?

-	Whether does UE apply the new allocated Network Slice into URSP when transported over NSCE layer?

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

# Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key issue 1 | Key issue 2 | Key issue 3 | Key issue 4 | Key issue 5 | Key issue 6 | Key issue 7 | Key issue 8 | Key issue 9 | Key issue 10 | Key issue 11 | Key issue 12 | Key issue 13 | Key issue 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solution 1 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Solution 2 | X |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Solution 3 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Solution 4 | X |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Solution 5 | X |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Solution 6 |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Solution 7 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Solution 8 |  |  | X |  |  |  |  |  |  |  |  |  | X |  |
| Solution 9 |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| Solution 10 |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| Solution 11 | X |  |  |  | X |  |  |  |  |  |  |  |  |  |
| Solution 12 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Solution 13 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| Solution 14 | X |  |  |  |  |  |  |  |  |  |  | X |  |  |
| Solution 15 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Solution 16 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Solution 17 | X |  |  |  |  |  |  |  |  |  | X |  |  |  |
| Solution 18 |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| Solution 19 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| Solution 20 |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| Solution 21 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.1	Solution 1: Automatic application layer network slice management

### 6.1.1	Solution description

#### 6.1.1.1	General

This solution aims to address the issues identified in Key Issue 2.

This solution provides a procedure for automatic application layer network slice management performed by the network slice capability enablement server based on network slice status collected from 5GS and QoE collected from application layer.

When network slice capability enablement server receives a request for automatic application layer network slice management from VAL server, the network slice capability enablement server performs the service operations including detecting and subscribing the event which may trigger the automatic network slice lifecycle management, making the network slice lifecycle management recommendation/decision, triggering the network slice management operations, notifying the consumer about the network slice information.

#### 6.1.1.2	Automatic application layer network slice lifecycle management

Figure 6.1.1.2-1 illustrates an automatic application layer network slice lifecycle management solution based on network slice related data and QoE collected from application layer.

Pre-conditions:

1.	The VAL client has requested a network slice provisioning;

2.	The VAL server has subscribed to the network slice capability enablement server for automatic network slice management;

3.	The network slice enabler layer is registered/capable for interacting with 5GS such as triggering network slice LCM operations, and has collected current network slice capabilities.

# Figure 6.1.1.2-1: Automatic application layer network slice lifecycle management

1.	The vertical server sends a request for automatic application layer network slice lifecycle management (auto-NS-LCM), with network slice requirements (e.g. delay, throughput, load, the maximum number of users supported, etc.). The request can indicate the level of auto-NS-LCM that is required, such as whether to notify the VAL server/consumer before performing the auto-NS-LCM. The request may also indicate the trigger conditions, such as by providing the monitored parameters and the corresponding thresholds.

2.	After receiving the request, the network slice capability enablement server checks that the user is authenticated and authorized to perform the corresponding auto-NS-LCM operations, and filters the unauthorized requests, if any.

3.	If authenticated and authorized, the network slice capability enablement server, acting as the network slice LCM service consumer, triggers the AllocateNsi request (see TS 28.531 clause 6.5.1 [5]) towards the respective management service provider, based on the network slice capabilities and network slice requirements.

4.	The network slice capability enablement server sends the auto-NS-LCM response to the VAL server with the result(s) of the network slice LCM operation(s).

5.	According to network slice requirements, network slice capability enablement server triggers the provision of network slice status and QoE metrics.

5a.	The network slice status could be collected through subscribing or requesting to 5GS. For example, to monitor the slice load, it could subscribe to/request the relevant service(s), such as AnalyticsExposure defined in TS 23.502 [4] clause 5.2.6.16, provisioning data report exposure for NSI in clause 5 of TS 28.532 [8]. If the trigger conditions are not indicated in the subscription, the network slice capability enablement server may help to configure an appropriate trigger condition, such as report period or thresholds.

5b.	Also, the network slice capability enablement server could get the information of QoE metrics from the application layer domain.

6.	Once the trigger condition or a combination of trigger conditions are met, based on requirements and network slice capabilities with updated information in Step 5, such as change in QoE, change in network slice status etc., the network slice capability enablement server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi/AllocateNsi/DeallocateNsi request as specified in TS 28.531 [5].

7.	Optionally, if it is indicated in the request to notify the VAL server/consumer before performing the auto-NS-LCM, the network slice capability enablement server sends the network slice LCM recommendation(s) with network slice status to VAL server, to see whether takes the recommendation(s) or not.

8.	Optionally, sending the response indicating the decision made by VAL server to network slice capability enablement server.

9.	Based on decision made by VAL server or network slice capability enablement server, the network slice capability enablement server performs the corresponding operation(s).

10. According to the corresponding operation(s) result, the network slice capability enablement server sends the response to the VAL server.

### 6.1.2	Solution evaluation

The proposed solution addresses Key Issue #2. SA5 provides the network slice lifecycle management, while this solution provides a procedure for automatic application layer network slice management performed by the network slice capability enablement server based on network slice status collected from 5GS and QoE collected from application layer. With automatic application layer network slice management, the consumer's requirement could be better met without having to interact with 5GS frequently. This solution does not introduce impact on 5GS architecture. It will possible need enhancement of SA6 SEAL architecture enhancement to support the interaction with OAM system, but the interface for interaction with the OAM system is still being studied in SA5.

## 6.2	Solution 2: Network slice fault management capability

### 6.2.1	Solution description

#### 6.2.1.1	General

This solution addresses the key issue 4 of network slice fault management described in clause 5.4.

This solution provides a possible procedure to illustrate the network slice fault management capability exposed by NSCE server.

To enable the VAL server to monitor the problems of a network, the NSCE layer should help the VAL server to scanning, diagnosing and identifying problems within a network. The performance data and alarm data from multiple sources is helpful to characterize the quality of the network connection.

#### 6.2.1.2	Network slice fault management capability exposure

Figure 6.2.1.2-1 illustrates the network slice fault management process to address the key issue 4 of network slice fault management described in clause 5.4.

Pre-conditions:

1.	The VAL client has subscribed to the network slice fault management capability.

2.	The network slice enabler layer is capable to interact with NEF and OAM system.

3.	The VAL client has checked the status of application layer.

# Figure 6.2.1.2-1: Network slice fault management process

1.	The VAL server sends a request to NSCE server to request for the fault diagnosis of the applications and networks when it detected a fault in the application layer, the services and the network slices. This request may be triggered by the errors of the applications detected by the VAL server itself, or the VAL server may periodically collect the fault diagnostic report subscribing to the network slice fault management capability. The fault information of applications may be included in this request. For example, the Service Availability Failure Events e.g, described as Fallback events carried out by the Dual SIM cellular device.

2.	On receiving the request from VAL server, NSCE server retrieves the performance data of network slice from 5GS. For OAM system, the APIs defined in clause 11.3, TS 28.532[8] is utilized. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[17] can be utilized.

The analytics data defined in clause 6.3 to clause 6.14, TS 23.288[17], network slice instance related performance data defined in clause 5, TS 28.552[12] and network analytics data in clause 8.3 and clause 8.4 TS 28.104[15] exposed by OAM system may be acquired.

3.	NSCE server retrieves the alarms of network slice instances from OAM system via the procedures defined in clause 6.1,TS 28.545[10], and the alarms are defined in clause 4.1.1.1, TS 32.111-1 [16], e.g., the fault of communication, environmental, equipment, processing error, QoS for device/resource/file/functionality/smallest.

4.	NSCE server may request the alarm information (e.g., the 5GS network is not work or the required performance is under the threshold which leads the service's problem) collected by NSCE client if possible. The information collected by the NSCE client depends on the third-parties' requirements and implementation.

5.	NSCE client report the requested fault information to NSCE server.

6.	NSCE server correlates the applications, services and the network resources of the specific network slice instances which supports the applications and services with the S-NSSAI, NSI ID (nSInstanceId defined in TS 28.541[14]), Application Identifier (defined in 23.501[3]), and then diagnoses the causes of the fault of the applications or services by analysing the fault information from different sources. For example, the RAN function of the slice instance which is utilized to support the service of the smart grid application, for a certain time duration, the smart grid suffered the bad experience caused by Service Availability Failure Events, and in the RAN function is detected continuously to report an alarm of environmental fault in the same time duration, then the environment fault may be the root cause of the Service Availability Failure and should be prioritized to be solved. The fault may be identified with "critical", "major", "minor", "ignore" to show its prioritization.

7.	NSCE server response to VAL server with the reports of fault diagnostic result to indicate the causes of the failures of the applications or services. If there is no problem within the 5GS network, the VAL server may be requested to check the application layer itself again.

8.	If the NSCE server detects that the application/service error is caused by the 5GS, the VAL server may send the fault diagnosis report to OAM system to indicate the server fault which causes the application/service failure by utilizing the NSCE-OAM interface.

NOTE: The APIs utilized to send the fault diagnosis report to OAM will re-utilize the fault management services in TS 28.532[8] exposed by EGMF as defined in TS 28.533[6].

### 6.2.2	Solution evaluation

The solution addresses the key issue #4 and this solution will have interaction with both the 5GC and OAM to collect the alarm information and network analytics data and also could acquire application fault data from VAL server or VAL client. For the NSCE server and NSCE client, it will possible need enhancement of the existed Rel-17 architecture to perform network and service level diagnosis. Also, fault diagnosis service in service level should also need to be added in the existed network architecture.

## 6.3	Solution 3: Slice API configuration and translation

### 6.3.1	Solution description

#### 6.3.1.1	General

This solution aims to address the issues identified in Key Issue 8.

This contribution provides a solution for the API configuration and translation and proposes the translation of the service API as invoked by the end applications to slice APIs based on the API configuration and application to slice mapping. Slice APIs can be defined as customized/tailored sets of service APIs (which can be either NEF northbound APIs or OAM provided APIs or enabler layer/SEAL provided APIs) and can be mapped to particular slice instances. The slice APIs can be a bundled or combined API comprising of different types of APIs, which will be used to expose the telco (5GS/SEAL)-provided services as needed by the applications of the slice customer. Each slice API may be configured per network slice instance.

This solution provides a support functionality to the vertical application specific layer and configures the exposure of APIs in a slice-tailored manner. It is assumed that the VAL server is not initially aware of the all the API exposure capabilities and information which will be needed for the given slice based on the SLA, and NSCE plays a vital role in configuring and translating the slice API based on the per slice requirements to service APIs.

This solution has two procedures:

-	a procedure on the slice API configuration

-	a procedure on the slice API translation

#### 6.3.1.2	slice API configuration

In this procedure, the VAL server initially provides an application requirement to enabler server including the service KPIs and the subscribed/preferred slices. Then, the slice enabler configures the mapping of the VAL application to a slice API which is a combination/bundling of northbound APIs (from both management and control plane). In particular, a slice API (or SDK) consists of telco-provided/platform dependent service APIs (e.g. NEF, OAM, SEAl, etc), and provides an abstraction/simplification on top of them. The procedure also covers the scenario where a trigger event occurs (e.g. QoS degradation, slice load) and the mapping configuration or the slice API configuration needs to change. In this scenario, the slice enabler updates the configuration of the API and provides a notification to the VAL server.

Figure 6.3.1.2-1 illustrates a solution for the slice API configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability enablement services

# Figure 6.3.1.2-1: Slice API configuration

1.	The VAL server sends a VAL application requirement request to the network slice capability enablement server. This request provides the service requirements / KPIs, the capability exposure requirements and a preferred/subscribed slice identification (e.g. S-NSSAI or ENSI)

2.	The network slice capability enablement server maps the VAL application requirement to a slice API which includes a list of APIs which are needed to be consumed as part of this service capability exposure (such APIs can be NEF APIs as specified in TS 29.522 clause 5 e.g. related to network monitoring, slice status, analytics exposure, SEAL APIs as specified in TS 23.434, OAM provided APIs). Such mapping can be determined at the network slice capability enablement server based on the VAL application exposure requirements or can be pre-configured per slice instance. The criteria for the mapping are the capability exposure requirement per slice (based on GST parameters, or from service/slice profile] as well as the capability exposure permissions/authorization for the API invoker.

The network slice capability enablement server may also store the mapping of the slice API to the service API list and per service API information (e.g. data encoding, transport technology, API protocol and versions)

NOTE: For OAM provided APIs, the consumption of the network slice management service related procedures are specified in TS 28.531[5].

3.	The network slice capability enablement server subscribes/registers to consume the corresponding APIs from the 5GS (NEF and OAM) and SEAL service producers. For example, network slice capability enablement server may subscribe to consume NEF monitoring events, or SLA monitoring from OAM.

4.	The network slice capability enablement server sends a VAL application requirement response to notify on the result of the request and indicate whether configuration of the slice API can be possible or not.

5.	The network slice capability enablement server sends the slice API information and optionally the slice to service API mapping to the VAL server.

6.	A trigger event is captured by the service/API providers (e.g. control plane, management plane, SEAL) or the VAL server side (application server relocation to different EDN/DN, UE mobility to different EDN, application change of behaviour) or any other API related event captured by the network slice capability enablement server (e.g. failure, unavailability, high load).

7.	The network slice capability enablement server processes the trigger event and checks and updates the mapping of service APIs to the slice APIs. The objective is to keep the slice APIs unchanged, so the VAL server is not aware of any change (if not triggered by VAL server). To accomplish this, the service APIs may need to be updated accordingly. For example, if one service API changes (e.g. due to high load, unavailability,  ..) the mapping to service APIs should be updated (e.g. if service API is a Location API which is provided by 5GC in the first place, the update would trigger the remapping to a Location API from SEAL LMS) to avoid affecting the slice API.

8.	The network slice capability enablement server updates the subscription/registration to the underlying 5GS and SEAL service producers, if an update on the service APIs (e.g. NEF APIs, SEAL APIs, OAM provided APIs) is needed.

9.	The network slice capability enablement server optionally notifies the VAL server on the slice/service API related updates.

#### 6.3.1.3	Slice API translation

This procedure follows the 6.3.1.2 and aims to describe how the slice API invocation request is translated to service API invocations after the slice API configuration mapping. In this procedure, the network slice capability enablement server initially receives a slice API invocation request from the vertical application based on step 5 of 6.3.1.2. Then, the network slice capability enablement server based on the slice API request fetches the service APIs to be invoked based on the slice API configuration and performs invocation requests to the corresponding service API providers.

As example, a slice API is requested for an IIOT slice. This may translate to: NSI Monitoring from Management Domain #1, NSSI Monitoring from Management Domain #2, network/QoS monitoring from NEF#1, Location monitoring from SEAL LMS, slice-related analytics from NWDAF (via NEF). Such translation could be based on the slice API configuration mapping of 6.3.1.2.

Figure 6.3.1.3-1 illustrates a solution for the slice API translation based on the configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability enablement services.

2.	The slice API mapping to the VAL server has been performed based on 6.3.1.2 step 2 and the slice API information is provided to the VAL server based on 6.3.1.2 step 5.

# Figure 6.3.1.3-1: Slice API translation

1.	The VAL server sends a slice API invocation request to the network slice capability enablement server. This request provides slice API information (name, type, communication methods, protocols,..),  based on the received information in step 5 of 6.3.1.2. Service APIs are indicated above can be NEF APIs as specified in TS 29.522, SEAL APIs as specified in TS 23.434, OAM provided APIs)

NOTE 1: For OAM provided APIs, the consumption of the network slice management service related procedures are specified in TS 28.531[5].

2.	After receiving the request, the network slice capability enablement server checks that the user is authenticated and authorized to perform the slice API invocation and maps the requested slice API to a service APIs (based on the VAL server ID / slice ID and procedure in clause 6.6.1.2).

NOTE 2:	If CAPIF is used, the network slice capability enablement server acts as AEF, and the authorization is obtained by CCF.

3.	The network slice capability enablement server generates a trigger for service API invocation requests to all the service APIs within the slice API.

4.	The network slice capability enablement server sends a service API invocation request to the producers of the service APIs within the slice API. This may include the originated service API invoker identity information (VAL server) as well as the direct API invoker information, authorization information, service API and slice API identification.

5.	The service API provider authenticates the API invoker and authorizes the request.

NOTE 3:	If CAPIF is used, the requests are sent to the corresponding AEFs of the API provider's domain, and the authorization is obtained by CCF.

6.	The network slice capability enablement server receives the service API invocation response as a result of the service API invocation.

7.	The network slice capability enablement server sends to the VAL server a slice API invocation response as a result of the slice API invocation.

### 6.3.2	Solution evaluation

The solution addresses the key issue #8 on requirements translation; and discusses the API translation and configuration aspects. Solution #3 requires interaction with API producers in 3GPP network systems, based on already defined procedures. This includes interaction with 5GC (acting as trusted AF) for subscribing to and invoking NEF services; the interaction with OAM / MnS producers (or EGMF) for consuming OAM services, and the interaction with SEAL for invoking SEAL APIs.

The solution is feasible and proposes the enhancement NSCE server capabilities to translate / bundle service APIs (of different producers and types) to slice APIs. The definition of the slice APIs and the interaction with the VAL server are expected to be specified during the normative phase.

## 6.4	Solution 4: QoS verification capability

### 6.4.1	Solution description

#### 6.4.1.1	General

This solution aims to address the issues identified in Key Issue 6.

This solution provides a procedure to illustrate the QoS verification capability by the network slice capability enablement server.

To enable the VAL server to verify QoS data of a network, the NSCE layer should help the VAL server to check whether the existing QoS data is able to satisfy the VAL client's requirement.

#### 6.4.1.2	QoS verification capability

Figure 6.4.1.2-1 illustrates the QoS verification capability process to address the key issue 6 of Application layer QoS verification capability exposure described in clause 5.6.

Pre-conditions:

1.	The VAL client has subscribed to the QoS verification capability.

2.	The VAL client has subscribed to the capability of Network slice related performance and analytics exposure.

3.	The network slice enabler layer is capable to interact with NEF/NWDAF and OAM system.

Figure 6.4.1.2-1: QoS verification process

1.	The VAL server sends a request to NSCE server to verify the service/application QoS data.

2.	On receiving the request form VAL server, NSCE server retrieves the QoS data of network slice from 5GC and OAM system. For OAM system, the APIs defined in clause 11.3, TS 28.532[15] is utilized, the performance data specified in TS 28.552[12] and analytics data specified in TS 28.104[16] can be collected. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[18] is utilized. The data of QoS information defined in TS 23.501[3].

3.	NSCE server sends a request to application layer to request the QoE data (e.g., MOS).

4.	VAL server sends the requested QoE data to NSCE server.

5.	NSCE server may send a request to NSCE client to collect the QoE data of network and services if possible.

6.	NSCE client report the requested data collected by itself if possible, the information collected by the NSCE client depends on the third-parties' implementations.

7. 	NSCE server verifies and analyses QoS data of network slice instance, Comparing QoS achieved status together with the OAM QoS data versus real customer QoE data (e.g., MOS) collected from VAL client to check whether the existing QoS data is able to satisfy the VAL client's to generate the QoS verification report as required by VAL server for specific period of time.

8. 	NSCE server sends the service/application QoS verification response to VAL server.

### 6.4.2	Solution evaluation

The proposed solution addresses Key Issue #6, the solution provides a procedure for application layer the capability of QoS verification by the network slice capability enablement server, which has interaction with both the 5GC and OAM to collect the QoS data from N33 and NSCE-OAM interface respectively, acquires network and service QoE data from NSCE client by NSCE-UU interface and obtains application QoE data from VAL server by NSCE-S interface to compare QoE date and QoS data, checking whether the existing QoS data is able to satisfy the real customer QoE data.

This solution does not introduce impact on 5GS architecture and SEAL architecture, the NSCE server has the capability to translate and compare QoE data and QoS data."

## 6.5	Solution 5: Network slice related performance and analytics exposure

### 6.5.1	Solution description

#### 6.5.1.1	General

This solution addresses the key issue 7 of performance and analytics exposure described in clause 5.7.

This solution provides a possible procedure to illustrate the network slice related performance and analytics exposure capability provided by network slice capability enablement server.

To enable the third-party to adjust the network slice or to configure the information which associates a service/UE to a network slice, the third-party is allowed to monitor the performance data and analytics data of the network slice, service or application. The network slice related performance data may be collected from multiple sources (e.g., OAM system or 5GC functions) to help the NSCE layer to perform data aggregation or analyses to generate the data required by the third-party. The solutions of performance data and analytics data exposed to the third-party needs to be studied.

#### 6.5.1.2	Network slice related performance and analytics exposure

Figure 6.5.1.2-1 illustrates the network slice related performance and analytics exposure process to address the key issue 7 of performance and analytics exposure described in clause 5.7.

Pre-conditions:

1.	The VAL client has subscribed to the capability of Network slice related performance and analytics exposure.

2.	The network slice enabler layer is capable to interact with NEF and OAM system.

# Figure 6.5.1.2-1: Network slice related performance and analytics exposure process

1.	The VAL server sends a request to NSCE server to collect the desired service/application specific performance data and analytics data, the detailed content of the reported data depends on the type of the application and services (the KQI and QoE data may be included with in the request). For example, the VAL may request the NSCE server to report the performance and analytics report of the service of the imaging for professional applications. The request may contain the interested geographical area.

NOTE 1:	It is not shown in the figure that VAL client could also trigger step1. It may be triggered by a specific event or request from the VAL client. Besides, the VAL client can trigger this procedure if the performance issue are detected by the VAL client itself.

NOTE 2:	Both periodical or ad-hoc reporting are possible, the VAL layer can request a periodical reporting or a one-off reporting. The result of the reporting can be subscribed by the VAL layer; it can request both or any one of them.

2.	The NSCE server shall check if the VAL server is authorized to get the network slice performance and analytics data.

3.	On receiving the request from VAL server, NSCE server retrieves the performance data and analytics data of network slice from 5GS. For OAM system, the APIs defined in clause 11.3, TS 28.532[8] is utilized. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[17] is utilized.

The performance and analytics data such as the Observed Service experience statistics, Load level information of a Network Slice defined in TS 23.288[18], network slice status defined in TS 23.502[4] clause 5.2.21, and analytics data defined in TS 23.288[17], network slice instance related performance data defined in TS 28.552[12] and network analytics data in TS 28.104[15] exposed by OAM system may be acquired. For example, when the performance and analytics report of the service of the imaging for a professional application is required, the imaging system latency as defined in TS 22.263[18] should be reported and analysed. In this case, the NSCE requests the OAM system to report the one-way/Round-trip packet delay of a specific S-NSSAI between PSA UPF and NG-RAN in clause 5.4, TS 28.552[12] and requests the NWDAF to report the User Data Congestion Analytics report defined in clause 6.8, TS 23.288[17].

4.	NSCE server retrieves the KQI data of services (e.g., the jitter duration of a video service), the network performance related data and the end users information (e.g., 5G UE's running time) by following the procedures defined in Figure 6.5.1.2.1-1 and Figure 6.5.1.2.1-2.

5.	NSCE server correlates and analyses the performance data of network slice instance, the analytics data of group of UEs and the KQI/QoE data to generate the performance data and analytics data report as required by VAL server for specific period of time.

NOTE 3:	How the data analytics correlation happens is out of scope of NSCALE study

6.  NSCE server sends the service/application related performance data and analytics response to VAL server. For example, when the performance and analytics report of the service of the imaging for a professional application is required, the report may acquire the calculated imaging system latency of the application, and level (e.g., good, normal, poor) of the latency.

##### 6.5.1.2.1	KQI and performance data report from NSCE client

Figure 6.5.1.2.1-1 and 6.5.1.2.1-2 illustrate the procedure to acquire instant KQI/performance data and KQI/performance data report from NSCE client respectively.

# Figure 6.5.1.2.1-1: Process of instant KQI/performance data reporting

1.	NSCE server sends a request to NSCE client to ask for instantly reporting of the KQI and performance (e.g., the jitter duration of a video service), the network performance related data (e.g., the downlink/uplink RSRP of serving cell measured by 5G UE) and the end users information (e.g., 5G UE's running time).

NOTE 1: The APIs over the NSCE-UU interface utilized to achieve the instant KQI/performance data from NSCE client is to be defined in normative work.

2.	On receiving the request described in step1, the NSCE client sends the response to notify NSCE server whether the request is received successfully.

3.	NSCE client reports the requested data (e.g., the jitter duration of a video service, 5G UE's running time) once the data is measured.

NOTE 2:	In this study, general functionalities and procedures of NSCE are defined, the required data depends on the concrete scenarios. The data measured by NSCE client may depend on the study of SEALDD.

# Figure 6.5.1.2.1-2: Process of reporting KQI/performance data

1.	NSCE server sends a request to NSCE client to request the report collection of KQI data (e.g., the jitter duration of a video service), the network performance related data and the end users information (e.g., 5G UE's running time), and the time period of the data collection may be included in this request.

NOTE 1: The APIs over the NSCE-UU interface utilized to achieve the instant KQI/performance data from NSCE client is to be defined in normative work.

2.	On receiving the request described in step1, the NSCE client sends the response to notify NSCE server whether the request is received successfully.

3.	NSCE server sends a request to retrieve the data desired report, the data type and the time duration may be indicated in this request.

4.	NSCE client sends the requested data report (e.g., the report of the jitter duration of a video service, 5G UE's running time) collected by itself.

NOTE 2:	In this study, general functionalities and procedures of NSCE are defined, the required data depends on the concrete scenarios. The data measured by NSCE client may depend on the study of SEALDD.

### 6.5.2 	Solution evaluation

The solution addresses the key issue #7 and this solution will have interaction with 5GC and OAM system to acquire KQI and performance data from N33 and NSCE-OAM interface, it also acquires KQI and performance data from NSCE Client by NSCE-UU interface. For the NSCE Client, it has the capabilities of KQI and performance data measurement and collection from Unsce-c interface. The data measurement may also depend with SEALDD. The NSCE Server could capture the KQI and performance data from 5GC and OAM system as well as NSCE Clients. The third applications or verticals can extract the performance data of network and analysis result from NSCE Server.

## 6.6	Solution 6: VAL server authorization and authentication via slice enabler layer

### 6.6.1	Solution description

#### 6.6.1.1	General

This solution aims to address the issues identified in Key Issue 9.

When a vertical application from third party wants to request management services as well as control plane services for a new slice on demand, it needs the authorization and authentication from 5GC. The vertical application may not be trusted by the OAM or the 5GC, it needs to interact with OAM and 5GC via the enabler server.

#### 6.6.1.2	VAL server authorization and authentication via slice enabler layer

Figure 6.6.1.2-1 illustrates the VAL server authorization and authentication via slice enabler layer.

Pre-conditions:

1.	The NSCE server can be seen as a trusted AF to interact with 5GC and has registered to the OAM.

2.	The NSCE server has been authorized to act as a middle layer to do the authentication and authorization.

# Figure 6.6.1.2-1: VAL server authorization and authentication via slice enabler layer

1.	The vertical server sends a request for authorization and authentication to the NSCE server with its identity and the requested network slice management and control capability (e.g. request for establish a new network slice).

2.	After receiving the request, the NSCE server authenticates and authorizes the VAL server based on its identity and requested capability.

The procedure of authentication and authorization could refer to API invoker authenticate and authorize as specified in TS 23.222[20] clause 8.10.3 and 8.11.3, if applicable.

NOTE: the authentication and authorization mechanism will be in scope of SA3.

3.	Upon authorized and authenticated, NSCE server sends the authorization and authentication response and information to VAL server with the token.

4.	The NSCE server triggers the network slice management and control operation.

### 6.6.2	Solution evaluation

The solution 6 proposed the producer of authorization and authentication for VAL server to address the Key Issue 9. As an authorized middle layer, the NSCE server authenticates and authorizes the VAL server based on its identity and requested capability. The API invoker authenticate and authorize functionalities of CAPIF could be utilized if applicable. After that, the VAL server can be trusted by 5GC and OAM.

The solution is feasible. And this solution does not introduce impact on 5GS architecture and SEAL architecture.

## 6.7	Solution 7:  network slice capability registration

### 6.7.1	Solution description

#### 6.7.1.1	 General

This solution aims to address the issues identified in Key Issue 3.

This solution provides a possible procedure to perform the VAL server registration.

The vertical enablement layer (SEAL, vertical-specific enablers) supports the exposure of telco provided services to the VAL. After registered in the enablement layer, the VAL server could be authorized to access network slice related exposure capabilities, e.g. monitoring of network/UE performance.

#### 6.7.1.2 	Capability exposure registration

# Figure 6.7.1.2-1: Capability exposure registration

1.	The VAL server sends the registration request to the NSCE server with its identity, and may also include interested service and level of exposure requirement.

2.	The NSCE server does the registration check by verifying whether all the necessary information has been provided and is reasonable. If the VAL server is already registered with the same registration information, the request will be rejected by the NSCE server. If the VAL server is already registered and updated information is received in the registration request, the request will be accepted by the NSCE server and the existing registration context of that VAL server will be updated based on the latest registration request.

The procedure of registration check could refer to API invoker on-boarding as specified in TS 23.222[20] clause 8.1.3 if applicable.

3.	After the registration procedure, the NSCE server sends the response to the VAL server indicating could initiate the authorization and authentication and further to do the service discovery.

### 6.7.2	Solution evaluation

This solution solves the Key Issue 7 by proposing a procedure to register the VAL server in the slice enabler layer. The on-boarding functionalities of CAPIF could be utilized if applicable. This solution does not introduce impact on 5GS architecture and SEAL architecture.

## 6.8	Solution 8: Discovery of management service exposure

### 6.8.1	Solution description

#### 6.8.1.1	General

This solution aims to address the issues identified in Key Issue 3 and more specifically the first open issue supporting the discovery of the offered slices and the offered management services related to these slices, to the vertical/ASP. This solution also addresses partially Key Issue #13, and in particular the point on "Whether and how management domain capabilities (e.g. MnS) corresponding to the network slice need to be delivered via the NSCE server?"

To be able to utilise capabilities/features of the management system the applications must be made aware of existence of such features and capabilities. All management domain (MD) features/capabilities come with a pre-configured exposure, where this can be configured by the operator for a given slice. This exposure is used to decide which application can see which information regarding the capability/feature. Exposure refers to the permissions that the 3rd party entity has gained over its use of the management service, e.g., the ability to read, or execute or modify or delete can be considered as different sorts of exposure.

A MD feature/capability is anything of use offered by the management system to the 3rd party application. Therefore, a new feature could be a managed entity, a MnS or management API, any software, hardware, or other functionality – for example, new technology support, new coverage area, new network slice type or instance, new NFs or new network slice subnet type or instance.

This solution provides a mechanism which is aligned with SA5 ongoing work related to the MnS registry exposure for external discovery (see TR 28.824 clauses 5.4 and 7.1), which enables the initial discovery of MnS for a given slice based on VAL server request, and the discovery of new/modified MnS.

#### 6.8.1.2	Procedure

In this procedure, the VAL server initially requests the new MnS which are supported by a target slice and based on this request the NSCE server requests from the OAM the MnS discovery (via EGMF). Then, the OAM/MnS registry derives the details to be exposed based on the NSCE server/VAL server permissions and provides the list of MnS for the given slice and the access details via the NSCE server to the VAL server. This procedure also includes the case when a new/modified MnS is deployed at the MD for the given slice, and the OAM/MnS registry provides this information directly to VAL server via EGMF (assuming that VAL server has registered to the MD).

Pre-conditions:

1.	The VAL server has registered to receive NSCE services

2.	The NSCE server is trusted by the 3GPP MD.

3.	MnS registry at OAM is aware of the allowed MnS and the exposure levels for a given slice

Figure 6.8.1.2-1 illustrates a solution for the MnS discovery support.

# Figure 6.8.1.2-1: MnS discovery support

The steps of this embodiment are as follows:

1.	The VAL server requests from NSCE server the expected exposure capability type and the related exposure level for a target slice or for a given VAL application, using the VAL application ID, the MD ID and optionally the slice ID.

2.	The NSCE server translates the expected exposure capability type in to MnS exposure requirement.

3.	The NSCE server coordinates with the 5GS and discovers the related network service(s), by triggering the MnS service discovery as specified in TS 28.533[6], clause5.1.1 and 6.1.1. The 5GS, acting as the discovery service producer, sends the discovery result including related exposure information such as management service identifier, management service information and management service producer information to the NSCE server.

4.	The NSCE server sends the service/management data based on step 3, including the list of MnS and the corresponding exposure details to the VAL server. Here to mention that based on exposure level, the exposure details may be different.

5.	VAL server registers to NSCE server for the management domain capability exposure for the subscribed slice.

The registration procedure of the VAL server to NSCE server for management capability exposure is based on Solution #7.

6.	The MnS registry discovers new or modified MnS in the 3GPP MD. The MnS registry derives the details to be exposed to the VAL server (via the NSCE server), based on the exposure level configured by the new or modified MnS and the exposure levels of registered VAL applications and NSCE server. The MnS change could be triggered due to changes on:

- New/modified management service producers

- New/modified managed entities – such as new radio, new technology or new NFs

- New/modified technical support – such as support in a new geography or coverage area

- Availability of new management data – e.g. related to slice performance

7.	The MnS registry sends to the NSCE server the new / modified MnS, as discovered in step 6.

8.	The information on the discovered new/modified MnS is sent from the NSCE server to VAL server.

### 6.8.2	Solution evaluation

The solution addresses the key issue #3 and key #13; and in particular the discovery aspects related to the MnS exposure. Solution #8 is non-overlapping and complementary to Solution #7 (which describes the Registration aspects). This solution is viable and important for allowing the VAL server to discover the MnS from OAM initially and based on MnS updates from the 3GPP MD. This solution will interact with OAM initially to trigger MnS service discovery and also OAM when new or modified MnS parameters change, it will inform NSCE server on the updated/new MnS info. This solution uses already supported SA5 capabilities when interacting with OAM.

## 6.9	Solution 9: Support for managing trusted third-party owned application(s)

### 6.9.1	Solution description

#### 6.9.1.1	General

This solution addresses the key issue 10 for managing trusted third-party owned application(s) as specified in clause 5.10.

This solution provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice quota to NSCE server. This solution enables the trusted third-party AF to provide application policy information to the operator's Service Hosting Environment. This solution also enables automatic management of application resources.

#### 6.9.1.2	Network slice quota management capability exposure

Figure 6.9.1.2-1 illustrates the network slice quota management process to address the key issue 10 for managing trusted third-party owned application(s).

Pre-conditions:

1.	The network slice enabler layer is capable to interact with NEF.

# Figure 6.9.1.2-1: Network slice quota management process

1.	The VAL server initiates network slice quota management request towards the NSCE server. The request includes VAL service ID, Slice identity, identity of the type of the quota, specific type of action to take (e.g. release low priority users as identified by 5GC, release list of users as identified by the VAL server, etc) and optionally list of UEs on which specific action to apply. The message also includes unique identity for the request in order to update or cancel the action based on change of the requirement or service provider's policy.

2.	Upon receiving the request from the VAL server to manage the network slice quota, the NSCE server authorises the VAL server and if VAL server is authorized, the NSCE server subscribes to the network slice quota events. The NSCE server includes possible actions (e.g. maximum number of UEs per slice is reached, etc) expected from 3GPP core network upon reaching the network slice quota threshold.

Editor's note:	The APIs to subscribe to the network slice quota related events is to be defined in SA2.

3.	The NSCE server sends the network slice quota management response specifying the result.

4.	Upon receiving notification from the 5GC/NEF about network slice quota threshold, the NSCE server may request the 5GC to perform specific action if not specified already.

### 6.9.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.10	Solution 10: Network slice application policy management capability

### 6.10.1	Solution description

#### 6.10.1.1	General

This solution aims to address the issues identified in Key Issue 10.

This solution provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice through NSCE server when reaching UEs slice quota threshold. This solution enables the trusted third-party AF to provide network slice access management policy based on the UE's priority, contract qualities level, etc. This solution also helps to avoid traffic loss in case of slice congestion and make adjustment beforehand considering the application policy from AF.

#### 6.10.1.2	Network slice application policy management capability

Figure 6.10.1.2-1 illustrates the application policy capability process to address the key issue 10 described in clause 5.10.

Pre-conditions:

1.	The VAL server is authorized to use the slice event reporting exposure and slice adaptation trigger capability.

2.	The network slice enabler layer is capable to interact with NEF/NWDAF/NSACF and OAM system.

3.	NSCE has added UEs to different slices based on UE priority, QoS reservation. High priority UEs with dedicated reserved resources are in one slice.

# Figure 6.10.1.2-1: Network slice application policy management process

1.	The VAL server initiates network slice event reporting exposure request towards the NSCE server. The request includes VAL service ID, Event Filter (such as slice identity, event type, etc.), Event Reporting filter (such as whether the notification is threshold based or periodical). The message also includes notification threshold or periodicity based on the Event Reporting filter settings.

2-3.	Upon receiving the request from the VAL server to manage the network slice quota event reporting, the NSCE server authorises the VAL server and if VAL server is authorized, the NSCE server subscribes to the network slice quota (the number of UE or PDU sessions) events, including the slice identity (S-NSSAI) and threshold/periodicity information. According to 3GPP TS 23.502 [4], there can be multiple NSACFs serving the same network slice and different threshold can be set based on the policy of trusted third-party AF. The event subscription procedure is described in clause 4.15.3.2.10 of TS 23.502 [4], and the APIs defined in clause 6.2 of TS 29.536 [22] can be utilized.

4.	The NSCE server receives the network slice event notification response specifying the result after 5GC confirms the subscription.

5-6.	When the event (e.g. the number of UE or PDU sessions has reached the threshold for specific slice) happens, the notification is sent from 5GC to NSCE server including the Event Reporting information (such as percentage of the maximum number of UEs or percentage of the maximum number of the PDU Sessions established on the network slice). The event notification procedure is specified in clause 4.15.3.2.10 of TS 23.502 [4].

7.	The notification is forwarded from NSCE server to VAL server including the Event ID, Event filter and Event reporting information.

8.	The VAL server initiates network slice adaptation trigger request to the NSCE server.

9.	The NSCE server retrieves the network slice related status information from 5GC and OAM. For OAM system, the services defined in clause 11.3 of TS 28.532 [8] and clause 5 of TS 28.552 [12] can be utilized. For CN functions, the services of Nnwdaf_AnalyticsInfo service defined in clause 7.3 of TS 23.288 [17] can be utilized.

10.	The NSCE server makes network slice adaptation analysis on network status information from 5GS and makes slice adjustment decision. NSCE server can make slice lifecycle management operations for specific slice as specified in step 11.

11. The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s) (e.g. to allocate more RAN resources for UEs with high priority), such as modifyNsi request as specified in TS 28.531 [5].

### 6.10.2	Solution evaluation

The proposed solution addresses Key Issue #10, the solution provides a high level procedure for the capability of network congestion handling when reaching UEs slice quota threshold. Based on VAL request, the NSCE server can define preferred slice maximum capacity threshold and get notification when the threshold is reached. Both the 5GC and OAM can be interacted by NSCE server to collect slice related status information. Based on VAL request, the NSCE server can make adjustment slice resource allocation (max number of registered UEs and max number of PDU sessions) having in mind the increased demand. SA5 has defined slice service profile parameters modification (as specified in TS 28.541 [14]) to change the maximum number of UEs/maximum number of PDU sessions in a slice. This solution does not introduce impact on 5GS architecture, the NSCE server has the capability to analyze network status data and decide how to make slice resource adjustment. The NSCE server can potentially have lack of knowledge of the actual hardware limitations of 5GC/NSACF. During normative phase it would be decided if NSCE server will manage RAN parameters for slice adaptation. It might be the case that SA5 has its own solution for adapting network slice capacity (max number of registered UEs and max number of PDU sessions per slice). During normative phase it will be decided if the NSCE server shall inform the VAL server about the result of NSCE slice adaptation. During the normative phase actual APIs will be described. The SA6 SEAL will possibly need enhancement to support the interaction with 5GS and OAM system.

## 6.11	Solution 11: Communication service management exposure

### 6.11.1	Solution description

#### 6.11.1.1	General

This solution aims to address the issues identified in Key Issue 5.

This solution provides a procedure for communication services management including the communication service lifecycle and communication assurance performed by the network slice capability enablement server.

To enable the verticals to create/modify/disengagement a communication service, the NSCE layer helps the VAL server to subscribe the slice services to allocate the network slices to fulfil the communication services. Also the NSCE server is able to perform the network slice related analyses to help the verticals to assure the communication service.

#### 6.11.1.2	Use Case of Communication service lifecycle management

Figure 6.11.1.2-1 illustrates one possible procedure of communication service management to address the communication service lifecycle management and communication service assurance

Pre-conditions:

1.	The VAL client ordered slice services to offer its communication services.

2.	The network slice enabler layer is registered/capable for interacting with 5GS such as triggering network slice LCM operations.

# Figure 6.11.1.2-1: Communication service creation

1.	The VAL server sends a request to NSCE server to create a communication service, e.g., the VAL server requests to create a video service in a future factory, the requirements of the video service (for instance, the service ID, the number of the UEs, the coverage area of the video service, the mobility information of the UEs, the resolution of the video service) is included in this request.

2.	NSCE server translates the communication service requirements (e.g., for a video service, the mobility information and the resolution of the video) then determines the network SLA (e.g., dLtThptPerSlice, uLtThptPerSlice, latency as defined in serviceProfile TS 28.541[14]). The NSCE may perform the translation by pre-configured industry profiles or by KQI-KPI translation algorithms which are out scope of standard.

3.	NSCE server initiates the Slice Service subscription procedures by utilizing the management service of network slice creation as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5. The slice creation request may fail due to the shortage of network resources.

4.	NCSE server sends the communication service creation response to VAL server. If the slice creation failed in step3, the NSCE server may ask the VAL sever to modify the service requirements.

# Figure 6.11.1.2-2: Communication service modification

1.	VAL server sends the communication service reconfiguration request to the NSCE server to reconfigure the properties of the communication service if the SLA is not able to satisfy current service requirements (e.g., reconfigure the resolution requirements of the communication service) or the VAL server predicts that service requirements changes sometime in the future (e.g., some new cameras are to be supported).

2.	NSCE server translates the communication requirements according the reconfigured communication properties then updates the network SLA.

3.	NSCE server initiates the Slice Service update procedures by utilize the management service of network slice modification as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5.

4.	NCSE server sends the communication service modification response to VAL server.

# Figure 6.11.1.2-3: Communication service disengagement

1.	The VAL server sends a request to NSCE server to disengage the communication service when the communication service ends.

2.	NSCE server initiates the Slice Service de-allocation procedures by utilizing the management service of network slice de-allocation as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5.

3.	NCSE server sends the communication service disengagement response to VAL server.

#### 6.11.1.3	Communication service creation

# Figure 6.11.1.3-1: Communication service creation

Figure 6.11.1.3-1 shows the APIs of service subscription of communication service creation as step 1 and step 4 of Figure 6.11.1.2-1.

#### 6.11.1.4	Communication service modification

# Figure 6.11.1.4-1: Communication service reconfiguration

Figure 6.11.1.4-1 shows the APIs of service subscription of communication service reconfiguration as step 1 and step 4 of Figure 6.11.1.2-2.

#### 6.11.1.5	Communication service disengagement

Figure 6.11.1.5-1: Communication service reconfiguration

Figure 6.11.1.5-1 shows the APIs of service subscription of communication service reconfiguration as step 1 and step 3 of Figure 6.11.1.2-3.

### 6.11.2	Solution evaluation

The solution addresses the key issue #5 of communication service management exposure. In this solution, the NSCE server interacts with VAL server to acquire/update the requirements of the communication service if a communication services is required to create/update. After the determination of the SLA of network slice according to the communication services requirements, the NSCE Server will have interactions with OAM system to trigger the network slice allocation/modification/deallocation by utilize the network slice related management services exposed by EGMF defined in SA5. The existing management services defined in SA5 are able to support the solution described in clause 6.11.1. The APIs over S-NSCE interface to be defined in normative phase:

1.  APIs of service subscription of communication service creation (for instance, to create a service, the service ID, the number of the UEs, the coverage area of the service, the mobility information of the UEs)

2.  APIs of service subscription of communication service reconfiguration

3. APIs of service subscription of communication service disengagement

## 6.12	Solution 12: SEAL enhancement

This solution aims to address the issues "which enhancement to SEAL network slice capability management service is required" identified in Key Issue 1.

This solution illustrates the impact on SEAL by analyzing the existing solutions.

### 6.12.1	Solution description

The following solutions provide the new service which could enhance the SEAL network slice capability management service defined in TS 23.434;

-	Solution 1 provides the Automatic application layer network slice management service based on the collected network slice status from 5GS and QoE from application layer;

-	Solution 2 provides the Network slice fault management capability based on collected alarm information from 5GS and application fault data from application layer;

-	Solution 3 provides the Slice API configuration by mapping the VAL application requirement to a slice API, and API translation service by mapping the slice API to service API;

-	Solution 4 and solution 5 provide the QoS verification service and the performance and analytics exposure based on the QoS and performance data 5GS, and KQI and QoE data from NSCE Client.

-	Solution 8 provides the Discovery of management service exposure by triggering the MnS service discovery;

-	Solution 9 provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice quota, but it does not specify specific follow up action(s). Solution 10 provides the Network slice application policy management capability based on slice quota by triggering the network slice lifecycle management. Solution 20 provides service of AF policy based network slice optimization by utilizing the network slice related management services. The three solutions may be merged in the normative work to enhance the SEAL.

-	Solution 11 provides the Communication service management exposure;

Solution 13 provides a Network Slice allocation service, where the NSCE layer performs the Network Slice allocation operation on behalf of the VAL server, and sends the allocated Network Slice information (e.g., S-NSSAI) to the NSCE Client.

-	Solution 14 provides a service of information collection among the NSCE servers. This service requires an interface between NSCE servers.

-	Solution 15 provides enhancements to the existing NSCE client functionality by separating the network slice adaptation subscription/notification functionality from the slice adaption triggering. Also, it enables the NSCE server to receive location requirements, schedule time window requirements and access type within the network slice adaptation request sent from the NSCE client.

-	Solution 16 and solution 17 provide Multi-Network slice monitoring service and Multi-Network slice resource optimization service for network slice from multi-networks (e.g. the PNI-NPN network and PLMN network) through NSCE server in a combined manner.

-	Solution 18 provides service of Network Slice Information delivery. It could be done via request and subscribe/notify operation, also it could be done along with the registration procedure.

-	Solution 19 provides service of slice requirement alignment by comparing the slice requirements with network current situations to estimate if the current slice requirements are properly configured.

-	Solution 21 provides service of predictive slice modification in edge based NSCE deployments, which is mainly for the NSaaS scenario when an expected migration happens for a UE or group of UEs from an EDN to another.

The following solutions could utilize the exiting services of CAPIF, and may not introduce enhancement to SEAL.

-	Solution 6 provides the authorization and authentication capability.

-	Solution 7 provides the registration capability.

Solution 1, 2, 3, 4, 5, 8, 10, 11, 13, 16, 17, 18, 19, 20, 21 require the interface between OAM and NSCE server (OAM-NSCE interface defined by SA5). Solution 14, 16, 17, 21 require the interface between the NSCE servers (NSCE-E interface). Therefore the SEAL NSCE architecture could consider to support the NSCE-E interface.

### 6.12.2	Solution evaluation

This solution analyzes all the solutions that could provide services to enhance SEAL service. The above analysis could be taken into consideration while working on the conclusion.

## 6.13	Solution 13: Network Slice Allocation by VAL server

### 6.13.1	Solution description

#### 6.13.1.1	General

The NSCE layer performs the Network Slice allocation operation on behalf of the VAL server. The non-trusted 3rd party application (i.e., VAL server) can not access to the 5GS management system directly. So, the NSCE server needs to perform the authentication and authorization for the registration of VAL server on behalf of 5G MnS. This procedure follows the clause 6.7 Solution 7 network slice capability registration.

The Network Slice creation that is triggered by the VAL server depends upon network operator policy. The network slice capabilities and management options offered to the customer are determined by a business agreement prior to and outside of the scope of 3GPP standards.

The VAL server can identify the Network slice in Network Slice creation request with the Network Slice indicator (e.g., S-NSSAI).

Upon network slice allocation, the NSCE server acts as the network slice provisioning MnS consumer. The NSCE server requests 'AllocacatedNsi' operation to the network slice provisioning MnS producer as specified in TS 28.531. When the 'AllocatedNsi' operation is received, the network slice provisioning MnS Producer in 5GS performs charging mechanism as specified in TS 28.202.

The NSCE server sends the allocated Network Slice information (e.g., S-NSSAI) to the NSCE Client, after the Network Slice allocation to the VAL server is successful.

NOTE:	If the UE is provisioned with URSP rules by the network operator, the UE handles the precedence between the delivered network slice info via NSCE layer info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. How the UE uses the Network Slice info delivered via NSCE layer in relation to the URSP is implementation dependent.

With the above regards, The NSCE layer performs the below for the Network Slice Allocation by the VAL server.

-	Network Slice Allocation operation on behalf of VAL Server as specified in TS 28.531

-	Delivery of the Network Slice Allocation result

-	Delivery of the allocated Network Slice Information to NSCE client

The VAL server as a Network Slice consumer makes use of the APIs provided from NSCE server to allocate the Network Slice for the Network Slice Lifecycle management for its service.

#### 6.13.1.2	Network Slice Allocation by VAL Server

This subclause depicts the procedure of the Network Slice Allocation by the VAL server, when the VAL server needs to allocate the Network Slice, interaction with 5GS via the NSCE layer.

Figure 6.13.1.2-1: Network Slice Allocation by the VAL server

1.	The VAL server makes an request to create the Network Slice.

The VAL server specifies the Network Slice requirements for the VAL service. The Network Slice requirement at the VAL server may be specified with the attributes of GST (which results in NEST) as specified in GSMA NG.116.

2.	The VAL server requests the Network Slice creation with the Network Slice requirements.

The Network Slice creation request includes the VAL Service ID, VAL UE's ID List, S-NSSAI and so on.

3.	The NSCE server performs the Network Slice Allocation. The NSCE server as network slice provisioning MnS consumer requests of 'AllocacatedNsi' operation as specified in TS 28.531.

When the 'AllocatedNsi' operation is received, the network slice provisioning MnS Producer performs charging mechanism as specified in TS 28.202.

The NSCE server performs AF-driven guidance for URSP determination to 5GS per VAL UE.

NOTE 1: 	According to the network operator policy, the NSCE server acting as AF may send the created network slice information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may create the new route selection parameters to indicate sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

4.	The NSCE server sends the result of Network Slice Allocation to the VAL server.

5.	The NSCE server delivers the Network Slice Allocation Information to the NSCE Clients of VAL UEs based on the VAL UE's ID list from step 2, in case the Network Slice Allocation is successful, if the NSCE server does not perform AF-driven guidance for URSP determination to 5GS in step 3.

The Network Slice Allocation Information contains the VAL service ID, S-NSSAI, DNN and so on.

6.	The NSCE client stores and applies the Network Slice Allocation Information.

7.	The NSCE client sends the Network Slice Allocation Information response to the NSCE server.

NOTE 2: 	If the UE is provisioned with URSP rules by the network operator, the UE handles the precedence between the delivered network slice info via NSCE layer info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. How the UE uses the Network Slice info delivered via NSCE layer in relation to the URSP is implementation dependent.

### 6.13.2	Solution evaluation

The proposed solution addresses the KI #14.

This solution provides the procedure for NSCE layer to operate the Network Slice allocation on behalf of the VAL server. This allows the NSCE layer to interact with both the VAL server and 5GS MnS for allocating Network Slice and makes use of the operation of 5GS MnS defined in SA5. The interaction between NSCE server and NSCE client is required for informing the Network slice to be used for the service provided from the VAL server.

The APIs of NSCE layer expect to be defined for the Network slice allocation for the VAL server in the normative phase.

## 6.14	Solution 14: Interaction between the NSCE servers

### 6.14.1	Solution description

This solution aims at addressing the KI #12 and KI #1. It proposes procedures of interaction between the NSCE servers.

### 6.14.2	Information collection from multiple NSCE servers

The owner of the centralized/national NSCE server including the MNO and vertical could have requirement to collect the slice status from edge/provincial NSCE server. The collected information could be used to optimize network resource allocation after some analysis.

Pre-condition:

1.	NSCE server#2 has agreement with NSCE server#1 to provide the collected slice information.

# Figure 6.14.2: Information collection from multiple NSCE servers

1.	The NSCE server#1 sends out the information collection subscription request with expected period and interested slice ID, e.g., List of SNSSAI. This step could be done by pre-configuration.

2.	The NSCE server#2 shall check if the NSCE server#1 is authorized to get the network slice information.

3.	After authenticated, the NSCE server#2 sends collected slice information to NSCE server#1. The Network slice related performance and analytics exposure could be re-utilized.

### 6.14.3	Solution evaluation

This solution addresses the KI #12 and KI #1 by providing a interaction procedure between the NSCE servers. In this solution, the NSCE server interacts with another NSCE server to monitor/collect the network slice information. The Network slice related performance and analytics exposure could be re-utilized to do the network slice information collection/monitoring.

The APIs over NSCE-E interface to be defined in normative phase:

1.	APIs of NSCE server registration on another NSCE server.

2.	APIs of service subscription of information collection across NSCE server.

## 6.15	Solution 15: UE triggered network slice adaptation

### 6.15.1	Solution description

#### 6.15.1.1	General

This pCR provides a solution for Key Issue #1 regarding UE triggered network slice adaptation.

The solution is provided via enhancements to existing TS 23.434 text as shown below, highlighted in yellow.

* * * Enhancement based on TS 23.434 v. 18.0.0   * * * *

#### 16.3.2.4	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.4-1 illustrates the VAL UE-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Pre-condition:

-	The NSCE client has connected to the NSCE server;

# Figure 16.3.2.4-1: Network slice adaptation for VAL application

1.	The VAL client provides a new application requirement to the NSCE client, indicating a new service profile for the VAL application. This may be in the form of a change at the application QoS requirements, location requirement, time window requirement, access type preference (e.g., 3GPP, non-3GPP or multi-access), service operation change, or other application-related parameters.

2.	The NSCE client sends a network slice adaptation trigger request to the NSCE server for the VAL application. This trigger may be in the form of exact requested network slice (and optionally DNN) for the VAL UE of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN). The trigger may also include additional parameters based on the step 1 request, including requested location criteria, time window, access type preference, associated group id or UE IP address preservation indicator.

NOTE 1：How the requested network slice is known by the NSCE client is out of scope of this release.

3.	The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

NOTE 2: How the NSCE server decides to trigger the network slice configuration is implementation dependent.

NOTE 3：Whether and how the NSCE server triggers the network slice adaptation for all the VAL UEs within the VAL Application is out of scope of this release.

4.	The NSCE server acting as AF provides the updated S-NSSAI, location criteria, time window, access type preference, associated group id or UE IP address preservation indicator and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI, location criteria, time window, access type preference, associated group id or UE IP address preservation indicator) that can be associated with applications matching the application traffic. 5GC uses this information to update the URSP to the affected UE(s).

NOTE 4:	NSCE server provides the updated S-NSSAI ,/DNN, location criteria, time window and access type preference, associated group id or UE IP address preservation indicator as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

5.	The NSCE server may sends a notification response to the NSCE client indicating success or failure.

#### 16.3.2.X	Procedure for VAL UE network slice adaptation subscription

Figure 16.3.2.X-1 illustrates the VAL UE network slice adaptation subscription procedure.

Pre-condition:

-	The NSCE client has connected to the NSCE server.

# Figure 16.3.2.X-1: VAL UE network slice adaptation subscription

1.	The NSCE client sends a network slice adaptation subscription request to the NSCE server to subscribe to network slice adaptations. The subscription request includes identifiers of the VAL UE, VAL application, network slice and a notification target address.

2.	The NSCE server processes the request by checking if the NSCE client is authorized to initiate the network slice adaptation subscription request. Further, the NSCE server acting as an AF may subscribe via the NEF to receive network slice adaptation notifications (i.e., changes to URSP rules) applicable to the VAL UE.

3.	The NSCE server replies with a network slice adaptation subscription response to the NSCE client indicating the subscription status.

#### 16.3.2.X	Procedure for VAL UE network slice adaptation notification

Figure 16.3.2.X-1 illustrates the VAL UE network slice adaptation notification procedure.

Pre-condition:

-	The NSCE client has connected to the NSCE server;

# Figure 16.3.2.4-1: VAL UE network slice adaptation notification

1.	The NSCE server detects network slice adaptation (i.e., changes to URSP rules) has occurred for the VAL UE.

2.	The NSCE server sends a network slice adaptation notification to the NSCE client. The NSCE server includes information regarding the type of network slice adaptation event that has occurred.

3.	The NSCE client optionally notifies the VAL client of the network slice adaptation.

##### 16.3.2.2.3	Network slice adaptation trigger request

Table 16.3.2.2.3-1 describes the information flow of network slice adaptation trigger request from the NSCE client to the NSCE server.

Table 16.3.2.2.3-1: Network slice adaptation trigger request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID(s) | M | The VAL UE ID(s) within the VAL service, for which the network slice adaptation trigger applies |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice configuration may corresponds to. |
| Requested S-NSSAI | M | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |
| Requested time window | O | Indication of the new scheduled time window that is requested |
| Requested location criteria | O | Indication of the new location criteria that is requested |
| Requested access type preference | O | Indication of the new access type (3GPP, non-3GPP or multi-access) preference that is requested. |
| Requested associated group id | O | Indication of the group id that is requested |
| Requested UE IP address preservation indicator | O | Indication that UE IP address preservation is requested |

##### 16.3.2.2.4	Network slice adaptation notification trigger response

Table 16.3.2.2.4-1 describes the information flow of network slice adaptation notification trigger response from the NSCE server to the NSCE client and optionally to the VAL client.

Table 16.3.2.2.4-1: Network slice adaptation notification trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation. |

##### 16.3.2.2.X	Network slice adaptation subscription request

Table 16.3.2.2.X-1 describes the information flow of network slice adaptation subscription request from the NSCE client to the NSCE server.

Table 16.3.2.2.X-1: Network slice adaptation subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID(s) | M | The VAL UE ID(s) within the VAL service, for which the network slice adaptation subscription applies |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice adaptation subscription corresponds to. |
| S-NSSAI | M | Indication of the applicable S-NSSAI. |
| DNN | O | Indication of the applicable DNN. |
| Notification Target Address | M | The Notification Target Address (e.g., URL) where the notifications destined for the NSCE client should be sent to. |

##### 16.3.2.2.X	Network slice adaptation subscription response

Table 16.3.2.2.X-1 describes the information flow of network slice adaptation subscription response from the NSCE server to the NSCE client.

Table 16.3.2.2.X-1: Network slice adaptation subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

##### 16.3.2.2.X	Network slice adaptation notification

Table 16.3.2.2.X-1 describes the information flow of network slice adaptation notification from the NSCE server to the NSCE client.

Table 16.3.2.2.X-1: Network slice adaptation notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Network slice adaptation information | M | Information regarding the type of network slice adaptation event that has occurred. |

#### * * * End of TS 23.434 v. 18.0.0 Enhancements  * * * *

### 6.15.2	Solution evaluation

The solution addresses key issue #1. The solution provides enhancements to the existing NSCE client functionality by separating the network slice adaptation subscription/notification functionality from the slice adaption triggering. This enables an NSCE client to subscribe and receive network slice adaptation notifications from a NSCE server independent of issuing a network slice adaptation trigger request to the NSCE server.  The solution also addresses the open issue on enhancements needed to the information to be provided in the UE triggered network slice adaptation procedure. These enhancements enable the NSCE server to receive location requirements, schedule time window requirements and access type within the network slice adaptation request sent from the NSCE client to NSCE server. This enables the NSCE server to provide this information within the AF-driven URSP guidance requests it sends to PCF.

## 6.16	Solution 16: Multi-Network slice management capability

### 6.16.1	Solution description

#### 6.16.1.1	General

This solution aims to address the issues identified in Key Issue 1.

This solution provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice from multi-networks (e.g. the PNI-NPN network and PLMN network) through NSCE server. This solution enables the trusted third-party AF to manage its own PNI-NPN (public network integrated non-public network) and its private slice(s) in the PLMN in a combined manner as specified in clause 6.10.2 of 3GPP TS 22.261 [7]. In the case that communication services of a PNI-NPN is extended through a PLMN (e.g. the service is supported by a slice in the PNI-NPN and a slice in the PLMN) as described in clause 6.1.2.4 of 3GPP TS 22.261 [7], the NSCE server can make the performance monitoring and analysis for dedicated service from multiple networks and offer consolidated service performance report to trusted third-party AF.

#### 6.16.1.2	Multi-Network slice management capability

Figure 6.16.1.2-1 illustrates the slice management process to address the key issue 1 described in clause 5.1.

Pre-conditions:

1.	The network slice enabler layer is capable to interact with both PNI-NPN and PLMN 5GC or OAM system.

2.	PNI-NPNs are deployed as network slices of the PLMN.

Figure 6.16.1.2-1: Network slice application QoS monitoring process

1.	The VAL server initiates service QoS monitoring report exposure request towards the NSCE server. The request includes VAL server ID, application ID, Network ID. The message also includes application key performance indicator (such as end-to-end delay, throughput, QoE data from UE, etc. ) list, monitoring period and monitoring zone.

2.	Upon receiving the request from the VAL server to manage the network slice QoS monitoring report, the NSCE server makes authentication and authorization of the VAL server and if VAL server is not authorized, the NSCE server replies with failure response.

3.	The NSCE server makes mapping from application ID that received from VAL server to slice identities (S-NSSAIs allocated in each network) and retrieves the network slice related status information from PNI-NPN networks, such as performance measurements in TS 28.552 [12] and analytics data as specified in 3GPP TS 28.104 [15].

4.	The NSCE server retrieves the network slice related performance information from PLMN 5GC networks (e.g. NWDAF) or OAM, For OAM system, the services defined in clause 11.3 of TS 28.532 [8] and clause 5 of TS 28.552 [12] can be utilized. For CN functions, the services of Nnwdaf_AnalyticsInfo service defined in clause 7.3 of TS 23.288 [17], performance measurements in TS 28.552 [12] and analytics data as specified in 3GPP TS 28.104 [15] can be utilized. The PLMN operator can choose to deploy NSCE-Server acting as the entry of PLMN capability exposure which can be optional.

5.	The NSCE server sends Service QoE data request to NSCE clients in monitoring zone which includes application ID to request the QoE data of specific application.

6.	The NSCE client sends the requested QoE data (e.g. MOS, stalling ratios, etc.) response to NSCE server.

NOTE 1:	In step 5 and step 6, the NSCE client may need to retrieve information from VAL client that is out of scope of the present document.

7.	The NSCE server verifies and analyses performance data of network slice instance that received from both PNI-NPN and PLMN networks as well as QoE data provided by VAL client, then NSCE server make consolidation performance report for specific application among different kinds of networks in specific period of time/location zone.

8.	The NSCE server sends the network slice QoS monitoring response towards VAL server.

### 6.16.2	Solution evaluation

This solution addresses the KI #1 by providing a procedure of performance monitoring and analysis for dedicated service from multiple networks by the NSCE server, to help the trusted third-party AF to manage its own PNI-NPN (public network integrated non-public network) and its private slice(s) in the PLMN in a combined manner. This solution has no impact on the 5GS architecture, and it may require the interactions between NSCE servers to retrieves the network slice related status information.

The APIs over NSCE-S interface to be defined in normative phase:

1.	APIs of network slice resource optimization request and response.

## 6.17	Solution 17: Multi-Network slice resource optimization

### 6.17.1	Solution description

#### 6.17.1.1	General

This solution aims to address the issues identified in Key Issue#1 and Key Issue#1.

This solution provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice from multi-networks (e.g. the PNI-NPN network and PLMN network) through NSCE server. This solution enables the trusted third-party AF to manage its own PNI-NPN (public network integrated non-public network) and its private slice(s) in the PLMN in a combined manner as specified in clause 6.10.2 of 3GPP TS 22.261 [7]. In the case that communication services of a PNI-NPN is extended through a PLMN (e.g. the service is supported by a slice in the PNI-NPN and a slice in the PLMN) as described in clause 6.1.2.4 of 3GPP TS 22.261 [7], the NSCE server can monitor the slice usage status of multiple networks and make resource adjustment between different networks to realize optimized and efficient resource usage among slices of multiple networks.

#### 6.17.1.2	Multi-Network slice management capability

Figure 6.17.1.2-1 illustrates the slice management process to address the key issue 1 described in clause 5.1.

Pre-conditions:

1.	The network slice enabler layer is capable to interact with both PNI-NPN and PLMN 5GC or OAM system.

2.	PNI-NPNs are deployed as network slices of the PLMN.

Figure 6.17.1.2-1 Network slice application resource optimization process

1.	The VAL server initiates network slice resource optimization request towards the NSCE server. The request includes VAL server ID, application ID, Network ID. The message also includes preferred optimization zone.

2.	Upon receiving the request from the VAL server to make the network slice resource optimization, the NSCE server makes authentication and authorization of the VAL server and if VAL server is not authorized, the NSCE server replies with failure response.

3.	The NSCE server makes mapping from application ID that received from VAL server to slice identities (S-NSSAIs allocated in multiple networks) and retrieves the network slice related status information from 5GC or OAM of PNI-NPN networks, such as NF(s) load in 5GC and network utilization in access network as defined in TS 28.535 [11].

4.	The NSCE server retrieves the network slice related status information from PLMN networks from 5GC or OAM of PLMN networks, such as NF(s) load in 5GC and network utilization in access network as defined in TS 28.535 [11]. The PLMN operator can choose to deploy NSCE-Server acting as single entry of PLMN capability exposure which can be optional.

5.	The NSCE server verifies and analyses status data of network slice instance that received from both PNI-NPN and PLMN networks as well as network slice QoS monitoring response (optional), then NSCE server makes resource optimization decision among different kinds of networks in specific location zone.

6-7.	The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [5] to PNI-NPN network. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s).

8-9.	The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [5] to PLMN network. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s). The PLMN operator can choose to deploy NSCE-S acting as single entry of PLMN capability exposure which can be optional.

10.	The NSCE server sends the network slice resource optimization response towards VAL server.

### 6.17.2	Solution evaluation

This solution addresses the KI #1 by providing a network slice resource optimization procedure to enable the NSCE server making multiple networks resource adjustment by utilizing the network slice related management services. This solution has no impact on the 5GS architecture, and it may require the interactions between NSCE servers to retrieves the network slice related status information.

The APIs over NSCE-S interface to be defined in normative phase:

1.	APIs of network slice resource optimization request and response.

## 6.18	Solution 18: Network Slice Information Delivery

### 6.18.1	Solution description

#### 6.18.1.1	General

This solution aims to address the issues identified in Key Issue 13.

The NSCE layer provides the feature of Network Slice information delivery. The Network Slice information is necessary for the VAL server to manage the network slice for their service such as preparation, creation, activation and termination (tear-down) of network slice.

The Network Slice information that is delivered to the VAL server depends upon network operator policy. The network slice capabilities and management options offered to the customer are determined by a business agreement prior to and outside of the scope of 3GPP standards.

In initial phase of Network slice Lifecycle, the network slice is defined by Network Slice Provider (NSP). When the Network Slice is defined, the NSP with 5G system may create Network Slice Service Profile with the appropriate values, which characterize the network slice. It refers to the Generic Network Slice Template (GST) and Network Slice Type (NEST) which are standardized by 3GPP, GSMA and is called S-NEST. In addition, the NEST can be defined specifically by NSP itself, which is called P-NEST. According to NSP's policy, the Network Slice Information can be derived from the information in Network Slice - ServiceProfile.

The VAL server needs to know which the characterized Network Slice service types and capabilities it is authorized to use.  The VAL server retrieves authorized Network Slice information defined by NSP (e.g., S-NEST, P-NEST) from the NSCE server.

The NSCE layer performs the below.

-	Retrieval of Network Slice ServiceProfile in 5GS (e.g., NSMF) as specified in TS 28.532[8]

-	Conversion of Network Slice ServiceProfile (specified in TS 28.541) to Network Slice Information

-	Creation of Network Slice Information

-	Storing of Network Slice Information

-	Delivery of Network Slice Information to VAL server that the Network Slice Customer is authorized to use.

NOTE:	The Network Slice Information provided to the VAL server depends on service agreements

The VAL server as a Network Slice consumer makes use of the delivered Network Slice information for the Network Slice Lifecycle management for its service.

#### 6.18.1.2	Network Slice Information delivery

This subclause depicts the procedure of the Network Slice Information delivery to the VAL server via NSCE layer, when the VAL server requests the Network Slice Information after registration.

NOTE:	The Network Slice Information provided to the VAL server depends on service agreements out of the scope of 3GPP.

Pre-condition

1. 	The NSCE server should have the agreement with MNO (NOP) for retrieval of ServiceProfile, if the NSCE server is the external entity.

Figure 6.18.1.2-1: Network Slice Information delivery

1.	The NSCE server retrieves the Network Slice ServiceProfile from 5GS (e.g., NSMF) when the NSCE server acting as a NSP prepares a Network Slice to be provided. The NSCE server follows the procedure to request/receive the Network Slice Service Profile with 'getMOIAttributes' operation as specified in TS 28.532[8].

NOTE:	If NSCE server and NSMF are in same operator, then the NSCE server gets access directly to NSMF.      The delivered Network Slice Service Profile contains the values of attributes such as PLMN, S-NSSAI, SST, maximum number of UEs, maximum number of PDU sessions, Coverage Area, Latency, Data volume, etc which specify the Network Slice characteristics, as specified in clause of ServiceProfile in TS 28.541.

2.	The NSCE server, as Network Slice as a Service, creates and stores the Network Slice information. When NSCE server retrieves the Network Slice Information, it is necessary for NSCE server to convert the attributes in Network Slice ServiceProfile to the Network Slice information for readable information and to compose the Network Slice information, according to the NSP's policy.

In order to reduce to request often the Network Slice Information Retrieval, the NSCE server stores the Network Slice information.

3.	The VAL server requests the Network Slice Information to the NSCE server. If the VAL server needs to know the specific attribute value for its service, then the attribute name of Network Slice (e.g., S-NSSAI, SST, Coverage Area, etc.) can be added in the Request message.

4.	The NSCE server performs to check whether the requesting VAL server is registered or not. The NSCE server identifies which the Network Slice Customer is authorized to use.

5.	The NSCE server sends the Network Slice Information, if the VAL server is registered and authorized. The NSCE server rejects to the request of the Network Slice Information, if not registered.

#### 6.18.1.3	Network Slice Information subscription

This subclause depicts the procedure of the Network Slice Information delivery to the VAL server via NSCE layer, when the VAL server subscribes to the Network Slice Information after registration.

The Network Slice information that is delivered to the VAL server depends upon network operator policy. The network slice capabilities and management options offered to the customer are determined by a business agreement prior to and outside of the scope of 3GPP standards.

Figure 6.18.1.3-1: Network Slice Information subscription

1.	The NSCE server retrieves Network Slice ServiceProfile from 5GS (e.g., NSMF). The NSCE server follows the procedure to request/receive the Network Slice Service Profile with 'getMOIAttributes' operation as specified in TS 28.532[8].

NOTE:	If NSCE server and NSMF are in same operator, then the NSCE server gets access directly to NSMF. The delivered Network Slice Service Profile contains the values of attributes such as PLMN, S-NSSAI, SST, maximum number of UEs, maximum number of PDU sessions, Coverage Area, Latency, Data volume, etc which specify the Network Slice characteristics, as specified in clause of ServiceProfile in TS 28.541.

2.	The NSCE server, as Network Slice as a Service, creates and stores the Network Slice information. When NSCE server retrieves the Network Slice Information, it is necessary for NSCE server to convert the attributes in Network Slice ServiceProfile to the Network Slice information for readable information and to compose the Network Slice information, according to the NSP's policy.

In order to reduce to request often the Network Slice ServiceProfile Retrieval, the NSCE server stores the Network Slice information.

3.	The VAL server subscribes to the Network Slice Information in the NSCE server. If the VAL server needs to know the specific attribute value for its service, then the attribute name of Network Slice Information can be added in the Subscription message.

4.	The NSCE server performs to check whether the requesting VAL server is registered or not. The NSCE server identifies which Network Slice Information the Network Slice Customer is authorized to use.

5.	The NSCE server sends the authorized Network Slice Information, if the VAL server is registered. The NSCE server rejects to the request of the Network Slice Information, if not registered.

Whenever NSCE server performs the Network Slice Information Retrieval and receives the updated values to the attributes of Network Slice Service Profile, the updated Network Slice information will be notified to the VAL server, if the NSC is authorized to receive this information.

#### 6.18.1.4	Network Slice Information Notify

This subclause depicts the procedure of the Network Slice Information delivery to the VAL server via NSCE layer, in case the NSCE server notifies the Network Slice Information after subscription. The notification occurs whenever the Network Slice information is updated.

Pre-conditions:

1.	The NSCE server performs to retrieve the Network Slice Information from 5GS (e.g., NSMF). The NSCE server follows the procedure to request/receive the Network Slice Service Profile with 'subscription' operation as specified in TS 28.532[8].

The delivered Network Slice Service Profile contains the values of attributes such as PLMNInfoList, S-NSSAI, SST, maximum number of UEs, maximum number of PDU sessions, Coverage Area, Latency, Data volume, etc which specify the Network Slice characteristics, as specified in clause of ServiceProfile in TS 28.541.

2.	When NSCE server retrieves the Network Slice ServiceProfile from 5GS (e.g., NSMF), it is necessary for NSCE server to convert the attributes in Network Slice ServiceProfile to the Network Slice information for readable information and to compose the Network Slice information, according to the NSP's policy.

In order to reduce to request often the Network Slice Information Retrieval, the NSCE server stores the Network Slice information.

3.	The VAL server subscribes to the Network Slice Information to the NSCE server.

NOTE:	Network Slice Provider may define the scope and condition for notification of Network Slice information to the 3rd party.

Figure 6.18.1.4-1: Network Slice Information Notify

1.	The NSCE server receives the Network Slice Service Profile from 5GS (e.g., NSMF) as a result of subscription of Network Slice Service Profile as specified in TS 28.531.

NOTE:	If NSCE server and NSMF are in same operator, then the NSCE server gets access directly to NSMF. The delivered Network Slice Service Profile contains the values of attributes such as ServiceProfileID, PLMNInfoList, S-NSSAI, SST, maximum number of UEs, maximum number of PDU sessions, Coverage Area, Latency, Data volume, etc which specify the Network Slice characteristics, as specified in clause of ServiceProfile in TS 28.541.

When NSCE server receives the Network Slice ServiceProfile, it is necessary for NSCE server to convert the attributes in Network Slice ServiceProfile to the Network Slice information for readable information.

2.	The NSCE server updates the Network Slice information.

3.	The NSCE server sends the updated Network Slice Information to the subscribed VAL servers, such as PLMNInfoList, S-NSSAI, SST, SliceQoS, UE density and so on.

#### 6.18.1.5	Network Slice Information delivery while registration

This subclause depicts the procedure of the Network Slice Information delivery to the VAL server via NSCE layer, when the VAL server registers to the NSCE server.

Pre-conditions:

1.	The NSCE server performs to retrieve the Network Slice ServiceProfile from 5GS (e.g., NSMF). The NSCE server follows the procedure to request/receive the Network Slice Service Profile with 'getMOIAttributes' operation as specified in TS 28.532[8].

NOTE:	If NSCE server and NSMF are in same operator, then the NSCE server gets access directly to NSMF.

The delivered Network Slice Service Profile contains the values of attributes such as PLMN, S-NSSAI, SST, maximum number of UEs, maximum number of PDU sessions, Coverage Area, Latency, Data volume, etc which specify the Network Slice characteristics, as specified in clause of ServiceProfile in TS 28.541.

2.	When NSCE server retrieves the Network Slice ServiceProfile, it is necessary for NSCE server to convert the attributes in Network Slice ServiceProfile to the Network Slice Information for readable information and to compose the Network Slice information according to the NSP's policy.

In order to reduce to request often the Network Slice Information Retrieval, the NSCE server stores the Network Slice information.

Figure 6.18.1.5-1: Network Slice Information delivery while registration

1.	The VAL server requests to register to the NSCE server with the indicator of the Network Slice information request, if necessary.

2.	The NSCE server performs the registration check as specified in clause 6.7 Solution 7: network slice capability registration.

3.	The NSCE server sends the Network Slice information (e.g., PLMNInfoList, S-NSSAI, SST, SliceQoS, UE density and so on), which corresponds to the indicator of the Network Slice information, in the Registration Response message if the Registration is accepted.

### 6.18.2	Solution evaluation

The proposed solution addresses the KI #13 on Network Slice Information delivery.

The solution provides the procedure for NSCE server to operate the Network Slice information delivery to the VAL server. This allows the NSCE layer to interact with both 5GS MnS and the VAL server for retrieval of Network Slice ServiceProfile and for delivery of Network Slice information. This solution makes use of the operation of 5GS MnS defined in SA5 for retrieval of Network Slice Service Profile. The Network slice information can be delivered to the VAL server with various ways according to the deployment

The solution is feasible and viable for NSaaS scenario when the VAL server needs to be informed on the Network Slice information. The APIs of the NSCE server capabilities expect to be defined for the Network Slice Information delivery to the VAL server in the normative phase.

## 6.19	Solution 19: Slice requirements alignment capability

### 6.19.1	Solution description

#### 6.19.1.1	General

This solution aims to address the issue identified in Key Issue 11 of slice requirement alignment.

This solution provides a procedure to illustrate slice requirements alignment capability by the network slice capability enablement server in case that the initial slice requirements are not configured properly or the slice requirements need modifications to meet the adjusted service requirements.

To enable the VAL server to configure the optimal slice requirements parameters to align with the real vertical needs, the NSCE layer is able to monitor the network performance statistics and to evaluate whether the current slice requirements parameters are reasonable to achieve the maximum return of investment. To enable the slice/service self-management, the NSCE server may have the capability to generate and configure more reasonable slice requirements parameters. To avoid frequently reconfiguration of network slice requirements, the action of alignment is triggered by the evaluation of network performance statistics during a certain time period but not by a single event.

#### 6.19.1.2	Slice requirements alignment capability

Figure 6.19.1.2-1 provides a possible procedure of slice parameters alignment to address the key issue 11 described in clause 5.11.

Pre-conditions:

1.	The VAL Server has subscribed to the slice requirements alignment capability

2.	The NSCE Server has subscribed to the service of network slice performance management provided by EGMF.

3.	The NSCE server has subscribed to the service of network slice provisioning provided by EGMF.

Figure 6.19.1.2-1: Slice requirements alignment process

1.	The VAL server sends a slice requirements alignment request to NSCE server to require the NSCE server to check whether the slice requirements (by configuring the attributes of serviceProfile) matches the real network slice usage status, the request may include the S-NSSAI, the ID of the VAL server, the slice requirements parameters (attributes of serviceProfile) which is requested to be aligned.

2.	The NSCE server checks if the VAL server is authorized to trigger the service of slice requirements alignment.

3.	The NSCE server response to VAL server with the result of slice requirements alignment request, e.g., the slice requirements alignment request is accepted.

4.	On receiving the request from VAL server, NSCE server retrieves the network slice related performance data and KPIs (e.g., the average PRB usage, the distribution of the PRB usage) defined in TS 28.552[12] by utilize performance assurance management services exposed by EGMF.

5.	NSCE server compares the slice requirement parameters of network slice (the values of attributes of serviceProfile, e.g., radioSpectrum, the maxNumberofUEs) with network slice performance statistics (e.g., active number of users, the average PRB usage, the distribution of the PRB usage of the S-NSSAI) to generate the optimal slice requirements for the required service (represented by S-NSSAI) of the vertical and determine whether the network slice modification operations should be taken. This step depends on implementation (e.g., if the VAL client experience are satisfied while the network slice resources are with low utilization, the resources required in the slice requirements could be reduced).

6.	If NSCE decides to update the slice requirements, the NSCE server performs the network slice modification operation, as defined in clause 7.6, TS 28.531[5] exposed by EGMF defined in SA5, the serviceProfile will be re-configured by utilizing the modifyMOIAttributes operation.

7.	NSCE server sends the notifications to VAL server to notify the change of the slice requirements parameters.

### 6.19.2	Solution evaluation

This solution addresses the key issue #11 of slice requirement alignment. In this solution, the NSCE server interacts with VAL Server and OAM system respectively to compare the slice requirements with network current situations to estimate if the current slice requirements are properly configured. The network situation can be monitored by utilize the performance assurance related management services exposed by EGMF. The alignment of slice requirements is a semi-static operation which is not triggered by a single event but considers the network performance statistics during a certain time period. Besides, this solution has no impact on the actual business arrangement between the third-party and the MNO, which only performs adjustments within the bounds of the actual SLA.

## 6.20	Solution 20: Network slice optimization based on AF policy

### 6.20.1	Solution description

#### 6.20.1.1	General

This solution aims at providing potential procedures for KI#10. It provides a mechanism to optimize the network slice parameters for the vertical applications (by triggering the slice modification), based on network slice usage patterns and policy of application, e.g., to trigger some slices operation when the pre-configured thresholds are met.

The AF policy can be in form of a policy profile which contains the listed trigger event and expected action.

For example, the trigger events can be:

-	Slice load exceeding the threshold (a numeric value or a percentage of the maximum number),

-	A specific time period (e.g., summer vacation or spring festival).

The event list above is not exhaustive list, and just something for information.

When one or more event(s) are triggered, then network slice optimization operations are expected to be triggered.

The expected network slice optimization operations could be:

-	Network slice modification for network slice(s),

-	Or just sending out the notification.

Then some examples of policy are:

-	In June, trigger the slice modification to decrease the slice with 10%, with updated network slice related requirement (e.g. maximum number of UEs, latency, and throughput).

-	When the max number of PDU sessions is reached, trigger the slice modification to expand with 20% this parameter for slice capacity.

-	When the uplink throughput threshold is reached, trigger the slice modification to expand with 20% this parameter.

According to the trigger event, slice optimization can be triggered by OAM as describe in 6.20.1.2, triggered by NWDAF as describe in 6.20.1.3, or triggered by NSCE server itself as describe in 6.20.1.4.

#### 6.20.1.2		OAM event triggered network slice optimization

Figure 6.20.1.2 illustrates the procedure of OAM event triggered Network slice optimization.

In this clause, the AF policy is triggered by the network slice management data reported from OAM. The measured object is defined in item f) of measurement definitions (3GPP TS 32.404 [23], TS 28.552 [12]) and in item d) of KPI definitions (TS 28.554 [24]).

The policy in this clause is when the network slice is vertical owned private slice, the vertical request to only activate part of the network slice at the beginning considering the energy saving. Based on the monitored connected UE or established PDU session or Downstream Throughput, more resource could be activated by triggering the modification of uLThptPerSlice/dLThptPerSlice/maximum number of UEs/maximum number of PDU session.

Pre-conditions:

1.	The NSCE server is authorized to get network slice management data notification from OAM;

2.	The VAL server is authorized to the NSCE server for network slice optimization.

3.	There is enough network capacity when the expected action is to expand the network slice.

4.	The AF policy has been pre-configured on the VAL server based on the agreement with NSCE server which is not conflicts with the SLA.

5.	The AF policy has been provided to the NSCE server as specified in clause 6.20.1.5.

Figure 6.20.1.2: Network slice optimization triggered by OAM

1.	VAL server sends network slice optimization service subscription request to NSCE server. The request contains the policy ID indicating the OAM based trigger event. The policy in this clause is: When the max number of PDU sessions is reached, trigger the slice modification to expand slice capacity by modify the throughput/maximum number of UEs/maximum number of PDU session.

2.	To monitor the trigger event, the NSCE server translates the trigger event to service API(s) with necessary parameter, and subscribe to it by sending network slice management data subscription request to OAM. To get the monitored performance metric, the notifyThresholdCrossing as defined in TS 28.532[8] clause 11.3.1.3 which is filled in with corresponding S-NSSAI in objectInstance could be used.

3.	NSCE server sends the network slice optimization response to the VAL server to confirm network slice optimization service subscription.

4.	Upon receiving the notification which indicating the trigger event is triggered, the NSCE server performs the expected action which is to trigger the slice modification/slice allocation by sending the modifyNsi/allocateNSI request as specified in TS 28.531 [5] with updated parameter  throughput/maximum number of UEs/maximum number of PDU session as specified in the AF policy. The OAM responds back to NSCE server that the requested slice modification was successful or not.

NOTE 1:	The parameters available for modification are based on the agreement with NSCE server.

NOTE 2:	The slice modification could be done by Auto-NS-LCM as defined in clause 6.1.

5.	The NSCE server provides a network slice optimization notification to the VAL server.

NOTE 3:	There is no expectation to have constant and exact mapping between slice configuration parameters and actual traffic load of the same slice.

#### 6.20.1.3		NWDAF event triggered network slice optimization

Figure 6.20.1.3 illustrates the procedure of NWDAF event triggered Network slice optimization.

The policy in this clause is when Network Slice load predictions (Predicted Number of PDU Session establishments at the Network Slice) exceeds the threshold with high confidence, then the OAM needs to be notified to prepare for network slice modification in advance with excepted parameters including the throughput/maximum number of UEs/maximum number of PDU session.

Pre-conditions:

1.	The NSCE server is authorized to get network slice analytics notification from NWDAF;

2.	The VAL server is authorized to the NSCE server for network slice optimization.

3.	There is enough network capacity when the expected action is to expand the network slice.

4.	The AF policy has been pre-configured on the VAL server based on the agreement with NSCE server which is not conflicts with the SLA.

5.	The AF policy has been provided to the NSCE server as specified in clause 6.20.1.5.

Figure 6.20.1.3: Network slice optimization triggered by NWDAF

1.	VAL server sends network slice optimization service subscription request to NSCE server. The request contains the policy ID indicating the NWDAF based trigger event. The policy in this clause is: when Network Slice load predictions (Predicted Number of PDU Session establishments at the Network Slice) exceeds the threshold with high confidence, then the OAM needs to be notified to prepare for network slice modification in advance with excepted parameters throughput/maximum number of UEs/maximum number of PDU session.

2.	To monitor the trigger event, the NSCE server translates the trigger event to service API(s) with necessary parameter, and subscribe to the NWDAF prediction by using the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request as defined in TS 23.288[17] clause 6.1.1, and the procedures are defined in TS 23.288[17] clause 6.3.4, and clause 6.8.

3.	NSCE server sends the network slice optimization response to the VAL server to confirm network slice optimization service subscription.

4.	Upon receiving the notification which indicating the trigger event is triggered, the NSCE server performs the expected action which is to trigger the slice modification by sending the modifyNsi request as specified in TS 28.531 [5] with updated parameter throughput/maximum number of UEs/maximum number of PDU session as specified in the AF policy. The OAM responds back to NSCE server that the requested slice modification was successful or not.

NOTE 1:	The parameters available for modification are based on the agreement with NSCE server.

NOTE 2:	The slice modification could be done by Auto-NS-LCM as defined in clause 6.1.

5.	The NSCE server provides a network slice optimization notification to the VAL server.

NOTE 3:	There is no expectation to have constant and exact mapping between slice configuration parameters and actual traffic load of the same slice.

#### 6.20.1.4		NSCE server triggered network slice optimization

Figure 6.20.1.4 illustrates the procedure of NSCE server triggered Network slice optimization.

The policy in this clause is in June, triggering the slice modification/slice de-allocation to decrease the slice with 10%, with updated network slice capacity by triggering the modification of uLThptPerSlice/dLThptPerSlice/maximum number of UEs/maximum number of PDU session.

Pre-conditions:

1.	The VAL server is authorized to the NSCE server for network slice optimization.

2.	There is enough network capacity when the expected action is to expand the network slice.3. The AF policy has been pre-configured on the VAL server based on the agreement with NSCE server which is not conflicts with the SLA.

3.	The AF policy has been provided to the NSCE server as specified in clause 6.20.1.5.

Figure 6.20.1.4: Network slice optimization triggered by NSCE server

1.	VAL server sends network slice optimization service subscription request to NSCE server. The request contains the policy ID indicating the NSCE server based trigger event.

2.	The NSCE server translates pre-configured policy to trigger event(s) with necessary parameter, by monitoring the time period and slice usage.

3.	NSCE server sends the network slice optimization response to the VAL server to confirm network slice optimization service subscription.

4.	Upon the trigger event is triggered, e.g. the arrival at a specific time period (summer vacation, spring festival etc.), the NSCE server takes actions  by triggering the slice modification with parameter uLThptPerSlice/dLThptPerSlice/maximum number of UEs/maximum number of PDU session as specified in the AF policy. The OAM responds back to NSCE server that the requested slice modification was successful or not.

NOTE 1:	The parameters available for modification are based on the agreement with NSCE server.

NOTE 2:	The slice modification could be done by Auto-NS-LCM as defined in clause 6.1.

5.	The NSCE server provides a network slice optimization response to the VAL server.

NOTE 3:	There is no expectation to have constant and exact mapping between slice configuration parameters and actual traffic load of the same slice.

#### 6.20.1.5		AF policy provisioning

Figure 6.20.1.5 illustrates the procedure of AF policy provisioning from VAL to NSCE server.

Figure 6.20.1.5: AF policy provisioning

1.	VAL server sends AF policy provisioning request to NSCE server. The request contains the policy, VAL server ID and S-NSSAI.

2.	The NSCE server checks whether the policy is conflict with the service profile and other pre-configured policy. If yes, the request could be rejected.

3.	NSCE server sends the AF policy provisioning response to the VAL server to indicating whether the request is   successful or not. If it is successful, policy ID is provided to VAL server.

### 6.20.2	Solution evaluation

The solution addresses the key issue #10. In this solution, the NSCE server optimizes the network slice for the vertical applications based on AF policy. After receiving the policy from AF, the NSCE Server may have interactions with 5GC and OAM system to monitor the network slice, and then trigger the network slice modification by utilize the network slice related management services exposed by EGMF defined in SA5. This solution is based on the capacity supported by SA2 and SA5 and does not introduce impact on 5GS architecture. While the SA6 SEAL will possibly need enhancement to support the interaction with 5GS and OAM system. The APIs interface needs to be defined in normative phase:

1.	APIs of policy/service subscription of network slice modification.

2.	APIs to provide the AF policy to NSCE server.

## 6.21	Solution 21: Solution on predictive slice modification in edge based NSCE deployments

### 6.21.1	Solution description

#### 6.21.1.1	General

The solution addresses the key issue #12 and in particular the open issues:

-	How could the NSCE server deployed inside the EDN interact with the NSCE server outside the EDN? Whether and how could the NSCE server interact with other NSCE server?

-	Whether and how does SEAL need to be enhanced to support NSCE client to interact with NSCE server in the EDN and NSCE server outside the EDN?

This scenario targets mainly deployments where the slice enabler is deployed at the edge and the migration to different DN will require that the ongoing slice is supported at the target area to ensure meeting the application session requirements. The slice parameters monitoring at the target area (e.g. for per NSI/NSSI resource situation) need to be known at the server NSCE server to allow for pro-active slice (or slice subnet) modification trigger to avoid degradation of the application service performance.

In this procedure, the NSCE server initially receives an expected/predicted UE location/mobility change request outside an EDN service area for one or more UEs within the VAL application session (e.g. such session can be an indirect V2V session or a multiplayer gaming session). Then, service or target NSCE server checks with 5GS (OAM, 5GC) whether the serving slice is available and can offer the same performance at the target EDN. Thereafter, NSCE server evaluates the need for a slice modification (e.g. a slice lifecycle related trigger change) e.g. a slice subnet resource adaptation to allow for optimizing the application performance at the target area. Based on this decision/recommendation, it provides the action to the OAM and supports the re-mapping of NSCE server for the NSCE client proactively, before UE mobility happens.

#### 6.21.1.1	Procedure

Figure 6.21.1.2-1 illustrates the solution to Key Issue #12 on network slice capability exposure in the edge data network to allow for slice modification when a vertical application migrates to a different EDN supported by different NSCE server.

Pre-conditions:

1.	The VAL server has subscribed to the network slice capability enablement server

2.	Initially, VAL client of VAL UE is mapped to Slice#1, and NSCE client of VAL UE has established a connection to NSCE server#1 (S-NSCE server).

3.	The S-NSCE server has already discovered the T-NSCE server and its area of coverage.

Figure 6.21.1.2-1: Support for predictive slice modification in distributed NSCE server deployments

1a/1b. The VAL server sends to S-NSCE server a VAL application service continuity requirement due to predicted/expected UE or group UE mobility to an area covered by a different EDN. Such UE predicted mobility can be based on analytics received by NWDAF or ADAES or can be predicted by the VAL layer (VAL server or VAL UE).

2.	S-NSCE server acting as AF, may interact with 5GC to query the UE specific information (location, UE connection capabilities) as well as network conditions (network monitoring from NEF) and/or slice related analytics on the slice load (from NWDAF as specified in TS 23.288).

3.	S-NSCE server may also interact with OAM to query on the up to data configured slice parameters e.g. slice RRM policies, modification of the NSI/NSSI resources (see TS 28.531[5], 5.1.12) at the target area and measurements for the slice at the target area.

4.	S-NSCE server checks whether new NSCE service area supports slice #1 and if slice #1 offers similar performance in target area. Here we assume that the S-NSCE is aware of the per application QoS requirements / KPIs (or has acquired this information from the VAL server).

If the current slice doesn't fulfil these requirements, S-NSCE interacts with T-NSCE server (by sending the VAL application service continuity requirement and optionally a proposed action) to negotiate and jointly determine the need for a slice lifecycle change at the target area and translates this to a trigger action towards OAM (slice provisioning MnS producer). This trigger action can be the outcome of the negotiation and can be a requested slice modification or the slice #1 creation/instantiation at the target area (this may happen if a group of UEs are moving to the target area and use slice #1, so it may be beneficial to create slice #1 at the target area). Such trigger action can be generated by either the S-NSCE server or the T-NSCE server based on the deployment.

5a/b.Then, S- or T-NSCE server may send the trigger action to the slice provisioning MnS at OAM (e.g. slice modification for network slice) based on the expected/predicted VAL UE or VAL group mobility. Following the MnS provides a response with a positive or negative result.

6.	After the slice lifecycle change execution (based on the indication in 5b), if the NSCE client needs to be remapped to different NSCE server (due to the expected change of UE location), the NSCE client establishes a new connection with T-NSCE and terminates the one with S-NSCE (in case of subscription-based interaction), or in case of request-based interaction, it updates the mapping at the client side, and maintains the new NSCE server address / ID and may also request for report configuration information for the target NSCE area.

### 6.21.2	Solution evaluation

The solution addresses the key issue #12 on network slice capability exposure in the edge data network, and in particular the interaction between NSCE servers (deployed at different EDNs) when an expected migration happens for a UE or group of UEs from an EDN to another. Solution #X requires interaction with 5GC (acting as trusted AF) for querying network/slice condition at the target area and the interaction with OAM / slice provisioning MnS producer for consuming OAM services.

The solution is feasible and viable mainly for the NSaaS scenario (where the NSCE server is the NSP and the VAL server is the NSC) and proposes the enhancement NSCE server capabilities to deal with scenarios when NSCE servers are deployed at the edge / distributed and a trigger can be related to UE/group predicted mobility from one edge area to another.

## 6.22	Solution #22: Slice policy and configuration alignment

### 6.22.1	Solution description

#### 6.22.1.1	General

This solution addresses the KI 11 bullet on resolving misalignments between VAL server policy (e.g., requirements provided when adding or changing slice) and the NSCE service provider policy (NSPP).

The VAL server policy can be in the form of a profile or configuration which the VAL server requests to be applied or maintained by NSCE, e.g., a profile including the configuration of several services using the same network slice. Alternatively, the VAL server policy can be provided together with the request for an action, e.g., adding a network slice. When the action requested also includes a VAL server policy to be applied, it needs to be aligned with the existing network slice policies available at NSCE i.e., NSPPs.

#### 6.22.1.2		Procedure

Figure 6.22.1.2 illustrates the procedure for slice lice policy and configuration alignment

Pre-conditions:

1.	The NSCE service provider policy (NSPP) is available at the NSCE Server.

NOTE 1:	It is assumed that NSPP includes ranges of NS capabilities which can be adapted via NSCE, along with conditions under which such adaptations are allowed. This policy is used by NSCE also in other TR solutions to determine whether parameters in NS management operations are within allowed ranges, e.g., clause 6.10.2.1 step 10, fig 6.11.1.2-1 steps 2 and/or 3, fig 6.11.1.2-2 steps 2 and/or 3, clause 6.13.1.2 step 3, clause 6.13.1.2 step 3, clause 6.17.1.2 step 5., clause 6.19.1.2 step 5., etc.

2.	The VAL server is authorized to receive NSCE services.

Figure 6.22.1.2: Policy alignment

1.	VAL server sends a provisioning or action request to NSCE server. The request contains the VAL policy.

NOTE 2:	The request in this step is described generically and can correspond to one or more requests to be defined in the normative phase to support other solutions. For example, the request corresponds to clause 6.1.1.2 step 1, clause 6.9.1.2 step 1, clause 6.10.1.2 step 8, clause 6.11.1.1 step 1, clause 6.11.1.2 step 1, clause 6.13.1.2 step 2, clause 6.17.1.2 step 1 etc. The response in step 3 is to be defined in a complementary manner.

2.	The NSCE server checks whether the provided VAL policy conflicts with the NSCE service provider policy. If not, the action requested, if any, is performed.

If the VAL policy conflicts with the NSCE service provider policy, the NSCE server can determine parameters harmonizing the two configurations. The NSCE server determines parameters harmonizing the configurations if previously authorized.

NOTE 3:	In the normative phase, this step can be implemented, for example, as part of the following: clause 6.1.1.2 steps 2, 6 or 9, clause 6.10.2.1 step 10, fig 6.11.1.2-1 steps 2 and/or 3, fig 6.11.1.2-2 steps 2 and/or 3, clause 6.13.1.2 step 3, clause 6.13.1.2 step 3, clause 6.17.1.2 step 5., clause 6.19.1.2 step 5., etc.

3.	NSCE server sends the VAL server a response indicating whether the VAL policy conforms with NSPP. If the policies conflicted, the NSCE server provides optional parameters values that allow the policies to be harmonized. If the policies did not conflict, the response indicates that there is alignment. If an action was also requested in step 1, the result of the action can be provided in the response.

4.	The NSCE Server can provide a notification to 5GS with the result of the compare and harmonize procedure.

NOTE 4:	In the normative phase the specification of this step is to be aligned with SA5 specifications

### 6.22.2	Solution evaluation

The solution addresses key issue #11 on slice requirement alignment by addressing misalignments between VAL server policy (e.g., requirements provided when adding or changing slices) and the NSCE service provider policy. The solution provides clarity to the how requirements and parameters from the two different domains may be harmonized/ aligned, in order to allow other NSCE functions to be performed.

In the normative phase, this solution can be used to provide clarifications and enhancements to a number of procedures. Alternatively, this solution can be implemented as a stand-alone process similar to the one described in clause 6.19., with the slice requirement updates resulting mainly from interactions between VAL Server and NSCE Server (not from interactions between NSCE and EGMF, as captured in the clause 6.19. solution),

The solution in this clause can be used to enhance request/response messaging and procedural steps specified within the scope of other solutions. Therefore, the solution is feasible and can be used to complement other key issues.

# 7	Identities and commonly used values

## 7.1	General

The following clauses list identities and commonly used values that are used in this technical report.

## 7.2	VAL server ID

The VAL server ID uniquely identifies the VAL server.

## 7.3	NSCE server ID

The NSCE server ID uniquely identifies the Network Slice Capability enablement server.

## 7.4	NSCE client ID

The Network Slice Capability enablement client ID identifies a particular NSCE client.

## 7.5	VAL client ID

The VAL Client ID identifies the client side of a particular vertical application.

In case that the UE is running mobile OS, the Application Client ID is a pair of OSId and OSAppId.

## 7.6	UE ID

The UE ID uniquely identifies a particular UE within a PLMN domain. Following identities can be used:

-	GPSI, as defined in 3GPP TS 23.501 [02].

## 7.7	slice coverage area

The slice coverage area is the area where the network slice is available in the whole PLMN or in one or more Tracking Areas of the PLMN.

## 7.8	NSCE service area

The NSCE service area is the area where the Network Slice Capability Enablement server owner provides its services. It is equal to the collection of coverage area of slices it can enable.

The NSCE service area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. Geographical coordinates) or both.

NOTE:	The NSCE server service area is not smaller than the collection of slice(s) coverage area(s) the NSCE server can enable.

# 8	Overall evaluation

## 8.1	General

The following clauses contain an overall evaluation of the solutions presented in this technical report, their applicability to the identified key issues and dependencies on other working groups which will need consideration.

## 8.2	Solution evaluations

### 8.2.1	General

All the key issues and solutions specified in this technical report are listed in Table 8.2.1-1. It includes the mapping of the key issues to the solutions and corresponding solution evaluations. It also lists the dependencies on other working groups that will need consideration during the Release 18 normative phase.

Table 8.2.1-1: Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Dependency on other working groups |
| --- | --- | --- |
| KI 1: Network slice capability management enhancements | Solution 12: SEAL enhancement |  |
|  | Solution 15: UE triggered network slice adaptation |  |
|  | Solution 16: Multi-Network slice management capability |  |
|  | Solution 17: Multi-Network slice resource optimization |  |
| KI 2: Application layer exposed network slice lifecycle management | Solution 1: Automatic application layer network slice management |  |
| KI 3: Discovery & registration aspects for management service exposure | Solution 7: Network slice capability registration |  |
|  | Solution 8: Discovery of management service exposure | SA 5 |
| KI 4: Network slice fault management capability | Solution 2: Network slice fault management capability |  |
| KI 5 : Communication service management exposure | Solution 11 :Communication service management exposure |  |
| KI 6: Application layer QoS verification capability enablement | Solution 4: QoS verification capability |  |
| KI 7: Network slice related performance and analytics exposure | Solution 5: Network slice related performance and analytics exposure |  |
| KI 8: Support for requirements translation | Solution 3: Slice API configuration and translation | SA 5 |
| KI 9: Support for trust enablement | Solution 6: VAL server authorization and authentication via slice enabler layer | SA 3 |
| KI 10: Support for managing trusted third-party owned application(s) | Solution 9: Support for managing trusted third-party owned application(s) | SA 2 |
|  | Solution 10: Network slice application policy management capability |  |
|  | Solution 20: Network slice optimization based on AF policy |  |
| KI 11: Slice requirement alignment | Solution 19: Slice requirements alignment capability |  |
| KI 12: Network slice capability exposure in the edge data network | 6.14 Solution 14: Interaction between the NSCE servers. |  |
|  | Solution 21: Solution on predictive slice modification in edge based NSCE deployments |  |
| KI 13: Delivery of the existing Network Slice information to the trusted third-party | Solution 18: Network Slice Information Delivery |  |
| KI 14: Network Slice creation to the third-party and UE | Solution 13: Network Slice Allocation by VAL server |  |

### 8.2.2	Overall Evaluation for KI#1

This clause provides an overall evaluation of the key issue #1 Network slice capability management enhancements.

The Solution #12 analyzes potential new services that could enhance the SEAL and the potential enhancement to SEAL NSCE architecture.

The solution #15 provides enhancements to the existing NSCE client functionality by separating the network slice adaptation subscription/notification functionality from the slice adaption triggering.

The solution #16 and solution #17 provides enhancements to the NSCE functionality by supporting the NSCE services including the network slice monitoring and network slice optimization in multi-Networks. Detailed signalling and APIs of making multiple networks resource adjustment and slice monitoring are to be addressed in normative phase.

Solution #15, solution #16 and solution #17 could be considered in the normative work.

### 8.2.3	Overall Evaluation for KI#10

This clause provides an overall evaluation of the key issue #10 supporting for managing trusted third-party owned application(s).

Solution #9 provides a possible procedure of the VAL server requesting to manage network slice quota by providing specific type of action to take (e.g. release low priority users as identified by 5GC, release list of users as identified by the VAL server, etc) and optionally list of UEs on which specific action to apply to NSCE Server. While the further action is dependent to the capability of SA2.

The Solution #10 illustrates the process of the VAL server requesting to manage network slice when reaching UEs slice quota threshold. The slice adjustment is mainly to modify the network slice and the decision is made based on network status information from 5GS.

Solution #20 illustrates the process of optimizing the network slice (modify the network slice ) based on policy of application, which could be triggered by not only predicted network slice status, but also network slice management data.

The solution #10 and solution#20 could be considered to be merged during the normative work, and the detailed APIs are to be addressed in normative phase.

Whether the Solution #9 is going to be considered in the normative work depends on the reply LS from SA2.

### 8.2.4	Overall Evaluation for KI#12

This clause provides an overall evaluation of the key issue #12 Network slice capability exposure in the edge data network. The solution #14 and solution #21 are complementary to each other.

The solution #14 is applicable when the monitoring of network slice supported by different edge deployed NSCE servers is needed to form an overview of the whole network.

The solution #21 is to optimize the application performance when the one or more UEs is predicted to migrate between different NSCE server.

Both solutions need the support of interaction between the NSCE servers, and the detailed API could be considered in the normative work.

## 8.3	Architecture evaluations

The architecture specified in clause 4.2 describes the application architecture for network slice capability enablement.

A summary of the architecture and key issues specified in this technical report are listed in Table 8.3-1.

Table 8.3-1: Architecture evaluation

| Architecture solution | Applicable key issues (clause reference) | Dependency on other working groups |
| --- | --- | --- |
| 4 Application architecture for network slice capability enablement | Supports all key issues specified in clause 5 | SA2 and SA5 |

The architecture is compliant to all the architectural requirements listed under clause 4.1.

# 9	Conclusions

This technical report fulfills the objectives of the study on application architecture for enabling Network Slice Capability Exposure. The report includes the following:

1.	Definition of terms and abbreviations used in the study (clause 3);

2.	Architectural requirements and detailed application architecture for enabling Network Slice Capability Enablement (clause 4);

3.	Deployment model and business model in Annex

3.	Key issues identified by the study (clause 5);

4.	Individual solutions addressing the key issues (clause 6);

5.	A list of identities and commonly used values (clause 7); and

6.	Overall evaluations of all the solutions (clause 8);

Some of the individual solutions have dependency on other working groups within 3GPP. This dependency is summarized in overall evaluations (clause 7).

For NSCALE in normative work in 3GPP Rel-18, it is recommended to define：

1.	Terms and abbreviations, the definition of terms and abbreviations captured in clause 3 will be reused;

2.	Common attributes/parameters provided by NSCE, the list of identities and commonly used values captured in clause 7 will be reused with appropriate enhancements;

3.	Requirements on NSCE, the architectural requirements identified in clause 4 will be used as baseline architectural requirements;

4.	Application architecture for enabling Network Slice Capability Exposure, the architectures as specified in clause 4 will be used as baseline architecture;

5.	APIs and attributes/parameters thereof provided by NSCE, following individual solutions, corresponding to the key issues, will be considered as candidate solutions:

a.	for Key issue #2 (Application layer exposed network slice lifecycle management):

i.	Solution #1 (Automatic application layer network slice management);

b.	for Key issue #3 (Discovery & registration aspects for management service exposure	):

i.	Solution #7 (Network slice capability registration);

ii.	Solution #8 (Discovery of management service exposure);

c.	for Key issue #4 (Network slice fault management capability):

i.	Solution #2 (Network slice fault management capability);

d.	for Key issue #5 (Communication service management exposure):

i.	Solution #11 (Communication service management exposure);

e.	for Key issue #6 (Application layer QoS verification capability enablement):

i.	Solution #4: QoS verification capability;

f.	for Key issue #7 (Network slice related performance and analytics exposure):

i.	Solution #5 (Network slice related performance and analytics exposure);

g.	for Key issue #8 (Support for requirements translation):

i.	Solution #3 (Slice API configuration and translation);

h.	for Key issue #9 (Support for trust enablement):

i.	Solution #6 (VAL server authorization and authentication via slice enabler layer);

i.	for Key issue #10 (Support for managing trusted third-party owned application(s)):

ii.		Solution #10 (Network slice application policy management capability);

iii.	Solution #20(Network slice optimization based on AF policy);

j.	for Key issue #11 (Slice requirements alignment):

i.		Solution #19 (Application layer slice SLA alignment);

k.	for Key issue #12 (Network slice capability exposure in the edge data network):

i.		Solution #14 (Interaction between the NSCE servers;

ii.		Solution #21 (Predictive slice modification in edge based NSCE deployments);

l.	for Key issue #1 (SEAL enhancement):

i.	Solution #15 (UE triggered network slice adaptation)

ii.	Application architecture;

iii.	Solution #16 (Multi-Network slice management capability);

iv.	Solution #17 (Multi-Network slice resource optimization);

m.	for Key issue #14 (Network Slice creation to the third-party and UE):

i.	Solution #13 (Network Slice Allocation by VAL server);

n.	for Key issue #13 (Delivery of the existing Network Slice information to the trusted third-party):

i	Solution #18 (Network Slice Information Delivery).

Individual solutions, not listed under bullet 5 may be adopted in technical specification with appropriate enhancements;

6.	Potential enhancement of SEAL services, the enhancement to the SEAL service is summarized in solution 12. The normative work of SEAL enhancement will take solution 12 and the conclusion of corresponding solutions into consideration.

Editor's Note: There is unconcluded solution to be dealt with; Section 9 Conclusion may be updated.

Annex A	Deployment models

## A.1	Deployment scenarios

### A.1.1	General

In addition to SA2 and SA5 network slicing capabilities, the NSCE service provides application layer enablement to support the network slice management and control in the granularity of slice which is identified by S-NSSAI.

A network slice can have only one owner and one NSCE service provider. NSCE service provider and slice owner can be different. For example the slice owner is VAL server, but the NSCE service provider is MNO.

This clause describes examples of deployment models with respect to different deployment scenarios as follows.

### A.1.2	Centralized NSCE deployment

Figure A.1.2 shows a fundamental deployment of NSCE server whose service area covering the whole PLMN. It is also possible slice coverage area to be smaller than the NSCE service area. One NSCE server can also be responsible for local slices covering only small service areas.

The network slice capability enablement service is provided with the view of whole PLMN in this scenario.

Figure A.1.2: Illustration of centralized NSCE deployment

### A.1.3	Distributed NSCE deployment

The distributed deployment refers to the deployment model in which the service area only covers some specific areas as shown below (based on geographical coordinates or TA list(s)).

Figure A.1.3: Illustration of distributed NSCE deployment

Examples of distributed deployment including NPN NSCE deployment and edge NSCE deployment which are shown below in A.1.3.1 and A.1.3.2.

When there are multiple NSCE servers managed by same provider, NSCE server(s) may be subscribed for providing the network slice statistics to another NSCE server to provide a global view.

There can be two use cases to provide the NSCE service in the distributed deployment:

One use case is that the edge/NPN deployed NSCE is about a slice service area which is equivalent to the edge/NPN area. For this scenario, if the edge/NPN deployed NSCE wants to access the NEF/NWDAF/NSACF services or to receive policies from OAM, it needs to interact to the global NSCE.

A further use case could be that some NSCE services (e.g. MnS discovery) are locally provided to edge -native VAL servers (for example as a micro-service), whereas other capabilities are provided for the whole PLMN area. So, the edge/NPN NSCE includes a subset of capabilities which are edge native. The local deployment of such capabilities can allow for more efficient services to the edge deployed VAL servers (e.g. for QoS verification, the edge deployed NSCE can receive more timely KQI/QoE measurements and can process them locally before triggering an event).

### A.1.3.1	NPN NSCE deployment

Figure X.1.3.1 shows the NPN NSCE server deployment. This case is valid if a geographical match between slice coverage area, NPN coverage area and NSCE service area is pre-configured. The matching may be pre-configured by network operator based on the TA list. The service area of the NSCE server equals to the area of the NPN in this case.  The slice(s) coverage area may equals to the area of the NPN, but it is also possible slice coverage area to be smaller than the NPN service area.

The NSCE server is deployed in Non-public network to provide the network slice capabilities exposure application service based on the interaction with NPN-5GC and NPN-OAM.

Figure A.1.3.1: Illustration of NPN NSCE deployment

### A.1.3.2	Edge NSCE deployment

Figure X.1.3.2 shows the edge NSCE deployment cases when the NSCE server is deployed in the EDN using LADNs as described in Annex A.2.4 of TS 23.558[x]. This case is valid if a geographical match between slice coverage area, LADN service area (which is EDN service area) and NSCE service area is pre-configured. The matching can based on the TA list or geographical coordinates. The service area of the NSCE server equals to the area of the LADN in this case. The slice(s) coverage area may equals to the area of the LADN(EDN), but it is also possible slice coverage area to be smaller than the LADN(EDN) service area.

The NSCE server is deployed in EDN to provide the network slice capabilities exposure application service based on the interaction with EDN-5GC and EDN-OAM.

Figure A.1.3.2: Illustration of edge NSCE deployment

## A.2	Deployment of NSCE server(s) in relation to VAL server and 3GPP system

To support the centralized/distributed, the NSCE server(s) will have different deployment models and different relation with VAL server and 3GPP system. This clause describes examples of deployment models of NSCE server(s) in relation to VAL server and 3GPP system.

Figure X.2-1 illustrates the centralized NSCE deployment. The NSCE server can be deployed in PLMN domain by MNO, or deployed in VAL service provider domain by vertical.

Figure A.2-1: Illustration of centralized NSCE deployment

Figure A.2-2 illustrates the distributed NSCE deployment. The NSCE servers can be deployed in PLMN domain by MNO, deployed in VAL service provider domain by vertical, or deployed in 3rd party domain by3rd party. The VAL server can communicate with multiple NSCE servers via NSCE-S as long as other NSCE servers are discovered and accessible. Or, the VAL server can communicate with other NSCE servers via NSCE-E if needed.

Figure A.2-2: Illustration of distributed NSCE deployment

Annex B (informative): Business models and relationships for NSCALE

## Annex B.1: Relevance to SA5 models

According to TS 28.530, the roles related to 5G networks and network slicing management include among others:

-	Communication Service Provider (CSP): Provides communication services. Designs, builds and operates its communication services. The CSP provided communication service can be built with or without network slice.

-	Communication Service Customer (CSC): Uses communication services.

-	Network Operator (NOP): Designs, builds and operates networks and provides related services, including network services and network slices.

Also, according to TS 28.530: "In case of Network Slice as a Service (NSaaS) (cf. clause 4.1.6), the Communication Service Provider (CSP) role can be refined into NSaaS Provider (NSaaSP) role – or, in short, Network Slice Provider (NSP) - and the Communication Service Customer (CSC) role can be refined into NSaaS Customer (NSaaSC) role – or, in short, Network Slice Customer (NSC). A NSC can, in turn, offer its own communication services to its own customers, being thus CSP at the same time. A tenant might take the role of a NSC."

Figure 4.1.6.1 of TS 28.530 illustrates some examples on how network slices can be utilized to deliver communication services, including network slice as a Service. For simplicity this figure omits the details of how NFs are being managed and does not show their groupings into network slice subnet:

a)	A Network Slice as a Service (NSaaS) is provided to CSC-A by CSP-A. Unlike the communication service delivered to end customers, in NSaaS, the offered service is the actual network slice.

b)	CSC-A can use the network slice obtained from CSP-A to support own Communication Services or may add additional network functions to the obtained NSaaS and offer the resulting combination as a new network slice to CSP-B. In this case, CSC-A plays the role of NOP-B and builds his own network. The network slice obtained by CSC-A from CSP-A becomes a "building block" or a network slice subnet of CSC-A in its role of NOP-B. The NOP-B (a.k.a. CSC-A) combines this network slice subnet with other network slice subnets and offers the new network slice subnet as network slice to CSP-B.

c) 	CSP-B can use the network slice obtained from CSC-A / NOP-B to deliver communication services to its end customers (as CSC-B).

Figure 4.1.6.1: Examples of Network Slice as a Service (NSaaS) being utilized to deliver communication services to end customers [TS 28.530]

[Observation B.1-1] For the NSaaS model, NSCE can be defined as an NSC of the NSP, which in turn offers its own communication services to its own customers, being thus CSP at the same time.

[Observation B.1-2] A Network Slice in NSaaS model can be combined with additional functions, and a new slice can be provided by the CSC-A which acts as NOP-B. In such model, the NSCE server can provide enablement services for the new slice provided by the NOP-A customer.

[Observation B.1-3] It is not necessary that the NOP-B "slice parameters" are the same as the NOP-A "slice parameters"; hence the slice areas for the resulting slice can be different from the slice area provided by NOP-B/CSC-A.

TS 28.530 also specifies the "network slices as NOP internals" model. In this model, the network slices are not part of the NOP service offering and hence are not visible to its customers. However, the NOP, to provide support to communication services, may decide to deploy network slices, e.g. for internal network optimization purposes. This model allows CSC to use the network as the end user or optionally allows CSC to monitor the service status (assurance of the SLA associated with the internally offered network slice). The CSP should be able to provide the service status information (e.g. service performance, fault information, traffic data, etc) to CSC via the management exposure interface.

Figure 4.1.7.1: Examples of network slice as NOP internals [TS 28.530]

[Observation B.1-4] For the network slices as NOP internals model, the NSCE server can be only defined as part of NOP to translate the slice specific requirements to communication service requirements and vice versa.

## Annex B.2: Business relationships

NSCE layer provides value added services to VAL customers, based on consuming 5GS services related to slicing (from OAM, 5GC) and based on interacting with the VAL UE side. The variety of services and the deployment aspects depend on the different assumptions for the slice owner / provider, the slice customer and the enablement service provider. With respect to NSCALE, the NSCE server can play different roles based on the business models. For example, NSCE server can be:

-	deployed by NOP / MNO

-	deployed by an Edge / Cloud Provider, as a trusted 3rd party

-	deployed by a vertical industry, which can be the end slice customer

From business perspective, if the NSCE server is not part of vertical or MNO, the following business model apply. In Figure X.2-1, the different interactions among all the involved entities are provided. More specifically, in this model the end user is the consumer of the applications provided by the vertical/ASP and can have app-level service agreement with vertical/ASP(s). The end user/UE also has a PLMN subscription arrangement with the MNO. The UE used by the end user is allowed to be registered on the MNO's network. MNO (via OAM) can have a slice SLA with the vertical / ASP, which is optional when vertical customer is the NSC. In addition, due to the involvement of a NSCE service provider, additional agreements can be possible between the NSCE server and VAL/ASP layer and the NOP/MNO(s):

-	the enablement service agreement between VAL/ASP layer and the NSCE service provider include the agreement on the value-added services, which in case on NSaaS these are services related to the consumed slice from NOP. So, the end customer (VAL) subscribes to NSCE server for receiving additional services for optimizing the slice utilization. In case that the NSCE server is a NSP towards VAL customer, then such agreement can relate to slice SLA (for the slice provided by the NSCE server).

-	the service agreement between MNO and NSCE service provider is for consuming 5GS services (and being also authorized and trusted to provide additional services on top). Such agreement could be also a slice SLA for the scenarios when NSCE server is the NSC of the MNO (in NSaaS model).

Figure B.2-1: Business relationships

Annex C:Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44-e |  |  |  |  | TR skeleton | 0.0.0 |
| 2021-07 | SA6#44-e |  |  |  |  | TR Skeleton agreed in SA6#44: S6-211848 Implemented pCRs approved in SA6#44: S6-211849，S6-211850，S6-211851，S6-211782 Editorial changes by the rapporteur | 0.1.0 |
| 2021-09 | SA6#45-e |  |  |  |  | Implemented pCRs approved in SA6#45: S6-212161，S6-212162，S6-212080，S6-212079，S6-211949，S6-211965，S6-211967，S6-212163，S6-212144，S6-212069 Editorial changes by the rapporteur | 0.2.0 |
| 2021-10 | SA6#45-bis-e |  |  |  |  | Implemented pCRs approved in SA6#45-bis: S6-212202, S6-212204, S6-212207, S6-212210, S6-212474, S6-212400 Editorial changes by the rapporteur | 0.3.0 |
| 2021-11 | SA6#46-e |  |  |  |  | Implemented pCRs approved in SA6#46: S6-212765, S6-212761, S6-212763, S6-212597, S6-212536, S6-212546, S6-212726, S6-212727, S6-212733, S6-212736, S6-212764, S6-212724, S6-212725, S6-212755, S6-212770; Editorial changes by the rapporteur. | 0.4.0 |
| 2022-02 | SA6#47-e |  |  |  |  | Implemented pCRs approved in SA6#47: S6-220421, S6-220098, S6-220338, S6-220305, S6-220097, S6-220307, S6-220445, S6-220446, S6-220447, S6-220126, S6-220334, S6-220448, S6-220337, S6-220339, S6-220308, S6-220322. Editorial changes by the rapporteur. | 0.5.0 |
| 2022-03 | SA#95-e | SP-220091 |  |  |  | Presentation for information at SA#95-e | 1.0.0 |
| 2022-04 | SA6#48-e |  |  |  |  | Implemented pCRs approved in SA6#48: S6-220783, S6-220793, S6-220941, S6-220942, S6-220786, S6-220787, S6-220943, S6-220791, S6-220812, S6-220603, S6-220815, S6-220816, S6-220940 Editorial changes by the rapporteur. | 1.1.0 |
| 2022-05 | SA6#49-e |  |  |  |  | Implemented pCRs approved in SA6#49: S6-221454,S6-221281,S6-221053,S6-221063,S6-221341, S6-221291, S6-221292, S6-221293, S6-221294, S6-221295, S6-221296, S6-221297, S6-221455,S6-221456, S6-221151, S6-221314, S6-221348 Editorial changes by the rapporteur. | 1.2.0 |
| 2022-06 | SA6#49-bis-e |  |  |  |  | Implemented pCRs approved in SA6#49-bis: S6-221962, S6-221961, S6-221963, S6-221964, S6-221965, S6-221966, S6-221967, S6-221602, S6-221606, S6-221645, S6-221823, S6-221786, S6-221863 | 1.3.0 |
| 2022-09 | SA6#50-e |  |  |  |  | Implemented pCRs approved in SA6#49-bis: S6-222082, S6-222804, S6-222153, S6-222188, S6-222187, S6-222462, S6-222190, S6-222191, S6-222192, S6-222193, S6-222194, S6-222463, S6-222464, S6-222574Editorial changes by the rapporteur. | 1.4.0 |
| 2022-09 | SA#97-e | SP-220911 |  |  |  | Presentation for approval at SA#97-e | 2.0.0 |