3GPP TSG SA Meeting #97e	SP-220937

- 13 - 19 September 2022, e-meeting

Source: 	SA1 (from S1-222345)

Title:		New WID on Minimization of Service Interruption During Core Network Failure Phase 2

Document for:	Approval

Agenda Item:		6.8

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Minimization of Service Interruption During Core Network Failure Phase 2

Acronym:	MINT_Ph2

Unique identifier:	970041

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830018 | Study on Support for Minimization of Service Interruption | This SID studies the minimization of service interruption during RAN failure. |
| 850036 | Support for Minimization of service Interruption | The WID specifies normative service requirements coming out of the study on Support for Minimization of service Interruption, which is to deal with RAN failure. This can be considered as "MINT Phase 1" |

Dependency on non-3GPP (draft) specification: N/A

# 3	Justification

In 5G system, ensuring the availability of communication service is critical. TS 22.261 highlights the need for disaster mitigation services when a 5G system fails to serve its users due to RAN failure. TS 22.261 also specifies the requirements of Disaster Roaming where a user that is applicable to Disaster Condition roams to another PLMN to resume connectivity service. However, the requirements of supporting communication service under core network failure are not covered. Therefore, it is proposed that two core network failure scenarios and their mitigation service requirements be included in TS 22.261. It is noted that any core network failure that leads to not operational user authentication is excluded as user authentication is required for disaster mitigation services.

In order to avoid the repeated construction of 5G network infrastructure and save cost, multi operator core network (MOCN) network sharing scenario, where only the RAN is shared, has been introduced to 5G system. With the shared-RAN configuration, when one operator’s 5G core network fails, the service interruption can be minimized by temporarily using the service provided by the core network of the cooperative operator.

National roaming is allowed in some regions to accelerate 5G rollout. That is, a UE roams into a 5G VPLMN which has a roaming agreement with the UE's 5G HPLMN, and the HPLMN and the VPLMN are in the same country. It is noted that the UE can also be an international roamer who has a 5G-only roaming access to the 5G VPLMN. Subject to roaming agreement, national roaming may be forbidden in EPS (i.e. a UE can register with a 5G VPLMN but it cannot register with the EPC of this VPLMN). The legacy 4G core network which is not virtualized and implemented isolated from 5G can function well when 5G core network fails. Such legacy 4G deployment is particularly common in the areas where 5G national roaming is allowed (e.g. rural areas). When a Disaster Condition applies to the VPLMN, the UE shall be allowed to register with the EPC of the VPLMN to resume 4G connectivity service.

# 4	Objective

Update the stage 1 specifications to support disaster mitigation services in shared-RAN and national roaming scenarios including disaster-roaming onto 4G.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 22.261 | Update the stage 1 specifications to support disaster mitigation services in shared-RAN and national roaming scenarios. | TSG#97 (Sept 2022) |  |

# 6	Work item Rapporteur(s)

Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

RAN 2: 4G cells to be able to broadcast support for inbound disaster roamers

CT1: update of 4G cell reselection/PLMN selection procedures

SA2: procedural updates similar to those for MINT in R17 5GC

CT4: signalling from MME to HSS

# 9	Supporting Individual Members

Supporting IM name

China Telecom

KDDI

CATT

vivo

ZTE