3GPP TSG-SA2 Meeting #159	S2-2311398

# Xiamen, P.R. China, 09th Oct 2023 – 13th Oct 2023	rev of S2-2311191

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0381 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameter update for Link Modification with L3 U2U Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Layer3 UE-to-UE Relay, end to end link modification is agreed at SA2 #156e meeting. And it needs update parameters in 6.4.3.7.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | For adding new QoS flow or modifying existing QoS flow and for removing QoS flow(s), Description of Link Modification Request and Link Modification Accept are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Link Modification procedure with L3 U2U Relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.7.3	Layer-2 link management over PC5 reference point for 5G ProSe Layer-3 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.7.1, the description in clause 6.4.3.7.1 applies with following differences and clarifications:

-	In the Security Procedure of the first hop PC5 reference point, the source 5G ProSe Layer-3 End UE provides the IP Address Configuration or Link-Local IPv6 Address and QoS Info of the end-to-end QoS to the 5G ProSe Layer-3 UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the source 5G ProSe Layer-3 End UE provides its Ethernet MAC address instead of IP related information.

-	In the Security Procedure of the second hop PC5 reference point, the 5G ProSe Layer-3 UE-to-UE Relay provides the IP Address Configuration or Link-Local IPv6 Address and QoS Info of the second hop QoS to the target 5G ProSe End UE. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE instead of IP related information.

-	The Direct Communication Accept message over the second hop PC5 reference point additionally includes IP Address Configuration or Link-Local IPv6 Address and QoS Info of the second hop QoS. If the PC5 link is used for transferring Ethernet traffic, the target 5G ProSe Layer-3 End UE provides its Ethernet MAC address instead of IP related information.

-	The 5G ProSe Layer-3 UE-to-UE Relay decides the QoS Info of the first hop QoS with considering the received second hop QoS, the Direct Communication Accept message over the first hop PC5 reference point additionally includes IP Address Configuration or Link-Local IPv6 Address, QoS Info of the first hop QoS and may include IP address of the target 5G ProSe End UE. If the PC5 link is used for transferring Ethernet traffic, 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the target 5G ProSe Layer-3 End UE instead of IP related information.

-	For adding new end-to-end QoS flow or modifying existing end-to-end QoS flow, in the Link Modification Request message over the first hop PC5 reference point, the source 5G ProSe End UE additionally includes QoS Info of the end-to-end QoS as described in the clause 6.4.3.4. For removing end-to-end QoS flows(s), in the Link Modification Request message over the first hop PC5 reference point, the source 5G ProSe End UE includes PFI(s) of the QoS flow(s) of the first hop as described in the clause 6.4.3.4. If the PC5 link is used for transferring Ethernet traffic, the source 5G ProSe Layer-3 End UE may provide its Ethernet MAC address.

-	For adding new end-to-end QoS flow or modifying existing end-to-end QoS flow, in the Link Modification Request message over the second hop PC5 reference point, the 5G ProSe Layer-3 UE-to-UE Relay additionally includes QoS Info of the second hop QoS to the target 5G ProSe End UE. For removing end-to-end QoS flow(s), in the Link Modification Request message over the second hop PC5 reference point, the 5G ProSe Layer-3 UE-to-UE Relay includes PFI(s) of the QoS flow(s) of the second hop. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE.

-	For adding new end-to-end QoS flow or modifying existing end-to-end QoS flow, the Link Modification Accept message over the second hop PC5 reference point additionally includes QoS Info of the second hop QoS. If the PC5 link is used for transferring Ethernet traffic, the target 5G ProSe Layer-3 End UE may provide its Ethernet MAC address.

-	For adding new end-to-end QoS flow or modifying existing end-to-end QoS flow, the 5G ProSe Layer-3 UE-to-UE Relay decides the QoS Info of the first hop QoS with considering the received second hop QoS, the Link Modification Accept message over the first hop PC5 reference point additionally includes QoS Info of the first hop QoS and may include IP address of the target 5G ProSe End UE. If the PC5 link is used for transferring Ethernet traffic, 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the target 5G ProSe Layer-3 End UE instead of IP related information.

When the PC5 link between a 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay is released, the 5G ProSe Layer-3 UE-to-UE Relay may initiate the PC5 link release to the peer 5G ProSe Layer-3 End UE(s) or notify the peer 5G ProSe Layer-3 End UE(s) the peer PC5 link is released.

# * * * * End of change * * * *