3GPP TSG-SA WG6 Meeting #57	S6-233158

Xiamen, China 9th – 13th October 2023	(revision of S6-233023)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0335 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of ad hoc group data communication participants involving multiple MCData systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification, the ad hoc group data communication is established using the list of participants or the criteria provided by the initiator of the data communication. Further, the authorized user can modify the ongoing ad hoc group data communication participants by providing one or more users to be added or removed from the ad hoc group data communication. The MCData server can also modify the list of participants based on one or more users meeting the criteria or no longer meeting the criteria. The involved participants can be part of both the primary and partner MCData systems. Currently modifying an ongoing ad hoc group data communication participants by authorized user or MCData server is supported in case of users belonging to the same MCData system and the modifying of an ongoing ad hoc group data communication participants by MCData server is supported in case of users belonging to multiple MCData systems. Modifying ad hoc group data communication participants involving multiple MCData systems in which users are added or removed from the partner MCData system by the authorized user is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A missing procedure is defined to modify the ongoing ad hoc group data communication participants by providing the list of users to be added or removed from the ad hoc group data communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Modifying the ad hoc group data communication participants involving multiple MCData systems in which users are added or removed from the partner MCData system is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.3.2.4, 7.17.3.2.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.17.3.2.4	Modifying of ad hoc group data communication participants by the MCData server

Figure 7.17.3.2.4-1 below illustrates the MCData server modifying the ad hoc group data communication participants procedure involving multiple MCData systems.

Pre-conditions:

1.	The MCData user at MCData client 1 is authorized to initiate ad hoc group data communication.

2.	The MCData server 1 of the primary and MCData server 2 of the partner MCData systems determined the participants for the ad hoc group data communication based on the criteria specified by the MCData client 1 while initiating the ad hoc group data communication.

3.	The ad hoc group data communication is established and on-going with the participants MCData client 1, MCData client 2, and MCData client 4. The participants list is determined by both primary and partner MC systems.

4.	The MCData server 1 of the primary and MCData server 2 of the partner MCData systems continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group data communication.

5.	The MCData server 1 of the primary MCData system detects that the MCData client 3 meets the criteria and MCData client 2 stops to meet the criteria specified by the MCData client 1.

6.	The MCData server 2 of the partner MCData system detects that the MCData client 5 meets the criteria and MCData client 4 stops to meet the criteria specified by the MCData client 1.

Figure 7.17.3.2.4-1: Modification of ad hoc group data communication participants by the MCData server

1.	The MCData server 1 detects that MCData client 3 now meets criteria and successfully added to group data communication.

2.	The MCData server 1 detects that MCData client 2 does not meet criteria any more and successfully removed from a group data communication.

3.	The MCData server 2 detects that the MCData client 5 meets the criteria specified by the MCData client 1.

4.	The MCData server 2 sends the notification to MCData server 1 of the primary MCData system to add the MCData user at MCData client 5 to on-going ad hoc group data communication.

5.	The MCData server 1 sends the ad hoc group data session request to the MCData client 5.

6.	The MCData client 5 notifies the user about the incoming ad hoc group data communication.

7.	The MCData client 5 accepts the ad hoc group data session request and sends the ad hoc group data session response to the MCData server 1.

8.	The on-going ad hoc group data communication is updated by adding MCData client 5 which meets the criteria specified by the MCData client 1.

9.	The MCData server 2 detects that the MCData client 4 is no longer satisfying the criteria to be the participant of the ad hoc group data communication.

10.	The MCData server 2 sends the notification to MCData server 1 of the primary MCData system to remove the MCData user at MCData client 4 from on-going ad hoc group data communication.

11.	The MCData server 1 sends the ad hoc group data session leave request to the MCData client 4 and removes it from the on-going ad hoc group data communication.

12.	The MCData client 4 notifies the user of the ad hoc group data session leave request.

13.	The MCData client 4 sends the ad hoc group data session leave response to the MCData server 1.

14.	The on-going ad hoc group data communication is updated by removing MCData client 4, which no longer meets the criteria specified by the MCData client 1.

NOTE:	The ad hoc group data session request and response exchanged between MCData server 1 of primary MC system and MCData client 4/MCData client 5 will always traversal through the MCData server 2.

* * * Next Change * * * *

##### 7.17.3.2.6	Modification of ad hoc group data communication participants by an authorized user

Figure 7.17.3.2.6-1 below illustrates the MCData user modifying the ad hoc group data communication participants procedure involving multiple MCData systems.

Pre-conditions:

1.	An MCData ad hoc group data communication is already in progress and the participants list provided by the originating MCData user while initiating the MCData ad hoc group data communication.

2.	The participants of the MCData ad hoc group data communication belongs to the multiple MCData system.

3.	The MCData user at MCData client 1 determines that the MCData client 3 needs to be invited to on-going ad hoc group data communication and MCData client 2 needs to be removed from the on-going ad hoc group data communication from the primary MCData system.

4.	The MCData user at MCData client 1 determines that the MCData client 5 needs to be invited to on-going ad hoc group data communication and MCData client 4 needs to be removed from the on-going ad hoc group data communication from the partner MCData system.

Figure 7.17.3.2.6-1: Modification of ad hoc group data communication participants by an authorized user

1.	The MCData client 1 sends the modify ad hoc group data session participants request to the MCData server 1 to remove both MCData client 2 and MCData client 4 from the on-going ad hoc group data communication and to add both MCData client 3 and MCData client 5 into on-going ad hoc group data communication.

2.	The MCData server 1 verifies whether the MCData client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group data communication.

3.	The MCData server 1 sends modify ad hoc group data session participants response to the MCData client 1.

4.	The MCData server 1 determines that MCData client 1 has requested that MCData client 3 from the primary MC system be invited to an on-going ad hoc group data communication and successfully added to the data communication according to the procedure defined in clause 7.17.3.1.4.

5.	The MCData server 1 determines that MCData client 1 has requested that MCData client 2 from the primary MC system be removed from the on-going ad hoc group data communication and successfully removed from data communication according to the procedure defined in clause 7.17.3.1.4.

6.	The MCData server 1 determines that MCData client 1 has requested that MCData client 5 from the partner MC system be invited to an on-going ad hoc group data communication. The MCData server 1 sends the ad hoc group data session request to the MCData client 5.

7.	The MCData client 5 notifies the user about the incoming ad hoc group data communication.

8.	The MCData client 5 accepts the ad hoc group data session request and sends the ad hoc group data session response to the MCData server 1.

9.	The on-going ad hoc group data communication is updated by adding MCData client 5 which is based on the modify participant list provided by the MCData client 1.

10.	The MCData server 1 determines that MCData client 1 has requested that MCData client 4 from the partner MC system be removed from the on-going ad hoc group data communication. The MCData server 1 sends the ad hoc group data session leave request to the MCData client 4.

11.	The MCData client 4 notifies the user of the ad hoc group data session leave request.

12.	The MCData client 4 sends the ad hoc group data session leave response to the MCData server 1.

13.	The on-going ad hoc group data communication is updated by removing MCData client 4, which is based on the modify participant list provided by the MCData client 1.

NOTE:	The ad hoc group data communication request and response exchanged between MCData server 1 of primary MC system and MCData client 4/MCData client 5 will always traversal through the MCData server 2.

14.	The MCData server 1 may notify the initiating MCData user of all the users who are added to the on-going ad hoc group data communication. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the ad hoc group data communication.

15.	The MCData server 1 may notify the authorized participants about the change in the participants list of on-going ad hoc group data communication.

* * * End of Changes * * * *