SA WG2 Meeting #S2-140E	S2-2005913

Aug 19 – Sep 01, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0181 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Analytics Exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is specified in current specification that "When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time." But it is not specified what the positive or negative offsets to the reporting time really mean for an analytics subscription. The relation between the positive or negative offsets to the reporting time and the nature of analytics information (i.e. statistics or prediction) needs to be clarified, to ensure a sound analytics subscription between the consumer NF and NWDAF. |  |  |  |  |  |  |  |  |
| FO |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the positive and negative offsets to the reporting time indicate subscription for prediction and statistics respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consumer NF and NWDAF may have different understandings on the expected analytics information (e.g. whether statistics or prediction is subscribed). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_AnalyticsSubscription) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	(Only for Nnwdaf_AnalyticsInfo_Request) Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: confidence in prediction.

******************** End of Changes ********************