3GPP TSG-SA5 Meeting #132e 	S5-204135

# e-meeting 17th-28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0059 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update description of MnS components used for configurable PM control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The MeasurementControl<<IOC>> and MeasurementReader<<IOC>> are used in the Table 7.6-1: MnS components used for configurable performance measurement control, however, in TS 28.622, these two <<IOC>> are merged to PerfMetricJob <<IOC>> |  |  |  |  |  |  |  |  |
| Summary of change: |  | Merge MeasurementControl<<IOC>> and MeasurementReader<<IOC>> in one PerfMetricJob <<IOC>> to align with TS 28.622 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Management service component type B used for configurable performance measurement control is not align with TS 28.622. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.6	Management service components used for configurable performance measurement control

The MnS components used for configurable performance measurement control are listed in table 7.6-1. The configurable performance measurement control approach and measurement job control service described in clause 7.1 are two alternative solutions used for controlling performance measurement.

Table 7.6-1: MnS components used for configurable performance measurement control

| Management purpose | Management service component type A | Management service component type B |
| --- | --- | --- |
| Configurable performance measurement control for NE/NF | Following operations/notifications defined in Clause 11.1.1 in TS 28.532[7]: Operations: - createMOI - getMOIAttributes - modifyMOIAttributes - deleteMOI Notifications: - notifyMOICreation - notifyMOIAttributeValueChanges - notifyMOIDeletion - notifyMOIChanges | Following IOCs defined in performance measurement control NRM fragment in TS 28.622 [5] /TS 28.541[3]: - PerfMetricJob - ManagedElement or concrete ManagedFunction |
| Configurable performance measurement control for NetworkSlice | Following operations/notifications defined in Clause 11.1.1 in TS 28.532[7]: Operations: - createMOI - getMOIAttributes - modifyMOIAttributes - deleteMOI Notifications: - notifyMOICreation - notifyMOIAttributeValueChanges - notifyMOIDeletion - notifyMOIChanges | Following IOCs defined in performance measurement control NRM fragment in TS 28.622 [5] /TS 28.541[3]: - PerfMetricJob - NetworkSlice |
| Configurable performance measurement control for NetworkSliceSubnet | Following operations/notifications defined in Clause 11.1.1 in TS 28.532[7]: Operations: - createMOI - getMOIAttributes - modifyMOIAttributes - deleteMOI Notifications: - notifyMOICreation - notifyMOIAttributeValueChanges - notifyMOIDeletion - notifyMOIChanges | Following IOCs defined in performance measurement control NRM fragment in TS 28.622 [5] / TS 28.541[3]: - PerfMetricJob - NetworkSliceSubnet |
| Configurable performance measurement control for SubNetwork | Following operations/notifications defined in Clause 11.1.1 in TS 28.532[7]: Operations: - createMOI - getMOIAttributes - modifyMOIAttributes - deleteMOI Notifications: - notifyMOICreation - notifyMOIAttributeValueChanges - notifyMOIDeletion - notifyMOIChanges | Following IOCs defined in performance measurement control NRM fragment in TS 28.622 [5]: - PerfMetricJob - SubNetwork |

End of Change