3GPP TSG-SA5 Meeting #132e	S5-204159

# 17 to 28 August 2020, E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0056 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RAN UE throughput KPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is unclear how the RAN UE throughput KPI is calculated for subnetwork level, and the KPI for network slice subnet level is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separated the RAN UE throughput KPI for NR Cell level and subnetwork level. Corrected the KPI calculation for subnetwork level. Added the missing KPI for network slice subnet level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN UE throughput KPI definition will lead to wrong implementation for subnetwork level and will not support the network slice subnet performance evaluation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6, 6.3.6.x (new), 6.3.6.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 6.3.6	RAN UE Throughput

#### 6.3.6.1	Void

#### 6.3.6.2	RAN UE Throughput definition

To achieve a Throughput measurement (below examples are given for DL) that is independent of file size and gives a relevant result it is important to remove the volume and time when the resource on the radio interface is not fully utilized. (Successful transmission, buffer empty in figure 1).

Time (slots)

Data arrives to

empty DL buffer

First data is

transmitted to the UE

The send buffer is

again empty

calculations since it

can be impacted

by packet size of

User Plane (UP) packets.

ThpTimeDl

Failed transmission (

”

Block

error

”

)

Successful transmission,

buffer not empty

Successful transmission,

buffer empty

ThpVolDl =

∑

Total DL transferred volume =

∑

(kbits)

+

(kbits)

UE Throughput in DL =

ThpVolDl / ThpTimeDl (kbits/s)

No transmission, buffer not

empty (e.g. due to contention)

The last slot shall always be removed from

Figure 1

To achieve a throughput measurement that is independent of bursty traffic pattern, it is important to make sure that idle gaps between incoming data is not included in the measurements. That shall be done as considering each burst of data as one sample.

Next modified section

#### 6.3.6.x	DL RAN UE throughput

##### 6.3.6.x.1	DL RAN UE throughput for a NRCellDU

a)	DlUeThroughput _Cell.

b)	This KPI describes the average DL RAN UE throughput for a NRCellDU. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)	Below is the equation for average DL RAN UE throughput for a NRCellDU:

DlUeThroughput _Cell = DRB.UEThpDl;

and optionally: DlUeThroughput _Cell.QOS = DRB.UEThpDl.QOS, where QOS identifies the target quality of service class;

and optionally: DlUeThroughput _Cell.SNSSAI = DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

d)	NRCellDU

##### 6.3.6.x.2	DL RAN UE throughput for a sub-network

a)	DlUeThroughput _SNw.

b)	This KPI describes the average DL RAN UE throughput for a sub-network. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)	Below is the equation for average DL RAN UE throughput for a sub-network, where

-	W is the measurement for the weighted average, it is one of the following:

-	the DL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s in the SubNetwork.

DlUeThroughput _SNw= 1#NRCellDUW1#NRCellDUWDRB.UEThpDl

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

DlUeThroughput _SNw.QoS= 1#NRCellDUW.QoS1#NRCellDUW.QoSDRB.UEThpDl.QoS

DlUeThroughput _SNw.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpDl.SNSSAI

d)	SubNetwork

##### 6.3.6.x.3	DL RAN UE throughput for a network slice subnet

a)	DlUeThroughput _Nss.

b)	This KPI describes the average DL RAN UE throughput for a network slice subnet. The KPI type is MEAN in kbit per second.

c)	Below is the equation for average DL RAN UE throughput for a network slice subnet, where

-	W is the measurement for the weighted average, it is one of the following:

-	the DL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s associated with the NetworkSliceSubnet.

DlUeThroughput_Nss.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpDl.SNSSAI, where the SNSSAI identifies the S-NSSAI that the NetworkSliceSubnet supports.

d)	NetworkSliceSubnet

Next modified section

#### 6.3.6.y	UL RAN UE throughput

##### 6.3.6.y.1	UL RAN UE throughput for a NRCellDU

a)	UlUeThroughput_Cell.

b)	This KPI describes the average UL RAN UE throughput for a NRCellDU. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)	Below is the equation for average UL RAN UE throughput for a NRCellDU:

UlUeThroughput _Cell = DRB.UEThpUl;

and optionally: UlUeThroughput_Cell.QOS = DRB.UEThpUl.QOS, where QOS identifies the target quality of service class;

and optionally: UlUeThroughput_Cell.SNSSAI = DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

d)	NRCellDU

##### 6.3.6.y.2	UL RAN UE throughput for a sub-network

a)	UlUeThroughput_SNw.

b)	This KPI describes the average UL RAN UE throughput for a sub-network. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)	Below is the equation for average UL RAN UE throughput for a sub-network, where

-	W is the measurement for the weighted average, it is one of the following:

-	the UL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s in the SubNetwork.

UlUeThroughput_SNw= 1#NRCellDUW1#NRCellDUWDRB.UEThpUl

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

UlUeThroughput_SNw.QoS= 1#NRCellDUW.QoS1#NRCellDUW.QoSDRB.UEThpUl.QoS

UlUeThroughput_SNw.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpUl.SNSSAI

d)	SubNetwork

##### 6.3.6.y.3	UL RAN UE throughput for a network slice subnet

a)	UlUeThroughput _Nss.

b)	This KPI describes the average UL RAN UE throughput for a network slice subnet. The KPI type is MEAN in kbit per second.

c)	Below is the equation for average UL RAN UE throughput for a network slice subnet, where

-	W is the measurement for the weighted average, it is one of the following:

-	the UL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s associated with the NetworkSliceSubnet.

UlUeThroughput_Nss.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpUl.SNSSAI, where the SNSSAI identifies the S-NSSAI that the NetworkSliceSubnet supports.

d)	NetworkSliceSubnet

End of modified section