3GPP TSG-SA WG6 Meeting #38-e	S6-201183

# e-meeting, 20th – 31st July 2020													(revision of S6-201071)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 261 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Editors Note related to functional alias resolution by LMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 10.9.1 exists and EN “It is for FFS how the location management server resolves the functional alias to the corresponding MC Service ID(s).” But we already have a procedure in 10.13.2.11 where in the LMS subscribes to the functional alias controlling MC Service server. This contribution enhances the existing procedure more generic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Since the procedure is already existing in section 10.13.2.11 this CR removes the Editors note from section 10.19.1 and makes the existing procedure more generic which can accommodate MC service server, LMS and any other entity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unwanted Editor’s note present in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.1, 10.3.10.1,10.3.10.2,10.3.10.3,10.3.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 10.9.1	General

Location information of MC service user shall be provided by the location management client to the location management server. The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

The location management client can be configured to store location information while not reporting location information to the location management server. The location management client may report stored location information to the location management server.

NOTE:	Configuration and location information transmission may occur while there is a communication link between the location management client and location management server.

* * * * * * * SECOND CHANGE * * * * * * *

### 10.13.10	Subscription and notification for functional alias

#### 10.13.10.1	General

The subscriber (e.g. MC service server, location management server) obtains the list of MC service users who have activated a functional alias from the functional alias controlling MC service server via subscription and notification mechanisms.

#### 10.13.10.2	Subscription for functional alias procedure

The procedure for subscription for functional alias is described in figure 10.13.10.2-1.

Pre-conditions:

-	The functional alias controlling MC service server holds a list of MC service users who has activated the functional alias.

Figure 10.13.10.2-1: Subscription for functional alias

1.	The subscriber sends a subscribe functional alias request to the functional alias controlling MC service server in order to receive notifications about the  list of MC service users who has activated the function alias.

2.	The functional alias controlling MC service server provides a subscribe functional alias response to the subscriber indicating success or failure of the request.

#### 10.13.10.3	Notification for functional alias procedure

The procedure for notification for functional alias as shown in figure 10.13.10.3-1 is used by the functional alias controlling MC service server to inform the subscriber about MC service users who has activated the functional alias.

Pre-conditions:

-	The subscriber has subscribed to the functional alias

-	The list of MC service users who has activated the functional alias is changed at functional alias controlling MC service server due to activation, deactivation or taken over.

Figure 10.13.10.3-1: Notification for the functional alias

1.	The functional alias controlling MC service server provides the list of MC service users who has activated the functional alias in the notify functional alias request to the subscriber.

2.	The subscriber provides a notify functional alias response to the functional alias controlling MC service server.

#### 10.13.10.4	Unsubscription for functional alias procedure

The procedure for unsubscription for functional alias is described in figure 10.13.10.4-1.

Pre-conditions:

-	The functional alias controlling MC service server holds a list of MC service users who has activated the functional alias.

Figure 10.13.10.4-1: Unsubscription for functional alias

1.	The subscriber sends a unsubscribe functional alias request to the functional alias controlling MC service server in order to stop receiving notifications about the  list of MC service IDs who has activated the function alias.

2.	The functional alias controlling MC service server provides a subscribe functional alias response to the subscriber indicating success or failure of the request.

* * * * * * * END CHANGE * * * * * * *