3GPP TSG-SA WG6 Meeting #41-e	S6-210070

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0288 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorized user clearing the entire floor request queue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, BDBOS, FirstNet, Police of the Netherlands |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An editor’s note has been captured in section 10.9.1.2.5 since there was no requirement available in stage 1 to support clearing of all the entries in the floor queue in single request. Recently in TS 22.280 the following requirement has been added : [R-6.19.1.1-005a] The MCX Service shall provide a mechanism for an authorised user to clear the entire MCX request queue. This CR proposes to resolve the editor’s note by addressing the above requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the existing floor request cancel information flow the list of MCPTT IDs IE is made optional with a NOTE clarifying that if the request is for clearing all the entries then there is no need to list the MCPTT IDs of the users whose floor request is queued. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement [R-6.19.1.1-005a] in TS 22.280 remains un-fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.1.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

##### 10.9.1.2.5	Floor request cancel

Table 10.9.1.2.5-1 describes the information flow floor request cancel, from the floor participant to the floor control server, which is used to request cancelling the floor request from the floor request queue. This information flow is sent in unicast to the floor control server.

Table 10.9.1.2.5-1: Floor request cancel

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity for the requester |
| Functional alias | O | Functional alias for the requester |
| List of MCPTT IDs (NOTE) | O | Target identity (identities) whose floor request is to be cancelled |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| NOTE: If this information element is not present all the entries in the floor request queue are cancelled. |  |  |

* * * * * * * END CHANGE * * * * * * *