3GPP TSG-SA1 Meeting #94e	S1-211390

Electronic Meeting, 10 – 20 May 2021	(revision of S1-211077r2)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.280 | CR | 0145 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of MCX UE de-affiliation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, The Police of the Netherlands |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY2 |  |  |  |  |  | Date: |  |  | 2021-05-20 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Concurrent login into multiple devices by one MCX User is supported in clause 5.10 Support for multiple devices. However, there remains a gap with reference to allowing de-affiliation of specific MCX UEs in case of multiple, simultaneous logins by one MCX User. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancement of current requirements to cover gaps in de-affiliation of specific MCX UEs, in case the MCX User has logged into multiple MCX UEs at the same time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case an MCX User has logged into multiple MCX UEs at the same time, that MCX User may not be able to de-affiliate specific MCX UEs from MCX Service Groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.4, 6.6.5.2, 8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

### 6.4.3	Identification

[R-6.4.3-001] The MCX Service shall provide the MCX Service User ID, associated MCX Service User ID alias(es), MCX Service Group ID, group aliases and, if available, the identity of the Mission Critical Organization name of the transmitting Participant to the receiving MCX UEs unless the transmitting Participant's identity is restricted.

[R-6.4.3-002] The MCX Service shall present users with human readable identifiers (with a minimum length of Nc3) for MCX Users (i.e., MCX Service User ID alias(es)) and for the MCX Service Groups (i.e., group alias(es)).

### 6.4.4	Membership/affiliation

[R-6.4.4-001] The MCX Service shall support automatic affiliation of the MCX UE to a Group-Broadcast Group or User-Broadcast Group within that MCX Service.

[R-6.4.4-002] The MCX Service shall support an MCX User's ability to revoke their affiliation with an MCX Service Group.

[R-6.4.4-002a] The MCX Service shall enable an MCX User to revoke their affiliation with a specific MCX Service Group on specific MCX UEs.

[R-6.4.4-003] The MCX Service shall be able to prevent the MCX User having registered a specific Functional Alias from revoking their affiliation with a specific MCX Service Group.

[R-6.4.4-004] The MCX Service shall be able to prevent the MCX User to revoke their affiliation with a specific MCX Service Group if the MCX User is the only MCX User in the MCX Service Group registered a specific Functional Alias.

* * * Next Change * * *

### 6.6.5	Dynamic Group Participation

#### 6.6.5.1 	Service description

Dynamic group participation enables control of group affiliation, de-affiliation, and selection on MCX Service Groups based on various combinations of criteria. For highly mobile MCX UEs, such as those used in railway communication, the group affiliation and selection needs to change, for example, when a train crosses a controlling area border.

#### 6.6.5.2	Requirements

[R-6.6.5.2-001] The MCX Service shall enable an MCX User to automatically affiliate to one or more MCX Service Groups based on one, or a combination of, criteria.

[R-6.6.5.2-002] The MCX Service shall be able to automatically affiliate an MCX User to one or more MCX Service Groups based one, or a combination of, criteria.

[R-6.6.5.2-003] The MCX Service shall enable an MCX User to automatically choose the Selected MCX Service Group based on one, or a combination of, criteria.

[R-6.6.5.2-004] The MCX Service shall be able to automatically choose the Selected MCX Service Group of an MCX User based on one, or a combination of, criteria.

[R-6.6.5.2-005] The MCX Service shall enable an MCX User to automatically de-affiliate from one or more currently affiliated groups based on a change to one, or a combination of, criteria.

[R-6.6.5.2-006] The MCX Service shall be able to automatically de-affiliate an MCX User from one or more currently affiliated groups based on a change to one, or a combination of, criteria.

[R-6.6.5.2-007] The MCX Service shall be able to prevent an MCX User de-affiliating from specific currently affiliated groups based on one, or a combination of, criteria.

NOTE: Examples of criteria for the above requirements could include one or more parameters from the list below:

-	A specific element or combination of specific elements in the functional alias of a MCX User

-	Location (including speed and heading) of a MCX User

-	Location or particular geographic area specified by an MCX Service Administrator

-	MCX Service configuration (e.g. MCX User responsible for a certain geographic area or MCX User responsible for a certain MCX Service Group)

[R-6.6.5.2-008] The MCX Service shall enable an MCX User to select on specific MCX UEs whether automatic affiliation and/or de-affiliation is allowed.

* * * Next Change * * *

## 8.4	Single group with multiple MCX Services

### 8.4.1	Overview

It is useful to be able to configure any single group to be able to handle multiple MCX Services. In sub-clause 8.4 requirements are on the service to provide a solution that looks as though the services are working in a coupled and coordinated manner. The exact means of achieving this is not implied here. So, where a User may affiliate to a single group for two services, this could be a single affiliation indicating two services but handled together or it could be that the UE sends two affiliations, one for each service, in response to the single command to affiliate. There may also be other possibilities. In essence, the solution should look and feel like a combined service.

### 8.4.2	Requirements

[R-8.4.2-001] Mission critical groups shall be able to use multiple MCX Services independently or in combination.

[R-8.4.2-002] The MCX Service shall provide a mechanism by which an authorized MCX User can affiliate to a mission critical group using that MCX Service through a single logical group affiliation for any subset of MCX Services used by the group, or independently by MCX Service.

[R-8.4.2-003] To support operation coordinated between mission critical services, an MCX Service used by a mission critical group shall be able to interact with another MCX Service used by the same group.

[R-8.4.2-004] To support combined services where the operation is intended to be coordinated, each MCX Service shall be able to apply similar geographic restrictions in use for one MCX Service to also operate in the coordinated MCX Service(s).

[R-8.4.2-005] The MCX Service shall provide a mechanism by which an authorized MCX User who is affiliated to multiple MCX Services in a single group can de-affiliate specific MCX UEs from each MCX Service independently.

* * * End Change * * *