TSG-SA Working Group 1 (Services) meeting #3	TSGS1#3(99)321

Hampton Court, Surrey, UK 10th-12th May 1999

Agenda Item:	6.9.2.2

Source: 	NTT DoCoMo

Title:	Proposed investigations for LS answer on “Identification of service 	capabilities” in S1-99251

Document for:	Discussion and decision

________________________________________________________________________________________________________

- Introduction

TSG-T2 asked TSG-S1 to identify Release 99 service capabilities and the correspondence between services and service capabilities in Tdoc S1-99251. We propose that S1 further investigates the various service capabilities which respect to their individual attributes as per TS 22.105.

Although 3GPP’s principle is to standardize service capabilities and not the services themselves, we consider that an investigation of the correspondence between services and bearer services as well as consideration for bearer services with respect to QoS, is necessary, particularly to ensure compatibility with roaming and to assess any QoS impact on the system complexity.

- Proposal

TS 22.100 defines service capabilities as “bearer services defined by parameters (e.g. QoS attributes) and mechanisms needed to realize services”. We propose to investigate the following items related to identification of service capabilities.

- List the types of bearer services that will be needed in 3GPP as in the table below.

| Type of bearers |  |  |  | ## Characteristics | ## Traffic Type |
| --- | --- | --- | --- | --- | --- |
| Circuit switched data |  | ## Speech |  | Real Time, Constant Delay, Maximum BER depends on CODEC | Real Time Conversation |
|  |  | Transparent(Unrestricted) data |  | Real Time, Constant Delay, High Reliability, Transparent | Real Time Conversation |
|  |  | Non- transparent data | Fax/Modem | Real Time, High Reliability(zero error with flow control) | Interactive? |
|  |  | Video |  | Real Time, Constant Delay, Maximum BER depends on CODEC | Real Time Conversation |
| Packet switched data |  |  |  | Variety | All type |
| SMS | SMS-PP |  |  | Non Real Time, High Reliability | Background |
|  | SMS-CB |  |  | Non Real Time, Uni-directional, Point-to-Multipoint | Background |

- Define the values for the attributes as in 22.105, that are necessary for each bearer, in particular bit rates (fixed bit rate for circuit switched data and peak, minimum, and mean bit rates for packet switched data) and quality attributes values correspondence to each bit rates.

[Note]  Among these values to be defined, T2 requires especially to prioritize the bit rates.

- Identify the correspondence between services and service capabilities. It is proposed that the correspondence of some typical services and standardized service capabilities would be identified.

The following is a proposed template with an initial attempt to highlight the correspondence between services and service capabilities.

| ## Type of bearer services |  |  |  | (e.g.)Services Service capability |  | Teleservice |  |  |  |  |  |  |  | #### Unrestricted data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Speech | Fax | Modem | Video | Packet |  | SMS |  |  |
|  |  |  |  |  |  |  |  |  |  | IP | X.25 | SMS-PP | SMS-CB |  |
|  |  |  |  | QoS Attributes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ### Fix/Max bit rate | Others |  |  |  |  |  |  |  |  |  |
| Circuit switched data |  | Speech |  | 8/16K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  | Transparent (Unrestricted) data |  | 16K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 32K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 64K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  | Non-transparent data | Fax/ Modem | 16K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 32K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 64K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  | Video |  | 32K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 64K | [TBD] |  |  |  |  |  |  |  |  |  |
| Packet switched data (*1) |  |  |  | 8K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 16K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 32K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 64K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 128K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 256K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 386K | [TBD] |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2048K | [TBD] |  |  |  |  |  |  |  |  |  |
| SMS | SMS-PP |  |  | --- | [TBD] |  |  |  |  |  |  |  |  |  |
|  | SMS-CB |  |  | --- | [TBD] |  |  |  |  |  |  |  |  |  |

(*1) The free combination of the bit rates for uplink and downlink will be available.

There may be other bit rates.