TSG-SA Working Group 1 (Services) meeting #4	TSGS1#4(99)

Québec, Canada, 5th-9th July 1999						Agenda 6.1.4.1

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.100 |  | Version: |  |  | 3.3.0 |  |  |  |
| Submitted to TSG_SA | 5 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  | X |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | UMTS phase 1 release 99 |
| --- | --- |

| Source: | S1 | Date: |  |
| --- | --- | --- | --- |

| Subject: | Editorial changes |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification | X |  | UMTS 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Editorial changes related to terminology. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

# 3	Definitions, and abbreviations

## 3.1	Definitions

Definitions applicable to current document :

CAC (Connection Admission Control) : is a set of measures taken by the network to balance between the QoS requirements of new connections request and the current network utilisation without affecting the grade of service of existing/already established connections.

Capability Class : is a piece of information which indicates general UMTS mobile station characteristics (e.g. supported radio interfaces,...) for the interest of the network.

Connection mode : characterizes the type of association between two points as required by the bearer service for the transfer of information. A bearer service is either connection-oriented or connectionless. In a connection oriented mode, a logical association called connection needs to be established between the source and the destination entities before information can be exchanged between them. Connection oriented bearer services lifetime is the period of time between the establishment and the release of the connection.

In a connectionless mode, no connection is established beforehand between the source and the destination entities ; the source and destination network addresses need to be specified in each message. Transferred information cannot be guaranteed of ordered delivery. Connectionless bearer services lifetime is reduced to the transport of one message.

FC (Flow Control) : is a set of mechanisms used to prevent the network from becoming overloaded by regulating the input rate transmissions.

GSM BSS : refers in this specification to the GSM/GPRS access network.

GSM core network : refers in this specification to the GSM NSS and GPRS backbone infrastructure.

Home environment : enables a user to obtain UMTS services in a consistent manner regardless of the user’s location or terminal used (within the limitations of the serving network and current terminal).

Performance : is concerned with the ability to track service and resource usage levels and provides feedback on the responsiveness and reliability of the network.

Serving network : provides the user with access to the services of home environment.

UMTS core network : refers in this specification to an evolved GSM core network infrastructure or any new UMTS core network infrastructures, integrating circuit and packet switched traffic..

UMTS mobile termination : part of the UMTS Mobile Station which provides functions specific to the management of the radio interface (Um).

UMTS network: refers to a network operated by a single network operator and consisting of :

UTRAN access networks (WCDMA and/or TD-CDMA),

optionally GSM BSS access networks,

an UMTS core network.

UPC (Usage Parameter Control) : is a set of actions taken by the network to monitor and control the offered traffic and the validity of the connection with respect to the traffic contract negotiated between the user and the network.

3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

BSS	Base Station System

CS	Circuit switched

GPRS	General Packet Radio Service

GSM	Global System for Mobile communications

NSS	Network Sub System

PC	Personal Computer

PS	Packet switched

QoS	Quality of Service

SIM	GSM Subscriber Identity Module

TD-CDMA	Time Division-Code Division Multiple Access

UICC	UMTS IC Card

UMTS	Universal Mobile Telecommunications System

USIM	User Service Identity Module

UTRAN	UMTS Terrestrial Radio Access Network

VHE	Virtual Home Environment

WCDMA	Wideband Code Division Multiple Access

# 7	UTRAN capabilities

NOTE :	The term performance refers in this clause to the realisation of the QoS objectives inside the UTRAN.

UTRAN capabilities for UMTS are the complete set of radio access bearer capabilities and radio access bearer control specified in UMTS 22.05. The UTRAN shall have the following capabilities :

1)	A UTRAN shall be contained within only one UMTS network. (In the case of a network with a phase 1 UMTS core network consisting of an evolved GSM core network, it shall be possible to connect the UTRAN to the GSM NSS and GPRS backbone infrastructures or only one of them.)

2)	The UTRAN shall support the set-up, re-negotiation and clearing of connections with a range of traffic and performance characteristics. The re-negotiation of QoS attributes / bearer attributes may result from an upper layer request or a change in the radio conditions (handover, cell load modification,...) and may be mobile station (e.g. by an application or the user via an application) or network initiated. It shall be possible for the UTRAN to apply the following traffic policing mechanisms such as :

. connection admission control (CAC) during connection set-up and re-negotiation,

. flow control (FC) on a connection during its lifetime,

.  usage parameter control (UPC) on a connection during its lifetime..

4) The UTRAN shall support radio access bearers for broadcast and multicast applications.

4)	The range of traffic and performance characteristics that shall be supported by UTRAN are indicated in TS 22.05.

5)	The UTRAN shall allow one mobile termination to handle more than one radio access bearer service simultaneously. It is nevertheless expected that the terminal and network capabilities will put some limitations on the number of radio access bearer services that can be handled simultaneously. It shall be possible for each radio access to have independent traffic and performance characteristics.

6)	Seamless handover of active radio access bearer service(s) from a single mobile termination, between cells of one UTRAN shall be supported. This shall result in an imperceptible loss of speech (if any) for the user of telephony services and without incurring degradation of QoS for data services.

7)	At least one Capability Class shall be standardised for mobile terminals supporting more than one  UTRA mode (e.g. UTRA FDD and TDD modes). It shall support monitoring of the different types of cells in idle mode (cell reselection procedure) and active mode (handover preparation procedure).

8)	For UMTS networks composed of UTRANs with different UTRA modes, the cell selection and the paging procedures shall accommodate to the fact that service areas may be covered by  cells supporting one specific mode (e.g. FDD or TDD mode),  and cells supporting more than one mode (e.g. FDD and TDD modes).

9)	Handover of one mobile termination handling one or more radio access bearer services between cells of two UTRANs using different UTRA modes and operated by one single UMTS network operator shall be supported in both directions. Furthermore, handover between cells using two different UTRA modes should be supported similarly to handover within one mode.

10) The UTRAN shall facilitate determination of the location of a UMTS mobile termination. The realisation of a positioning service can be determined by several methodologies, namely mobile-based positioning, network-based positioning, or a hybrid position architecture. It shall be possible for the location precision to be a UMTS network operator choice, with the precision of the location varying from one part of the service area to another. It shall be possible to achieve a minimum precision of around 50 meters in all types of terrestrial radio environments. Location requirements are detailed in UMTS 22.05 subclause 8.5.

11)	 The UTRAN shall support the Localised Service Area (LSA) concept. It shall facilitate user-dependent radio resource selection based on LSA (e.g. when user is located at his office, radio coverage provided with indoor radio solutions should be preferred). Corresponding GSM feature has been specified in GSM 02.43.

12) The optimisation of the UTRAN radio interface shall be based upon the objectives expressed in UMTS 22.05 clause 5.

13)	 Standardised protocols shall be defined for the operation, administration and maintenance of each of the UTRAN components in UMTS phase 1 in cooperation with ETSI TMN.

14)	 The USIM requirements defined for later releases of UMTS should be taken into account in the design of UTRAN (for any impact).

# 8	UTRAN and GSM BSS relationship

There is a special relationship between the UTRAN and GSM access networks as it is expected that UTRANs will start as islands in a sea of GSM BSS. GSM BSS access networks will be a key element for service continuity in UMTS networks. The requirements are the following for UMTS phase 1 :

1)	UMTS phase 1 shall support dual mode UMTS/GSM terminals. At least one Capability Class shall be standardised for mobile terminals supporting the GSM and UTRA modes. It shall support monitoring of cells belonging to the two types of access networks in idle mode (cell reselection procedure) and active mode (handover preparation procedure).

2)	Cell selection and paging procedures shall be designed to accommodate to the fact that networks may consist of GSM BSS cells, UTRAN cells or a combination of both.

3)	For UMTS networks composed of both GSM BSS and UTRAN access networks, handover of radio access bearer services shall be supported between GSM BSS and UTRAN cells, in both directions (i.e. UTRAN to GSM BSS and GSM BSS to UTRAN). Some traffic flows may be re-negotiated, temporarily released or re-established during these handover procedures because of the different radio access bearer capabilities of the GSM BSS and UTRAN access networks.

# 9	UMTS Core Network

NOTE 1:	The term performance refers in this clause to the resource level usage and reliability of the UMTS core network.

NOTE 2:	It is not required for phase 1 UMTS core networks to support calls with multiple connections. Multiple connections for a single mobile could be realised through several calls.

In the first phase of UMTS, the UMTS core network capabilities are a superset of the phase 2+ release 99 GSM core network capabilities. The additional requirements for the phase 1 UMTS core network are the following :

1)	The phase 1 UMTS core network shall support circuit switched data service capability of at least 64 kbit/s per user. This shall not limit the user from choosing lower data rates.

2)	The phase 1 UMTS core network shall support packet switched data service capabilities of at least 2 Mbit/s peak bit rate per user. This shall not limit the user from choosing lower data rates.

3)	The phase 1 UMTS core network shall enable set-up, re-negotiation and clearing of connections (i.e. CS calls or PS sessions) with a range of traffic and performance characteristics . The re-negotiation of QoS attributes / bearer may be caused by an application or the user via an application (see UTRAN capability section). It shall be possible to apply traffic policing (e.g. connection admission control, flow control, usage parameter control...) on a connection during its set-up and lifetime.

4)	The phase 1 UMTS core network shall support a range of traffic and performance characteristics for connectionless (e.g. unicast, broadcast, and multicast) traffic.

5)	The range of traffic and performance characteristics that shall be supported by the phase 1 UMTS core network shall be at least those of GPRS phase 2+ release 99. This means that the support of the full set of bearer services defined in TS 22.05 is not required for the phase 1 UMTS core network.

6)	Point to multipoint communication configurations as defined in TS 22.05 shall be supported by the phase 1 UMTS core network.

7)  Established bearers shall not prevent the set-up of a new bearer. These new bearers can be of any type (e.g. PS, CS). 	It is nevertheless expected that the terminal and network capabilities will put some limitations on the number of bearer services that can be handled simultaneously. It shall be possible for each bearer to have independent traffic and performance characteristics.

8)	In order to facilitate the development of new applications, it shall be possible to address applications to/from a phase 1 UMTS mobile termination (e.g. the notion of Internet port).

9)	Operator specific services based on the VHE concept shall be provided by the phase 1 UMTS core network. This functionality could be provided through available toolkits (such as CAMEL, MExE, WAP and SIM Toolkit).

10)	 If UMTS authentication is invoked while a user has services active, the authentication shall not degrade the user services.

11)	 The phase 1 UMTS core network shall support the generation of standardised charging records based upon parameters such as the dialled number, call duration, traffic (volume, bit rate) and perceived Quality of Service provided to the user.

12)	 The phase 1 UMTS core network shall support on-line billing. Billing of 3rd party value added services with the concept of one-stop-billing shall be supported by the phase 1 UMTS core network through standardised procedures.