TSG-SA Working Group 1 (Services) meeting #4	TSGS1#4(99)529

Quebec City, Canada, 5-9 July 1999									Agenda Item 6.3.1

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.105 |  | Version: |  |  | 3.5.0 |  |  |  |
| Submitted to TSG_SA |  |  | for approval |  |  |  | without presentation ("non-strategic") |  |  |  | X |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | UMTS Quality of Service |
| --- | --- |

| Source: | Nortel Networks | Date: | 08.07.99 |
| --- | --- | --- | --- |

| Subject: | Dynamically variable rate QoS requirements |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification |  |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | There is a need for the QoS mechanism in UMTS to accommodate applications that require dynamically variable bit rates between a minimum guaranteed bit rate and a maximum bit rate for some real time applications. Hence this is made a requirement of the bearer service. Text is introduced which clarifies that the tables in sections 5.5 define specifically end to end quality of service requirements. |
| --- | --- |

| Clauses affected: | 5.2.1, 5.5 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 1st Change

### 5.2.1	Information transfer

Connection oriented / conectionless services

Both Connection oriented and connectionless services shall be supported.

Traffic type-  It is required that the bearer service provides one of the following:

- guaranteed/constant bit rate,

- non-guaranteed/dynamically variable bit rate and

- real time dynamically variable bit rate with a minimum guaranteed bit rate.

- Real time and non real time applications shall be supported.

- Real time video, audio and speech must be supported. This implies the:

- ability to provide a real time stream of guaranteed bit rate, end to end delay and delay variation.

- ability to provide a real time conversational service of guaranteed bit rate, end to end delay and delay variation.

- Non real time interactive and file transfer service must be supported. This implies the:

- ability to support message transport with differentiation as regards QoS between different users.

- Multimedia applications shall be supported. This implies the:

- ability to support several user flows to/from one user having different traffic types (e.g. real time, non real time)

- Traffic characteristics

It shall be possible for an application to specify its traffic requirements to the network by requesting a bearer service with one of the following configurations

- 1)	Point-to-Point

-	Uni-Directional

-	Bi-Directional

-	Symmetric

-	Asymmetric

- 2)	Uni-Directional Point-to-Multipoint

-	Multicast

-	Broadcast

A multicast topology is one in which sink parties are specified before the connection is established, or by subsequent operations to add or remove parties from the connection. The source of the connection will always be aware of all parties to which the connection travels.

A broadcast topology is one in which the sink parties are not always known to the source. The connection to individual sink parties is not under the control of the source, but is by request of each sink party.

In the case of a mobile termination with several active bearer services simultaneously, it shall be possible for each bearer service to have independent configurations and source/sink parties.5.2.2	Information quality

### 2nd Change

## Supported End User QoS

This section outlines the QoS that shall be provided to the end user / applications. This section defines QoS requirements from end to end. The values in the tables are end to end, including mobile to mobile calls and satellite components. Figure 2 below summarises the major groups of application in terms of QoS requirements. Applications and new applications may be applicable to one more groups.

- The following tables further elaborate UMTS end user / application QoS requirements.

### Table 1: End-user Performance Expectations - Conversational / Real-time Services

| Medium | Application | Degree of symmetry | Data rate | Key performance parameters and target values |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | One-way Delay | Delay Variation | Information loss |
| Audio | Conversational voice | Two-way | 4-25 kb/s | <150 msec preferred <400 msec limit | < 1 msec | < 3% FER |
| Video | Videophone | Two-way | 32-384 kb/s | < 150 msec preferred <400 msec limit Lip-synch : < 100 msec |  | < 1% FER |
| Data | Telemetry - two-way control | Two-way | <28.8 kb/s | < 250 msec | N.A | Zero |
| Data | Interactive games | Two-way | < 1 KB | < 250 msec | N.A | Zero |
| Data | Telnet | Two-way (asymmetric) | < 1 KB | < 250 msec | N.A | Zero |

[ Note: The values provided in table 1 need to be further reviewed and may be revised in the next version of this specification]

## Table 2: End-user Performance Expectations - Interactive Services

| Medium | Application | Degree of symmetry | Data rate | Key performance parameters and target values |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | One-way Delay | Delay Variation | Information loss |
| Audio | Voice messaging | Primarily one-way | 4-13 kb/s | < 1 sec for playback < 2 sec for record | < 1 msec | < 3% FER |
| Data | Web-browsing - HTML | Primarily one-way |  | < 4 sec /page | N.A | Zero |
| Data | Transaction services – high priority e.g. e-commerce, ATM | Two-way |  | < 4 sec | N.A | Zero |
| Data | E-mail (server access) | Primarily One-way |  | < 4 sec | N.A | Zero |

[ Note: The values provided in table 2 need to be further reviewed and may be revised in the next version of this specification]

## Table 3: End-user Performance Expectations - Streaming Services

| Medium | Application | Degree of symmetry | Data rate | Key performance parameters and target values |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | One-way Delay | Delay Variation | Information loss |
| Audio | High quality streaming audio | Primarily one-way | 32-128 kb/s | < 10 sec | < 1 msec | < 1% FER |
| Video | One-way | One-way | 32-384 kb/s | < 10 sec |  | < 1% FER |
| Data | Bulk data transfer/retrieval | Primarily one-way |  | < 10 sec | N.A | Zero |
| Data | Still image | One-way |  | < 10 sec | N.A | Zero |
| Data | Telemetry - monitoring | One-way | <28.8 kb/s | < 10 sec | N.A | Zero |

[ Note: The values provided in table 3 need to be further reviewed and may be revised in the next version of this specification]