| TSG-SA Working Group 1 (Services) meeting#6 TSGS1#6(99)xxx San Diego, USA, 29th November – 3rd December 1999 | TSG S1 | (99)1065 |
| --- | --- | --- |
|  | Agenda: |  |

| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 22.135 |  |  |  |  | CR |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.0.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submision to TSG |  |  |  |  | SA#6 |  |  |  | for approval |  |  |  |  |  | X |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  |  | ME |  |  |  |  |  |  |  | X | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | TSG SA WG1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 1999-12-03 |  |  |  |  |
| Subject: |  |  | Clarification on handling of multiple bearers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | The current Stage 1 contains a mixture the use of “bearer” and “call”. The handling of multiple voice calls in handover for future releases is clarified. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | 4.2.1, 5.1.1, 5.4, 5.7, 7.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

Note:	The protocol architecture in GSM allows several parallel CS calls, the limitation being that there is only one traffic channel, which the different CS calls share. This is facilitated by e.g. the Call Waiting, Call Hold, Call Transfer and Multiparty SSs. Call configurations related to GSM supplementary services are not considered as Multicall. See section 6 for interworking.

It shall be possible for each CS call / PS session to have independent traffic and performance characteristics. It shall be possible that each active CS call and PS session shall be terminated individually.

It shall be also possible that each of the CS calls and PS sessions have different priorities.

Note: 	Priority mechanism for CS calls and PS sessions at release 1999 is for further study.

The basic idea with CS multicall is that each CS call may have one dedicated bearer, i.e. it is possible that each new call (MO and MT) generate a new bearer.

Figure 1: Multicall concept

It is a requirement, that the current GSM supplementary services are preserved when suitable. Support of UMTS-GSM interworking and handovers, GSM evolution, GSM user conventions etc. are reasons for this requirement.

## 4.2	Multicall service scenarios

### 4.2.1	Terminating CS call

The indication of terminating CS call to mobile terminal will be done until the maximum number of total CS calls (Ncs) has been reached.

If the maximum number to total CS  call has been reached an  additional terminating call (Ncs+1) will be only indicated to the user if the user have the SS Call Waiting active. (Ncs is specified in chapter 5.2.) See chapter 6.4.2 for interworking with Call Waiting SS. The maximum value for Ncs=7 and no more calls may be supported, irrespective of whether the SS Call Waiting is active or not, once this has been reached.

If the Ncs  is not been reached  and a terminating call is indicated to the user she may reacted in the following way:

a)	accepting the terminating call

-	the user/user applications shall have the possibility to allocate a new bearer for the terminating call

-	the user/user applications shall have the possibility to reuse/share an already established bearer (e.g. release existing calls or  put an speech on hold and accept the terminating call.

b)	rejecting the terminating call

If the user/user application rejects the terminating call the call shall be released in a normal way

c)	ignoring the incoming call

If the user/user application ignores the indication of the terminating call (i.e. not accepts nor rejects) the terminating call the normal call handling shall apply, e.g.. after the Alerting Timer expires the call will be released.

### 4.2.2	Originating CS call

If the Ncs  is not reached already and the user/user application wants to establish a new originating CS call she may act in the following way:

- allocate a new bearer for the originating call

- reuse/share an already established bearer (e.g. to put an speech on hold and set-up a new call.

### 4.2.3	PS sessions

It shall be possible to have several PS sessions active simultaneously. See TS22.060 for further details.

PS sessions shall be handled independently of any CS calls.

Note:	There are no new PS related requirements from TS 22.060 point of view but there might be issues related to stage 2 and stage 3 that need to be considered.

### 4.2.4 Connectionless traffic

Multicall shall not impact the usage of SMS-PP, SMS-CB and USSD.

## 4.4	Charging aspects

It shall be possible to charge each call / session independently.

# 5	Functional requirements

## 5.1	Provision and withdrawal

### 5.1.1	Provision

The provision of multicall is provided by prior arrangement with home environment. If the multicall service is provisioned the limits for Ncs and Nps shall be set as subscription options. Ncs is equal to the number of bearers that a user is allowed when Call Hold or Call Waiting are not invoked.

### 5.1.2	Withdrawal

The multicall service subscription will be withdrawn on subscribers request or at administrative reasons.

## 5.2	Registration

User shall be able to  modify the maximum number of CS calls and number of PS sessions within the limitations set  at provision of the service.

## 5.3		Interrogation

User shall be able to interrogate the maximum number of CS calls and number of PS sessions set by user.

User shall be able to interrogate the maximum number of CS calls and number of PS sessions supported by serving network.

## 5.4	Limiting the number of multicalls

It should be possible for the number of active calls or sessions supported simultaneously to be restricted and selected by network operator, by the capabilities of the used terminal, by user subscription and/or user setting. The maximum number of CS calls and PS sessions should be set respectively. It shall be possible to have one or more CS calls simultaneously with one or more parallel PS sessions.

Standard shall be able to support up to 7  simultaneous CS Calls (Ncs). This is equal to the number of bearers when the Call Hold or Call Waiting are not invoked.

Standard shall be able to support up to 7 PS Sessions (Nps).

Terminals and networks may support any number of CS calls and PS Sessions within these limits.

It shall be possible to limit the maximum number of simultaneous bearers for CS calls to one, at this case GSM rel'98 functionality shall be shall provided.

The value of the maximum number of active speech calls is 1. Network shall not allow more than bearers allocated for speech.

## 5.6	Handover

The handover event can trigger changes to individual calls in any multicall scenario.

Priority setting of CS Calls and PS Sessions shall influence the handover  process.  It shall be possible to handover all calls and sessions. If the target cell is not able to accommodate all calls/sessions, then the calls/sessions that are handed over shall be selected in following order and the calls/sessions that cannot be handed over will be released.

The selection criteria shall be based on the following order:

- The call of teleservice emergency call

- If the user has set the priority, the call or session of highest priority marking. If there is more than one call or session of highest priority marking, then the call of teleservice telephony shall be chosen in preference to those of equal highest priority. If there are multiple calls where priority is the same, how to treat the calls depends on the operator.

- The call of teleservice telephony

- The call of any other type

If no single call can be selected according to the above criteria, handover shall be rejected.

Note:	Requirements shall be considered in an intra and inter system (e.g. UMTS to GSM) handover situations. In case of intra UMTS handover it shall be possible handover all calls when resources permit. In case of UMTS to GSM handover only one call can be handed over.  A change in the availability of suitable radio resources may also occur for other reasons in addition to handover.

For further handover requirements please refer to TS 22.129.

NOTE 	This section may be later transferred to TS 22.129.

## 5.7	Busy Definition

NDUB (Network Determined User Busy) occurs when:

- The maximum number of calls has been reached. In the case where Call Waiting or Call Hold are not subscribed to, this is equal to maximum number of bearers. In the case with Multiparty this can occur even if bearers are still available.

- In the case where Call Waiting or Call Hold are subscribed to, the maximum number of bearers is reached, and the maximum number of both waiting and held calls has been reached.

NOTE:	This implies that CFB according to NDUB will only be invoked if the maximum number of CS calls is reached.

For User Determined User Busy (UDUB) condition see GSM 02.01 Annex C.

## 5.8	Exceptional procedures or unsuccessful outcome

If the subscriber requests to set the limits of Ncs and/or Nps to higher values as allowed according to the provision (subscription option), this request shall be rejected and the subscriber shall be informed on the unsuccessful outcome of the request.

Roaming into networks not supporting multicall shall be possible and at this case GSM rel'98 functionality shall apply .

In case there is a difference between the maximum numbers (Ncs, Nps) , supported by the serving network,  by the capabilities of the used terminal and/or by the user setting (according to the user subscription options), the smallest value should be applied as the maximum number.

# 6	Interaction with other services

## 6.1	General on Supplementary Services

Relation between multicall and supplementary services are considered only for CS calls. .

## 6.2	Line Identification

### 6.2.1	Calling Line Identification Presentation (CLIP)

No impact, i.e. CLIP shall be provided with all calls.

### 6.2.2	Calling Line Identification Restriction (CLIR)

No impact, i.e. CLIR shall be provided with all calls.

### 6.2.3	Connected Line Identification Presentation (COLP)

No impact, i.e. COLP shall be provided with all calls.

### 6.2.4	Connected Line Identification Restriction (COLR)

No impact, i.e. COLR shall be provided with all calls.

## 6.3	Call Forwarding

### 6.3.1	Call Forwarding Unconditional (CFU)

No impact.

### 6.3.2	Call Forwarding on Busy (CFB)

No impact.

### 6.3.3	Call Forwarding on No Reply (CFNRy)

No impact.

### 6.3.4	Call Forwarding on Not Reachable (CFNRc)

No impact.

## 6.4	Call Completion

### 6.4.1		Call Hold (CH)

No impact, i.e. it shall be possible to put  an established speech call on hold.

### 6.4.2	Call Waiting (CW)

The indication of a terminating CS call within the maximum number of calls is done by multicall feature; whereas the indication of a terminating call to the user / terminal by Call Waiting function is done when  multicall limit (Ncs) has been reached and the subscriber has CW active.

When the supplementary service “Call Waiting” is applicable the maximum number of  calls is M+ Ncs , where M is maximum number of waiting calls, which is specified in 22.083.

NOTE:	Due to that there is no change to the maximum number of waiting calls required from the multicall service the maximum number of waiting calls is still 1.

## 6.5	Multi Party (MPTY)

No Impact.

The number of MPTY member may be limited because of number of existing CS calls.

## 6.6	Closed User Group (CUG)

No impact.

## 6.7	Advice of Charge (AoC)

It shall be possible for network to indicate the AoC parameters for each CS call.

It shall be possible for mobile terminal to count each CS Call charges respectively and to have overall ACM (Accumulated Call Meter as defined in 22.024) for all the calls.

## 6.8	Call Barring

No impact.

### 6.8.1	Barring of all outgoing calls

No impact.

### 6.8.2	Barring of outgoing international calls

No impact.

### 6.8.3	Barring of outgoing international calls except those directed to the HPLMN country

No impact.

### 6.8.4	Barring of all incoming calls

No impact.

### 6.8.5	Barring of incoming calls when roaming

No impact.

## 6.9	Explicit Call Transfer (ECT)

No impact.

## 6.10	Completion of Call to Busy Subscriber (CCBS)

No impact.

## 6.11	Multiple Subscriber Profile (MSP)

No impact.

## 6.12	Calling Name Presentation (CNAP)

No impact.

## 6.13	User-to-User Signalling (UUS)

No Impact

## 6.14	enhanced Multi-Level Precedence and Pre-emption service (eMLPP)

No impact.

## Call Deflection (CD)

No impact.

## 6.16	CAMEL

No impact.

## 6.17	IST

No impact.

# 7	Cross Phase Compatibility for R99

This section details the cross phase compatibility requirements relating to the service requirements in this document.

Note: when a change is introduced which affects the 3GPP specifications, it is said to be 'backward compatible' if existing equipment can continue to operate and perform correctly with equipment that conforms to the new implementation.

## 7.1		Compatibility With Existing Standards

Where the service and operational requirements in this document relate to a core network functionality, compatibility is required.

Multicall mechanisms is not applicable for GSM BSS.

## 7.2	Compatibility With Future Releases

It is envisaged that 3GPP standards will evolve beyond R99, for example with the addition of new service requirements. The standards which define the technical implementation of R99 should be developed in such a way that it is practical to add the requirements in this section in a backward compatible manner.

Following chapters include requirements that are foreseen for future release.

### 7.2.1	Multicall configuration

When having one active CS call and one held call on the same bearer. It shall be possible to create a new CS bearer and to move one of the calls to the new bearer, resulting both calls being active within the limits set by the operator/user and within the capability of the terminal. See figure 2: Split of bearer.

When having two calls (multicall) on the separate bearers. It shall be possible to join both calls to one of the two  bearers, put the one of the calls to hold and to release unused CS bearer. It shall be possible to select which call to put on hold. See figure 2: Combination of bearers. (Note: there is no clear end-user service requirement for this feature at time being.)

NOTE: Due to that only speech calls can be put on hold, so one of the two active Cs calls has to be a speech call

Figure2: Illustration for split of bearer and combination of bearers.

### 7.2.2	Several simultaneous speech calls / bearers

Key requirements for multicall is to allow several simultaneous CS call. The most important usage scenario is to allow several CS data bearers to be bind at application level resulting to higher than 64kbits/s data rates. Other important feature is just general flexibility allowing e.g. simultaneous speech and data call. It's been also required to have several simultaneous active speech calls.

It's been proposed that the multicall feature could be introduced in a phased manner, meaning that in the first phase, i.e. UMTS phase 1, release 99 only one active speech call would be supported. However, Call control should not prohibit a complete set of multiple speech bearer services in future releases and UTRAN shall be designed in a flexible way to support multiple speech bearers. In Release 99, GSM SS Call Wait, Multiparty and Call Hold are used to offer simultaneous speech calls to user.

If multiple simultaneous voice calls are supported then the Call Hold service shall be used to reconfigure the number of bearers supporting voice calls if required during handover. e.g. in the case of handover to GSM where only one voice call can be active at a time. This requirement is dependent on the user subscribing to Call Hold.

### 7.2.3	CCBS

At release 1999 CCBS  no enhancements for CCBS is required.

In the future releases the definition of IDLE state of subscriber A and destination B should be modified in away that the IDLE state is reach even if there are active CS calls but the maximum limit of CS calls is not reached.