TSG-SA Working Group 1 (Services) meeting #6	TSG S1 (99) 1073

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item: 4.1

# proposed Liaison Statement

To:	SMG9 / TSG-T3, T2, S1 MMS ad-hoc

From:	TSG-S1

Subject:	RE: Introduction of Mailbox Presence & Count Indication Service

___________________________________________________________________________

TSG-S1 has received the attached LS on the above-mentioned topic from SMG9.

S1 thanks SMG9 for their proposal on what is believed to be a useful feature. However, S1 would like to point out that there has been substantial work done on messaging services in rel. ´99, e.g. the introduction of the Multimedia Messaging Service (MMS). It is therefore felt that the service requirements for a mailbox field on SIM, and the count indication, must take also this new messaging service into account, so that SIM/ USIM as well as mobiles need not support two different versions for what could be the same type of service.

S1 therefore also invites the S1 MMS ad-hoc group as well as TSG-T2 to study the service requirements for Mailbox notifications and storage, and recommends SMG9/ T3 not to progress their work until these service requirements are elaborated.

TSG-SA Working Group 1 (Services) meeting #6	TSG S1 (99) 1058

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item: 4.1

| ETSI SMG9 Meeting #19 Munich, Germany, September 20 - 23, 1999 | Tdoc 9-99-276 |
| --- | --- |

# Liaison Statement

To:	SMG1

From:	SMG9

Subject:	Introduction of Mailbox Presence & Count Indication Service

___________________________________________________________________________

A request was received to define a file on the SIM to support mailbox information. Currently this information has to be stored in the Mobile for GSM900 systems.

SMG9 have considered the support of a Mailbox file for various Mailbox types. e.g. Voice,Fax,Email.  They have agreed that this would be a useful feature. Although SMG9 can specify a file on the SIM card to store the mailbox data as for phonebook entries, it was realised that this feature could be expanded to include data indicating the number of unread messages for each type of mailbox along with information of how this indication is presented to the user. However for this feature to be considered a service requirement must be specified and the appropriate procedures to handle such a service defined.

SMG9 ask SMG1 to consider if this is a required service and if so, make recommendations to the relevant groups to put in place the procedures necessary to support such a service.

SMG9 look forward to hearing SMG1’s response to this service request.

ETSI SMG9 Meeting #19	Tdoc SMG 9-99-273

Munich, Germany,

September 20-23, 1999

| CHANGE REQUEST No : |  |  |  |  | Axxx |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 11.11 |  | Version: |  |  | 8.0.0 |  |  |  |
| Submitted to SMG | #30 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3a.doc |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM | X | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | SMG9 | Date: | 22nd Sept 1999 |
| --- | --- | --- | --- |

| Subject: | Interoperability between GSM 900, 1800 and 1900 MS for Mail Box Numbers |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Currently there are a number of places where mail box numbers are stored: in the handset's non-volatile memory or in a non-standardised SIM file, however, there is no standardised method defined in GSM recommendation 11.11 for such purposes. The following contribution proposes the addition of a new Elementary File (EF) that is to be used as the source of numbers for various types of mailboxes. The proposal is to have a standardised file, which resides under the TELECOM directory [7F10] that would be available for use with any SIM based mobile. By placing the file under the TELECOM directory, the proposed file would enable the following benefits: - Help facilitate the migration from non-standard methods (i.e. CPHS, currently being used by GSM 1800 and 1900) to a more formally defined method - Provide a consistent solution for all GSM terminals (single, dual and triple band 900/1800/1900) - Provide a path forward for SIM based multi-mode terminals (GAIT, GPRS-136HS) |
| --- | --- |

| Clauses affected: | 10.3.7, 10.5.16 (new), 10.7, 11, 11.5.1, Annex D |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 10.3.7	EFSST (SIM service table)

This EF indicates which services are allocated, and whether, if allocated, the service is activated. If a service is not allocated or not activated in the SIM, the ME shall not select this service.

| Identifier: '6F3F' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: X bytes, X ≥ 2 |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n°1 to n°4 |  |  | M |  | 1 byte |
| 2 | Services n°5 to n°8 |  |  | M |  | 1 byte |
| 3 | Services n°9 to n°12 |  |  | O |  | 1 byte |
| 4 | Services n°13 to n°16 |  |  | O |  | 1 byte |
| 5 | Services n°17 to n°20 |  |  | O |  | 1 byte |
| 6 | Services n°21 to n°24 |  |  | O |  | 1 byte |
| 7 | Services n°25 to n°28 |  |  | O |  | 1 byte |
| 8 | Services n°29 to n°32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services (4X-3) to (4X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1 : | CHV1 disable function |
|  | Service n°2 : | Abbreviated Dialling Numbers (ADN) |
|  | Service n°3 : | Fixed Dialling Numbers (FDN) |
|  | Service n°4 : | Short Message Storage (SMS) |
|  | Service n°5 : | Advice of Charge (AoC) |
|  | Service n°6 : | Capability Configuration Parameters (CCP) |
|  | Service n°7 : | PLMN selector |
|  | Service n°8 : | RFU |
|  | Service n°9 : | MSISDN |
|  | Service n°10: | Extension1 |
|  | Service n°11: | Extension2 |
|  | Service n°12: | SMS Parameters |
|  | Service n°13: | Last Number Dialled (LND) |
|  | Service n°14: | Cell Broadcast Message Identifier |
|  | Service n°15: | Group Identifier Level 1 |
|  | Service n°16: | Group Identifier Level 2 |
|  | Service n°17: | Service Provider Name |
|  | Service n°18: | Service Dialling Numbers (SDN) |
|  | Service n°19: | Extension3 |
|  | Service n°20: | RFU |
|  | Service n°21: | VGCS Group Identifier List (EFVGCS and EFVGCSS) |
|  | Service n°22: | VBS Group Identifier List (EFVBS and EFVBSS) |
|  | Service n°23: | enhanced Multi-Level Precedence and Pre-emption Service |
|  | Service n°24: | Automatic Answer for eMLPP |
|  | Service n°25: | Data download via SMS-CB |
|  | Service n°26: | Data download via SMS-PP |
|  | Service n°27: | Menu selection |
|  | Service n°28: | Call control |
|  | Service n°29: | Proactive SIM |
|  | Service n°30: | Cell Broadcast Message Identifier Ranges |
|  | Service n°31: | Barred Dialling Numbers (BDN) |
|  | Service n°32: | Extension4 |
|  | Service n°33: | De-personalization Control Keys |
|  | Service n°34: | Co-operative Network List |
|  | Service n°35: | Short Message Status Reports |
|  | Service n°36: | Network's indication of alerting in the MS |
|  | Service n°37: | Mobile Originated Short Message control by SIM |
|  | Service n°38: | GPRS |
|  | Service n°39: | Image (IMG) |
|  | Service n°40: | SoLSA (Support of Local Service Area) |
|  | Service n°41: | USSD string data object supported in Call Control |
|  | Service n°42: | Support of Mail Box Dialling Numbers |

[..]

### 10.5.16	EFMBXDN (Mail Box Dialling Numbers)

This EF contains dialling numbers for access to mailboxes for voice mail, fax mail and e-mail. It may also contain associated alpha-tags for each supported mailbox.

| Identifier: '6F4F' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X+15 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1 INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | Alpha Identifier |  |  | O |  | X bytes |
| X+1 | Length of BCD number/SSC contents |  |  | M |  | 1 byte |
| X+2 | TON and NPI |  |  | M |  | 1 byte |
| X+3 to X+12 | Dialling Number |  |  | M |  | 10 bytes |
| X+13 | Capability/Configuration Identifier |  |  | M |  | 1 byte |
| X+14 | Extension 1 Record Identifier |  |  | M |  | 1 byte |
| X+15 | Mail Box Type Identifier |  |  | M |  | 1 byte |

-	Mail Box Type Identifier

Contents:

This byte identifies the mailbox type(s) associated with the Dialling Number. It shall be possible to associate a mailbox type with multiple Dialling Numbers. In this case the first Dialling Number being associated with a mailbox type shall be used as the primary Dialling Number and the second shall be used for any proprietary implementations (e.g. two Voice Mail records could exist, the first record for Telephony services and the second for Auxiliary Telephony)

Coding:

1 bit is used to associate a Mail Box type:

bit = 1: Mail Box type associated with Dialling Number

bit = 0: Mail Box type not associated with Dialling Number

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Voice Mail |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fax Mail |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E-mail |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |

- For contents and coding of data items 1 to (X+14) see the respective data items of the EFADN (GSM 11.11 sub-clause 10.5.1).

NOTE:	The value of X (the number of bytes in the alpha-identifier) may be different to the length denoted X in EFADN.

[..]

## 10.7 Files of GSM

This subclause contains a figure depicting the file structure of the SIM. DFGSM shall be selected using the identifier '7F20'. If selection by this means fails, then DCS 1800 MEs shall, and optionally GSM MEs may then select DFGSM with '7F21'.

NOTE 1:	The selection of the GSM application using the identifier '7F21', if selection by means of the identifier '7F20' fails, is to ensure backwards compatibility with those Phase 1 SIMs which only support the DCS 1800 application using the Phase 1 directory DFDCS1800 coded '7F21'.

NOTE 2:	To ensure backwards compatibility with those Phase 1 DCS 1800 MEs which have no means to select DFGSM two options have been specified. These options are given in GSM 09.91 [17]

| . |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |
| DFGSM |  |  | DFTELECOM |  |  | DFIS-41 |  |  | DFFP-CTS |  |  |  |  |  | EFICCID |  |  | EFELP |  |
| '7F20' |  |  | '7F10' |  |  | '7F22' |  |  | '7F23' |  |  |  |  |  | '2FE2' |  |  | '2F05' |  |
|  |  |  |  |  |  |  |  |  | see GSM 11.19 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | EFADN |  |  | EFFDN |  |  | EFSMS |  |  | EFCCP |  |  | EFMSISDN |  |
|  |  |  |  |  |  | '6F3A' |  |  | '6F3B' |  |  | '6F3C' |  |  | '6F3D' |  |  | '6F40' |  |
|  |  |  |  |  |  | EFSMSP |  |  | EFSMSS |  |  | EFLND |  |  | EFSMSR |  |  | EFSDN |  |
|  |  |  |  |  |  | '6F42' |  |  | '6F43' |  |  | '6F44' |  |  | '6F47' |  |  | '6F49' |  |
|  |  |  |  |  |  | EFEXT1 |  |  | EFEXT2 |  |  | EFEXT3 |  |  | EFBDN |  |  | EFEXT4 |  |
|  |  |  |  |  |  | '6F4A' |  |  | '6F4B' |  |  | '6F4C' |  |  | '6F4D' |  |  | '6F4E' |  |
|  |  |  |  |  |  | EFMBXDN |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '6F4F' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DFGRAPHICS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '5F50' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | EFIMG |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | '4F20' |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DFIRIDIUM |  |  | DFGLOBST |  |  | DFICO |  |  | DFACeS |  |  |  |  |  |  |  |
|  |  |  | '5F30' |  |  | '5F31' |  |  | '5F32' |  |  | '5F33' |  |  |  |  |  |  |  |
|  |  |  | DFPCS1900 |  |  | DFCTS |  |  | DFSoLSA |  |  |  |  |  |  |  |  |  |  |
|  |  |  | '5F40' |  |  | '5F60' |  |  | '5F70' |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | see GSM 11.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | EFSAI |  |  | EFSLL |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | '4F30' |  |  | '4F31' |  |  |  |  |
|  |  |  | EFLP |  |  | EFIMSI |  |  | EFKc |  |  | EFPLMNsel |  |  | EFHPLMN |  |  | EFACMmax |  |
|  |  |  | '6F05' |  |  | '6F07' |  |  | '6F20' |  |  | '6F30' |  |  | '6F31' |  |  | '6F37' |  |
|  |  |  | EFSST |  |  | EFACM |  |  | EFGID1 |  |  | EFGID2 |  |  | EFPUCT |  |  | EFCBMI |  |
|  |  |  | '6F38' |  |  | '6F39' |  |  | '6F3E' |  |  | '6F3F' |  |  | '6F41' |  |  | '6F45' |  |
|  |  |  | EFSPN |  |  | EFCBMID |  |  | EFBCCH |  |  | EFACC |  |  | EFFPLMN |  |  | EFLOCI |  |
|  |  |  | '6F46' |  |  | '6F48' |  |  | '6F74' |  |  | '6F78' |  |  | '6F7B' |  |  | '6F7E' |  |
|  |  |  | EFAD |  |  | EFPHASE |  |  | EFVGCS |  |  | EFVGCSS |  |  | EFVBS |  |  | EFVBSS |  |
|  |  |  | '6FAD' |  |  | '6FAE' |  |  | '6FB1' |  |  | '6FB2' |  |  | '6FB3' |  |  | '6FB4' |  |
|  |  |  | EFeMLPP |  |  | EFAAeM |  |  | EFECC |  |  | EFCBMIR |  |  | EFNIA |  |  | EFKcGPRS |  |
|  |  |  | '6FB5' |  |  | '6FB6' |  |  | '6FB7' |  |  | '6F50' |  |  | '6F51' |  |  | '6F52' |  |
|  |  |  | EFLOCIGPRS |  |  | EFSUME |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | '6F53' |  |  | '6F54' |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 8: File identifiers and directory structures of GSM

# 11	Application protocol

[..]

Subscription related procedures:

-	Dialling Numbers (ADN, FDN, MSISDN, LND, SDN, BDN, MBXDN)	MMI/ME

-	Short messages (SMS)	MMI

-	Advice of Charge (AoC)	MMI

-	Capability Configuration Parameters (CCP)	MMI

-	PLMN Selector	MMI

-	Cell Broadcast Message Identifier (CBMI)	MMI

-	Group Identifier Level 1 (GID1)	MMI/ME

-	Group Identifier Level 2 (GID2)	MMI/ME

-	Service Provider Name (SPN)	ME

-	Voice Group Call Service (VGCS)	MMI/ME

-	Voice Broadcast Service (VBS)	MMI/ME

-	Enhanced Multi Level Pre-emption and Priority (eMLPP)	MMI/ME

-	Depersonalisation Control Keys	ME

-	Short message status reports (SMSR)	MMI

-	Network's indication of alerting	ME

[..]

[..]

## 11.5	Subscription related procedures

### 11.5.1	Dialling numbers

The following procedures may not only be applied to EFADN and its associated extension files EFCCP and EFEXT1 as described in the procedures below, but also to EFFDN, EFMSISDN, EFLND, EFBDN,EFSDN and EFMBXDN and their associated extension files. If these files are not allocated and activated, as denoted in the SIM service table, the current procedure shall be aborted and the appropriate EFs shall remain unchanged.

As an example, the following procedures are described as applied to ADN.

Requirement:	Service n°2 "allocated and activated"

(Service n°3 for FDN,

Service n°9 for MSISDN,

Service n°13 for LND,

Service n°18 for SDN,

Service n°31 for BDN,

Service n°42 for MBXDN)

Update:	The ME analyses and assembles the information to be stored as follows (the byte identifiers used below correspond to those in the description of the EFs in subclauses 10.5.1, 10.5.4 and 10.5.10):

i)	The ME identifies the Alpha-tagging, Capability/Configuration Identifier and Extension1 Record Identifier.

ii)	The dialling number/SSC string shall be analysed and allocated to the bytes of the EF as follows:

-	if a "+" is found, the TON identifier is set to "International";

-	if 20 or less "digits" remain, they shall form the dialling number/SSC string;

-	if more than 20 "digits" remain, the procedure shall be as follows:

Requirement:

Service n°10 "allocated and activated"

(Service n°10 applies also for MSISDN, LND and MBXDN;

Service n°11 for FDN;

Service n°19 for SDN;

Service n°32 for BDN.)

The ME seeks for a free record in EFEXT1. If an Extension1 record is not marked as "free", the ME runs the Purge procedure. If an Extension1 record is still unavailable, the procedure is aborted.

The first 20 "digits" are stored in the dialling number/SSC string. The value of the length of BCD number/SSC contents is set to the maximum value, which is 11. The Extension1 record identifier is coded with the associated record number in the EFEXT1. The remaining digits are stored in the selected Extension1 record where the type of the record is set to "additional data". The first byte of the Extension1 record is set with the number of bytes of the remaining additional data. The number of bytes containing digit information is the sum of the length of BCD number/SSC contents of EFADN and byte 2 of all associated chained Extension1 records containing additional data (see subclauses 10.5.1 and 10.5.10).

iii)	If a called party subaddress is associated to the ADN/SSC the procedure shall proceed as follows:

Requirement:

Service n°10 "allocated and activated"

(Service n°10 applies also for MSISDN, LND and MBXDN;

Service n°11 for FDN;

Service n°19 for SDN;

Service n°32 for BDN.)

If the length of the called party subaddress is less than or equal to 11 bytes (see GSM 04.08 [15] for coding):

The ME seeks for a free record in EFEXT1. If an Extension1 record is not marked as "free", the ME runs the Purge procedure. If an Extension1 record is still unavailable, the procedure is aborted.

The ME stores the called party subaddress in the Extension1 record, and sets the Extension1 record type to "called party subaddress".

If the length of the called party subaddress is greater than 11 bytes (see GSM 04.08 [15] for coding):

The ME seeks for two free records in EFEXT1. If no such two records are found, the ME runs the Purge procedure. If two Extension1 records are still unavailable, the procedure is aborted.

The ME stores the called party subaddress in the two Extension1 records. The identifier field in the Extension1 record containing the first part of the subaddress data is coded with the associated EFEXT1 record number containing the second part of the subaddress data. Both Extension1 record types are set to "called party subaddress".

Once i), ii), and iii) have been considered the ME performs the updating procedure with EFADN. If the SIM has no available empty space to store the received ADN/SSC, or if the procedure has been aborted, the ME advises the user.

NOTE 1:	For reasons of memory efficiency the ME is allowed to analyse all Extension1 records to recognize if the additional or subaddress data to be stored is already existing in EFEXT1. In this case the ME may use the existing chain or the last part of the existing chain from more than one ADN (LND, MSISDN, MBXDN). The ME is only allowed to store extension data in unused records. If existing records are used for multiple access, the ME shall not change any data in those records to prevent corruption of existing chains.

Erasure:	The ME sends the identification of the information to be erased. The content of the identified record in EFADN is marked as "free".

Request:	The ME sends the identification of the information to be read. The ME shall analyse the data of EFADN (subclause 10.5.1) to ascertain, whether additional data is associated in EFEXT1 or EFCCP. If necessary, then the ME performs the reading procedure on these EFs to assemble the complete ADN/SSC.

Purge:	The ME shall access each EF which references EFEXT1 (EFEXT2) for storage and shall identify records in these files using extension data (additional data or called party subaddress). Note that existing chains have to be followed to the end. All referred Extension1 (Extension2) records are noted by the ME. All Extension1 (Extension2) records not noted are then marked by the ME as "free" by setting the whole record to 'FF'.

NOTE 2:	Dependent upon the implementation of the ME, and in particular the possibility of erasure of ADN/SSC records by Phase 1 MEs, which have no knowledge of the EFEXT1, it is possible for Extension1 records to be marked as "used space" (not equal to 'FF'), although in fact they are no longer associated with an ADN/SSC record.

The following three procedures are only applicable to service n°3 (FDN).

FDN capability request. The ME has to check the state of service n°3, i.e. if FDN is "enabled" or "disabled". In case of enabled FDN, the ME has to switch to a restrictive terminal mode (see GSM 02.07). To ascertain the state of FDN, the ME checks in EFSST whether or not ADN is activated. If ADN is not activated, service n°3 is enabled. If ADN is activated, the ME checks the response data of EFADN. If EFADN is invalidated, service n°3 is enabled. In all other cases service n°3 is disabled.

FDN disabling. The FDN disabling procedure requires that CHV2 verification procedure has been performed successfully and that ADN is activated. If not, FDN disabling procedure will not be executed successfully. To disable FDN capability, the ME rehabilitates EFADN. The invalidate/rehabilitate flag of EFADN, which is implicitly set by the REHABILITATE command, is at the same time the indicator for the state of the service n°3. If ADN is not activated, disabling of FDN is not possible and thus service n°3 is always enabled (see FDN capability request).

NOTE 3:	If FDN is disabled (by rehabilitating EFADN) using an administrative terminal then the FDN disabling procedure of this administrative terminal need also to rehabilitate EFIMSI and EFLOCI to ensure normal operation of the SIM in a phase 1 ME or a phase 2 ME which does not support FDN.

FDN enabling. The FDN enabling procedure requires that CHV2 verification procedure has been performed successfully. If not, FDN enabling procedure will not be executed successfully. To enable FDN capability, the ME invalidates EFADN. The invalidate/rehabilitate flag of EFADN, which is implicitly cleared by the invalidate command, is at the same time the indicator for the state of the service n°3 (see FDN capability request). If ADN is not activated, service n°3 is always enabled.

Invalidated ADNs may optionally still be readable and updatable depending on the file status (see clause 9.3)

The following three procedures are only applicable to service n°31 (BDN).

BDN capability request. The ME has to check the state of service n°31, i.e. if BDN is "enabled" or "disabled". BDN service is "enabled" only if service n°31 is allocated and activated, and EFBDN is not invalidated. In all other cases, the BDN service is "disabled".

BDN disabling. The BDN disabling procedure requires that CHV2 verification procedure has been performed successfully. If not, BDN disabling procedure will not be executed successfully. To disable BDN capability, the ME invalidates EFBDN. The invalidate/rehabilitate flag of EFBDN, which is implicitly cleared by the INVALIDATE command, is at the same time the indicator for the state of the service n°31 (see BDN capability request).

BDN enabling. The BDN enabling procedure requires that CHV2 verification procedure has been performed successfully. If not, BDN enabling procedure will not be executed successfully. To enable BDN capability, the ME rehabilitates EFBDN. The invalidate/rehabilitate flag of EFBDN, which is implicitly set by the REHABILITate command, is at the same time the indicator for the state of the service n°31 (see BDN capability request).

Invalidated BDNs (when BDN capability is disabled) may optionally still be readable and updatable depending on the file status (see clause 9.3).

Annex D (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification | Description | Value |
| --- | --- | --- |
| '2F E2' | ICC identification | operator dependant (see 10.1.1) |
| '2F 05' | Extended Language preference | 'FF…FF' |
| '6F 05' | Language preference | 'FF' |
| '6F 07' | IMSI | operator dependant (see 10.3.2) |
| '6F 20' | Ciphering key Kc | 'FF...FF07' |
| '6F 30' | PLMN selector | 'FF...FF' |
| '6F 31' | HPLMN search period | 'FF' |
| '6F 37' | ACM maximum value | '000000' (see note 1) |
| '6F 38' | SIM service table | operator dependant (see 10.3.7) |
| '6F 39' | Accumulated call meter | '000000' |
| '6F 3E' | Group identifier level 1 | operator dependant |
| '6F 3F' | Group identifier level 2 | operator dependant |
| '6F 41' | PUCT | 'FFFFFF0000' |
| '6F 45' | CBMI | 'FF...FF' |
| '6F 46' | Service provider name | 'FF...FF' |
| '6F 48' | CBMID | 'FF...FF' |
| '6F 49' | Service Dialling Numbers | 'FF...FF' |
| '6F 74' | BCCH | 'FF...FF' |
| '6F 78' | Access control class | operator dependant (see 10.1.12) |
| '6F 7B' | Forbidden PLMNs | 'FF...FF' |
| '6F 7E | Location information | 'FFFFFFFF xxFxxx 0000 FF 01' (see note 2) |
| '6F AD' | Administrative data | operator dependant (see 10.3.15) |
| '6F AE' | Phase identification | see 10.3.16 |
| '6F 3A' | Abbreviated dialling numbers | 'FF...FF' |
| '6F 3B' | Fixed dialling numbers | 'FF...FF' |
| '6F 3C' | Short messages | '00FF...FF' |
| '6F 3D' | Capability configuration parameters | 'FF...FF' |
| '6F 40' | MSISDN storage | 'FF...FF' |
| '6F 42' | SMS parameters | 'FF...FF' |
| '6F 43' | SMS status | 'FF...FF' |
| '6F 44' | Last number dialled | 'FF...FF' |
| '6F 47' | Short message status reports | '00FF…FF' |
| '6F 4A' | Extension 1 | 'FF...FF' |
| '6F 4B' | Extension 2 | 'FF...FF' |
| '6F 4C' | Extension 3 | 'FF...FF' |
| '6F 4D' | Barred dialling numbers | 'FF...FF' |
| '6F 4E' | Extension 4 | 'FF...FF' |
| '6F 4F' | Mail box dialling numbers | 'FF...FF' |
| '6F 51' | Network's indication of alerting | 'FF...FF' |
| '6F 52' | GPRS Ciphering key KcGPRS | 'FF...FF07' |
| '6F 53' | GPRS Location Information | 'FFFFFFFF FFFFFF xxFxxx 0000 FF 01' |
| '6F 54' | SetUpMenu Elements | operator dependant (see 10.3.34) |
| '4F 20' | Image data | '00FF...FF' |
| '4F 30' | SoLSA Access Indicator) | '00FF...FF' |
| '4F 31' | SoLSA LSA List | 'FF...FF' |

NOTE 1:	The value '000000' means that ACMmax is not valid, i.e. there is no restriction on the ACM. When assigning a value to ACMmax, care should be taken not to use values too close to the maximum possible value 'FFFFFF', because the INCREASE command does not update EFACM if the units to be added would exceed 'FFFFFF'. This could affect the call termination procedure of the Advice of Charge function.

NOTE 2:	xxFxxx stands for any valid MCC and MNC, coded according to GSM 04.08 [15].