TSG-SA Working Group 1 (Services) meeting #5	TSG S1 (99) 871

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item: 6.4

3GPP TSG-S1 / T2 MMS AdHoc

Ascot, UK, 9-10 November 1999	T2M(99)012

From:	TSG SA WG1 MMS adhoc

To:	TSG SA WG3

CC:

Liaison Statement on security requirements for
Multimedia Messaging Service (MMS)

In the TSG SA-WG1 MMS adhoc meeting  (9. Nov. Ascot, UK) substantial progress has been made in finalizing  stage 1 of the Multi Media Messaging  Service (3G TS 22.140 version 1.1.0)  for release ’99.

Chapter 7 of the specification contains security requirements of MMS.

Some background information on the document:

MMS enables a unified application which integrates the composition, storage, access, and delivery of different kinds of media, e.g. text, voice, image or video in combination with additional mobile requirements.

It should be noted that, as it is stated in the introduction of the document, “ 3GPP shall not standardise new services themselves, but instead uses the standardised set of service capabilities features on which the new services will be built”.  
In the light of this general 3GPP principle the specification lists requirements which need to be considered when specifying the means to allow (third party) applications to implement the service.

A question which arose while discussing security issues was the following:

Chapter 7  states that  “A recipient shall be informed of the reliability of the identity of the sender in case the sender as authorised his identity to be transmitted”
This was seen to be useful e.g. in the case of a Multimedia Message received through e-mail (in which case the reliability of the identity of the sender is low) as opposed to a MM received from a terminal.

There was uncertainty if:

- The identity of the sender of a Multimedia Message is always to be transmitted (as it happens now with SMS) by the terminal or

- The identity may be withheld by the sender  (CLIP/CLIR like)

Conclusion:

TSG SA-WG1 MMS adhoc would like to invite TSG SA-WG3 to review the security aspects of MMS and if appropriate contribute to it.  In particular we appreciate your comment on the above mentioned question.

For the convenience of the reader  chapter  7 of  3G TS 22.140 version 1.1.0 is  duplicated below while the complete specification is included as a separate document.

- Security

The user shall be able to use and access MM in a secure manner. It shall be possible for the contents of MMs to be read only by the intended recipient(s).  A recipient shall be informed of the reliability of the identity of the sender in case the sender as authorised his identity to be transmitted.

The integrity of MMs during transit shall be assured to extent of the network capabilities.

The MMS shall be intrinsically resistant to attempts of malicious or fraudulent use.

The “Security Threats and Requirements” specified in 22.133 [3] shall not be compromised.

3G TS 22.140 1.1.0 (1999-11)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects

Service aspects;

Stage 1

Multimedia Messaging Service

(3G TS 22.140 version 1.1.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTR/TSGS-0122971U

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

# Contents

Contents	3

Foreword	3

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	High level Requirements	6

5	General Requirements	6

5.1	Multimedia message management	6

5.2	Multimedia message delivery	6

5.3	Notification and Acknowledgement	6

5.4	Addressing	6

6	Profile	6

7	Security	6

8	Charging	6

9	External Interface	6

10	Interworking	6

History	6

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TR, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# Introduction

SMS has been very successful in the GSM second generation system, as all mobiles have supported the application level and it is possible to send to any GSM handset without the need to check for individual support.  This easy to use service for non realtime text transmission between GSM users shall be succeeded to in third generation mobile systems by a non real-time Multimedia Message Service, MMS. The MMS will allow users to send and receive messages exploiting the whole array of media types available today e.g. text, images, audio, video while also making it possible to support new content types as they become popular.

3GPP shall not standardise new services themselves, but instead uses the standardised set of service capabilities features on which the new services will be built.

Multimedia technology  a rapidly developing allowing new capabilities, such as multimedia messages, games, presentations and services that are now considered to be a part of every day life. Multimedia consists of one or more media elements (such as text, voice, image and video), and it is the combination of these media elements in a ordered synchronised manner that creates a multimedia presentation.

A non-realtime multimedia message as observed by the user is a combination of one or more different media elements in a multimedia presentation, that can be transferred between users without the requirement for the need to be transferred in realtime.  The non-real-time multimedia messaging service shall be capable of supporting current and future multimedia messaging services, and exploit the advances being made in the world multimedia community, with additional mobile requirements.

# Scope

This Technical Specification defines the stage one description of the non real-time Multimedia Messaging Service, MMS. Stage one is  the set of requirements which shall be supported for the provision of non real-time multimedia messaging service, seen primarily from the subscriber’s and service providers’ points of view.

This TS includes information applicable to network operators, service providers, terminal and network manufacturers.

This TS contains the core requirements for the Multimedia Messaging Service, which are sufficient to provide a complete service.

Additional functionalities not documented in this TS may implement requirements which are considered outside the scope of this TS. Such additional functionality may be on a network-wide basis, nation-wide basis or particular to a group of users. Such additional functionality shall not compromise conformance to the core requirements of the service.

This TS defines the requirements for MMS to be understood as a framework to enable non real-time transmissions for different types of media including such functionality as:-

- multiple media elements per single message

- individual handling of message elements

- different delivery methods for each message element

- negotiate different terminal and network MM capabilities

- notification and acknowledgement of MM related events (e.g. delivery, deletion, ...)

- handling of undeliverable MM

- personalised MMS configuration

- flexible charging

The above list is not exhaustive.

Thus the MMS enables a unified application which integrates the composition, storage, access, and delivery of different kinds of media, e.g. text, voice, image or video in combination with additional mobile requirements.

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	3G TS 22.101 (V3.6.0 onwards): “Service Principles”

[2]	3G TS 22.121 (V3.0.0 onwards): “The Virtual home Environment”

[3]	3G TS 21.133 (V3.0.0 onwards): "3G Security; Security Threats and Requirements"

[4]	3G TS 22.975 (V3.0.0 onwards): "Advanced Addressing"

# Definitions and abbreviations

## Definitions

- Recipient : the recipient is the entity to which  a MM has been sent

Sender : the sender is the  entity that sent a MM

User : the user is the MM sender or the MM recipient

message element : a message element is a part of a MM consisting of only one media type

multimedia message : a multimedia message is a message composed of one or more message elements.

multimedia message service : A  multimedia message service allows transfer of multimedia messages between users without the requirement for the multimedia messages to be transferred in real-time.

media types:

media formats:

network : for the purposes of supporting multimedia messaging, the term network shall be considered to include the mobile operator's network and any functionality which may exist outside the mobile operator's network (i.e.fixed, internet and multimedia technologies etc.), and the support provided by that functionality for multimedia messaging.

service capabilities features : see Reference [2]

## Abbreviations

For the purposes of this document the following abbreviations apply:

MM			Multimedia Message

MMS			Multimedia Message Service

SMS			Short Message Service

# High level Requirements

The following list gives the high level requirements of the MMS. These are requirements which are independent of the user’s perception of the service:-

- Forward compatible multimedia messaging

Multimedia messaging mechanisms shall provide the capability to support current and evolving multimedia messaging by re-useing existing standards as far as possible and proposing extensions (as necessary) to existing standards  (i.e. the multimedia messaging service shall support the evolution of multimedia messaging technologies).

- Consistent messaging

Regardless of the message type / format, MMS shall be capable of supporting integration of all types of messaging (e.g. fax, SMS, Multimedia , voicemail, e-mail etc.) in a consistent manner.

- Universal messaging access

Within the capabilities of networks and terminals, the user shall be able to experience consistent access to the MMS regardless of the access point.

For example the user should be capable of accessing her multimedia messages through a number of different access points, which should include 3G and 2G networks, fixed networks, the Internet etc..

- Interoperability

The MMS shall support a minimum set of functionality and message formats to ensure interoperability. (e.g. deletion of MM, identified standardised message notification, message media types and message content formats)

The MMS shall provide a minimum set of supported formats to ensure full interoperability between different terminals and networks from the very beginning of service provisioning (e.g. JPEG for pictures, MP3 for audio, MPEG for motion pictures, etc.).

- The MMS shall comply with the Virtual Home Environment specified in 22.121[2]

The non-real-time multimedia messaging service shall be supported using the standardised set of service capabilities features.

# General Requirements

Network operators have many differing requirements, and MMS shall be supported in the network in a manner which allows network operators to consider different configurations depending on their network and commercial requirements.  Thus, an identified set of functionalities and formats shall be standardised to ensure interoperability across networks and terminals to support MMS.

However, some network operators may wish to design and configure networks in different ways, and the subsequent requirements are identified to allow flexibility in how the MMS functionality is supported.  For example in some networks the network operators may wish to implement the MMS functionality within the core network, whereas other may wish to place the MMS functionality on the periphery of the core network (e.g. a centralised network model instead of a distributed architecture).  Further, some network operators may wish to support a limited set of MMS functionality, while others may require extensive and elaborate MMS support according to their business models (e.g. basic MMS instead of advanced MMS).  Interoperability shall always be maintained within this flexible architecture.

The following sub-clauses use the term "The MMS shall be able to support a request for …" and similar phrases to allow network operators to consider these different network models and business requirements, to permit flexible architectures and ensure MMS interoperability.

The following general requirements shall be supported via the use of service capability features.

## Multimedia message management

- Terminal-sensitive MM management

The MMS shall be able to support the capability for the terminal and network to take account of the capability of the user's terminal (e.g. deliver a MM / notification in a manner compatible with the terminals capability).

- User status-sensitive MM Management

The MMS shall be able to support the capability for the terminal and network to take account of the availability, changes of the state of availability of the user (e.g. store messages if the recipient is not available).

- MMS Control by the operator

The MMS shall be able to support a request from the operator to enable/disable MM delivery and submission.

- MMS Control by the user

The MMS shall be able to support a request from the user to enable/disable MM delivery and submission.

- Personalise multimedia messaging

The MMS shall be able to support a request by the user to manage the Service Preferences of his User Service Profile related to this MMS [2](e.g. customise his MM environment within the capabilities of the terminal, network and MM application. This could be unconditional or conditional e.g. depending on roaming conditions or operator restrictions).

- MM creation

The MMS shall be able to support the request to create a MM by the user or an application.

- MM recall

The MMS shall be able to support the request to delete a MM once submitted (e.g. recalling a message).

- Multiple Media

Multimedia messages may be composed of either a single medium (e.g. voice) or multi-media (e.g. Voice and video). The MMS shall be able to support a request for media synchronisation / sequencing.

- Media Type Conversion

The MMS shall be able to support a request to convert between  media types (e.g. Fax to image).

- Media Format Conversion

The MMS shall be able to support a request by the user or the application to  convert between MM media formats (e.g. JPEG to GIF).

- Message content retrieval by the network messaging application

The MMS shall be able to support a request by the messaging application to retrieve MM elements.

- Message forwarding

The MMS shall be able to support a request to forward multimedia messages or multimedia message elements without having to first download the MM to the terminal.

- Storage of Multi-Media Messages

The MMS shall be able to support a request for multimedia messages or message elements to be stored until delivered to the recipient’s terminal, until they expire, or until they are deleted by the user (unless configured differently). The MMS shall be able to support a request to store and manage all MMs in a network based repository rather than on the mobile terminal.

Note: There is no requirement for the MMS  to be responsible for the processing/presentation of the MM message, after it has been delivered to the terminal.

- Prioritisation of Messages

The MMS shall be able to support a request for MM prioritisation subject to the capabilities of the network (e.g. the sender of the MM may request to prioritise the importance of the multimedia messages).

- Screening of Messages

The MMS shall be able to support a request for MM screening subject to the capabilities of the network (e.g. automatically delete “junk mail” without delivery to the recipient’s terminal)

- Validity Period

The MMS shall be able to support a request to define validity periods for message delivery (e.g. if a message can not be delivered within a certain time it will be deleted).

## Multimedia message delivery

- Submission mechanism

The MMS shall support multimedia messages or messages elements to be submitted to the recipient’s terminal.

- Push Mechanism

The MMS shall be able to support a request for multimedia messages or messages elements to be automatically delivered to the recipient's terminal.

- Pull Mechanism

The MMS shall be able to support a request for multimedia messages or messages elements to be delivered to the recipient’s terminal on request by the recipient.

For Release 99, streaming will be considered only in the case of the pull mechanism

Editor’s Note: push and pull delivery mechanisms could be identical; the criteria which decide on the type of mechanism (push / pull) are either described in the User Services Profile or out of the scope of this specification.

- Concurrency

The MMS shall be able to support MM delivery to and from the user’s terminal not be restricted during other active services (subject to the capabilities of the terminal and the network).

## Notification and Acknowledgement

- The MMS shall be able to support a request to send generic notification and acknowledgement capability to inform the user in an appropriate manner of MMS events Examples may include:-

- notify the recipient about received  messages (including a description of the message, e.g. content, size, type).

- notify the recipient about actions taken by the MMS, (e.g. due to profile settings like automatic MM forwarding, deletion, etc.)

- acknowledge the sender about successful or failed MMor storage of a MM.

- acknowledge the sender about successful or failed MM submission

- acknowledge the sender about successful or failed MM delivery to the recipient terminal.

- acknowledge the sender about successful or failed MM deletion.

- acknowledge the sender, upon request, about the status of a submitted MM (i.e. delivered / not delivered).

## Addressing

The MMS shall support different addressing formats to identify the sender and recipient as specified in 22.975 [4] where applicable. It shall be possible to submit one message to multiple recipients.

# Profile

The MMS shall be able to support the ability to create, update, store, transfer, interrogate, manage and retrieve  a user’s multimedia messaging profiles.

The multimedia messaging profiles shall allow a user to configure and personalise his multimedia messaging environment with the multimedia messaging profiles (e.g. which media types and notifications that shall be delivered to the recipient , such as voice only or text only).

The multimedia messaging profiles shall form part of the user's virtual home environment.

# Security

The user shall be able to use and access MM in a secure manner. It shall be possible for the contents of MMs to be read only by the intended recipient(s).  A recipient shall be informed of the reliability of the identity of the sender in case the sender as authorised his identity to be transmitted.

The integrity of MMs during transit shall be assured to extent of the network capabilities.

The MMS shall be intrinsically resistant to attempts of malicious or fraudulent use.

The “Security Threats and Requirements” specified in 22.133 [3] shall not be compromised.

# Charging

- The MMS shall be able to support various charging mechanisms. The following charging characteristics may be considered:-

- message types, length, storage time in the network, etc

- delivering time, upload / download method,

- MM-sender / -recipient

- number of messages sent

- number of messages received.

- roaming conditions

- location conditions

# External Interface

External interfaces for controlling and delivering MM between the terminal and an external device e.g. portable computer should be supported.

# Interworking

The standard shall permit interworking with other or existing messaging technologies, messaging services, intelligent network services and  supplementary services, either located within or outside a mobile network.

# History

| Document history |  |  |
| --- | --- | --- |
| V. 0.0.1 | June 1999 | First Draft (Presented at TSG-T-WG2 SWG3 14th – 16th June 99) |
| V. 0.0.2 | June 1999 | Interim Draft for SWG3 discussion |
| V. 0.0.3 | July 1999 | Draft version to be submitted to TSG-SA-WG1, 5th – 9th July 99 |
| V. 0.0.4 | July 1999 | 2nd draft for submission to TSG-SA-WG1, 5th – 9th July 1999 |
| V. 0.1.0 | July 1999 | Version to be submitted to TSG-SA-WG1, 5th – 9th July 99 |
| V. 0.2.0 | September 1999 | Presented at TSG-T-WG2#5 6th – 9th September 99 |
| V. 0.2.2 | September 1999 | Version after post TSG-T-WG2#5 discussion / Presentation to S1 |
| V. 0.3.0 | September 1999 | Version after TSG-SA-WG1 MMS ad-hoc |
| V 0.3.1 | September 1999 | "3G" and "UMTS" removed to indicate availability to 2G systems Proposed version 1.0.0 Version sent to SA for information |
| V 1.1.0 | November 1999 | Version after TSG-SA-WG1 MMS AdHoc #2 |

Rapporteur for 3G TS 22.140 is: Gunnar Schmidt Bosch Telecom GmbH Tel. : +49 5341 285 853 Fax : +49 5341 285 140 Email : gunnar.schmidt@de.bosch.com

This document is written in Microsoft Word version 97