| 3GPP SA1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  | S1-0095 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France, 9-11 Feb 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Replaces 73/00 |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.078 |  |  |  |  |  | CR |  |  | 032 r1 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Nokia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | 7 Feb 2000 |  |  |  |  |  |
| Subject: |  |  | Call gapping / congestion control in HPLMN only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | CAMEL Phase 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  | Release: |  |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | N2A discussed the principle in Kyoto meeting, and it was then found acceptable to have gall gapping in the Home network only. This simplifies the implementation. It shall be noted also, that most of the load comes from the HPLMN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 4.2, 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

****    FIRST MODIFIED SECTION    ****

## 4.2	General Procedures

Each process is made up of a series of telecommunication events, some of which are service events. At a service event, the IPLMN or VPLMN may suspend the process and make contact with a CSE to ask for instructions or to send a notification. When a service event occurs, the IPLMN or VPLMN shall send to the CSE the information listed in this specification. All information sent to the CSE relates to the served CAMEL subscriber unless otherwise stated. The initial service events, and the corresponding CSE identity, which can initiate contact with the CSE are defined in the CAMEL Subscription Information.

The serving network shall accept the instruction from the CSE and continue call processing with the received information.

The CAMEL feature is applicable in a PLMN when the CAMEL subscription information is handled properly and when the communication to the CSE is compliant with the CAMEL protocol [8].

- The CAMEL network capabilities are used at a PLMN when the CAMEL feature is applicable and:

-	the CSI is received from the HPLMN; or

-	the CSE requests congestion control in the VPLMN or IPLMN; or - $(CAMEL3$);

-	the CSE creates a call - $(CAMEL3$).

In addition dialled network-based services may be applicable in a PLMN if so administered. -$(CAMEL3$)

The CSE shall be capable of responding to the CAMEL request with instructions on how to resume the suspended process. In the case of subscriber-based services the CSE shall be possible to instruct the IPLMN or VPLMN to:

-	Activate further service events for potential invocation. These events shall remain active only for the life-time of the telecommunication service;

-	Alter information relating to the suspended process;

-	Alter information relating to the parties involved in the process;

-	Indicate which of the possible parts of the process should occur next (e.g. terminate the call);

-	Perform Charging activities -$(CAMEL2$);

-	Order in band user interaction -$(CAMEL2$);

-	 Create additional parties in the call - $(CAMEL3$);

-	 Change the configuration of the connections between call parties - $(CAMEL3$).

$(begin$(CAMEL3$)

It shall be possible in the case of subscribed dialled services for the CSE to instruct the  serving PLMN to:

-	Continue the processing of the call, or;

-	Connect the calling party to a specified called party;

- After one of the above instructions, the relation between the serving network and the CSE shall be released. Any other behaviour may cause misoperation of CAMEL based services.

Serving network-based service numbers may be treated after the above described behaviour. These services are outside the scope of the CAMEL specification.

Serving network based service numbers may be provided on the descretion of the network operator but the are outside this specification.

$(end$(CAMEL3$)

CAMEL features shall form an integral part of the following processes:

- Mobile Originated call (MO call);

- Mobile Terminated call in GMSC (MT call);

- Mobile Terminated call in VMSC (MT call); $(CAMEL3$);

- Mobile Forwarded call (MF call) - early call forwarding; early forwarded calls are treated as MO calls;

- Mobile Forwarded call (MF call) - late call forwarding; late forwarded calls are treated as MO calls;

-	supplementary service invocation -$(CAMEL2$);

-	USSD user interaction. The of service codes for CAMEL services can be allocated on subscriber basis or globally for all subscribers of the HPLMN. -$(CAMEL2$);

- MO SM service; both via MSC and SGSN of GPRS $(CAMEL3$);

- GPRS $(CAMEL3$).

- Mobility Management events $(CAMEL3$)

The CSE shall be able to interrogate the HPLMN for information about a particular subscriber at any time.

It shall be possible for the CSE to originate calls on behalf of a CAMEL subscriber - $(CAMEL3$).

****   NEXT MODIFIED SECTION    ****

# 17	CSE related congestion control $(CAMEL3$)

It shall be possible for the CSE to drop or reject either all or some CAMEL interrogations from a V/IPLMN, when the V/IPLMN is the subscriber’s HPLMN. With a bilateral agreement the operators can also apply congestion control between different networks.

In the case where the contact from the V/IPLMN is not confirmed by the CSE, the V/IPLMN shall proceed in accordance to the Default Call Handling, Default SMS Handling or Default Session Handling.

In the case where the congestion control denies the contact to the CSE, the V/IPLMN shall proceed in accordance to the Default Call Handling.