TSG-SA Working Group 1 (Services) meeting #7	TSG S1 (00) 012

Sophia Antipolis, France 7 - 11 February 2000	Agenda Item: 6.3

3GPP TSG-N SS ad hoc #4	Tdoc 3GPP N_SS-99146

Henley-on-Thames, UK

30th November – 2nd December 1999

Work Item:	Call Forwarding Enhancements

From:

LS to TSG-S1:

Service Requirements for Long Forwarded-to Numbers

TSG-N SS ad hoc is currently working on the Call Forwarding Enhancements Work Item, to allow a subscriber to register a Forwarded-to Number with a maximum length of 28 digits.

TSG-N SS ad hoc have identified problems with the following scenarios:

- When a subscriber, with a long forwarded-to number registered, roams to an MSC/VLR not supporting long forwarded-to numbers, either:

- The MSC/VLR stores the Call Forwarding subscription information as “not provisioned”. This means CFB, CFNRy and late CFNRc can not be applied while the subscriber remains in the non-supporting MSC/VLR. If the subscriber interrogates Call Forwarding, he will receive incorrect information.

- The HLR includes a default forwarded-to number of less than 15 digits. This could be a number that plays a recorded announcement or invokes an IN service to complete the call or another operator specific solution.

- An incoming call subject to CFU or early CFNRc to a subscriber with a long forwarded-to number registered will, if the GMSC does not support long forwarded-to numbers, either

- the call forwarding shall fail and the call shall be dropped.

- a default forwarded-to number, received from the HLR, shall be used as in (1b)

- On invocation of Optimal Routeing to a long forwarded-to number, routed via a network entity not supporting long forwarded-to numbers, either

- the call forwarding shall fail and the call shall be dropped

- a default forwarded-to number, received from the HLR, shall be used as in (1b)

- A subscriber, with a long forwarded-to number registered, has an MS that does not support long forwarded-to numbers. If the subscriber interrogates call forwarding, either

- no information is displayed to the subscriber

- the first 15 digits of the long forwarded-to number are displayed

- the last 15 digits of the long forwarded-to number are displayed

TSG-N SS ad hoc ask TSG-S1 which of the options are preferred in cases 1-4 above.

TSG-N SS ad hoc also ask TSG-S1 if the Call Forwarding Enhancements work item should be extended to include Call Deflection? Currently, the deflected-to number must be less than 16 digits.