TSG-SA Working Group 1 (Services) meeting #7	TSG S1 (00)xxx

Sophia Antipolis, 9th – 11th February 2000	Agenda Item: 5.x.y

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.094 |  | Version: |  |  | 3.1.0 |  |  |  |
| Submitted to TSG_SA | 1 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Follow Me |
| --- | --- |

| Source: | Siemens | Date: | 9/2/2000 |
| --- | --- | --- | --- |

| Subject: | Renaming of the “Follow Me” Feature to “Follow Me for Remote Party” due to name clash with a DECT service |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| And one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification | X |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | Introduction of Service Code 212 for the “Follow Me for Remote Party” Feature. |
| --- | --- |

| Clauses affected: | all |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR

3G TS 22.094 V3.1.1 (2000-02)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Follow Me for Remote Party Service description;

Stage 1

(3G TS 22.094 version 3.1.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGSA-0122094U

Keywords

3GPP, SA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

All rights reserved.

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 1999, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

Contents

Foreword	5

1	Scope	6

2	References	6

2.1	Normative references	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Description	7

4.1	Normal Operation	8

5	Applicability to telecommunication services	8

6	Normal procedures with successful outcome	9

6.1	Provision	9

6.2	Withdrawal	9

6.3	Registration	10

6.4	Erasure	10

6.5	Activation	10

6.6	Deactivation	10

6.7	Invocation	10

6.8	Interrogation	11

7	Exceptional procedures or unsuccessful outcome	11

7.1	Registration	11

7.2	Erasure	11

7.3	Interrogation	12

7.4	Invocation	12

8	Alternate procedures	12

9	Charging requirements	12

10	Interactions with supplementary services	12

10.1	Call Forwarding unconditional	12

11	Interaction with Operator Determined barring	13

12	Interaction with Optimal Routeing	13

13	Interaction with CAMEL	13

14	Interworking considerations	13

Annex A (informative) : Change history	14

History	15

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# 1	Scope

The present document specifies the stage 1 description for the Follow Me for Remote Party feature.

The Follow Me for Remote Party feature enables a mobile subscriber A to manipulate the Follow Me for Remote Party data of a party B in such a way that - under certain conditions - subsequent calls directed to party B will be forwarded to subscriber A.

The feature is described from the service subscriber's and user's point of view, in particular:

-	the procedure for normal operation with successful outcome;

-	the action to be taken in exceptional circumstances;

-	the interaction with other GSM services and features.

This TS does not deal with the Man-Machine Interface (MMI) requirements, but makes reference to the appropriate  specifications.

Any interaction with other services and/or networks not dealt with in this specification are outside the scope of this TS.

# 2	References

References may be made to:

a)	specific versions of publications (identified by date of publication, edition number, version number, etc.), in which case, subsequent revisions to the referenced document do not apply; or

b)	all versions up to and including the identified version (identified by "up to and including" before the version identity); or

c)	all versions subsequent to and including the identified version (identified by "onwards" following the version identity); or

d)	publications without mention of a specific version, in which case the latest version applies.

A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

- For this Release 1999 document, references to GSM documents are for Release 1999 versions (version 8.x.y).

## 2.1	Normative references

This TS incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this TS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	GSM 01.04: "Digital cellular telecommunication system (Phase 2+); Definitions, abbreviations and acronyms".

[2]	TS 22.004: "General on supplementary services".

[4]	TS 22.030: "Man-Machine Interface (MMI) of the Mobile Station (MS)".

[5]	TS 22.082: "Call Forwarding (CF) supplementary services - Stage 1".

[6]	TS 22.085: "Closed User Group (CUG) supplementary services - Stage 1".

[7]	TS 22.041: "Closed User Group (CUG) supplementary services - Stage 1".

[8]	TS 22.078: " Customized Applications for Mobile network Enhanced Logic (CAMEL); Service definition (Stage 1).

[9]	TS 22.079: "Support of Optimal Routeing (SOR); Service definition (Stage 1)".

[10]	TS 22.090: " Unstructured Supplementary Service Data (USSD) Stage 1"

[11]	GSM 02.31: "Fraud Information Gathering System (FIGS); Service description - Stage 1"

[12]	TS 22.101: "UMTS Service Principles".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

initiating subscriber:	the mobile subscriber which modifies the Follow Me for Remote Party data of the remote party.

remote party:	Is characterised by a number in E.164 format defined in the numbering plan of a PLMN operator. The Follow Me for Remote Party feature enables the initiating subscriber to modify the Follow Me for Remote Party data associated with this number. In general this number is not assigned to a subscriber. In particular cases the remote party is a  subscriber of the PLMN and the number denotes her basic MSISDN.

FMRP service supervisor:	Is an initiating subscriber who is allowed to modify the Follow Me for Remote Party data of a remote party who has been registered to another initiating subscriber for the Follow Me for Remote Party application. The FMRP service supervisor shall be authorised by her network operator.

## 3.2	Abbreviations

For the purposes of this TS the following abbreviations apply, in addition, abbreviations used in this TS are listed in GSM 01.04 [1].

FMRP	Follow Me for Remote Party feature

# 4	Description

Follow Me for Remote Party enables an initiating mobile subscriber A to have control over the Follow Me for Remote Party data of a remote party B. Initiating Subscriber A shall be able to manipulate the Follow Me for Remote Party data of remote party B such that subsequent calls directed to remote party B are forwarded to initiating subscriber A.

Initiating subscriber A needs to have FMRP provisioned by her service provider. In case remote party B is a subscriber she needs to have FMRP provisioned by her service provider too.

The FMRP service supervisor shall be able to modify the Follow Me for Remote Party data of the remote party who has been registered to another initiating subscriber.

It remains in the responsibility of a FMRP provisioning service provider to confine

a.)	the range of remote parties for which a FMRP provisioned subscriber may become initiating subscriber

b.)	the range of potential initiating subscribers for a remote party.

The mobile subscriber who has been provisioned Follow Me for Remote Party is able to use the following control operations:

Registration	available to the initiating subscriber

Interrogation	available to the initiating and remote party in case the remote party corresponds to a subscriber

Erasure	available to the initiating and remote party in case the remote party corresponds to a subscriber

(There is no explicit activation or deactivation.)

The FMRP service supervisor is able to use the following control operations:

Interrogation

Erasure	(forced erasure)

NOTE 1:  The functionality of the Follow Me for Remote Party feature is equivalent to the Call Forwarding Unconditional Supplementary Service (see TS 22.082 [5]), controlled by initiating subscriber A and effecting remote party B.

NOTE 2:	The Follow Me for Remote Party feature is designed in such a way that the requirements of "functional addressing", which is needed for GSM-Railway operators, can be met. The concept of "functional addressing" enables a subscriber to set up a call to a "function" - e.g. the conductor - on a "train". 
The Follow Me for Remote Party feature identifies this "function" with the remote party B. Consequently a "functional number" is the equivalent to the number of  remote party B (e.g. the MSISDN of a subscriber B).  A call to that "functional number" (the conductor on a particular train - to stay with the example) will be forwarded to the MS of the person, who is on duty as a conductor at this time for this train.
In the context of the Follow Me for Remote Party feature, this person (the train conductor) constitutes the "initiating subscriber" A, who has registered Follow Me for Remote Party with respect to the remote party B (the "function" train conductor).

## 4.1	Normal Operation

Due to reduced security requirements to the Follow Me for Remote Party feature the service shall not be offered in standard operations of public GSM networks.  In railway operations it must be enabled by definitive action on the part of the operator.

Fraud Information Gathering System (FIGS) as specified in [11] can be used to protect  in CAMEL based inter-working networks.

The Follow Me for Remote Party feature shall be controlled by means of  Subscriber Controlled Input using USSD.

# 5	Applicability to telecommunication services

GSM 02.01 describes the principles of the Telecommunication services provided in a GSM PLMN. It also defines the concepts of Telecommunication services and describes their characterization by appropriate attributes. Bearer services and Teleservices, which are offered by a PLMN in connection with other networks, are defined in TS 22.002, GSM 02.03 and TS 22.101. A list of basic service groups can be found in GSM TS 22.004.

Follow Me for Remote Party is applicable to the following telecommunication services:

Table 1: Applicability of the Follow Me for Remote Party feature to telecommunication services

| Feature | Telephony | Emergency Call | SMS PTP |  | SMS-CB | Fax | cct Data | PAD/Packet Access |  | Voice Group Services |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TS11 | TS12 | TS21 | TS22 | TS 23 | TS 6x | BS2x, BS3x | BS4x | BS5x | TS 91 | TS 92 |
| Follow Me for Remote Party | Yes |  |  |  |  | Yes | Yes |  |  | No1 | No1 |
| NOTE 1: The feature is in general not applicable for Voice Group Services. However a dispatcher may be an initiating subscriber or a remote party for the Follow Me for Remote Party feature. EDITOR's Note: The applicability of FMRP to SMS is for further study. |  |  |  |  |  |  |  |  |  |  |  |

# 6	Normal procedures with successful outcome

## 6.1	Provision

Follow Me for Remote Party shall be provided after prior arrangement with the service provider. Follow Me for Remote Party will be provisioned for all Basic Services subscribed to and to which it is applicable.

NOTES:

- A Follow Me for Remote Party provision is required for the initiating subscriber and for the remote party in case the remote party corresponds to a subscriber.

- If Follow Me for Remote Party is provisioned for a mobile subscriber she may assume the role of an initiating subscriber and/or the role of a remote party.

- A Follow Me for Remote Party provisioning service provider may confine:

a.)	the range of remote parties for which a FMRP provisioned subscriber may become initiating subscriber

b.)	the range of potential initiating subscribers for a remote party.

c)	the range of remote parties for which a FMRP provisioned subscriber may become FMRP service supervisor.

NOTE:  (Health warning on security)  The Follow Me for Remote Party feature is designed in such a way as to meet the security requirements of GSM-Railway operators. If the Follow Me for Remote Party feature is to be used by other operators - in particular if the remote party corresponds to a subscriber - operators are strongly advised to care for additional suitable security mechanisms to protect the remote party from potential harm by misuse of FMRP. These mechanisms are out of scope of this specification.
To support such mechanisms the possibility is foreseen to convey additional operator-specific information in the registration, interrogation and erasure procedures to the PLMN of the remote party.  It should however be noted, that this is not a very secure mechanism. Operators should try to use existing security mechanisms where possible

## 6.2	Withdrawal

Follow Me for Remote Party will be withdrawn at the subscriber's request or for administrative reasons.

## 6.3	Registration

Follow Me for Remote Party shall be registered with an appropriate control procedure by the initiating subscriber.

The initiating Subscriber registers the Follow Me for Remote Party feature with respect to a particular remote party.

The initiating subscriber shall provide the following information to the network:

the number of the remote party (e.g. the Basic MSISDN of a subscriber);

The initiating subscriber shall receive an indication if the FMRP registration request was accepted or rejected by the network.

As an operator's option additional information (such as passwords) for registration may be required from the initiating subscriber. The registration procedure shall transport this information from the initiating subscriber to the network of the remote party. This information shall be coded as a USSD string with a length not exceeding 30 characters. The content of this information and the use of it are out of scope of this specification.

NOTE 1:	The Follow Me for Remote Party feature may, at an instant, be registered to an initiating subscriber with respect to several remote parties. Conversely, a remote party can be registered only to one initiating subscriber at a time.

## 6.4	Erasure

A previous FMRP registration can be erased in either of the following ways:

a)	The initiating subscriber can specifically erase her previous registration with an appropriate control procedure.

b)	In case the remote party corresponds to a subscriber the remote party

can erase any previous registration with an appropriate control procedure.

c)	The FMRP service supervisor can erase any previous registration to any remote party with an appropriate control procedure (forced erasure).

The subscriber who initiates the FMRP erase request shall be informed of the outcome of the request by the network.

As an operator's option additional information (such as passwords) for erasure may be required from the subscriber. The registration procedure shall transport this information from the subscriber to the network of the remote party. This information shall be coded as a USSD string with a length not exceeding 30 characters. The content of this information and the use of it are out of scope of this specification.

## 6.5	Activation

Follow Me for Remote Party is activated by registration.

## 6.6	Deactivation

Follow Me for Remote Party is deactivated by erasure.

## 6.7	Invocation

Follow Me for Remote Party is automatically invoked by the network if it was previously registered. If Follow Me for Remote Party is registered all incoming calls to the remote party will be forwarded to the initiating subscriber by network invocation.

When Follow Me for Remote Party is active and operative and if the remote party is a subscriber, the ability of the remote party  to originate calls is not affected.

## 6.8	Interrogation

An initiating subscriber (also the FMRP service supervisor) shall be able to interrogate the Follow Me for Remote Party data of any remote party, for which she is authorised to become initiating subscriber.

In case the remote party corresponds to a subscriber the remote party shall be able to interrogate her own Follow Me for Remote Party data stored in the network.

The network shall return the basic MSISDN of the initiating subscriber for which the remote party is registered to Follow Me for Remote Party.

As an operator's option additional information (such as passwords) for interrogation may be required from the subscriber. The registration procedure shall transport this information from the subscriber to the network of the remote party. This information shall be coded as a USSD string with a length not exceeding 30 characters. The content of this information and the use of it are out of scope of this specification.

# 7	Exceptional procedures or unsuccessful outcome

## 7.1	Registration

If the system cannot accept a registration request, the initiating subscriber shall receive a notification that Follow Me for Remote Party registration was not successful. Possible causes are:

-	insufficient information;

-	feature not provisioned to initiating subscriber;

-	feature not provisioned to remote party (in case the remote party is a subscriber);

-	conflicting situation with supplementary services (e.g. incoming call barring has been activated);

- remote party already registered to a different initiating subscriber

- insufficient authorisation for initiating subscriber or remote party

## 7.2	Erasure

If the network cannot accept a mobile initiating or remote subscriber's request  for erasure, cause will be returned to the subscriber, such as:

-	insufficient information;

- remote party not registered

- remote party registered to a different initiating subscriber

- insufficient authorisation for initiating subscriber or remote party

-	feature not provisioned to initiating subscriber;

-	feature not provisioned to remote party (in case the remote party is a subscriber);

## Interrogation

If the network cannot accept a mobile initiating or remote subscriber's request  for interrogation, cause will be returned to the subscriber, such as:

-	insufficient information;

- insufficient authorisation for initiating subscriber or remote party

- feature not provisioned to initiating subscriber;

-	feature not provisioned to remote party (in case the remote party is a subscriber);

## 7.4	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections (including call forwarding, call deflection, Follow Me for Remote Party) should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call has already been reached, an unsuccessful call set up indication is sent backwards.

If the call cannot be completed to the Follow Me for Remote Party destination, then the network will clear the forwarded part of the call and an unsuccessful call set up indication is sent backwards.

# 8	Alternate procedures

None identified.

# 9	Charging requirements

None identified.

# 10	Interactions with supplementary services

Interactions of the Follow Me for Remote Party feature with supplementary services may occur only for the remote party.

The interactions of the Follow Me for Remote Party feature with supplementary services other than Call Forwarding Unconditional (CFU) are the same as the interactions of CFU with supplementary services (see TS 22.082 [5]).

NOTE:	The “served mobile subscriber” in TS 22.082 [5] corresponds to the “remote party” in this document. The “forwarded-to subscriber” in TS 22.082 [5] corresponds to the “initiating subscriber” in this document

## 10.1	Call Forwarding unconditional

Follow Me for Remote Party and the Call Forwarding Unconditional Supplementary Service are mutually exclusive. That is:

If Follow Me for Remote Party is registered with respect to a remote party, registration or activation of Call forwarding unconditional by the remote party is rejected. The remote party shall be informed of this incompatibility.

If Call forwarding unconditional is registered or active for the remote party, registration of Follow Me for Remote Party is rejected. The initiating subscriber shall be informed of this incompatibility.

# 11	Interaction with Operator Determined barring

Same as interaction between Operator Determined barring and call forwarding unconditional as defined in TS 22.041.

# 12	Interaction with Optimal Routeing

Same as interaction between Optimal Routeing and call forwarding unconditional as defined in TS 22.079.

# 13	Interaction with CAMEL

Same as interaction between CAMEL and call forwarding unconditional as defined in TS 22.078.

# 14	Interworking considerations

If a forwarded call due to invocation of Follow Me for Remote Party meets an interworking situation, then an interworking indication should be sent to the calling party. When interworking with non-PLMN or non-ISDN networks, tones and announcements will be required.

NOTE1:	The number of times a call has been forwarded once it has exited the PLMN or ISDN may not be limited.

NOTE2:	Tones and/or announcements should not be provided to the calling party if the information transfer capability is set to UDI.

Annex A (informative) :
Change history

| Change history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SMG No. / TSG SA# | TDoc. No. | CR. No. | Section affected | New version | Subject/Comments |
| TSGT SA#6 | SP99-595 | 001 | 3.1, 4, 6.1, 6.4, 6.8 | Version 3.1.0 | Introduction of the rôle of a “ Follow Me for Remote Party service supervisor:” |

# History

| Document history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | Version | Comment |  |  |  |
| August 1999 | V2.0.1 | Transferred to TSG SA at ETSI SMG#29. |  |  |  |
| September 1999 | V2.1.0 | Ch 4.1, 6,1: Warning added due to reduced security requirements. (SMG10) |  |  |  |
| December 1999 | V3.0.0 | Approved at SA #5 in Nice, France |  |  |  |
| December 1999 | V3.1.0 | Inclusion of CRs at SA #5 in Nice, France |  |  |  |
| February 2000 | V 3.1.1 | Renaming of the “Follow Me” Feature to “Follow Me for Remote Party” due to name clash with a DECT service |  |  |  |