| 3GPP TSG SA1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  | TSG SG (00) 111 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France, 9-11 Feb 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 02.07 |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 7.1.0 |  |  |  |
| SMG specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submision to SMG |  |  |  |  | SMG# |  |  |  | for approval |  |  |  |  |  | X |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list SMG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  |  | ME |  |  |  |  |  |  |  | X |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Vodafone-AirTouch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 9-02-2000 |  |  |  |  |
| Subject: |  |  | Support of encryption in GPRS mobile stations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | Currently it is not explicitly stated that support for encryption and no-encryption modes is mandatory for GPRS terminals. Networks may be operating either with encryption on or off and therefore terminals must support both modes to ensure consistent access when roaming. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | 2, new B.1.xx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

*** First Modified Section ***

# 2	Requirements for implementing MS features

MS features are qualified as mandatory or optional. Mandatory features have to be implemented as long as they are relevant to the MS type, and will be subject to Type Approval when applied according to GSM 11.10 [13]. Whether or not an optional feature is implemented is left to the manufacturers' discretion. The method of implementation of all MS features must be done in accordance with the appropriate GSM specifications. For all present and future MS features, manufacturers have the responsibility to ensure that the MS features will neither conflict with the air interface nor cause any interference to the network or any other MS or its own MS, and these requirements shall be recognized during the Type Approval process.

In the following tables 1, 2 and 3 the basic, supplementary and additional MS features are listed. Mandatory features are marked by "M". Optional features are marked by "0".

Additional MS features not listed in table 3 are permitted without the requirement for this table to be amended, provided that these new features do not affect the mandatory air interface requirements.

Unless otherwise stated for a particular feature, the feature supported by the Subscriber Identity Module (SIM) takes priority over the same feature supported by the Mobile Equipment (ME).

Table 1: Basic MS features

|  | Name | Mandatory (M) Optional (O) |  |
| --- | --- | --- | --- |
| 1.1 | Display of Called Number | M* |  |
| 1.2 | Indication of Call Progress Signals | M* |  |
| 1.3 | Country/PLMN Indication | M* |  |
| 1.4 | Country/PLMN Selection | M |  |
| 1.5 | Keypad | O | (note 1) |
| 1.6 | IMEI | M |  |
| 1.7 | Short Message | M | (note 4) |
| 1.8 | Short Message Overflow Indication | M |  |
| 1.9 | DTE/DCE Interface | O |  |
| 1.10 | ISDN "S" Interface | O |  |
| 1.11 | International Access Function ("+" key) | O | (note 1) |
| 1.12 | Service Indicator | M* |  |
| 1.13 | Autocalling restriction capabilities |  | (note 2) |
| 1.14 | Emergency Calls capabilities | M | (note 3) |
| 1.15 | Dual Tone Multi Frequency function (DTMF) | M | (note 5) |
| 1.16 | Subscription Identity Management | M |  |
| 1.17 | On/Off switch | O |  |
| 1.18 | Subaddress | O |  |
| 1.19 | Support of Encryption A5/1 and A5/2 | M |  |
| 1.20 | Support of GPRS Encryption | M | (note 6) |
| 1.21 | Short Message Service Cell Broadcast | M |  |
| 1.22 | Short Message Service Cell Broadcast DRX | O |  |
| 1.23 | Service Provider Indication | O |  |
| 1.24 | Support of the extended SMS CB channel | O |  |
| 1.25 | Support of Additional Call Set-up MMI Procedures | O |  |
| 1.26 1.27 1.28 1.29 1.30 | Network Identity and Timezone Ciphering Indicator Network's indication of alerting in the MS Network initiated Mobile Originated connection Support of Localised Service Area | O M* O O O | (NI Alert in MS) |

Descriptions are given in annex B.

*	Mandatory where a human interface is provided, i.e. may be in-appropriate for MS driven by external equipment.

NOTE 1:	The physical means of entering the characters 0-9, +, * and # may be keypad, voice input device, DTE or others, but it is mandatory that there shall be the means to enter this information.

NOTE 2:	MTs with capabilities for Autocalling, or to which call initiating equipment can be connected via the "R" or "S" interface, shall restrict repeated call attempts according to the procedures described in annex A.

NOTE 3:	Emergency calls shall be possible according to Teleservice 12 (see GSM 02.03 [2] and GSM 02.30 [7]). This feature is only required to be provided by ME supporting Telephony.

NOTE 4:	Support of reception by the ME and storage of SMS MT in the SIM is mandatory, but its display is optional. Reception and storage of a message shall be indicated by the MS.

NOTE 5:	The use of DTMF is only mandatory when the speech teleservice is being used or during the speech phase of alternate speech/data and alternate speech/facsimile teleservices.

NOTE 6:	The implementation of GPRS encryption algorithm is mandatory for terminals supporting GPRS

Table 2: Supplementary MS features

|  | Name | Mandatory (M) Optional (O) |  |
| --- | --- | --- | --- |
| 2.1 | Control of Supplementary Services | (note 1) |  |

NOTE 1:	See annex B, subclause B.2.1.

Descriptions are given in annex B to GSM 02.07.

***Next Modified Section ***

## B.1.18	Sub-Address

This feature allows the mobile to append and/or receive a sub-address to a Directory Number, for use in call set-up, and in those supplementary services that use a Directory Number.

## B.1.19	Support of encryption A5/1 and A5/2

Provision is made for support of up to 7 different algorithms, and the support of no encryption. It is mandatory for A5/1, A5/2 and non encrypted mode to be implemented on mobile stations. Other algorithms are optional.

## B.1.20 Support of GPRS encryption

Provision is made for support of up to 7 different algorithms, and the support of no encryption. It is mandatory for a GPRS encryption algorithm and non encrypted mode to be implemented on mobile stations supporting GPRS.

## B.1.21	Short Message Service Cell Broadcast

The Short Message Service Cell Broadcast enables the mobile station to receive short messages from a message handling system.

The short message service cell broadcast teleservice is described in specification GSM 02.03 [2].