| 3GPP SA1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  | S1-00237 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France, 5-6 April 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Agenda Item 6.2 |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.078 |  |  |  |  |  | CR |  |  | xxx |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | x |  |
| Source: |  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | 04/04/2000 |  |  |  |  |  |
| Subject: |  |  | CAMEL interaction with Location Services for GPRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | CAMEL Phase 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
|  | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
|  | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
|  | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  | X |  |  |
| Reason for change: |  |  | The Location Services specification (3G TS 22.071) for Release ’00 specifies the support of Packet Switched terminals. It shall be possible for an external client to interrogate the GMLC for the accurate location of a GPRS terminal. The present CR proposes that the CSE be able to interrogate the GMLC for the location of a GPRS terminal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Other comments: |  |
| --- | --- |

****First Modified Section ****

# 22	Location Information

The purpose of this procedure is to obtain the location of a particular subscriber. The resolution of the location information may be based on the Cell Identity of the subscriber’s location, or may be based on more accurate positioning information ($CAMEL3$).

The CSE may interrogate the HLR in order to obtain a particular subscriber’s location based on the cell identity. The HLR may return the cell identity stored in the VLR ($CAMEL1$) or may return the current cell identity as a result of paging the subscriber ($CAMEL3$).

($begin(CAMEL3$).

The CSE may interrogate the GMLC in order to obtain a particular subscriber’s current location based on accurate geographical information as defined by LCS in TS 22.071 [8]. The GMLC has the possibility to reject any interrogation from any CSE.

($end(CAMEL3$).

($begin(CAMEL4$).

The CSE may interrogate the GMLC to obtain the position of a packet switched terminal (GPRS Terminal). The GMLC has the possibility to reject any interrogation from any CSE.

($end(CAMEL4$).