TSG-SA Working Group 1 (Services) meeting #8	TSG S1 (00) 257

Beijing, China 10 - 14 March 2000	Agenda Item: 6.11

Source: 	NTT DoCoMo

Title:	Emergency Calls for R00

Document for:	Discussion and Decision

- Introduction

Since S1 discuss about how to deal an Emergency call for R99, and decided to support GSM approach for at least R99.  As discussing enhancement of emergency call for R00 on the E-mail, S1 seems to support call type of emergency center categorizations such as police, fire, and ambulance).  NTT DoCoMo would like to show how to deal with emergency call for R00.

- Discussion

When supports call types of emergency center categorization, we need to consider below points.

① When correspondence of emergency number and center is not 1 to 1, it may be unable to communicate with the emergency center that a user overlooks at the time of roaming.  (For instance, Japanese emergency number 119 is for both fire and ambulance.  When this user does roaming to France where fire and ambulance are different emergency centers, it does not know where a network should be connected in France

These three points show what we discussed on the S1 mailing list.

1) When a connection center is not clear, it connects with a default emergency center in the country if they have the default emergency center.

- In the country where redirect forward is possible, it connects with one of emergency centers.  It depends on an operator which emergency center it connects first.

Note. Emergency call that originates by default number does not include information of call type.

- In the country where redirect forward is not possible, it depends on an operator which emergency centers connect.

Since the number used for emergency call in each country differs when premised on mapping of a number and a category, the connection place at the time of roaming does NOT become clear.   Therefore, NTT DoCoMo suggests following solutions.

・A number and a category are made to correspond by 1 to 1.

・The interface of a number is not used at the time of roaming.

If a number and a category would be made to correspond by 1 to 1, specification of the emergency     center at the time of roaming also was clarified.  However this is not realistic in order to be dependent on the situation of each country.

The menu is used instead of not using the number interface at the time of roaming, it work well.  But

it is not desirable to make it indispensable to specify MMI other than this number from the viewpoint of ME.  Therefore, the interface of the number used is recommended at a home network and with the time of roaming, and the interface which can carry out direct specification of the category by simple operation of a menu etc, at the time of roaming is recommended.

Note: Also emergency call by operation of a menu etc. is also possible at the home network.

② When a UE does not insert SIM/USIM, the category assigned to the default number of a UE cannot be used widely. (For instance, 110 is carried out police in Japan, but fire in Norway).

Regarding to the Emergency call without SIM/USIM, because of service requirement, it is also should be possible to indicate categories.  NTT DoCoMo recommend using interfaces to indicate category such as menu.

3.Conclusion

NTT DoCoMo proposes about operation to consider at the time of the roaming and UE without SIM/USIM in the case of using a category for call type of an emergency center as mentioned and prepares CR for TS22.101 about Emergency call.