| 3GPP SA Working Group 1 Meeting #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  | S1-000274 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beijing, 10 April - 14 April 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.002 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | TSG#8 |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Nokia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | 10/4/00 |  |  |  |  |  |
| Subject: |  |  | Lower User Rates in UMTS for Circuit Switched Data Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  | Release: |  |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Corrections and clarifications based on N3 proposal. This CR alligns stage 1 with rel 99 system functionality. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 3.1.X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

## 3.1	General bearer service user data characteristics

The tables below describe the characteristics of the General Bearer Services. The indicated fixed network user rates are possible, but support of General Bearer Service does not imply support of all rates.

### 3.1.1	3,1 kHz Audio

| Fixed Network User Rate | Access Structure | Information Transfer Capability | QoS attributes | Note |
| --- | --- | --- | --- | --- |
| 0,3 kbit/s | Asynch | 3,1 kHz | NT or T | note 2 and 4 |
| 1,2 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | Notes 1,2 and 4 |
| 2,4 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | Note 2 and 4 |
| 4,8 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | note 2 and 4 |
| 9,6 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | Note 5 |
| 14,4 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | Note 5 |
| 19,2 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | Note 5 |
| 28,8 kbit/s | Asynch, Synch | 3,1 kHz | NT or T |  |
|  | Asynch | 3,1 kHz | NT | note 3 |
| 'NOTE 1: Not applicable to synchronous NT service. NOTE 2: These services are also supported by the GSM Phase 2 Specifications. NOTE 3: This is used with high speed modems such as V.90 (56kbit/s). Modem type = 'Autobauding Type 1' is selected. FNUR has no meaning in this case. NOTE 4: In case of UTRAN the FNURs 300, 1200, 2400 and 4800 bit/s towards the fixed network can be provided only in Async non-transparent (NT) mode with modem type = 'Autobauding Type 1' is selected. NOTE 5: In case of UTRAN the FNURs 9.6, 14.4 and 19.2 kbit/s towards the fixed network shall be provided only in non-transparent (NT) mode. |  |  |  |  |

### 3.1.2	V.110 UDI

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 0,3 kbit/s | Asynch | V.110 | NT or T | Note 2 and 3 |
| 1,2 kbit/s | Asynch, Synch | V.110 | NT or T | Note 1, 2 and 3 |
| 2,4 kbit/s | Asynch, Synch | V.110 | NT or T | Note 2 and 3 |
| 4,8 kbit/s | Asynch, Synch | V.110 | NT or T | Note 2 and 3 |
| 9,6 kbit/s | Asynch, Synch | V.110 | NT or T | Note 2 and 4 |
| 14,4 kbit/s | Asynch, Synch | V.110 | NT or T | Note 4 |
| 19,2 kbit/s | Asynch, Synch | V.110 | NT or T | Note 4 |
| 28,8 kbit/s | Asynch, Synch | V.110 | NT or T |  |
| 38,4 kbit/s | Asynch, Synch | V.110 | NT or T | Note 4 |
| 48 kbit/s | Synch | V.110 | T |  |
| 56 kbit/s | Synch | V.110 | T (in a 64 kbit/s environment) |  |
| NOTE 1: Not applicable to synchronous NT service. NOTE 2: These services are also supported by the GSM Phase 2 Specifications. NOTE 3: In case of UTRAN the user rates 300, 1200, 2400 and 4800 bit/s towards the fixed network can be provided only for mobile terminated calls and only in Async non-transparent (NT) mode. NOTE 4: In case of UTRAN the FNURs 9.6, 14.4, 19.2 and 38.4 kbit/s towards the fixed network shall be provided only in non-transparent (NT) mode. |  |  |  |  |

### 3.1.3	X.31 Flag Stuffing UDI

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 2.4 kbit/s | Synch | X.31 Flag Stuffing | NT | Note 1 and 2 |
| 4.8 kbit/s | Synch | X.31 Flag Stuffing | NT | Note 1 and 2 |
| 9.6 kbit/s | Synch | X.31 Flag Stuffing | NT | Note 1 |
| 14.4 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 19.2 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 28.8 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 38.4 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 48 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 56 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| NOTE 1: These services are also supported by the GSM Phase 2 Specifications. NOTE 2: In case of UTRAN the user rates 2400 and 4800 bit/s are not supported. |  |  |  |  |

### 3.1.4	V.120

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 1,2 kbit/s | Asynch | V.120 | NT | Note 3 |
| 2,4 kbit/s | Asynch, Synch | V.120 | NT | Note 3 |
| 4,8 kbit/s | Asynch, Synch | V.120 | NT | Note 3 |
| 9,6 kbit/s | Asynch, Synch | V.120 | NT |  |
| 14,4 kbit/s | Asynch, Synch | V.120 | NT |  |
| 19,2 kbit/s | Asynch, Synch | V.120 | NT |  |
| 28,8 kbit/s | Asynch, Synch | V.120 | NT | note 1 |
| 38,4 kbit/s | Asynch, Synch | V.120 | NT |  |
| 48 kbit/s | Asynch, Synch | V.120 | NT |  |
| 56 kbit/s | Asynch, Synch | V.120 | NT | note 2 |
| NOTE 1: Requires a new code point in V.120 specification to be defined. NOTE 2: Not applicable in a 56 kbit/s environment. NOTE 3: In case of UTRAN the user rates 1200, 2400 and 4800 bit/s toward the fixed network can be provided only for asynchronous non-transparent (NT) mobile terminated calls. |  |  |  |  |

### 3.1.5	Bit Transparent Mode

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 56 kbit/s | Synch | Bit transparent | T (RDI) (in a 56 kbit/s environment) |  |
| 64 kbit/s | Synch | Bit transparent | T (UDI) (in a 64 kbit/s environment) |  |

### 3.1.6	PIAFS

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 32 kbit/s | Asynch | PIAFS | NT |  |
| 64 kbit/s | Asynch | PIAFS | NT |  |

### 3.1.7	Frame Tunnelling Mode

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 56kbit/s | Asynch | X.31 flag stuffing | NT |  |
| 64 kbit/s | Asynch | X.31 flag stuffing | NT | note |
| NOTE: Not applicable in a 56kbit/s environment. |  |  |  |  |

### 3.1.8	Multimedia Call

| Fixed Network User Rate | Access Structure | Information Transfer Capability | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- | --- |
| 28,8 kbit/s | Synch | 3.1kHz Audio | H.223 & H.245 | T |  |
| 32,0 kbit/s | Synch | UDI | H.223 & H.245 | T | Note |
| 33,6 kbit/s | Synch | 3.1kHz Audio | H.223 & H.245 | T | Note |
| 56 kbit/s | Synch | RDI | H.223 & H.245 | T |  |
| 64 kbit/s | Synch | UDI | H.223 & H.245 | T |  |
| Note: 32kbit/s and 33.6kbit/s FNURs are applicable only for UTRAN. |  |  |  |  |  |