3GPP TSG SA WG 1 Meeting #102 	S1-231703

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231570)

Source:	Novamint, Huawei, ETRI, Nokia, Gatehouse, China Telecom, CATT

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.865 – for next meeting

Draft Spec:	3GPP TR 22.865 1.0.0

Agenda item:	7.7

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in S1-231086.

1. Introduction

The “Consolidated Requirements” of TR 22.865 are currently not included in version 1.0.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in  S1-231382.

The document  was updated into S1-231383 to merge the documents S1-231072 & S1-231224.

Then, S1-231383 was revised and split into 3 documents:

- S1-231568 – potential CPR agreaable at this meeting

- S1-231569 - potential CPR for further discussion at this meeting

- S1-231570/1703 - potential CPR for next meeting (present document)

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865 v1.0.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

EN: The following CPR have been consolidated at this meeting and they need further discussion.

## 6.2	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below

Table 6.2-1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # |  | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- | --- |
| CPR 6.2-7 | A 5G system with satellite access shall be able to inform an authorised 3rd party whether Store & Forward Satellite operation is applied for communication with a UE. |  | [PR 5.2.6-001] | To be aligned (merge) with the new PR |

## 6.3	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication (without going through the ground network) are listed in the table below

Table 6.3-1 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-6 | Subject to the operator’s policy, a 5G system shall be able to support two UEs using different satellite access paths to communicate with each other without going through the ground network. (e.g. one UE using a direct satellite connection path, and one UE using a connection via a terrestrial relay with satellite access) | [PR 5.10.6-002] | Related PR still under discussion |

## 6.4	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below

Table 6.4-1 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.5	General and other aspects for satellite access

The potential requirements corresponding to the support of enhancements of general and other aspects of satellite access are listed in the table below

Table 6.5-1 – Consolidated Requirements for general and other aspects of satellite access

## 6.6	Security aspects

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for security aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.6-3 | A 5G system with satellite access shall be able to support mechanisms to authorize the communication between UEs using satellite access without going through the ground network, based on e.g., location information and subscription. NOTE: UEs can use satellite access directly or via a terrestrial relay. | [PR 5.6.6-001] [PR 5.10.6-001] [PR 5.1.6-006] [PR 5.2.6-005] | X |

## 6.7	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.7-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7-2 | A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, without the user traffic going through the ground network. | [PR 5.6.6-002] [PR 5.7.6-002] | X |

EN: The following CPR are based on PR agreed at this meeting and they will be considered for the next meeting.

| CPR 6.2-x | Potential New requirement from S1-231563 [PR.5.1.6-007] Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite, which has an available feeder link to the ground network, through Inter-Satellite Links. NOTE: It is assumed that the constellation knows which satellite has a feeder link available. However, this is outside the scope of 3GPP. | Potential new PR [PR.5.1.6-007] | Potential new PR from S1-231563 Is a complement to CPR 6.2-1 Or addressed by CPR 6.6-1? |
| --- | --- | --- | --- |
| CPR 6.2-x | Potential New requirement from S1-231577 [PR.5.1.6-008] The 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support suitable means to resume communication once the feeder link becomes available. | Potential new PR [PR.5.1.6-008] [PR 5.2.6-006] | Potential new PR from S1-231577 & from S1-231578 |
| CPR 6.2-x | Potential New requirement from S1-231579 [PR 5.2.6-005] The 5G system with satellite access shall be able to provide to a trusted third party application the information about the store and forward operation applied to a UE (e.g. estimated delivery time to the UE). | Potential new PR [PR 5.2.6-005] | Potential new PR from S1-231579 See CPR 6.2-7 |

| CPR 6.3-x | Potential New requirement from S1-231700 [PR 5.6.6-005] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support UE-Satellite-UE communication when the feeder link is temporarily unavailable. | Potential new PR [PR 5.6.6-005] | Potential new PR from S1-231700 Partially already covered covered by CPR 6.2-1? |
| --- | --- | --- | --- |
| CPR 6.3-x | Potential New requirement from S1-231121 [PR 5.14.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support the establishment of a communication path between UEs via one or multiple serving satellites without going through the ground network. | Potential new PR [PR 5.14.6-001] | Potential new PR from S1-231121 May be merged in CPR 6.3-1 |

| CPR 6.4-x | Potential new PR from S1-231573 [PR 5.13.6-003] Subject to regulatory requirements, the 5G system with satellite access shall be able to provide positioning services (e.g. with the availability of 99%, the accuracy of several kilometres) independently of UE’s GNSS capability when the UE is using only satellite access. NOTE: The regulatory requirements for positioning (e.g. service requirements of Public Safety by GSMA[X]) could be taken into account. | Potential new PR [PR 5.13.6-003] | Potential new PR from S1-231573 |
| --- | --- | --- | --- |

EN: The following CPR are based on revised requirements which were removed with the understanding that this topic can be re discussed in next meeting:

| CPR 6.2-x | Potential revised requirement from S1-231235 which was removed in 1574 with the understanding that this topic can be re discussed in August. Subject to regulatory requirement, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to know the data sent from a UE is for emergency purpose, shall forward the emergency report from an UE as soon as possible, and shall immediately notify the UE about the successful forwarding. | Potential revised PR [PR.5.16.6-00x] | To review in August |
| --- | --- | --- | --- |