3GPP TSG SA WG 1 Meeting #104		 S1-233015

Chicago, USA,  13 - 17 November 2023

Title:	LS on the requirements for slice-based PLMN selection

Response to:	none

Release:	Rel-18

Work Item:	PLMNsel NS

Source:	SA1

To:	CT1, SA, CT

Cc:	SA2

Contact person:

Name:		Juergen Merkel

E-mail Address:		juergen.merkel (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# Overall description

SA1 would like to thank CT1 for their LS and responds as follows to the questions raised by CT1:

CT1 would like to ask the following questions:

Issue 1

Question 1: Would SA1 be able to provide any additional guidance regarding Scenario 1? In particular:

- Whether the "prioritization information" defines priorities between the PLMNs or between the slices?

SA1 Answer: The prioritization information in the stage 1 requirements refers to prioritization information of the VPLMNs for a network slice. In case there are multiple network slices, there should be prioritization information of the VPLMNs per network slice. In other words, the UE may receive several slice-specific PLMN lists.

- If the answer to a) is “between the slices”, should the prioritization of slices be determined by the network or by the UE?

SA1 Answer: In case there are multiple network slices, the operator should be able to assign relative importance between the PLMN lists provided to the UE.

Question 2: Should a slice determined according to Scenario 2 be considered as being offered by the serving network or not?

SA1 Answer: No, a slice determined according to Scenario 2 may or may not be offered by the serving network.

Issue 2

Regarding the prioritization information of the VPLMNs with which the UE may register for the network slice, CT1 has discussed the interaction between the legacy prioritization of VPLMNs (“Operator controlled PLMN selector”) and the new prioritization information of the VPLMNs with which the UE may register for the network slice.

Question 3: Would SA1 be able to provide any additional guidance regarding the interaction between legacy PLMN selection prioritization and new prioritization information for slice based PLMN selection? In particular:

- which list has a higher priority while performing PLMN selection?

SA1 Answer: The operator should be able to indicate relative importance between the “Operator controlled PLMN selector” and slice-based PLMN selection information.

- does the trigger to perform SOR apply when the UE uses slice-based PLMN selection?

SA1 Answer: Yes.

# 2	Actions

To CT1

ACTION: 	SA1 kindly asks CT1 to take the answers above into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#105	26Feb -01 Mar 2024	Athens, Greece

SA1#106	27-31 May 2024 	Korea, Jeyju