3GPP TSG-SA WG1 Meeting #104	S1-233370

# Chicago, USA, 13 –17 Novembre 2023				(revision of S1-233361, revision of S1-233205)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.179 | CR | 0078 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of multi-talker control due to updates of FRMCS functional requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Alignement with UIC FRMCS – FRS FU-7120 requirements, for multi-talker control |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modification of talker priority by authorized MCPTT user - Modification of the maximum number of simultaneous users and impact on the talker permissions of the participants of an MCPTT group configured for multi-talker control |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirement identified in gap analysis between UIC FRMCS – FRS FU-7120 and 3GPP specifications. Downstream groups will not be able to complete the implementation of multi-talker control mainly for Ad hoc Group Emergency Communications due to missing stage 1 requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.3.7.2	General aspects

[R-6.2.3.7.2-001] An MCPTT Group shall be configurable to allow multi-talker control.

[R-6.2.3.7.2-002] The MCPTT Service shall provide a mechanism for multiple MCPTT Users to talk simultaneously in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-003] The MCPTT Service shall determine which Participant(s) are allowed to transmit to all other Participant(s) in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-004] The MCPTT Service shall support all Participant(s) to receive audio from all other Participant(s) that are transmitting in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-005] The MCPTT Service shall provide a mechanism for the MCPTT Administrator to configure the maximum number of simultaneous talkers in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-006] The MCPTT Service shall allow an authorized MCPTT User to change the maximum number of simultaneous talkers at any time during a group call in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-007] The MCPTT Service shall allow an authorized MCPTT User to change the priority hierarchy of simultaneous talkers at any time during a group call in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-008] In case the maximum number of simultaneous talkers is reduced by an authorized MCPTT User, the permission to talk will be revoked, until the new maximum number of simultaneous talkers is met.

NOTE:	The selection of the participants for revoking the permission to talk is left for implementation.

* * * End of Changes * * * *